The Universal VESA VBE

----------------------

Copyright (C) 1993 Kendall Bennett

All rights reserved.

What is it?

-----------

The Universal VESA VBE is a small Terminate and Stay Resident (TSR) program

that extends the video BIOS of SuperVGA video cards to make them compatible

with the Video Electonics Standards Association (VESA) recommended

Video BIOS Extensions (VBE).

By having a video card that has a VESA VBE compatible BIOS, or by

installing this small TSR, programs that support the VESA VBE programming

interface will work correctly for your SuperVGA. This helps to solve a

lot of problems with compatability between different SuperVGA cards on the

market. Note that this program will not take advantage of any of the

acceleration features that your video card may provide.

Why use it?

-----------

You may already have a VESA VBE TSR for you video card, or you video card

may have a VESA VBE compatible BIOS already, so why would you want to use

this program?

The simple answer is that most of the TSR's that exist today, and a lot

of the not-so-new SuperVGA cards that are around implement an older version

of the VESA VBE programming interface (if any at all). For this reason, some

programs that use the advanced features of the new VESA VBE standard will

not work with the TSR or BIOS that you currently have.

So what advanced features does this TSR provide that other's don't? The

Universal VESA VBE implements the VESA VBE 1.2 programming interface,

which supports the following features:

. SuperVGA page flipping. High performance animation programs can

use your card to full potential using the VESA BIOS to implement

extended page flipping (imagine, 1024x768x16 double buffered

animation, or even 800x600x256 and 640x400x32k/64k on a 1Mb video

card!!).

. SuperVGA virtual screens. Programs may set up a virtual display

resolution of say 1024x1024 pixels, and smoothly scroll a window

with less physical resolution around within this buffer.

. Support for the 32k, 64k and 16 million color video modes. As well

as supporting the industry standard 16 and 256 color video

modes.

. Speed. The bank switching code in this package tends to run faster

than the routines embedded in the VIDEO BIOS of some video cards.

. Programs that know about the Universal VESA VBE protected mode

extensions can use the high performance protected mode programming

interface provided by this TSR.

. Extremely small size. When the TSR is resident in your computer it

only requires about 3k of memory which is smaller than the

size of most commerical VESA VBE TSR's. It may also be loaded high

to save even more memory.

Hardware Requirements

---------------------

The Universal VESA VBE will run on any 80x86 based IBM PC compatible

running MSDOS. Currently the Universal VESA VBE supports the following

SuperVGA cards (note that not all of these have been fully tested):

- ATI Technologies 18800, 28800

- Ahead A & B

- Chips & Technologies 82c451/452/453

- Everex

- Genoa Systems GVGA

- OAK Technologies OTI-037C, OTI-067, OTI-077, OTI-087

- Paradise PVGA1A, WD90C00/10/11/20/21/30/31

- NCR 77C20/21/22E

- Trident 88/8900

- Video7 V7VGA versions 1-5

- Tseng Labs ET3000, ET4000, ET4000/W32

- AcuMos AVGA2

- S3 86c911/924/801/805/928

- Advance Logic AL2101 SuperVGA

- MXIC 86010 SuperVGA

- Primus 2000 SuperVGA

- RealTek 3106 SuperVGA

- Cirrus Logic CL-GD 5420, 5422, 5424, 5426, 5428

How do I install it?

--------------------

You can install the TSR from the command line, or from your autoexec.bat

file. Don't worry about installing it twice, it will automatically detect

if you have already installed it.

To install it simply type:

univbe

from the command line. It will determine what type of SuperVGA card you

have installed and install itself accordingly. If you wish to remove the

TSR from memory after it has been installed, simply type:

univbe -u

This will remove the program from memory, even if it was not the last

TSR to be installed. However if you installed another TSR after this

one that has hooked into the Int 10h interface the old TSR will cease

to function.

You may also use command line options to modify the the installation

process used by the program. The following is produced by typing

'univesa -h' at the command line:

Options are:

-s<name> - Force detection of SuperVGA 'name'

-c<x>    - Force detection of SuperVGA chipset (numerical id)

-m<size> - Force memory size to 'size'

-d<x>    - Force DAC type (0:VGA, 1:15 bit, 2:16 bit, 3:24 bit)

-i       - Do not perform SuperVGA detection

-q       - Quiet mode

-g       - Regenerate default .ini file

-u       - Unload the Universal VESA VBE from memory

-t       - Don't go TSR, just return the detected SuperVGA id

It is possible the SuperVGA detection code will hang the machine on old

VGA/SVGA cards. You can optionally force the program to work with any

combination of card, chipset, memory size and DAC, but unless you specify

the -i option, unspecified values will be filled in automatically for you.

Thus you if the detection code did not correctly identify you video card

configuration, you can force the TSR to work with the correct values.

For some SuperVGA cards, I have not been able to work out a solid method

of detecting the amount of video memory on board. If this is the case,

the Universal VESA VBE will refuse to install itself and ask that you

specify the amount of video memory on your card from the command line.

The Universal VESA VBE does not provide support for any of the extended

text modes that your video card may provide, but will provide support

for any extended text modes supported by an underlying VESA VBE compliant

BIOS. Thus if you video card already has a VBE BIOS or you install

your favorite VESA VBE TSR for your video card _before_ installing the

Universal VESA VBE, the Universal VESA VBE will use a pass-through

mechanism to ensure the extended text modes still work correctly.

What's the .ini file for?

-------------------------

For most installations, the Universal VESA VBE will automatically

determine the video modes that you card supports. However on some older

video cards some video modes that the Universal VESA VBE thinks are

available aren't and so an error will occur if a program tries to set

the video mode. The .ini file is used to remove support for those video

modes. You can edit the .ini file yourself or you can get the TSR

to automatically generate the default .ini file for you.

In order to generate the default .ini file for you, you must remove the

TSR from memory, delete the old .ini file, re-install the TSR in memory

and run the program with the -g option. The default .ini file is now

generated. You should then remove the TSR again and re-install it using

the values in the new .ini file. A batch file called 'makeini.bat'

is provided which will do this for you.

The format of the .ini file is simple. All the video modes removed by

the program are listed under the section '[remove]' in the .ini file.

They are listed as the hexadecimal internal mode numbers used by the

TSR (see the next section for a list of the mode numbers used). An

example .ini file might be:

[remove]

10D

Video modes supported by this TSR

---------------------------------

The Universal VESA VBE supports the VESA VBE 1.2 programming interface,

which defines a number of standard video modes numbers. The new version

of the VESA VBE specs will allow the vendor to extend the video numbers

used to whatever values they wish, so I have taken the liberty to

extend the mode numbers used by the Universal VESA VBE to include

modes not originally specified in the VBE 1.2 specs, but supported by

a number of popular video cards on the market. Depending on how well

your applications have been written, they may or may not be able to

use those modes.

The following is a list of the internal mode numbers used by the Universal

VESA VBE and a description of each video mode:

102   - 800x600 16 color

104   - 1024x768 16 color

106   - 1280x1024 16 color

11C   - 640x350 256 color

100   - 640x400 256 color

101   - 640x480 256 color

103   - 800x600 256 color

105   - 1024x768 256 color

107   - 1280x1024 256 color

10D   - 320x200 32k color

11D   - 640x350 32k color

11E   - 640x400 32k color

110   - 640x480 32k color

113   - 800x600 32k color

116   - 1024x768 32k color

119   - 1280x1024 32k color

10E   - 320x200 64k color

11F   - 640x350 64k color

120   - 640x400 64k color

111   - 640x480 64k color

114   - 800x600 64k color

117   - 1024x768 64k color

11A   - 1280x1024 64k color

10F   - 320x200 16m color

121   - 640x350 16m color

122   - 640x400 16m color

112   - 640x480 16m color

115   - 800x600 16m color

118   - 1024x768 16m color

11B   - 1280x1024 16m color

Protected mode interface

------------------------

In order to be able to write high performance programs that utilise the

VESA VBE programming interface from 16 bit and 32 bit protected mode,

I have provided a special interface to allow application programs to

copy the bank switching and page flipping code into their own code space,

so it can be executed directly as a protected mode routine.

Please refer to the code in the SuperVGA test kit to see how to use

this interface.

Problems and known bugs

-----------------------

The Universal VESA TSR seems to have problems with QEMM, since QEMM does

something weird with the Video BIOS. In order to fix this problem,

you should use the QEMM 'exclude' options to exclude the Video BIOS

area. Do this by including the following in the QEMM options:

EXCLUDE=C000-C1FF

Can I get the source code?

--------------------------

Yes, the full source code for this product is available, along with a

number of test programs (with source) for programming SuperVGA cards

using the VESA VBE interface, called the SuperVGA Test Kit Library. You

can probably download this from your local BBS, or by using ftp on the

internet from Simtel20 or garbo (or one of it's mirrors). You can

also ftp the latest version from:

godzilla.cgl.rmit.oz.au (131.170.14.2): kjb/MGL/svgakt??.zip

How can I contact the Author?

-----------------------------

If you have any corrections or updates to the code in this package, or

you have any questions you would like asked, feel free to contact me

through one of the following methods.

Internet:   kjb@citri.edu.au

Snail Mail: Kendall Bennett

15 Stevenson Street

Kew Victoria 3101

AUSTRALIA