ARJ TECHNICAL INFORMATION                               November 1994

** IMPORTANT NEWS ****************************************************

Some archiver support programs have designed ARJ archive

identification schemes that are not reliable.  At ARJ 2.39, there are

now two versions of the large self-extraction module.  The 17K byte

ARJSFX module supports ARJ-SECURITY.  The standard 15K byte ARJSFX

module does NOT support ARJ-SECURITY.  For the first time, the

ARJ-SECURITY fields in the main ARJ header are publicly defined.

In addition, the main ARJ header in self-extracting archives MAY NOT

immediately follow the EXE module now.

ARJ has used the same ARJ archive identification scheme since ARJ

1.0.  The following is the algorithm:

(1) find the ARJ header id bytes 0x60, 0xEA,

(2) read the next two bytes as the header record size in bytes,

(3) if the record size is greater than 2600, go back to the header

id file position, increment the file position, and go back to

step (1),

(4) read the header record based upon the previous byte count,

(5) calculate the 32 bit CRC of the header record data,

(6) read the next four bytes as the actual header record CRC,

(7) if the actual CRC does not equal the calculated CRC, go back

to the header id file position, increment the file position,

and go back to step (1).

It is acceptable to start this identification algorithm at a point

just after the ARJ self-extraction portion of an archive.  This

algorithm is fully demonstrated in the UNARJ C source code. A portion

of that source code is excerpted at the end of this document.

When using listfiles with ARJ, ARJ support programs should use the

"-p" option to ensure that ARJ will only extract the selected files

from an ARJ archive.  In addition, the listfiles should contain the

full pathname information as stored in the ARJ archive.  This avoids

the problem accessing files that have the same filename but different

paths.

There is an extended header bug in older versions of ARJ, AV.C and

UNARJ.C.  The extended header processing in read_header() should

skip 4 bytes for the extended header CRC and not 2.  This is NOT a

current problem as no versions of ARJ use the extended header.

**********************************************************************

Modification history:

Date      Description of modification:

--------  ------------------------------------------------------------

11/03/94  Improved SFX identification information.

01/21/94  Added find_header() routine.

03/17/93  Added information about ARJSFX change.

02/17/93  Added description of ARJ security fields.

Added archive date-modified field.

12/03/91  Added BACKUP flag to header arj flags.

11/21/91  Described the two types of headers separately.

11/11/91  Added information about the change in text mode processing.

06/28/91  Added several new HOST OS numbers.

05/19/91  Improved the description of extended header processing.

05/11/91  Simplified this document.  Added volume label type.

03/11/91  Added directory file type.

02/23/91  Added more comments.

01/10/91  Corrected timestamp description and header order of file mode.

10/30/90  Corrected values of flags in ARJ flags.

ARJ archives contains two types of header blocks:

Archive main header - This is located at the head of the archive

Local file header   - This is located before each archived file

Structure of main header (low order byte first):

Bytes Description

----- -------------------------------------------------------------------

2   header id (main and local file) = 0x60 0xEA

2   basic header size (from 'first_hdr_size' thru 'comment' below)

= first_hdr_size + strlen(filename) + 1 + strlen(comment) + 1

= 0 if end of archive

maximum header size is 2600

1   first_hdr_size (size up to and including 'extra data')

1   archiver version number

1   minimum archiver version to extract

1   host OS   (0 = MSDOS, 1 = PRIMOS, 2 = UNIX, 3 = AMIGA, 4 = MAC-OS)

(5 = OS/2, 6 = APPLE GS, 7 = ATARI ST, 8 = NEXT)

(9 = VAX VMS)

1   arj flags

(0x01 = NOT USED)

(0x02 = OLD_SECURED_FLAG)

(0x04 = VOLUME_FLAG)  indicates presence of succeeding

volume

(0x08 = NOT USED)

(0x10 = PATHSYM_FLAG) indicates archive name translated

("\" changed to "/")

(0x20 = BACKUP_FLAG) indicates backup type archive

(0x40 = SECURED_FLAG)

1   security version (2 = current)

1   file type	    (must equal 2)

1   reserved

4   date time when original archive was created

4   date time when archive was last modified

4   archive size (currently used only for secured archives)

4   security envelope file position

2   filespec position in filename

2   length in bytes of security envelope data

2   (currently not used)

?   (currently none)

?   filename of archive when created (null-terminated string)

?   archive comment  (null-terminated string)

4   basic header CRC

2   1st extended header size (0 if none)

?   1st extended header (currently not used)

4   1st extended header's CRC (not present when 0 extended header size)

Structure of local file header (low order byte first):

Bytes Description

----- -------------------------------------------------------------------

2   header id (main and local file) = 0x60 0xEA

2   basic header size (from 'first_hdr_size' thru 'comment' below)

= first_hdr_size + strlen(filename) + 1 + strlen(comment) + 1

= 0 if end of archive

maximum header size is 2600

1   first_hdr_size (size up to and including 'extra data')

1   archiver version number

1   minimum archiver version to extract

1   host OS   (0 = MSDOS, 1 = PRIMOS, 2 = UNIX, 3 = AMIGA, 4 = MAC-OS)

(5 = OS/2, 6 = APPLE GS, 7 = ATARI ST, 8 = NEXT)

(9 = VAX VMS)

1   arj flags (0x01 = GARBLED_FLAG) indicates passworded file

(0x02 = NOT USED)

(0x04 = VOLUME_FLAG)  indicates continued file to next

volume (file is split)

(0x08 = EXTFILE_FLAG) indicates file starting position

field (for split files)

(0x10 = PATHSYM_FLAG) indicates filename translated

("\" changed to "/")

(0x20 = BACKUP_FLAG)  indicates file marked as backup

1   method    (0 = stored, 1 = compressed most ... 4 compressed fastest)

1   file type (0 = binary,    1 = 7-bit text)

(3 = directory, 4 = volume label)

1   reserved

4   date time modified

4   compressed size

4   original size (this will be different for text mode compression)

4   original file's CRC

2   filespec position in filename

2   file access mode

2   host data (currently not used)

?   extra data

4 bytes for extended file starting position when used

(these bytes are present when EXTFILE_FLAG is set).

0 bytes otherwise.

?   filename (null-terminated string)

?   comment  (null-terminated string)

4   basic header CRC

2   1st extended header size (0 if none)

?   1st extended header (currently not used)

4   1st extended header's CRC (not present when 0 extended header size)

...

?   compressed file

Time stamp format:

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16

|<---- year-1980 --->|<- month ->|<--- day ---->|

15 14 13 12 11 10  9  8  7  6  5  4  3	2  1  0

|<--- hour --->|<---- minute --->|<- second/2 ->|

Routine to find an ARJ Header record:

long find_header(FILE *fd)

{

long arcpos, lastpos;

int c;

arcpos = file_tell(fd);

file_seek(fd, 0L, SEEK_END);

lastpos = file_tell(fd) - 2;

for ( ; arcpos < lastpos; arcpos++)

{

file_seek(fd, arcpos, SEEK_SET);

c = fget_byte(fd);

while (arcpos < lastpos)

{

if (c != HEADER_ID_LO)  /* low order first */

c = fget_byte(fd);

else if ((c = fget_byte(fd)) == HEADER_ID_HI)

break;

arcpos++;

}

if (arcpos >= lastpos)

break;

if ((headersize = fget_word(fd)) <= HEADERSIZE_MAX)

{

crc = CRC_MASK;

fread_crc(header, (int) headersize, fd);

if ((crc ^ CRC_MASK) == fget_crc(fd))

{

file_seek(fd, arcpos, SEEK_SET);

return arcpos;

}

}

}

return -1;          /* could not find a valid header */

}

end of document