V7-MIRAGE VIDEO

november  95

High-Performance Multimedia Accelerator

- High-Resolution Graphics-Acceleration up to 1280 x 1024, 75 Hz

- High Performance Graphics Accelerator with 2 MB EDODRAM

- PCI Version 2.0 Bus

- DPMS Power Management

- Simple jumperless installation

- OPTIONAL MPEG Modul Add-on:

- TrueColour,  High-Resolution Hardware

MPEG Video and Audio Playback

- SPEA MovieBus

Hardware

High-Performance Graphics Accelerator

- 1280 x 1024 pixels with 256 colours  at 75 Hz ergonomic display mode

- 1024 x 768 pixels with High Colour (16 bit - 65536 colours) at 85 Hz display mode

- 800 x 600 pixel with TrueColour (24 bit - 16.7 mio. colours) at 85 Hz display mode

- S3 Trio64V+ graphics processor & 135 MHz DAC

- 2 MByte EDODRAM

- 32 bit PCI 2.0 Bus Version

- VESA Power Management Support

- Scaling of digital Video like MPEG or Video for Windowsä

- hardware double buffering

- Chromakey, Colorkey and Sprite support

- supports MPEG software playback

Software

SPEA basic software:

- Driver for WindowsÒ 95 and WindowsÒ 3.1x

- SPEAview Displaymanagement Software

- SPEAenergy Power Management Software

- SPEAtune Monitorkonfigurations Software

- SPEA MediaStation easy to use control software

- CD with MPEG Videoclips

- NT and OS/2 graphics driver

- optional available: BigFocus DOS/Windows:    high performance displaylist driver for AutoCad

OPTIONAL MPEG MODUL:

simple plug in MPEG hardware decoder:

- S3 Scenic/MX2 Video/Audio Decoder

- full compliance with MPEG-1 Standard

- playback of  VideoCDä, Karaoke CD

- True Colour Playback

- 30 fps (NTSC), 25 fps (PAL) sustained frame rate

- Output 16x16 to 1024x768 resizable, fullscreen or in a window

- Output in realtime and maximum framerate from any analog or digital source

- High quality scaling through interpolation in X and Y direction

- Support of foreground windows on overlapping video playback

- Simultaneous display of graphics and video of different color depth

Audio

- 16-Bit Stereo, up to 44.1 kHz

- MPEG Audio Layer I and II

- Line level to 3,5 mm mini phone jack

- Hook up to powered speakers or soundcard

SPEA Movie Bus

- True Colour Digital Video Bus

- External Hardware Interface, eg. Motion  JPEG board

Additional Software

- MCI Windows driver for hardware MPEG audio and video

- CD Media Maestro LE presentation software

- CD Media Styler SE Authoring Software

System Requirements

- IBM 486 or better or other 100% compatibles

- 4 MB RAM

- MS Windows Ò95 or DOS 5.0 or higher and Windows Ò 3.1x

- 1 free PCI Slot

- No other active VGA or SVGA compatible graphics accelerator in the PC

Compatibility

- 100% compatible with VGA, SVGA and VESA Standard

- White Book 2.0 / 1.1compatible

- with MPEG Modul: MultimediaPC 3 compatible

Service & Support

- 3 years warranty

- free Software updates via SPEA BBS and CompuServe (GO SPEA)

- Technical support through SPEA Expert Line

Display modes

Resolution	Colours	Line 	Frequency	Refresh Rate 		Modes with video

(kHz)		(Hz)			(with optional MPEG Modul)

640 x 480	256		31.5 - 52.5	60 - 100			*

640 x 480	65.536		31.5 - 52.5	60 - 100			*

640 x 480	16.7 Mill	31.5 - 52.5	60 - 100			*

800 x 600	256		35.2 - 64.0	56 - 100			*

800 x 600	65.536		37.9 - 64.4	60 - 100			*

800 x 600	16.7 Mill.	37.9 - 56,6	60 - 85			*

1024 x 768	256		49.0 - 80.9	60 - 100			*

1024 x 768	65.536		49.0 - 68.7	60 - 85			*

1280 x 1024	256		64.4 - 79.8	60 - 75

first customer shipment

november 1995

SPEA Software AG

Moosstraße 18 b, D-82319 Starnberg

copyright:

©  SPEA  Software AG   Subject to technical alterations

Trademarks

(ä) The brands or product names mentioned are/may be trademarks or registered trademarks of their respective holders.