#

640K Memory Size Support

A Modification to PC-DOS 2.0 BIOS

By

John A. Chapman

CIS [70205,1217]

844 S. Madison St.

Hinsdale, Illinois  60521

Machine Configuration

This  modification  applies ONLY to those machines which  have  installed

more  than 544K byte of contiguous main storage,  AND DO NOT HAVE THE NEW

BIOS  CHIP.  The  new BIOS chip IBM supplies,  both  with  the  expansion

interface  and new 256K motherboard PC's already supports 640K of memory.

Please note that attempts to use this modification on machines with  less

than 608K storage may cause the system to fail.

Limitation and Disclaimer

Since  this modification alters the behavior of BIOS in a static fashion,

use  of  the modified system is thereafter LIMITED to machines  WITH  THE

SAME MEMORY SIZE, since there is no dynamic size determination capability

in this version of the mod.

THIS MODIFICATION IS SUPPLIED ON AN AS-IS BASIS. EACH USER MUST DETERMINE

THE FITNESS AND APPLICABILITY OF THIS CHANGE FOR THE SPECIFIC ENVIRONMENT

IN  WHICH  IT  WILL BE INSTALLED.   THE AUTHOR ASSUMES NO  LIABILITY  FOR

FAILURE  OR  DAMAGE  CAUSED  BY  THIS  MODIFICATION.

This  modification  is  in  use  on the  author's  system,  which  is  an

"original"  PC  (the old 16K mother board with the OLD bios  chip)  which

has the following equipment installed on it:

Vista MAXI-RAM     (576K memory board)

ABM OMNI-Board     (Clock 2-serial, 1-parallel, game adapter)

IBM Disk Controller (2 TEAC FD-55B's 1 Tandon 101-4)

IBM Color Graphics Adapter

This  modification  works on the configuration described  above.  If  you

report a bug,  please include a description of the configuration on which

the  bug  was encountered.   No known bugs exist in this version  of  the

modification,  which has been in use since DOS 2.0 was released.   Please

report any bugs you find to the author, at the address shown above.

1

Overview

Older  PC's are limited by the BIOS code and switch setting  support  (in

the  ROM  BIOS chip) to 544K.  It is possible to add 576K to the machine,

giving a total memory of 640K, or even 704K, if you don't have, and don't

need  the monochrome display.   To access this memory it is necessary  to

have it recognized at BOOT time.   DOS 2.0 maintains pointers to the  top

of memory which are set at initialization.  This modification to the INIT

routine of DOS 2.0 BIOS,  initializes the memorsize field in low memory

to 640K, and leaves a "footprint" similar to the one left by the keyboard

routine when ALT-CTRL-DEL sequence is detected.

The code added to BIOS does three things:

1) Checks for the "footprint" and skips all MEM640 processing

if the footprint is present (We've been here before) -

This prevents conflict with RAMDRIVE software that causes

a "warm" re-boot to alter memory size, to allocate memory

for the ramdrive.

2) Modifies the memory locations containing memory size and

I/O-bus memory size, in the system data segment at X'40'

3) Clears the memory above 544K to zeros.

Installing the Modification

To use this modification on a 640K machine, simply run the DEBUG script

contained in the file MEM640.ZAP, as follows:

a) B: drive must contain the DOS 2.0 formatted disk to be modified

b) A: drive must contain the DEBUG command, and the file MEM640.ZAP

When the disks are in the correct drives, the command:

a:debug <b:MEM640.ZAP

will  invoke  the DEBUG script supplied.  A listing of the actions  taken

will  display on the screen,  and eventually the DOS prompt will  return.

After the script has run,  the newly modified diskette should be  booted,

to  assure  that the modification was correctly  installed.   The  CHKDSK

command can be used to verify that the additional memory is accessable.

The  code  sequences to support greater that 544K are simple and  static.

The code should be easy to understand,  and easy to modify.  A listing of

the debug script, with the comments and indications of where changes must

be  made to support other memory sizes,  is included as Attachment  I  to

this document.

2

ATTACHMENT I: The DEBUG Script

l100,1,5,1              <======== First line of script

e10b,20

w100,1,5,1

nb:ibmbio.com

l

r                       <==== Action of script begins here

R CX

1230                    <== change length of BIOS before rewriting

A12d0,12ff              <== Begin assembly entry of mod

PUSH    AX

PUSH    ES

PUSH    DI

XOR     AX,AX

MOV     ES,AX

MOV     DI,0472          Change the two values pointed to below if

MOV     AX,4321           you wish to support a memory size between

ES:                       544 and 640.

CMP     AX,[DI]

JZ      12F7

MOV     DI,0413

MOV     AX,0280         <-- TOTAL MEMORY SIZE IN HEX        <<CHANGE>>

STOSW

MOV     AX,0240        <-- I/O bus Memory  (Total - 64K)    <<CHANGE>>

STOSW

CALL    1300           <-- invoke memory clear routine to prevent

MOV     DI,0472            parity checks on read references to memory

MOV     AX,4321        <-- OUR FOOTPRINT !!!

STOSW

POP     DI

POP     ES

POP     AX

MOV     AL,02

STOSB

JMP     08DC             <-- Jump back to BIOS init code

PUSH    CX   <===========Routine to clear ememory above 544K

PUSH    DX                (If you have less than 640K you don't have

PUSH    SI                 to change this routine, but If you have

MOV     DX,8800            704K or 768K, start and length values

MOV     AH,03              must be changed.

MOV     AL,00

MOV     ES,DX

MOV     CX,8000

SUB     DI,DI

REPZ

STOSB

ADD     DX,0800

DEC     AH

JNZ     130A

XOR     AX,AX

MOV     ES,AX

POP     SI

POP     DX

POP     CX

RET

NOP                    [[[ Continued on next Page ]]]

3

ATTACHMENT I: The DEBUG Script

NOP

NOP

NOP       <======= This section pads out the MOD to an even boundary

NOP

NOP

E,3E7,B0,00

A8D9

JMP     12D0       <====== Jump to the additional INIT code

W

l100,1,5,1

e10b,27            <====== This sequence resets IBMBIO.COM to Hidden

w100,1,5,1                 and System attributes, updating the directory

Q

###########################################