RESIDENT COMMAND INTERPRETER

John Chapman

844 S. Madison St.

Hinsdale, Illinois 60521

COMPUSERV ID 70205,1217

DISCLAIMER

This  modification is provided without warranty of any kind.  The

author  assumes no liability for failure of this code to  perform

in  any  environment.  The  user is  completely  responsible  for

determining   the  applicability  of  this  modification   to   a

particular environment.

LIMITATIONS

This modification causes the transient portion of COMMAND.COM  to

be  made resident at system IPL.   Re-booting the system with  an

unmodified   COMMAND.COM   is  the  only  way  to   remove   this

modification.  This  modification  alters  the  starting  segment

[paragraph] address available to programs, lowering the amount of

storage  available  by 4288 bytes [for PC/DOS 1.1 with  a  normal

load   segment  address  of  02C1].    The  starting  segment  of

COMMAND.COM can be found at location 0000:9E,  and should display

[debug] as       0000:009E 2C 01

CONTENTS

This   modification,   supplied  as  a  BASIC   program    called

"RESCMD.COM"  (Or  CLOCKCMD.BAS for realtime clock users)  which

will read in an UNMODIFIED DOS 1.1 COMMAND.COM and produce a  new

file, "COMMAND.CMD" which has all the mods installed. This set of

modifications  alters  the  IBM  Personal  Computer  DOS  command

interpreter  (COMMAND.COM)  to remain entirely  resident,  rather

than  be half resident and half transient.  The whole purpose  of

this  modification is to suppress 99% of the occurrances  of  the

refresh   of  the  transient  portion.   This  will  remove   the

requirement  to  ALWAYS have a disk with a valid  COMMAND.COM  in

drive  A.  This  change has been especially useful with  multiple

DISKCOPY  type activities,  and with those  application  packages

which  use all of memory,  including the top 4880 bytes where the

transient COMMAND.COM resides.

The  modification  merely omits the relocation of  the  transient

portion of COMMAND.COM, and sets the first free paragraph address

to  be  the area behind the transient routines,  rather than  the

small "resident" portion.

1

PC/DOS Initialization Overview

The  IBM  Personal  Computer DOS is brought in from disk  by  the

bootstrap loader read from track 0 of drive A by the ROM routines

after the initial hardware tests are performed at  power-on,  and

subsequently  when  ALT-CTRL-DEL re-boot is  requested.  In  both

cases, the following sequence occurs:

1) The "boot" program is read from track 0.

2) It reads IBMBIO.COM  from  a  specific location

3) IBMBIO is given control and builds the environmental

control blocks, initializes peripherals, and then

reads IBMDOS.COM from its specific location on disk

4) IBMDOS is passed control. After initializing the

remainder of the operating system, IBMDOS loads

the COMMAND.COM file [the command interpreter], and

passes it control

5) The command interpreter immediately relocates all of the

main line code for command prompting,  parsing,  and  the

code which supports the resident commands to the high end

of memory, where it can be more easily destroyed by

wandering programs.

6) Resident COMMAND.COM will now [pretend to] detect a

checksum error in the transient portion of itself,

immediately causing a Re-load from disk, thus setting

the tone for a long and tedious interaction, interrupted

frequently by disk I/O and requests for a DOS disk

in the default drive.

REAL-TIME CLOCK VERSION

An   alternate  version  of  the  modification  is  supplied   as

"CLOCKCMD.BAS".   This version REQUIRES THE PRESENCE OF A  CLOCK.

The  DOS  DATE  and TIME commands ARE DISABLED by  this  version,

requiring the user to employ the software provided with the clock

card  to access date and time.   The space formerly used by  DATE

and time is reused for three new RESIDENT commands,  BEEP,  PAGE,

and CLS.  PAGE causes the printer (LPT1:) to skip to top-of form.

CLS clears the active display.  BEEP causes a warbling tone to be

produced.

###################################################################################################################################