PRNSTATS.BAS

PRNSTATS.BAS  is  a  BASICA program to dynamically examine the status of

the line printer through port 957.  The particular program I wrote  just

continues  polling  the  port  until  is finds a change at which time it

updates the screen with the new status.

However, it was written with the purpose of calling  this  routine  just

before  requesting  some printer function within the driver program.  If

bits 1, 2, 3 and 5 are set low (0) then the printer is ready to  perform

some  function.   Otherwise some fault has occurred with it.  I selected

the message "Intervention required." However, to determine what is wrong

with  is  examine  each  bit  position  in the array PSB (program status

byte):  If bit 1 is high (1) then the printer is currently busy.  If the

second bit is high (1) then it probably has no electrical energy supply.

If bit 3 is set then it is out of forms.  If Bit 4 is low (0)  then  the

printer  is  not  on line (depress button on printer);  if bit 5 is high

then a print fault occurred (no power or off line).  Bits 6 and  7  have

no  meaning  assigned  yet,  and  are always high (1);  a time out fault

occured if the printer returns status bit 8 low (0), otherwise  it  will

be high.

This  program  should be more informative in the application that having

BASIC terminate execution, or is better than playing with the  on  error

command and guessing which err will be returned.

If problems contact me:  201 Delaware Avenue

Kingston, New York 12401

Currently  working  on a similar program for Disk usage, if you have one

completed similar to this, please let me know.