The program VM-SUPER is a simple supervisor program that will put a

80386 machine into virtual 8086 mode.  The code is from Appendix G of the book

"80386 Technical Reference" by Edmond Strauss. I have found this book to be

very informative.

The program is run from DOS real mode and returns to DOS in virtual 8086 mode.

The supervisor allows programs full access of all ports, and interrupts are

channeled through their original vectors.  Interrupts 01,06,07,0A,0B,0C

halt the machine. These are:

01: Debug Exception

06: Invalid Opcode

07: Coprocessor Not Available

0A: Invalid TSS

0B: Segment not present

0C: SS not present or Limit Violation

The debug and coprocessor ones have given me problems but I would assume

most simple programs should run fine from virtual mode.

Some possible enhancements:

Simulation of EMS/EEMS hardware by preventing access of the I/O ports

normally used by these boards and then emulating the memory paging in an

exception routine.

Multiple DOS environments, multitasking ...

I can be of some very limited help on how this supervisor works having spent

a lot of time with the text.  If you need leave me a message on

ASTRO   (202) 524-1837

Tom Dengler

5422 Forest Ave

Alexandria, VA  22310