S P L O T  -  The Pen Plotter Simulator V2.5

--------------------------------------------

Copyright (C) 1989-93 by Alexandr NOVY

All rights reserved.

April 1993

This file  contains technical  information about  pen plotter

types supported by the  SPLOT program. It also  contains list

of  supported  plotter  instructions  for each listed plotter

type.

1. General Information:

The  SPLOT  program  is   available  in  various   versions

according to the type  of simulated  plotter. This means that

each SPLOT.EXE  module is  designated for  a certain  plotter

type.   This  principle  gives  the  maximum  reliability  of

simulation of the given plotter type. At present versions for

the following plotter types are at your disposal:

Plotter type                          | Plotter syntax

------------------------------------------------------------

ROLAND DXY 1100/1200/1300             | DXY-GL / RD-GL I

HP 7470A                              | HP-GL

HP 7475A                              | HP-GL

HP 7440A ColorPro                     | HP-GL

HP 7550A                              | HP-GL

HP 7570A/7575A/7576A DraftPro DXL/EXL | HP-GL

HP 7595A/7596A DraftMaster I/II       | HP-GL

An UNREGISTERED COPY  of the program  is available only  in

the version for the HP 7475A plotter. The versions for  other

plotter types can  be received only  on the basis  of regular

registration.  See  the  file  LICENCE.DOC  for  registration

information.

2. Description of Supported Plotters:

-------------------------

HP 7470A Graphics Plotter

-------------------------

Syntax: HP-GL

Number of pens: 2

Paper formats: ISO A4 (210 x 297 mm)

ANSI A (8.5 x 11 in.)

Plotter unit: 0.025 mm

Maximal number of digits in decimal fraction to be read: 4

Number of character sets: 5 (0-4)

(all supported by SPLOT)

Alphabetical list of plotter instructions supported by

the HP7470A version of SPLOT:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PR  - The Plot Relative Instruction

PU  - The Pen Up Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The following instructions are  NOT supported by the  HP7470A

version of  SPLOT (they  are checked  for syntax  only and no

action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OI, OO, OP, OS and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

-------------------------

HP 7475A Graphics Plotter

-------------------------

Syntax: HP-GL

Number of pens: 6

Paper formats: ISO A4 (210 x 297 mm)

ISO A3 (297 x 420 mm)

ANSI A (8.5 x 11 in.)

ANSI B (11  x 17 in.)

Plotter unit: 0.02488 mm (0.025 mm in SPLOT)

Maximal number of digits in decimal fraction to be read: 4

Number of character sets: 19 (0-4,6-9,30-39)

(all supported by SPLOT)

Alphabetical list of plotter instructions supported by

the HP7475A version of SPLOT:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

EA  - The Edge Rectangle Absolute Instruction

ER  - The Edge Rectangle Relative Instruction

EW  - The Edge Wedge Instruction

FT  - The Fill Type Instruction (Note, that for HP7475A

plotter fill angle may be specified in  45 degree

increments only.)

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The following instructions are  NOT supported by the  HP7475A

version of  SPLOT (they  are checked  for syntax  only and no

action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OH, OI, OO, OP, OS and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

- The Page Size Instruction PS - in the SPLOT paper size

may be changed only via program control panel.

----------------------------------

HP 7440A ColorPro Graphics Plotter

----------------------------------

Including HP 17440 Graphics Enhancement Cartridge

Syntax: HP-GL

Number of pens: 8

Paper formats: ISO A4 (210 x 297 mm)

ANSI A (8.5 x 11 in.)

Plotter unit: 0.025 mm

Maximal number of digits in decimal fraction to be read: 4

Number of character sets:

5 without HP 17440 (0-4)

19 with   HP 17440 (0-4,6-9,30-39)

(all supported by SPLOT)

Alphabetical list of plotter instructions supported by

the HP7440A ColorPro version of SPLOT:

AA  - The Arc Absolute Instruction (HP 17440)

AR  - The Arc Relative Instruction (HP 17440)

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction (HP 17440)

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

EA  - The Edge Rectangle Absolute Instruction (HP 17440)

EP  - The Edge Polygon Instruction (HP 17440)

ER  - The Edge Rectangle Relative Instruction (HP 17440)

EW  - The Edge Wedge Instruction (HP 17440)

FP  - The Fill Polygon Instruction (HP 17440)

FT  - The Fill Type Instruction (HP 17440)

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PM  - The Polygon Mode Instruction (HP 17440)

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction (HP 17440)

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction (HP 17440)

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction (HP 17440)

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction (HP 17440)

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The following instructions are  NOT supported by the  HP7440A

ColorPro version of SPLOT  (they are checked for  syntax only

and no action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OH, OI, OO, OP, OS and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

-------------------------

HP 7550A Graphics Plotter

-------------------------

Syntax: HP-GL

Number of pens: 8

Paper formats: ISO A4 (210 x 297 mm)

ISO A3 (297 x 420 mm)

ANSI A (8.5 x 11 in.)

ANSI B (11  x 17 in.)

Plotter unit: 0.025 mm

Maximal number of digits in decimal fraction to be read: 8

Number of character sets: 40 (0-9,30-39,10-19,40-49)

(20 supported by SPLOT (0-9,30-39))

Alphabetical list of plotter instructions supported by

the HP7550A version of SPLOT:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

CT  - The Chord Tolerance Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

EA  - The Edge Rectangle Absolute Instruction

EP  - The Edge Polygon Instruction

ER  - The Edge Rectangle Relative Instruction

ES  - The Extra Space Instruction

EW  - The Edge Wedge Instruction

FP  - The Fill Polygon Instruction

FT  - The Fill Type Instruction

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LO  - The Label Origin Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PM  - The Polygon Mode Instruction

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The following instructions are  NOT supported by the  HP7550A

version of  SPLOT (they  are checked  for syntax  only and no

action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OG, OH, OI, OK, OL, OO, OP, OS, OT

and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

- And this:

AF or AH - The Advance Page Instruction *

AP  - The Automatic Pen Operations Instruction

AS  - The Acceleration Select Instruction

BF  - The Buffer Plot Instruction

BL  - The Buffer Label Instruction *

CC  - The Character Chord Angle Instruction *

CM  - The Character Selection Mode Instruction *

CV  - The Curved Line Generator Instruction

DL  - The Define Downloadable Character Instruction *

DS  - The Designate Character Set into Slot Instruction *

FS  - The Force Select Instruction

GC  - The Group Count Instruction

GM  - The Graphics Memory Instruction *

GP  - The Group Pen Instruction *

IV  - The Invoke Character Slot Instruction *

KY  - The Define Key Instruction

NR  - The Not-Ready Instruction

PB  - The Print Buffered Label Instruction *

PG  - The Page Feed Instruction *

RP  - The Replot Instruction

SG  - The Select Pen Group Instruction *

UF  - The User-Defined Fill Type Instruction *

WD  - The Write to Display Instruction

* - may be supported in next versions

----------------------------

HP DraftPro Graphics Plotter

----------------------------

HP 7570A DraftPro

HP 7575A DraftPro DXL

HP 7576A DraftPro EXL

Syntax: HP-GL

Number of pens: 8

Paper formats: ISO A4 (DXL/EXL only)

ISO A3 (DXL/EXL only)

ISO A2

ISO A1

ISO A0 (EXL only)

ANSI A (DXL/EXL only)

ANSI B (DXL/EXL only)

ANSI C

ANSI D

ANSI E (EXL only)

ARCH A (DXL/EXL only)

ARCH B (DXL/EXL only)

ARCH C

ARCH D

ARCH E (EXL only)

Supports EXPAND mode for each format.

Note - origin of plotter coordinates is situated to the

center of the plotting area (paper).

Plotter unit: 0.025 mm

Maximal number of digits in decimal fraction to be read: 4

Number of character sets: 21 (0-9,30-39,99)

(all supported by SPLOT)

Alphabetical list of plotter instructions supported by

the HP DraftPro version of SPLOT:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

CT  - The Chord Tolerance Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

DV  - The Direction Vertical Instruction

EA  - The Edge Rectangle Absolute Instruction

EP  - The Edge Polygon Instruction

ER  - The Edge Rectangle Relative Instruction

ES  - The Extra Space Instruction

EW  - The Edge Wedge Instruction

FP  - The Fill Polygon Instruction

FT  - The Fill Type Instruction

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LO  - The Label Origin Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PM  - The Polygon Mode Instruction

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SG  - The Select Pen Group Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The  following  instructions  are  NOT  supported  by  the HP

DraftPro version of SPLOT  (they are checked for  syntax only

and no action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OH, OI, OO, OP, OS and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

- And this:

AP  - The Automatic Pen Operations Instruction

CM  - The Character Selection Mode Instruction *

DS  - The Designate Character Set into Slot Instruction *

GM  - The Graphics Memory Instruction *

IV  - The Invoke Character Slot Instruction *

NR  - The Not-Ready Instruction

PE  - The Encoded Polyline Instruction *

* - may be supported in next versions

-------------------------------

HP DraftMaster Graphics Plotter

-------------------------------

HP 7595A DraftMaster I

HP 7596A DraftMaster II

Syntax: HP-GL

Number of pens: 8

Paper formats: ISO A4,A3,A2,A1,A0

ANSI A,B,C,D,E

ARCH A,B,C,D,E

Supports EXPAND mode for each format.

Note - origin of plotter coordinates is situated to the

center of the plotting area (paper).

Plotter unit: 0.025 mm

Maximal number of digits in decimal fraction to be read: 8

Number of character sets: 64 (0-9,30-39,99,10-19,40-49,

20-29,50-59,60,70,80)

(21 supported by SPLOT (0-9,30-39,99))

Alphabetical list of plotter instructions supported by

the HP DraftMaster version of SPLOT:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

BF  - The Buffer Plot Instruction (NOP)

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

CT  - The Chord Tolerance Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

DV  - The Direction Vertical Instruction

EA  - The Edge Rectangle Absolute Instruction

EP  - The Edge Polygon Instruction

ER  - The Edge Rectangle Relative Instruction

ES  - The Extra Space Instruction

EW  - The Edge Wedge Instruction

FP  - The Fill Polygon Instruction

FT  - The Fill Type Instruction

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LO  - The Label Origin Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PM  - The Polygon Mode Instruction

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VA  - The Adaptive Velocity Instruction (NOP)

VN  - The Normal Velocity Instruction (NOP)

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The  following  instructions  are  NOT  supported  by  the HP

DraftMaster version  of SPLOT  (they are  checked for  syntax

only and no action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OG, OH, OI, OK, OL, OO, OP, OS, OT

and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

- The Page Size Instruction PS - in the SPLOT paper size

may be changed only via program control panel.

- And this:

AF or AH - The Advance Page Instruction *

AP  - The Automatic Pen Operations Instruction

AS  - The Acceleration Select Instruction

BL  - The Buffer Label Instruction *

CC  - The Character Chord Angle Instruction *

CM  - The Character Selection Mode Instruction *

CV  - The Curved Line Generator Instruction

DL  - The Define Downloadable Character Instruction *

DS  - The Designate Character Set into Slot Instruction *

EC  - The Enable Cut Line Instruction

FR  - The Frame Advance Instruction

FS  - The Force Select Instruction

GC  - The Group Count Instruction

GM  - The Graphics Memory Instruction *

GP  - The Group Pen Instruction *

IV  - The Invoke Character Slot Instruction *

KY  - The Define Key Instruction

NR  - The Not-Ready Instruction

PB  - The Print Buffered Label Instruction *

PG  - The Page Feed Instruction *

SG  - The Select Pen Group Instruction *

UF  - The User-Defined Fill Type Instruction *

WD  - The Write to Display Instruction

* - may be supported in next versions

-------------------------

ROLAND DXY 1100/1200/1300

-------------------------

Syntax: RD-GL I (corresponds with the HP-GL syntax)

DXY-GL

Number of pens: 8

Paper formats: ISO A4 (210 x 297 mm)

ISO A3 (297 x 420 mm)

ANSI A (8.5 x 11 in.)

ANSI B (11  x 17 in.)

Supports EXPAND mode for each format.

Plotter unit: 0.025 mm in RD-GL mode

0.1 or 0.025 mm in DXY-GL mode

Maximal number of digits in decimal fraction to be read: 4

Number of character sets: 19 (0-4,6-9,30-39)

(all supported by SPLOT)

Alphabetical list of plotter instructions supported by

the ROLAND version of SPLOT in RD-GL mode:

AA  - The Arc Absolute Instruction

AR  - The Arc Relative Instruction

CA  - The Designate Alternate Character Set Instruction

CI  - The Circle Instruction

CP  - The Character Plot Instruction

CS  - The Designate Standard Character Set Instruction

DF  - The Default Instruction

DI  - The Absolute Direction Instruction

DR  - The Relative Direction Instruction

DT  - The Define Terminator Instruction

EA  - The Edge Rectangle Absolute Instruction

ER  - The Edge Rectangle Relative Instruction

EW  - The Edge Wedge Instruction

FT  - The Fill Type Instruction (Note, that for ROLAND

plotter fill angle may be specified in 45 degree

increments only.)

IN  - The Initialize Instruction

IP  - The Input P1 and P2 Instruction

IW  - The Input Window Instruction

LB  - The Label Instruction

LT  - The Line Type Instruction

PA  - The Plot Absolute Instruction

PD  - The Pen Down Instruction

PR  - The Plot Relative Instruction

PT  - The Pen Thickness Instruction

PU  - The Pen Up Instruction

RA  - The Shade Rectangle Absolute Instruction

RO  - The Rotate Coordinate Instruction

RR  - The Shade Rectangle Relative Instruction

SA  - The Select Alternate Character Set Instruction

SC  - The Scale Instruction

SI  - The Absolute Character Size Instruction

SL  - The Character Slant Instruction

SM  - The Symbol Mode Instruction

SP  - The Select Pen Instruction

SR  - The Relative Character Size Instruction

SS  - The Select Standard Character Set Instruction

TL  - The Tick Length Instruction

UC  - The User Defined Character Instruction

VS  - The Velocity Select Instruction (for information only)

WG  - The Shade Wedge Instruction

XT  - The X-Tick Instruction

YT  - The Y-Tick Instruction

The following  instructions are  NOT supported  by the ROLAND

version of SPLOT in RD-GL  mode (they are checked for  syntax

only and no action is performed):

- All escape code sequences.

- All output instructions:

OA, OC, OD, OE, OF, OH, OI, OO, OP, OS and OW.

- Digitizing instructions DP and DC.

- The Input Mask Instruction IM.

- The Page Size Instruction PS - in the SPLOT paper size

may be changed only via program control panel.

Alphabetical list of plotter instructions supported by

the ROLAND version of SPLOT in DXY-GL mode:

A  - The Circle Center Instruction

B  - The Line Scale Instruction

C  - The Circle Instruction

D  - The Draw Instruction

E  - The Relative Circle Instruction

G  - The A + Circle Instruction

H  - The Home Instruction

I  - The Relative Draw Instruction

J  - The Pen Change Instruction

K  - The A + % Instruction

L  - The Line Type Instruction

M  - The Move Instruction

N  - The Mark Instruction

P  - The Print Instruction

Q  - The Alpha Rotate Instruction

R  - The Relative Move Instruction

S  - The Alpha Scale Instruction

T  - The Hatching Instruction

X  - The Axis Instruction

^  - The Up Instruction

The following  instructions are  NOT supported  by the ROLAND

version of SPLOT in DXY-GL mode (they are checked for  syntax

only and no action is performed):

Y  - The Curve Instruction *

_  - The Relative Curve Instruction *

* - may be supported in next versions

End of list.