Chapter 9: At Grade Intersection Capacity and Level of Service

9.1

Given:		2-lane intersection approach; CBD area

No cross street pedestrians during left turns

Lane width = 11 ft

+2% grade

No parking lanes; No bus stops

6% heavy vehicles

Protected phasing

PLT = 0.30

N = 2

fw = 0.967  (Table 9-5)

fHV = 0.943  (Table 9-6)

fg = 0.990  (Table 9-7)

fp = 1.0  (Table 9-8)

fbb = 1.0  (Table 9-9)

fa = 0.90  (Table 9-10)

fRT = 1  (Table 9-11a, no right turns)

fLT=1.01.0+0.05PLT=1.01.0+0.05(0.30)=0.985  (Table 9-12)

s = s0 N fw fHV fg fp fbb fa fRT fLT

s = 1900(2)(0.967)(0.943)(0.990)(1.0)(1.0)(0.90)(1.0)(0.985) = 3042 vph green time

9.2

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-08-1996  1

=======================================================================

Streets: (E-W) Third Ave.              (N-S) Main St.

Analyst:                               File Name: Exerc_02.HC9

Area Type: CBD                         7-18-96 4-5 PM

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  |    > 2  <    |    > 2  <    |    > 1  <    |    > 1  <

Volumes    |  60  700   50|  50  550   20|  40  500   30|  45  500   15

PHF or PK15|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90

Lane Width |     11.0     |     11.0     |     15.0     |     15.0

Grade      |        0     |        0     |        0     |        0

% Heavy Veh|   5    5    5|   5    5    5|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |           100|           100|           100|           100

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |        4     |        2     |        3     |        3

RTOR Vols  |            15|            10|            20|            10

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          27.0P                  |Green    37.0P

Yellow/AR       3.0                   |Yellow/AR 3.0

Cycle Length:  70 sec  Phase combination order: #1 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left    60  0.90    67

Thru   700  0.90   778   LTR   884  2  1.050   1.000   928  0.08  0.04

Right   50  0.90    39

WB

Left    50  0.90    56

Thru   550  0.90   611   LTR   678  2  1.050   1.000   712  0.08  0.02

Right   20  0.90    11

NB

Left    40  0.90    44

Thru   500  0.90   556   LTR   611  1  1.000   1.000   611  0.07  0.02

Right   30  0.90    11

SB

Left    45  0.90    50

Thru   500  0.90   556   LTR   612  1  1.000   1.000   612  0.08  0.01

Right   15  0.90     6

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

LTR    1900   2  0.97  0.95  1.00  1.00  1.00  0.90  0.99  0.74  2326

WB

LTR    1900   2  0.97  0.95  1.00  1.00  1.00  0.90  1.00  0.66  2078

NB

LTR    1900   1  1.10  0.95  1.00  1.00  1.00  0.90  0.90  0.81  1302

SB

LTR    1900   1  1.10  0.95  1.00  1.00  1.00  0.90  0.90  0.79  1263

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                              EB

Cycle Length, C                                         70

Actual Green Time for Lane Group, G                     27

Effective Green Time for Lane Group, g                  27

Opposing Effective Green Time, go                       27

Number of Opposing Lanes, No                             2

Number of Lanes in Lane Group, N                         2

Adjusted Left-Turn Flow Rate, Vlt                       67

Proportion of Left Turns in Lane Group, Plt              0.08

Left Turns per Cycle: LTC=Vlt*C/3600                     1.30

Adjusted Opposing Flow Rate, Vo                        712

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No       6.92

Opposing Platoon Ratio, Rpo                              0.67

Lost time per phase, tl                                  3

gf=Gexp(-0.882*LTC^0.717)-tl                             6.30

Opposing Queue Ratio: qro=1-Rpo(go/C)                    0.74

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl        8.83

gu=g-gq (or g-gf)                                       18.17

fs=(875-0.625Vo)/1000                                    0.43

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                            0.24

El1                                                      4.94

fmin                                                     0.09

fm,(min=fmin;max=1.00)                                   0.58

flt=[fm+0.91(N-1)]/N                                     0.74

APPROACH                                              WB

Cycle Length, C                                         70

Actual Green Time for Lane Group, G                     27

Effective Green Time for Lane Group, g                  27

Opposing Effective Green Time, go                       27

Number of Opposing Lanes, No                             2

Number of Lanes in Lane Group, N                         2

Adjusted Left-Turn Flow Rate, Vlt                       56

Proportion of Left Turns in Lane Group, Plt              0.08

Left Turns per Cycle: LTC=Vlt*C/3600                     1.09

Adjusted Opposing Flow Rate, Vo                        928

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No       9.02

Opposing Platoon Ratio, Rpo                              1.33

Lost time per phase, tl                                  3

gf=Gexp(-0.882*LTC^0.717)-tl                             7.57

Opposing Queue Ratio: qro=1-Rpo(go/C)                    0.49

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       10.37

gu=g-gq (or g-gf)                                       16.63

fs=(875-0.625Vo)/1000                                    0.29

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                            0.32

El1                                                     12.40

fmin                                                     0.10

fm,(min=fmin;max=1.00)                                   0.41

flt=[fm+0.91(N-1)]/N                                     0.66

APPROACH                                              NB

Cycle Length, C                                         70

Actual Green Time for Lane Group, G                     37

Effective Green Time for Lane Group, g                  37

Opposing Effective Green Time, go                       37

Number of Lanes in Lane Group, N                         1

Proportion of Left Turns in Lane Group, Plt              0.07

Proportion of Left Turns in Opposing Flow, Plto          0.08

Adjusted Left-Turn Flow Rate, Vlt                       44

Left Turns per Cycle: LTC=Vlt*C/3600                     0.86

Adjusted Opposing Flow Rate, Vo                        612

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      11.90

Opposing Platoon Ratio, Rpo                              1

Lost time per phase, tl                                  3

gf=Gexp(-0.860*LTC^0.629)-tl                            13.97

Opposing Queue Ratio: qro=1-Rpo(go/C)                    0.47

gq=4.943volc^0.762*qro^1.061-tl                         11.69

gu=g-gq (or g-gf)                                       23.03

n=(gq-gf)/2                                              0.00

Ptho=1-Plto                                              0.92

El2                                                      0.00

El1                                                      7.07

fmin=2(1+Pl)/g                                           0.06

flt=fm,(min=fmin; max=1)                                 0.81

APPROACH                                              SB

Cycle Length, C                                         70

Actual Green Time for Lane Group, G                     37

Effective Green Time for Lane Group, g                  37

Opposing Effective Green Time, go                       37

Number of Lanes in Lane Group, N                         1

Proportion of Left Turns in Lane Group, Plt              0.08

Proportion of Left Turns in Opposing Flow, Plto          0.07

Adjusted Left-Turn Flow Rate, Vlt                       50

Left Turns per Cycle: LTC=Vlt*C/3600                     0.97

Adjusted Opposing Flow Rate, Vo                        611

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      11.88

Opposing Platoon Ratio, Rpo                              1

Lost time per phase, tl                                  3

gf=Gexp(-0.860*LTC^0.629)-tl                            12.90

Opposing Queue Ratio: qro=1-Rpo(go/C)                    0.47

gq=4.943volc^0.762*qro^1.061-tl                         11.67

gu=g-gq (or g-gf)                                       24.10

n=(gq-gf)/2                                              0.00

Ptho=1-Plto                                              0.93

El2                                                      0.00

El1                                                      7.02

fmin=2(1+Pl)/g                                           0.06

flt=fm,(min=fmin; max=1)                                 0.79

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

LTR        928       2326    0.399     0.386        897    1.034 *

WB

LTR        712       2078    0.343     0.386        802    0.888

NB

LTR        611       1302    0.469     0.529        688    0.888

SB

LTR        612       1263    0.485     0.529        668    0.917 *

Sum (v/s) critical = 0.884

Lost Time/Cycle, L =   6.0 sec   Critical v/c(x)    = 0.966

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

LTR     1.034 0.386  16.3 0.910   897   12   29.0  43.9  E  43.9  E

WB

LTR     0.888 0.386  15.3 1.124   802   12    6.7  23.8  C  23.8  C

NB

LTR     0.888 0.529  11.1 1.000   688   16    9.6  20.7  C  20.7  C

SB

LTR     0.917 0.529  11.5 1.000   668   16   12.6  24.1  C  24.1  C

Intersection Delay =  29.7 sec/veh Intersection LOS = D

-----------------------------------------------------------------------

9.3

A complete operational analysis based on the Highway Capacity Software (HCS) is included here. Exercise 3 requests the left-turn adjustment factor only for the EB approach, in a case where there is an exclusive left-turn lane. The left-turn adjustment-factor computation is highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-02-1996  1

=======================================================================

Streets: (E-W) Ryder St.               (N-S) Moss Ave.

Analyst:                               File Name: Exerc_03.HC9

Area Type: Other                       8-1-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    2  <    | 1    2  <    |    > 1  <    |    > 1  <

Volumes    |  80  870   50|  80  756   53|  35  425   25|  15  500   30

PHF or PK15|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95

Lane Width |12.0 12.0     |12.0 12.0     |     12.0     |     12.0

Grade      |        3     |       -3     |        3     |       -3

% Heavy Veh|   5    5    5|   5    5    5|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |             0|             0|             0|             0

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |   3    3     |   3    3     |        3     |        3

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          35.0P                  |Green    27.0P

Yellow/AR       4.0                   |Yellow/AR 4.0

Cycle Length:  70 sec  Phase combination order: #1 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left    80  0.95    84   L      84  1  1.000   1.000    84  1.00  0.00

Thru   870  0.95   916   TR    969  2  1.050   1.000  1017  0.00  0.05

Right   50  0.95    53

WB

Left    80  0.95    84   L      84  1  1.000   1.000    84  1.00  0.00

Thru   756  0.95   796   TR    852  2  1.050   1.000   895  0.00  0.07

Right   53  0.95    56

NB

Left    35  0.95    37

Thru   425  0.95   447   LTR   510  1  1.000   1.000   510  0.07  0.05

Right   25  0.95    26

SB

Left    15  0.95    16

Thru   500  0.95   526   LTR   574  1  1.000   1.000   574  0.03  0.06

Right   30  0.95    32

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

L      1900   1  1.00  0.95  0.99  1.00  1.00  1.00  1.00  0.15   270

TR     1900   2  1.00  0.95  0.99  1.00  1.00  1.00  0.99  1.00  3538

WB

L      1900   1  1.00  0.95  1.01  1.00  1.00  1.00  1.00  0.11   204

TR     1900   2  1.00  0.95  1.01  1.00  1.00  1.00  0.99  1.00  3635

NB

LTR    1900   1  1.00  0.95  0.99  1.00  1.00  1.00  0.89  0.86  1372

SB

LTR    1900   1  1.00  0.95  1.01  1.00  1.00  1.00  0.89  0.95  1560

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             EB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    35

Effective Green Time for Lane Group, g                 36

Opposing Effective Green Time, go                      36

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                      84

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    1.63

Adjusted Opposing Flow Rate, Vo                       895

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      8.70

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.49

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       8.25

gu=g-gq (or g-gf)                                      27.75

fs=(875-0.625Vo)/1000                                   0.32

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     5.10

fmin                                                    0.11

fm,(min=fmin;max=1.00)                                  0.15

flt=[fm+0.91(N-1)]/N                                    0.15

APPROACH                                             WB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    35

Effective Green Time for Lane Group, g                 36

Opposing Effective Green Time, go                      36

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                      84

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    1.63

Adjusted Opposing Flow Rate, Vo                      1017

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      9.89

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.49

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      10.39

gu=g-gq (or g-gf)                                      25.61

fs=(875-0.625Vo)/1000                                   0.24

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     6.56

fmin                                                    0.11

fm,(min=fmin;max=1.00)                                  0.11

flt=[fm+0.91(N-1)]/N                                    0.11

APPROACH                                             NB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.07

Proportion of Left Turns in Opposing Flow, Plto         0.03

Adjusted Left-Turn Flow Rate, Vlt                      37

Left Turns per Cycle: LTC=Vlt*C/3600                    0.72

Adjusted Opposing Flow Rate, Vo                       574

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     11.16

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           10.42

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.60

gq=4.943volc^0.762*qro^1.061-tl                        15.07

gu=g-gq (or g-gf)                                      12.93

n=(gq-gf)/2                                             2.33

Ptho=1-Plto                                             0.97

El2                                                     2.28

El1                                                     6.08

fmin=2(1+Pl)/g                                          0.08

flt=fm,(min=fmin; max=1)                                0.86

APPROACH                                             SB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.03

Proportion of Left Turns in Opposing Flow, Plto         0.07

Adjusted Left-Turn Flow Rate, Vlt                      16

Left Turns per Cycle: LTC=Vlt*C/3600                    0.31

Adjusted Opposing Flow Rate, Vo                       510

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      9.92

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           14.87

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.60

gq=4.943volc^0.762*qro^1.061-tl                        13.51

gu=g-gq (or g-gf)                                      13.13

n=(gq-gf)/2                                             0.00

Ptho=1-Plto                                             0.93

El2                                                     0.00

El1                                                     5.06

fmin=2(1+Pl)/g                                          0.07

flt=fm,(min=fmin; max=1)                                0.95

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

L           84        270    0.311     0.514        139    0.605

TR        1017       3538    0.287     0.514       1820    0.559

WB

L           84        204    0.412     0.514        105    0.801 *

TR         895       3635    0.246     0.514       1869    0.479

NB

LTR        510       1372    0.372     0.400        549    0.929 *

SB

LTR        574       1560    0.368     0.400        624    0.920

Sum (v/s) critical = 0.783

Lost Time/Cycle, L =   6.0 sec   Critical v/c(x)    = 0.857

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       0.605 0.514   9.1 1.000   139   16    5.1  14.2  B   9.5  B

TR      0.559 0.514   8.8 1.000  1820   16    0.3   9.1  B

WB

L       0.801 0.514  10.7 1.000   105   16   22.5  33.2  D  10.6  B

TR      0.479 0.514   8.3 1.000  1869   16    0.2   8.5  B

NB

LTR     0.929 0.400  15.2 1.000   549   16   16.2  31.4  D  31.4  D

SB

LTR     0.920 0.400  15.2 1.000   624   16   13.6  28.8  D  28.8  D

Intersection Delay =  16.9 sec/veh Intersection LOS = C

-----------------------------------------------------------------------

9.4

The following is the Highway Capacity Software (HCS) solution to Exercise 9. Exercise 4 requests the left-turn adjustment factor only for the EB approach, in a case where the left turns are made from a shared lane. Computation of the left-turn adjustment factor is highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-02-1996  1

======================================================================

Streets: (E-W) Ryder St.               (N-S) Moss Ave.

Analyst:                               File Name: Exer_4&9.HC9

Area Type: Other                       8-1-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  |    > 2  <    |    > 2  <    |    > 1  <    |    > 1  <

Volumes    |  80  870   50|  80  756   53|  35  425   25|  15  500   30

PHF or PK15|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95

Lane Width |     12.0     |     12.0     |     12.0     |     12.0

Grade      |        3     |       -3     |        3     |       -3

% Heavy Veh|   5    5    5|   5    5    5|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|            0

Con. Peds  |             0|             0|             0|            0

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |        3     |        3     |        3     |        3

RTOR Vols  |             0|             0|             0|            0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          35.0P                  |Green    27.0P

Yellow/AR       4.0                   |Yellow/AR 4.0

Cycle Length:  70 sec  Phase combination order: #1 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left    80  0.95    84

Thru   870  0.95   916   LTR  1053  2  1.050   1.000  1106  0.08  0.05

Right   50  0.95    53

WB

Left    80  0.95    84

Thru   756  0.95   796   LTR   936  2  1.050   1.000   983  0.09  0.06

Right   53  0.95    56

NB

Left    35  0.95    37

Thru   425  0.95   447   LTR   510  1  1.000   1.000   510  0.07  0.05

Right   25  0.95    26

SB

Left    15  0.95    16

Thru   500  0.95   526   LTR   574  1  1.000   1.000   574  0.03  0.06

Right   30  0.95    32

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

LTR    1900   2  1.00  0.95  0.99  1.00  1.00  1.00  0.99  0.62  2179

WB

LTR    1900   2  1.00  0.95  1.01  1.00  1.00  1.00  0.99  0.59  2165

NB

LTR    1900   1  1.00  0.95  0.99  1.00  1.00  1.00  0.89  0.86  1372

SB

LTR    1900   1  1.00  0.95  1.01  1.00  1.00  1.00  0.89  0.95  1560

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             EB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    35

Effective Green Time for Lane Group, g                 36

Opposing Effective Green Time, go                      36

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        2

Adjusted Left-Turn Flow Rate, Vlt                      84

Proportion of Left Turns in Lane Group, Plt             0.08

Left Turns per Cycle: LTC=Vlt*C/3600                    1.63

Adjusted Opposing Flow Rate, Vo                       983

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      9.56

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            6.99

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.49

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       9.77

gu=g-gq (or g-gf)                                      26.23

fs=(875-0.625Vo)/1000                                   0.26

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           0.33

El1                                                    15.15

fmin                                                    0.07

fm,(min=fmin;max=1.00)                                  0.32

flt=[fm+0.91(N-1)]/N                                    0.62

APPROACH                                             WB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    35

Effective Green Time for Lane Group, g                 36

Opposing Effective Green Time, go                      36

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        2

Adjusted Left-Turn Flow Rate, Vlt                      84

Proportion of Left Turns in Lane Group, Plt             0.09

Left Turns per Cycle: LTC=Vlt*C/3600                    1.63

Adjusted Opposing Flow Rate, Vo                      1106

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     10.75

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            6.99

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.49

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      12.08

gu=g-gq (or g-gf)                                      23.92

fs=(875-0.625Vo)/1000                                   0.18

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           0.45

El1                                                    16.00

fmin                                                    0.08

fm,(min=fmin;max=1.00)                                  0.28

flt=[fm+0.91(N-1)]/N                                    0.59

APPROACH                                             NB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.07

Proportion of Left Turns in Opposing Flow, Plto         0.03

Adjusted Left-Turn Flow Rate, Vlt                      37

Left Turns per Cycle: LTC=Vlt*C/3600                    0.72

Adjusted Opposing Flow Rate, Vo                       574

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     11.16

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           10.42

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.60

gq=4.943volc^0.762*qro^1.061-tl                        15.07

gu=g-gq (or g-gf)                                      12.93

n=(gq-gf)/2                                             2.33

Ptho=1-Plto                                             0.97

El2                                                     2.28

El1                                                     6.08

fmin=2(1+Pl)/g                                          0.08

flt=fm,(min=fmin; max=1)                                0.86

APPROACH                                             SB

Cycle Length, C                                        70

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.03

Proportion of Left Turns in Opposing Flow, Plto         0.07

Adjusted Left-Turn Flow Rate, Vlt                      16

Left Turns per Cycle: LTC=Vlt*C/3600                    0.31

Adjusted Opposing Flow Rate, Vo                       510

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      9.92

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           14.87

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.60

gq=4.943volc^0.762*qro^1.061-tl                        13.51

gu=g-gq (or g-gf)                                      13.13

n=(gq-gf)/2                                             0.00

Ptho=1-Plto                                             0.93

El2                                                     0.00

El1                                                     5.06

fmin=2(1+Pl)/g                                          0.07

flt=fm,(min=fmin; max=1)                                0.95

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

LTR       1106       2179    0.508     0.514       1121    0.987 *

WB

LTR        983       2165    0.454     0.514       1113    0.883

NB

LTR        510       1372    0.372     0.400        549    0.929 *

SB

LTR        574       1560    0.368     0.400        624    0.920

Sum (v/s) critical = 0.879

Lost Time/Cycle, L =   6.0 sec   Critical v/c(x)    = 0.962

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

LTR     0.987 0.514  12.7 1.000  1121   16   17.9  30.7  D  30.7  D

WB

LTR     0.883 0.514  11.5 1.000  1113   16    6.1  17.6  C  17.6  C

NB

LTR     0.929 0.400  15.2 1.000   549   16   16.2  31.4  D  31.4  D

SB

LTR     0.920 0.400  15.2 1.000   624   16   13.6  28.8  D  28.8  D

Intersection Delay =  26.4 sec/veh Intersection LOS = D

-----------------------------------------------------------------------

9.5

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-08-1996  1

=======================================================================

Streets: (E-W) Grand Ave.              (N-S) 4th St.

Analyst:                               File Name: Exerc_05.HC9

Area Type: CBD                         7-30-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  |    > 2  <    |    > 2  <    |    > 1  <    |    > 1  <

Volumes    |  60  570   30|  25  635   18|  27  330   20|  35  450   45

PHF or PK15|0.85 0.85 0.85|0.85 0.85 0.85|0.85 0.85 0.85|0.85 0.85 0.85

Lane Width |     12.0     |     12.0     |     14.0     |     14.0

Grade      |        0     |        0     |        0     |        0

% Heavy Veh|   5    5    5|   5    5    5|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |            80|            80|            80|            80

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |        4     |        2     |        3     |        3

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          30.0P                  |Green    27.0P

Yellow/AR       4.0                   |Yellow/AR 4.0

Cycle Length:  65 sec  Phase combination order: #1 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left    60  0.85    71

Thru   570  0.85   671   LTR   777  2  1.050   1.000   816  0.09  0.05

Right   30  0.85    35

WB

Left    25  0.85    29

Thru   635  0.85   747   LTR   797  2  1.050   1.000   837  0.04  0.03

Right   18  0.85    21

NB

Left    27  0.85    32

Thru   330  0.85   388   LTR   444  1  1.000   1.000   444  0.07  0.05

Right   20  0.85    24

SB

Left    35  0.85    41

Thru   450  0.85   529   LTR   623  1  1.000   1.000   623  0.07  0.09

Right   45  0.85    53

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

LTR    1900   2  1.00  0.95  1.00  1.00  1.00  0.90  0.99  0.68  2215

WB

LTR    1900   2  1.00  0.95  1.00  1.00  1.00  0.90  1.00  0.83  2701

NB

LTR    1900   1  1.07  0.95  1.00  1.00  1.00  0.90  0.89  0.84  1299

SB

LTR    1900   1  1.07  0.95  1.00  1.00  1.00  0.90  0.89  0.90  1377

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             EB

Cycle Length, C                                        65

Actual Green Time for Lane Group, G                    30

Effective Green Time for Lane Group, g                 31

Opposing Effective Green Time, go                      31

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        2

Adjusted Left-Turn Flow Rate, Vlt                      71

Proportion of Left Turns in Lane Group, Plt             0.09

Left Turns per Cycle: LTC=Vlt*C/3600                    1.28

Adjusted Opposing Flow Rate, Vo                       837

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      7.56

Opposing Platoon Ratio, Rpo                             0.67

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            7.46

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.68

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       9.18

gu=g-gq (or g-gf)                                      21.82

fs=(875-0.625Vo)/1000                                   0.35

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           0.32

El1                                                     7.85

fmin                                                    0.09

fm,(min=fmin;max=1.00)                                  0.46

flt=[fm+0.91(N-1)]/N                                    0.68

APPROACH                                             WB

Cycle Length, C                                        65

Actual Green Time for Lane Group, G                    30

Effective Green Time for Lane Group, g                 31

Opposing Effective Green Time, go                      31

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        2

Adjusted Left-Turn Flow Rate, Vlt                      29

Proportion of Left Turns in Lane Group, Plt             0.04

Left Turns per Cycle: LTC=Vlt*C/3600                    0.52

Adjusted Opposing Flow Rate, Vo                       816

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      7.37

Opposing Platoon Ratio, Rpo                             1.33

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                           14.23

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.37

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       4.71

gu=g-gq (or g-gf)                                      16.77

fs=(875-0.625Vo)/1000                                   0.37

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           0.14

El1                                                     6.80

fmin                                                    0.07

fm,(min=fmin;max=1.00)                                  0.76

flt=[fm+0.91(N-1)]/N                                    0.83

APPROACH                                             NB

Cycle Length, C                                        65

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.07

Proportion of Left Turns in Opposing Flow, Plto         0.07

Adjusted Left-Turn Flow Rate, Vlt                      32

Left Turns per Cycle: LTC=Vlt*C/3600                    0.58

Adjusted Opposing Flow Rate, Vo                       623

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     11.25

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           11.68

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.57

gq=4.943volc^0.762*qro^1.061-tl                        14.19

gu=g-gq (or g-gf)                                      13.81

n=(gq-gf)/2                                             1.25

Ptho=1-Plto                                             0.93

El2                                                     1.24

El1                                                     7.59

fmin=2(1+Pl)/g                                          0.08

flt=fm,(min=fmin; max=1)                                0.84

APPROACH                                             SB

Cycle Length, C                                        65

Actual Green Time for Lane Group, G                    27

Effective Green Time for Lane Group, g                 28

Opposing Effective Green Time, go                      28

Number of Lanes in Lane Group, N                        1

Proportion of Left Turns in Lane Group, Plt             0.07

Proportion of Left Turns in Opposing Flow, Plto         0.07

Adjusted Left-Turn Flow Rate, Vlt                      41

Left Turns per Cycle: LTC=Vlt*C/3600                    0.74

Adjusted Opposing Flow Rate, Vo                       444

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      8.02

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.860*LTC^0.629)-tl                           10.25

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.57

gq=4.943volc^0.762*qro^1.061-tl                        10.28

gu=g-gq (or g-gf)                                      17.72

n=(gq-gf)/2                                             0.01

Ptho=1-Plto                                             0.93

El2                                                     0.02

El1                                                     4.00

fmin=2(1+Pl)/g                                          0.08

flt=fm,(min=fmin; max=1)                                0.90

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

LTR        816       2215    0.368     0.477       1056    0.772 *

WB

LTR        837       2701    0.310     0.477       1288    0.650

NB

LTR        444       1299    0.342     0.431        560    0.793

SB

LTR        623       1377    0.452     0.431        593    1.050 *

Sum (v/s) critical = 0.821

Lost Time/Cycle, L =   6.0 sec   Critical v/c(x)    = 0.904

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

LTR     0.772 0.477  10.7 0.801  1056   12    1.9  10.5  B  10.5  B

WB

LTR     0.650 0.477   9.8 1.212  1288   12    0.6  12.5  B  12.5  B

NB

LTR     0.793 0.431  12.2 1.000   560   16    5.3  17.5  C  17.5  C

SB

LTR     1.050 0.431  14.1 1.000   593   16   43.1  57.2  E  57.2  E

Intersection Delay =  22.9 sec/veh Intersection LOS = C

-----------------------------------------------------------------------

Even though the SB approach is at capacity, the overall intersection level of service is C. Therefore, the existing design is acceptable.

9.6

The following is the Highway Capacity Software (HCS) solution to the exercise.  This is one suitable three-phase signal plan.  This plan is acceptable without any improvements to the given geometry.

HCS:  Signalized Intersection    Version 2.4a             08-08-1996  1

=======================================================================

Streets: (E-W) Pine Blvd.              (N-S) Oak St.

Analyst:                               File Name: Exerc_06.HC9

Area Type: Other                       7-30-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    2       |      2  <    |    > 2  <    |

Volumes    | 100  900     |      650  100|  35  700   25|

PHF or PK15|0.95 0.95     |     0.95 0.95|0.95 0.95 0.95|

Lane Width |12.0 12.0     |     12.0     |     12.0     |

Grade      |       -1     |        1     |        0     |

% Heavy Veh|  10   10     |       10   10|   5    5    5|

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) Y    10 |

Bus Stops  |            10|            10|             0|

Con. Peds  |             0|            30|            30|            0

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |

Arr Type   |   3    3     |        3     |        3     |

RTOR Vols  |             0|             0|             0|

Lost Time  |3.00 3.00     |     3.00 3.00|3.00 3.00 3.00|

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *     *             |NB  Left    *

Thru          *     *             |    Thru    *

Right                             |    Right   *

Peds                              |    Peds

|

WB  Left                              |SB  Left

Thru                *             |    Thru

Right               *             |    Right

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          10.0P 30.0P            |Green    26.0P

Yellow/AR       4.0   4.0             |Yellow/AR 4.0

Cycle Length:  78 sec  Phase combination order: #1 #2 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left   100  0.95   105   L     105  1  1.000   1.000   105  1.00  0.00

Thru   900  0.95   947   T     947  2  1.050   1.000   994  0.00  0.00

WB

Thru   650  0.95   684   TR    789  2  1.050   1.000   828  0.00  0.13

Right  100  0.95   105

NB

Left    35  0.95    37

Thru   700  0.95   737   LTR   800  2  1.050   1.000   840  0.05  0.03

Right   25  0.95    26

SB

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB                                                            0.12   204

L      1900   1  1.00  0.91  1.00  1.00  1.00  1.00  1.00  0.95  1649

T      1900   2  1.00  0.91  1.00  1.00  0.98  1.00  1.00  1.00  3402

WB

TR     1900   2  1.00  0.91  1.00  1.00  0.98  1.00  0.98  1.00  3301

NB

LTR    1900   2  1.00  0.95  1.00  0.93  1.00  1.00  1.00  1.00  3324

SB

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             EB

Cycle Length, C                                        78

Actual Green Time for Lane Group, G                    44

Effective Green Time for Lane Group, g                 34

Opposing Effective Green Time, go                      31

Number of Opposing Lanes, No                            2

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                     105

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    2.27

Adjusted Opposing Flow Rate, Vo                       828

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      8.97

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 0

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.60

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      14.04

gu=g-gq (or g-gf)                                      19.96

fs=(875-0.625Vo)/1000                                   0.36

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     6.22

fmin                                                    0.12

fm,(min=fmin;max=1.00)                                  0.12

flt=[fm+0.91(N-1)]/N                                    0.12

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB Lsec.        0        204    0.000     0.436         92    0.000

Lpri.      105       1649    0.064     0.141        233    0.451 *

Ltot.      105                                      325    0.323

T          994       3402    0.292     0.577       1963    0.506

WB

TR         828       3301    0.251     0.397       1312    0.631 *

NB

LTR        840       3324    0.253     0.346       1151    0.730 *

SB

Sum (v/s) critical = 0.567

Lost Time/Cycle, L =   9.0 sec   Critical v/c(x)    = 0.641

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       0.323 0.321   7.0 1.000   325   16    0.2   7.3  B   7.6  B

T       0.506 0.577   7.5 1.000  1963   16    0.2   7.7  B

WB

TR      0.631 0.397  14.4 1.000  1312   16    0.7  15.1  C  15.1  C

NB

LTR     0.730 0.346  17.0 1.000  1151   16    1.7  18.6  C  18.6  C

SB

Intersection Delay =  13.2 sec/veh Intersection LOS = B

=======================================================================

Supplemental Uniform Delay Worksheet

Approach                                                   Eastbound

Adj. LT Vol  (v)                                           105

v/c ratio  (x)                                               0.32

Primary phase effective green                               11.00

gq from Supplemental LT Worksheet                           14.04

gu from Supplemental LT Worksheet                           19.96

Red time  (r)                                               33.00

Arrivals qa = v/(3600(max(x,1)                               0.03

Primary Ph. Departures Sp=s/3600                             0.46

Secondary Ph. Departures Ss=S(Gq+Gu)/(Gu*3600)               0.10

Xperm                                                        0.51

Xprot                                                        0.25

Case                                                         1

Queue at beginning of green arrow (Qa)                       0.96

Queue at beginning of unsaturated green (Qu)                 0.41

Residual queue (Qr)                                          0.00

Uniform Delay                                                7.04

9.7

The following is the Highway Capacity Software (HCS) solution to the exercise.

HCS:  Signalized Intersection    Version 2.4a             08-16-1996  1

=======================================================================

Streets: (E-W) Main St.                (N-S) Johnson St.

Analyst:                               File Name: Exerc_07.HC9

Area Type: Other                       8-5-96

Comment: Problem 7

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    2  <    | 1    2  <    | 1    2  <    | 1    2  <

Volumes    |  50  250   75|  75  500   25| 100 1250   50| 125  750   50

PHF or PK15|0.85 0.85 0.85|0.85 0.85 0.85|0.85 0.85 0.85|0.85 0.85 0.85

Lane Width |11.0 11.0     |11.0 11.0     |11.0 11.0     |11.0 11.0

Grade      |        0     |        0     |        0     |        0

% Heavy Veh|   5    5    5|   5    5    5|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |           100|           100|           100|           100

Ped Button |(Y/N) Y 20.0 s|(Y/N) Y 20.0 s|(Y/N) Y 20.0 s|(Y/N) Y 20.0

Arr Type   |   3    3     |   3    3     |   3    3     |   3    3

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru                *             |    Thru          *

Right               *             |    Right         *

Peds                *             |    Peds    *

|

WB  Left          *                   |SB  Left    *

Thru                *             |    Thru          *

Right               *             |    Right         *

Peds                *             |    Peds    *

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green           7.0P 19.0P            |Green    18.0P 45.0P

Yellow/AR       4.0   4.0             |Yellow/AR 4.0   4.0

Cycle Length: 105 sec  Phase combination order: #1 #2 #5 #6

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left    50  0.85    59   L      59  1  1.000   1.000    59  1.00  0.00

Thru   250  0.85   294   TR    382  2  1.050   1.000   401  0.00  0.23

Right   75  0.85    88

WB

Left    75  0.85    88   L      88  1  1.000   1.000    88  1.00  0.00

Thru   500  0.85   588   TR    617  2  1.050   1.000   648  0.00  0.05

Right   25  0.85    29

NB

Left   100  0.85   118   L     118  1  1.000   1.000   118  1.00  0.00

Thru  1250  0.85  1471   TR   1530  2  1.050   1.000  1606  0.00  0.04

Right   50  0.85    59

SB

Left   125  0.85   147   L     147  1  1.000   1.000   147  1.00  0.00

Thru   750  0.85   882   TR    941  2  1.050   1.000   988  0.00  0.06

Right   50  0.85    59

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

L      1900   1  0.97  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1662

TR     1900   2  0.97  0.95  1.00  1.00  1.00  1.00  0.95  1.00  3339

WB

L      1900   1  0.97  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1662

TR     1900   2  0.97  0.95  1.00  1.00  1.00  1.00  0.99  1.00  3466

NB

L      1900   1  0.97  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1662

TR     1900   2  0.97  0.95  1.00  1.00  1.00  1.00  0.99  1.00  3478

SB

L      1900   1  0.97  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1662

TR     1900   2  0.97  0.95  1.00  1.00  1.00  1.00  0.99  1.00  3466

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

L           59       1662    0.035     0.076        127    0.466

TR         401       3339    0.120     0.190        636    0.631

WB

L           88       1662    0.053     0.076        127    0.695 *

TR         648       3466    0.187     0.190        660    0.982 *

NB

L          118       1662    0.071     0.181        301    0.392

TR        1606       3478    0.462     0.438       1524    1.054 *

SB

L          147       1662    0.088     0.181        301    0.489 *

TR         988       3466    0.285     0.438       1518    0.651

Sum (v/s) critical = 0.790

Lost Time/Cycle, L =  12.0 sec   Critical v/c(x)    = 0.892

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       0.466 0.076  35.3 1.000   127   16    2.0  37.3  D  31.9  D

TR      0.631 0.190  29.7 1.000   636   16    1.4  31.2  D

WB

L       0.695 0.076  36.0 1.000   127   16   10.0  46.0  E  53.9  E

TR      0.982 0.190  32.2 1.000   660   16   22.8  55.0  E

NB

L       0.392 0.181  28.8 1.000   301   16    0.5  29.3  D  53.7  E

TR      1.054 0.438  22.4 1.000  1524   16   33.1  55.5  E

SB

L       0.489 0.181  29.4 1.000   301   16    1.0  30.4  D  19.9  C

TR      0.651 0.438  17.6 1.000  1518   16    0.7  18.3  C

Intersection Delay =  41.8 sec/veh Intersection LOS = E

-----------------------------------------------------------------------

9.8

The following is the Highway Capacity Software (HCS) solution to the exercise.  This is one suitable intersection layout and phasing sequence that provides a condition of under capacity.

HIGHWAY CAPACITY MANUAL SIGNALIZED INTERSECTION PLANNING METHOD

LANE VOLUME WORKSHEET

File name:    Date:   8 -  2 - 19 96  Time Period:

(E/W):                      (N/S):                      Analyst :

Peak hour factor:  .9  Comment: Exercise 08

EAST   WEST   NORTH  SOUTH

BOUND  BOUND  BOUND  BOUND

LEFT TURN MOVEMENT

1. LT volume                                    100    125    150    200

2. Opposing mainline volume                     645    700    1200   1150

3. Number of exclusive LT lanes                 0      0      1      1

Cross Product [2] * [1]                         64500  87500  180000 230000

Left Lane Configuration (E=Excl, S=Shrd):        S      S      E      E

Left Turn Treatment Type:                       Perm   Perm   Perm   Perm

4. LT adjustment factor                         N/A    N/A    1.0    1.0

5. LT lane vol                                  N/A    N/A    N/A    N/A

RIGHT TURN MOVEMENT

Right Lane Configuration (E=Excl, S=Shrd)       S      S      S      S

6. RT volume                                    100    120    250    200

7. Exclusive lanes                              N/A    N/A    N/A    N/A

8. RT adjustment factor                         .85    .85    .85    .85

9. Exclusive RT lane volume                     0      0      0      0

10. Shared lane vol                              118    141    294    235

THROUGH MOVEMENT

11. Thru volume                                  500    400    900    1000

12. Parking adjustment factor                    1      1      1      1

13. No. of thru lanes including shared           2      2      2      2

14. Total approach volume                        618    541    1194   1235

15. Prop. of left turns in lane group            N/A    N/A    0      0

16. Left turn equivalence                        4.14   4.8    10.4   9.35

17. LT adj. factor:                              .63    .57    N/A    N/A

18. Through lane volume                          494    477    597    618

19. Critical lane volume                         494    477    597    618

Left Turn Check (if [16] > 8)

20. Permitted left turn sneaker capacity: 7200/Cmax

N/A    N/A    60     60

HIGHWAY CAPACITY MANUAL SIGNALIZED INTERSECTION PLANNING METHOD

SIGNAL OPERATIONS WORKSHEET

File name:    Date:   8 -  2 - 19 96  Time Period:

(E/W):                      (N/S):                      Analyst :

EAST   WEST   NORTH  SOUTH

BOUND  BOUND  BOUND  BOUND

Phase Plan Selection from Lane Volume Worksheet

Critical through-RT vol: [19]                       494    477    597    618

LT lane vol: [5]                                    N/A    N/A    N/A    N/A

Left turn protection: (P/U/N)                        U      U      U      U

Dominant left turn: (Indicate by '*')                       *             *

Selection Criteria based on the          Plan 1:   U      U      U      U

specified left turn protection           Plan 2a:  U      P      U      P

Plan 2b:  P      U      P      U

* Indicates the dominant left turn       Plan 3a: *P      P     *P      P

for each opposing pair                   Plan 3b:  P     *P      P     *P

Plan 4:   N      N      N      N

1             1

Min. cycle (Cmin)    60           Max. cycle (Cmax)    120

Timing Plan                           --- EAST-WEST ----   -- NORTH-SOUTH ---

Value      Ph 1   Ph 2   Ph 3   Ph 1   Ph 2   Ph 3

Movement codes                        EWG                  NSG

Critical phase vol [CV]               494    0      0      618    0      0

Critical sum [CS]          1112

CBD adjustment [CBD]       .9

Reference sum [RS]         1385

Lost time/phase [PL]                  3      0      0      3      0      0

Lost time/cycle [TL]       6

Cycle length [CYC]         60

Green time                            27     0      0      33     0      0

Critical v/c ratio [Xcm]   0.76

Status                     Under capacity.

9.9

Refer to the solution to Exercise 4.  A critical V/c ratio of 0.90 or lower is not achieved.  The overall intersection LOS is D.

9.10

a.)  2-phase signal, pretimed

E-W flow ratio, V/s = 0.50

N-S flow ratio, V/s = 0.25

Lost time = change interval = 4 sec/phase (8 sec/cycle)

The minimum cycle length is computed using Xc = 1.00

Cmin=LXcXc-åV/sci=8(1.0)1.0−(0.5+0.25)=32 sec

If a V/c of no more than 0.8 were desired, then X = 0.8

C=8(0.8)0.8−(0.5+0.25)=128 sec  (cycle too long)

Try X = 0.85

C=8(0.85)0.85−(0.5+0.25)=68 sec » 70 sec  (rounding to nearest 5 sec)

The actual V/c ratio provided by the 70-sec cycle is

Xc=iVsciCC−L=(0.5+0.25)7070−8=0.85

1.)  If the greens are allocated so that the V/c ratios for critical movements in each phase are equal, then

gi=VsiCXig1=0.5700.85=41 secg2=0.25700.85=21 sec

Total = 62 sec

2.)  If the minimum green time is assigned to the minor approach, and the balance to the major approach, then

g2=0.25701.0=17.5 sec

g1 = 70 - 8 - 17.5 = 44.5 sec

Total = 62 sec

- 2-phase semi-actuated signal

E-W flow ratio, V/s = 0.65

N-S flow ratio, V/s = 0.10

Designed to operate at critical V/c = 0.85

Lost time = 9 sec/cycle

C=9(0.85)0.85−(0.65+0.1)=76.5 sec

Because the signal is semi-actuated, the cycle length is not rounded off.

The green time for the side street is estimated using V/c = 1.0

g2=0.176.51.0=7.65 sec

g1 = 76.5 - 9 - 7.65 = 59.85 sec

9.11

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

Center For Microcomputers In Transportation

HCS: Unsignalized Intersection   Release 2.1              Page 1

****************************************************************

File Name ................ Exerc_11.HC0

Streets: (N-S) Molalla Ave.              (E-W) Main St.

Analyst...................

Date of Analysis.......... 7/19/96

Other Information.........

All-way Stop-controlled Intersection

========================================================================

|  Eastbound   |  Westbound   |   Northbound |  Southbound

|   L    T    R|   L    T    R|   L    T    R|   L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  |   0>   2<   0|   0>   2<   0|   0>   1<   0|   0>   1<   0

Volumes    |  50  325   75|  50  225   75|  45  245   60|  35  225   50

PHF        | .95  .95  .95| .95  .95  .95| .95  .95  .95| .95  .95  .95

Grade      |        0     |        0     |        0     |        0

MC's*(%)   |   0    0    0|   0    0    0|   0    0    0|   0    0    0

SU/RV's (%)|   0    0    0|   0    0    0|   0    0    0|   0    0    0

CV's*(%)   |   0    0    0|   0    0    0|   0    0    0|   0    0    0

PCE's      | 1.1  1.1  1.1| 1.1  1.1  1.1| 1.1  1.1  1.1| 1.1  1.1  1.1

*MC’s Motor Cycles		CV’s Combination Vehicles

Volume Summary and Capacity Analysis Worksheet

-----------------------------------------------------------------------

EB      WB      NB      SB

-----------------------------------------------------------------------

LT Flow Rate                                 53      53      47      37

RT Flow Rate                                 79      79      63      53

Approach Flow Rate                          474     369     368     327

Proportion LT                              0.11    0.14    0.13    0.11

Proportion RT                              0.17    0.21    0.17    0.16

Opposing Approach Flow Rate                 369     474     327     368

Conflicting Approaches Flow Rate            695     695     843     843

Proportion, Subject Approach Flow Rate     0.31    0.24    0.24    0.21

Proportion, Opposing Approach Flow Rate    0.24    0.31    0.21    0.24

Lanes on Subject Approach                     2       2       1       1

Lanes on Opposing Approach                    2       2       1       1

LT, Opposing Approach                        53      53      37      47

RT, Opposing Approach                        79      79      53      63

LT, Conflicting Approaches                   84      84     106     106

RT, Conflicting Approaches                  116     116     158     158

Proportion LT, Opposing Approach           0.14    0.11    0.11    0.13

Proportion RT, Opposing Approach           0.21    0.17    0.16    0.17

Proportion LT, Conflicting Approaches      0.12    0.12    0.13    0.13

Proportion RT, Conflicting Approaches      0.17    0.17    0.19    0.19

Approach Capacity                           690     669     505     495

****************************************************************

Intersection Performance Summary

Approach     Approach     V/C      Average

Movement     Flow Rate    Capacity     Ratio     Total Delay     LOS

----------   ----------   ---------    -------    -----------     ----

EB           474            690      0.69           13.6         C

WB           369            669      0.55            8.1         B

NB           368            505      0.73           15.9         C

SB           327            495      0.66           12.3         C

Intersection Delay  =   12.58

Level of Service (Intersection) =  C

9.12

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-07-1996  1

=======================================================================

Streets: (E-W) Moss Avenue             (N-S) Ryder Street

Analyst:                               File Name: Exerc_12.HC9

Area Type: Other                       8-7-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    1    1  | 1    1    1  | 1    1    1  | 1    1    1

Volumes    | 105  450  105| 125  500   80| 100  560   90|  95  525   40

PHF or PK15|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90

Lane Width |12.0 12.0 12.0|12.0 12.0 12.0|12.0 12.0 12.0|12.0 12.0 12.0

Grade      |        0     |        0     |        0     |        0

% Heavy Veh|   3    3    3|   3    3    3|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |             0|             0|             0|             0

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |   3    3    3|   3    3    3|   5    5    5|   5    5    5

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru          *                   |    Thru    *

Right         *                   |    Right   *

Peds                              |    Peds

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          46.0P                  |Green    46.0P

Yellow/AR       4.0                   |Yellow/AR 4.0

Cycle Length: 100 sec  Phase combination order: #1 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left   105  0.90   117   L     117  1  1.000   1.000   117  1.00  0.00

Thru   450  0.90   500   T     500  1  1.000   1.000   500  0.00  0.00

Right  105  0.90   117   R     117  1  1.000   1.000   117  0.00  1.00

WB

Left   125  0.90   139   L     139  1  1.000   1.000   139  1.00  0.00

Thru   500  0.90   556   T     556  1  1.000   1.000   556  0.00  0.00

Right   80  0.90    89   R      89  1  1.000   1.000    89  0.00  1.00

NB

Left   100  0.90   111   L     111  1  1.000   1.000   111  1.00  0.00

Thru   560  0.90   622   T     622  1  1.000   1.000   622  0.00  0.00

Right   90  0.90   100   R     100  1  1.000   1.000   100  0.00  1.00

SB

Left    95  0.90   106   L     106  1  1.000   1.000   106  1.00  0.00

Thru   525  0.90   583   T     583  1  1.000   1.000   583  0.00  0.00

Right   40  0.90    44   R      44  1  1.000   1.000    44  0.00  1.00

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

L      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  0.13   244

T      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  1.00  1845

R      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  0.85  1.00  1568

WB

L      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  0.17   318

T      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  1.00  1845

R      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  0.85  1.00  1568

NB

L      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  0.16   297

T      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  1.00  1810

R      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  0.85  1.00  1538

SB

L      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  0.13   234

T      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  1.00  1810

R      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  0.85  1.00  1538

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             EB

Cycle Length, C                                        100

Actual Green Time for Lane Group, G                    46

Effective Green Time for Lane Group, g                 47

Opposing Effective Green Time, go                      47

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                     117

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    3.25

Adjusted Opposing Flow Rate, Vo                       556

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     15.44

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.53

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      20.69

gu=g-gq (or g-gf)                                      26.31

fs=(875-0.625Vo)/1000                                   0.53

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     4.24

fmin                                                    0.09

fm,(min=fmin;max=1.00)                                  0.13

flt=[fm+0.91(N-1)]/N                                    0.13

APPROACH                                             WB

Cycle Length, C                                       100

Actual Green Time for Lane Group, G                    46

Effective Green Time for Lane Group, g                 47

Opposing Effective Green Time, go                      47

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                     139

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    3.86

Adjusted Opposing Flow Rate, Vo                       500

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     13.89

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.53

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      17.38

gu=g-gq (or g-gf)                                      29.62

fs=(875-0.625Vo)/1000                                   0.56

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     3.65

fmin                                                    0.09

fm,(min=fmin;max=1.00)                                  0.17

flt=[fm+0.91(N-1)]/N                                    0.17

APPROACH                                             NB

Cycle Length, C                                       100

Actual Green Time for Lane Group, G                    46

Effective Green Time for Lane Group, g                 47

Opposing Effective Green Time, go                      47

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                     111

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    3.08

Adjusted Opposing Flow Rate, Vo                       583

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     16.19

Opposing Platoon Ratio, Rpo                             1.67

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.22

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      12.17

gu=g-gq (or g-gf)                                      34.83

fs=(875-0.625Vo)/1000                                   0.51

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     4.52

fmin                                                    0.09

fm,(min=fmin;max=1.00)                                  0.16

flt=[fm+0.91(N-1)]/N                                    0.16

APPROACH                                             SB

Cycle Length, C                                       100

Actual Green Time for Lane Group, G                    46

Effective Green Time for Lane Group, g                 47

Opposing Effective Green Time, go                      47

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                     106

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    2.94

Adjusted Opposing Flow Rate, Vo                       622

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No     17.28

Opposing Platoon Ratio, Rpo                             1.67

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.22

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl      14.58

gu=g-gq (or g-gf)                                      32.42

fs=(875-0.625Vo)/1000                                   0.49

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     5.33

fmin                                                    0.09

fm,(min=fmin;max=1.00)                                  0.13

flt=[fm+0.91(N-1)]/N                                    0.13

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

L          117        244    0.480     0.470        115    1.020 *

T          500       1845    0.271     0.470        867    0.577

R          117       1568    0.075     0.470        737    0.159

WB

L          139        318    0.437     0.470        149    0.930

T          556       1845    0.301     0.470        867    0.641

R           89       1568    0.057     0.470        737    0.121

NB

L          111        297    0.374     0.470        140    0.795

T          622       1810    0.344     0.470        851    0.731

R          100       1538    0.065     0.470        723    0.138

SB

L          106        234    0.453     0.470        110    0.964 *

T          583       1810    0.322     0.470        851    0.685

R           44       1538    0.029     0.470        723    0.061

Sum (v/s) critical = 0.932

Lost Time/Cycle, L =   6.0 sec   Critical v/c(x)    = 0.992

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       1.020 0.470  20.1 1.000   115   16   71.7  91.8  F  26.9  D

T       0.577 0.470  14.6 1.000   867   16    0.7  15.4  C

R       0.159 0.470  11.5 1.000   737   16    0.0  11.5  B

WB

L       0.930 0.470  19.0 1.000   149   16   37.9  56.9  E  23.0  C

T       0.641 0.470  15.3 1.000   867   16    1.1  16.4  C

R       0.121 0.470  11.3 1.000   737   16    0.0  11.3  B

NB

L       0.795 0.470  17.0 0.409   140    8   10.0  16.9  C   8.6  B

T       0.731 0.470  16.3 0.409   851    8    1.2   7.8  B

R       0.138 0.470  11.4 0.409   723    8    0.0   4.7  A

SB

L       0.964 0.470  19.5 0.409   110    8   37.1  45.1  E  12.6  B

T       0.685 0.470  15.7 0.409   851    8    0.8   7.3  B

R       0.061 0.470  11.0 0.409   723    8    0.0   4.5  A

Intersection Delay =  17.6 sec/veh Intersection LOS = C

-----------------------------------------------------------------------

9.13

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-07-1996  1

=======================================================================

Streets: (E-W) Moss Avenue             (N-S) Ryder Street

Analyst:                               File Name: Exerc_13.HC9

Area Type: Other                       8-7-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    1    1  | 1    1    1  | 1    1    1  | 1    1    1

Volumes    | 105  450  105| 125  500   80| 100  560   90|  95  525   40

PHF or PK15|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90|0.90 0.90 0.90

Lane Width |12.0 12.0 12.0|12.0 12.0 12.0|12.0 12.0 12.0|12.0 12.0 12.0

Grade      |        0     |        0     |        0     |        0

% Heavy Veh|   3    3    3|   3    3    3|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Bus Stops  |             0|             0|             0|             0

Con. Peds  |             0|             0|             0|             0

Ped Button |(Y/N) N       |(Y/N) N       |(Y/N) N       |(Y/N) N

Arr Type   |   3    3    3|   3    3    3|   5    5    5|   5    5    5

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru                *             |    Thru          *

Right               *             |    Right         *

Peds                              |    Peds

|

WB  Left          *                   |SB  Left    *

Thru                *             |    Thru          *

Right               *             |    Right         *

Peds                              |    Peds

|

NB  Right         *                   |EB  Right   *

SB  Right         *                   |WB  Right   *

|

Green           7.0P 32.0P            |Green     7.0P 38.0P

Yellow/AR       4.0   4.0             |Yellow/AR 4.0   4.0

Cycle Length: 100 sec  Phase combination order: #1 #2 #5 #6

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left   105  0.90   117   L     117  1  1.000   1.000   117  1.00  0.00

Thru   450  0.90   500   T     500  1  1.000   1.000   500  0.00  0.00

Right  105  0.90   117   R     117  1  1.000   1.000   117  0.00  1.00

WB

Left   125  0.90   139   L     139  1  1.000   1.000   139  1.00  0.00

Thru   500  0.90   556   T     556  1  1.000   1.000   556  0.00  0.00

Right   80  0.90    89   R      89  1  1.000   1.000    89  0.00  1.00

NB

Left   100  0.90   111   L     111  1  1.000   1.000   111  1.00  0.00

Thru   560  0.90   622   T     622  1  1.000   1.000   622  0.00  0.00

Right   90  0.90   100   R     100  1  1.000   1.000   100  0.00  1.00

SB

Left    95  0.90   106   L     106  1  1.000   1.000   106  1.00  0.00

Thru   525  0.90   583   T     583  1  1.000   1.000   583  0.00  0.00

Right   40  0.90    44   R      44  1  1.000   1.000    44  0.00  1.00

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

L      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  0.95  1752

T      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  1.00  1845

R      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  0.85  1.00  1568

WB

L      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  0.95  1752

T      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  1.00  1.00  1845

R      1900   1  1.00  0.97  1.00  1.00  1.00  1.00  0.85  1.00  1568

NB

L      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1719

T      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  1.00  1810

R      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  0.85  1.00  1538

SB

L      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  0.95  1719

T      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  1.00  1.00  1810

R      1900   1  1.00  0.95  1.00  1.00  1.00  1.00  0.85  1.00  1538

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

L          117       1752    0.067     0.080        140    0.835

T          500       1845    0.271     0.330        609    0.821

R          117       1568    0.075     0.440        690    0.170

WB

L          139       1752    0.079     0.080        140    0.992 *

T          556       1845    0.301     0.330        609    0.913 *

R           89       1568    0.057     0.440        690    0.129

NB

L          111       1719    0.065     0.080        138    0.807 *

T          622       1810    0.344     0.390        706    0.881 *

R          100       1538    0.065     0.500        769    0.130

SB

L          106       1719    0.062     0.080        138    0.771

T          583       1810    0.322     0.390        706    0.826

R           44       1538    0.029     0.500        769    0.057

Sum (v/s) critical = 0.789

Lost Time/Cycle, L =  12.0 sec   Critical v/c(x)    = 0.896

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       0.835 0.080  34.5 1.000   140   16   22.3  56.8  E  31.2  D

T       0.821 0.330  23.4 1.000   609   16    6.1  29.5  D

R       0.170 0.440  12.9 1.000   690   16    0.0  12.9  B

WB

L       0.992 0.080  34.9 1.000   140   16   55.9  90.8  F  44.1  E

T       0.913 0.330  24.4 1.000   609   16   13.1  37.5  D

R       0.129 0.440  12.6 1.000   690   16    0.0  12.6  B

NB

L       0.807 0.080  34.4 0.942   138    8   11.0  43.3  E  19.1  C

T       0.881 0.390  21.5 0.574   706    8    4.9  17.2  C

R       0.130 0.500  10.2 0.333   769    8    0.0   3.4  A

SB

L       0.771 0.080  34.3 0.942   138    8    8.5  40.8  E  18.0  C

T       0.826 0.390  20.9 0.574   706    8    3.0  14.9  B

R       0.057 0.500   9.8 0.333   769    8    0.0   3.3  A

Intersection Delay =  28.1 sec/veh Intersection LOS = D

-----------------------------------------------------------------------

9.14

The following is the Highway Capacity Software (HCS) solution to the exercise.  The specific answers requested in the problem statement are highlighted in bold.

HCS:  Signalized Intersection    Version 2.4a             08-07-1996  1

=======================================================================

Streets: (E-W) 5th Avenue              (N-S) James Street

Analyst:                               File Name: Exerc_14.HC9

Area Type: Other                       8-7-96

=======================================================================

Traffic and Roadway Conditions

-----------------------------------------------------------------------

|   Eastbound  |  Westbound   |  Northbound  |  Southbound

| L    T    R  | L    T    R  | L    T    R  | L    T    R

|---- ---- ----|---- ---- ----|---- ---- ----|---- ---- ----

No. Lanes  | 1    2  <    | 1    2    1  | 1    1  <    | 1    1    1

Volumes    | 175  980   75| 140 1020  205|  75  245   25|  50  160   95

PHF or PK15|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95|0.95 0.95 0.95

Lane Width |11.0 11.0     |11.0 11.0 11.0|11.0 11.0     |10.0 10.0 10.0

Grade      |        2     |       -2     |        0     |        0

% Heavy Veh|   2    2    2|   2    2    2|   5    5    5|   5    5    5

Parking    |(Y/N) N       |(Y/N) N       |(Y/N) Y    40 |(Y/N) Y    25

Bus Stops  |             4|             2|             0|             0

Con. Peds  |            50|            50|            50|            50

Ped Button |(Y/N) Y 15.0 s|(Y/N) Y 15.0 s|(Y/N) Y 20.0 s|(Y/N) Y 20.0

Arr Type   |   4    4     |   4    4    4|   3    3     |   3    3    3

RTOR Vols  |             0|             0|             0|             0

Lost Time  |3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00|3.00 3.00 3.00

-----------------------------------------------------------------------

Signal Operations

-----------------------------------------------------------------------

Phase Combination 1     2     3     4 |            5     6     7     8

EB  Left          *                   |NB  Left    *

Thru                *             |    Thru    *

Right               *             |    Right   *

Peds                *             |    Peds    *

|

WB  Left          *                   |SB  Left    *

Thru                *             |    Thru    *

Right               *             |    Right   *

Peds                *             |    Peds    *

|

NB  Right                             |EB  Right

SB  Right                             |WB  Right

|

Green          10.0P 44.0P            |Green    24.0P

Yellow/AR       4.0   4.0             |Yellow/AR 4.0

Cycle Length:  90 sec  Phase combination order: #1 #2 #5

=======================================================================

Volume Adjustment Worksheet

-----------------------------------------------------------------------

Direc-                        Lane     Lane           Adj

tion/ Mvt         Adj   Lane  Grp  No. Util   Growth  Grp   Prop  Prop

Mvt   Vol   PHF   Vol   Grp   Vol  Ln  Fact   Fact    Vol    LT    RT

----- ----  ----  ----  ----  ---- --  -----  ------  ----  ----  ----

EB

Left   175  0.95   184   L     184  1  1.000   1.000   184  1.00  0.00

Thru   980  0.95  1032   TR   1111  2  1.050   1.000  1167  0.00  0.07

Right   75  0.95    79

WB

Left   140  0.95   147   L     147  1  1.000   1.000   147  1.00  0.00

Thru  1020  0.95  1074   T    1074  2  1.050   1.000  1128  0.00  0.00

Right  205  0.95   216   R     216  1  1.000   1.000   216  0.00  1.00

NB

Left    75  0.95    79   L      79  1  1.000   1.000    79  1.00  0.00

Thru   245  0.95   258   TR    284  1  1.000   1.000   284  0.00  0.09

Right   25  0.95    26

SB

Left    50  0.95    53   L      53  1  1.000   1.000    53  1.00  0.00

Thru   160  0.95   168   T     168  1  1.000   1.000   168  0.00  0.00

Right   95  0.95   100   R     100  1  1.000   1.000   100  0.00  1.00

-----------------------------------------------------------------------

Saturation Flow Adjustment Worksheet

-----------------------------------------------------------------------

Ideal                                                     Adj

Direction Sat   No.  f     f     f     f     f     f     f     f    Sat

/LnGrp    Flow  Lns   W     HV    G     p     BB    A     RT    LT  Flow

------    ----  --- ----- ----- ----- ----- ----- ----- ----- ----- ----

EB

L      1900   1  0.97  0.98  0.99  1.00  1.00  1.00  1.00  0.95  1694

TR     1900   2  0.97  0.98  0.99  1.00  0.99  1.00  0.99  1.00  3494

WB

L      1900   1  0.97  0.98  1.01  1.00  1.00  1.00  1.00  0.95  1728

T      1900   2  0.97  0.98  1.01  1.00  1.00  1.00  1.00  1.00  3637

R      1900   1  0.97  0.98  1.01  1.00  1.00  1.00  0.83  1.00  1497

NB

L      1900   1  0.97  0.95  1.00  1.00  1.00  1.00  1.00  0.51   887

TR     1900   1  0.97  0.95  1.00  0.70  1.00  1.00  0.98  1.00  1205

SB

L      1900   1  0.93  0.95  1.00  1.00  1.00  1.00  1.00  0.26   440

T      1900   1  0.93  0.95  1.00  1.00  1.00  1.00  1.00  1.00  1689

R      1900   1  0.93  0.95  1.00  0.77  1.00  1.00  0.83  1.00  1081

=======================================================================

Supplemental Permitted LT Worksheet

APPROACH                                             NB

Cycle Length, C                                        90

Actual Green Time for Lane Group, G                    24

Effective Green Time for Lane Group, g                 25

Opposing Effective Green Time, go                      25

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                      79

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    1.98

Adjusted Opposing Flow Rate, Vo                       168

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      4.20

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.72

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       3.69

gu=g-gq (or g-gf)                                      21.31

fs=(875-0.625Vo)/1000                                   0.77

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     1.68

fmin                                                    0.16

fm,(min=fmin;max=1.00)                                  0.51

flt=[fm+0.91(N-1)]/N                                    0.51

APPROACH                                             SB

Cycle Length, C                                        90

Actual Green Time for Lane Group, G                    24

Effective Green Time for Lane Group, g                 25

Opposing Effective Green Time, go                      25

Number of Opposing Lanes, No                            1

Number of Lanes in Lane Group, N                        1

Adjusted Left-Turn Flow Rate, Vlt                      53

Proportion of Left Turns in Lane Group, Plt             1.00

Left Turns per Cycle: LTC=Vlt*C/3600                    1.33

Adjusted Opposing Flow Rate, Vo                       284

Opposing Flow per Lane, Per Cycle: Volc=VoC/3600No      7.10

Opposing Platoon Ratio, Rpo                             1

Lost time per phase, tl                                 3

gf=Gexp(-0.882*LTC^0.717)-tl                            0.00

Opposing Queue Ratio: qro=1-Rpo(go/C)                   0.72

gq = Volc * qro / (.5 - Volc * (1 - qro) / go)-tl       9.18

gu=g-gq (or g-gf)                                      15.82

fs=(875-0.625Vo)/1000                                   0.70

Pl=Plt[1+{(N-1)g/(fsgu+4.5)}]                           1.00

El1                                                     2.43

fmin                                                    0.16

fm,(min=fmin;max=1.00)                                  0.26

flt=[fm+0.91(N-1)]/N                                    0.26

=======================================================================

Capacity Analysis Worksheet

-----------------------------------------------------------------------

Adj     Adj  Sat   Flow               Lane Group

Direction Flow Rate  Flow Rate  Ratio  Green Ratio  Capacity   v/c

/LnGrp       (v)        (s)     (v/s)     (g/C)        (c)    Ratio

--------- ---------  ---------  -----  ----------- ---------- -----

EB

L          184       1694    0.109     0.122        207    0.889 *

TR        1167       3494    0.334     0.500       1747    0.668 *

WB

L          147       1728    0.085     0.122        211    0.696

T         1128       3637    0.310     0.500       1818    0.620

R          216       1497    0.144     0.500        748    0.289

NB

L           79        887    0.089     0.278        246    0.321

TR         284       1205    0.236     0.278        335    0.848 *

SB

L           53        440    0.120     0.278        122    0.434

T          168       1689    0.099     0.278        469    0.358

R          100       1081    0.093     0.278        300    0.333

Sum (v/s) critical = 0.678

Lost Time/Cycle, L =   9.0 sec   Critical v/c(x)    = 0.754

-----------------------------------------------------------------------

Level of Service Worksheet

-----------------------------------------------------------------------

Delay Del   Lane  Calib Delay Lane Lane Delay LOS

Direction  v/c   g/C    d    Adj   Group  d     d    Grp  Grp  By    By

/LnGrp    Ratio Ratio    1   Fact  Cap     2     2   Del  LOS  App   App

--------- ----- -----  ----- ----- ----- ----- ----- ---- ---- ----- ---

EB

L       0.889 0.122  29.6 1.000   207   12   19.3  48.9  E  15.6  C

TR      0.668 0.500  12.8 0.767  1747   12    0.5  10.4  B

WB

L       0.696 0.122  28.8 1.000   211   12    5.0  33.8  D  11.9  B

T       0.620 0.500  12.4 0.767  1818   12    0.4   9.9  B

R       0.289 0.500  10.0 0.767   748   12    0.0   7.7  B

NB

L       0.321 0.278  19.6 1.000   246   16    0.3  19.9  C  32.4  D

TR      0.848 0.278  23.3 1.000   335   16   12.5  35.9  D

SB

L       0.434 0.278  20.3 1.000   122   16    1.6  21.8  C  20.3  C

T       0.358 0.278  19.8 1.000   469   16    0.2  20.0  C

R       0.333 0.278  19.7 1.000   300   16    0.3  19.9  C

Intersection Delay =  16.2 sec/veh Intersection LOS = C

-----------------------------------------------------------------------