** TAKING 35mm  SLIDES OFF THE AMIGA SCREEN **

Contributed by Robert Goldberg, M.D.  <EYEDOC>

I have shot some two dozen rolls of film now off the Amiga screen, and

I wanted to share my experience using various techniques.  My current setup

produces spectacular color slides which project very well, even onto a large

screen.  I personally use the slides to illustrate medical lectures; anyone

who needs to make presentations or promotions of any kind should try using

their Amiga to produce a set of lecture slides.  Amiga's striking graphics

are absolutely stunning when projected on a large screen and, with a little

care exercised in the production, you ought to be able to completely dazzle

your audience (especially if they have only been exposed to the somnolent

graphics produced by Amiga's competition!)  I'm sure that Amiga slides have

abundant applications in other areas as well, such as advertising, art, and

so forth.

After a great deal of experimentation I have arrived at the following

setup for slide production.  As with anything, there are endless ways to

accomplish the same goal, and your own tastes and equipment will dictate any

necessary modifications to these recommendations.  The important thing is to

refine your setup to the point that it is simple and repeatable.

- More (Y/N/NS)? ns

-- CAMERA --

Any 35mm single lens reflex will do fine.  I use a Nikon N2000.  A

sturdy tripod is key, although I suppose an extremely patient individual

could use a table with a bunch of books or something to provide a platform

for the camera.  You need to be able to manipulate the camera position very

precisely, though, as I will explain below, and a tripod is the only

efficient way to do this.

Also, a shutter release will make your life much easier since the

exposures tend to be long, and directly pressing the camera button can

sometimes induce unacceptable vibration; if you flat refuse to get a shutter

release, a good poor man's trick is to use the camera's self timer so that

you can keep your hands off.

-- LENS --

Your computer screen is not flat.  The curvature of the screen can

induce distortion; this is particularly noticeable when your graphics

include a large rectangular border near the edge of the screen.  This

curvature effect can be minimized (not eliminated) by using a lens of

moderately long focal length.  I have not noticed much improvement with

longer focal length than 150mm; at very long focal lenghts, the camera ends

up inconveniently far from the screen and also more sensitive to vibration.

I would recommend a lens in the 120mm to 200mm range.

If you want to take shots of less than the full screen, you will need a

lens with some sort of close-focusing (macro) ability.  Some of the new zoom

lenses allow pretty close focusing (and the zoom capablility makes it very

easy to fine-tune the desired magnification.)

There are three less expensive alternatives if you do not already own a

close-focusing zoom lens:

1)  (My choice:)   Use a tele-extender.  ($25-50)  I use a Vivitar 3X

tele-extender on my 50mm standard lens; this nets me a 150mm lens with

ability to focus to 2.5 feet from the screen, which is about the most

magnification that is useful (more than that and the dot pattern of the CRT

display becomes very noticeable.)  These are inexpensive and sharp; they do

reduce the effective aperature 3 stops (allow 3 times less light to get to

the film for you non-photographers) but you are on a tripod anyway, so a

slow shutter speed is no problem.

2)  Use a close-up lens. ($10-20)  These are inexpensive lenses that screw

onto the front of your lens and allow closer focusing.  A +1 lens allows

plenty of magnification, and does not reduce the aperature of the lens.  The

image quality is not as sharp, and the flatness suffers; they actually do

pretty reasonably well, though, especially if you stop down the lens (say to

f8 or f11.)  If you already have a telephoto or zoom lens, you might at

least try screwing one onto the front of the lens and give it a try.

3)  Extension tubes.  ($30-100)  These go in back of the lens and allow you

to focus closer to the screen.  These would be used with a telephoto or zoom

lens (as opposed to the teleconverter, which turns your regular lens into a

telephoto.)  The advantage over closeup lenses: no loss of sharpness.  The

disadvantage: cost, and reduction of effective aperature 1-2 stops.

-- FILM --

This is the most important thing I have to teach you!  I used a great

deal of Ektachrome 400 for my slides, and this works pretty well (slight

bluish cast.)  I have also used Kodachrome 64  and this is also reasonable.

Any slide film is reasonable.

But there is a new film out that is more than just reasonable; it is

FANTASTIC!  After discovering it, I will never use another roll of anything

else for screen photos.  It is 3-M CRT Recording Film, which comes in 100

ASA.  This film was designed specifically for computer screen photography,

and although I was skeptical, I am now a fanatic.  It is basically very high

contrast, high resolution, color slide film.  It also somehow allows the

dots in the CRT image to "bleed" a little so that they are not as

noticeable.  The Amiga screen comes out in a resplendant saturated rainbow

of color.  And the black background comes out inky jet black.  I don't own

stock in the company, it's just that this film really is an order of

magnitude better than any others I have tried.

To ice the cake, it can be processed in E-6 chemistry, which simply

means that shops that advertise 2 hour slide developing can handle this

film.  I buy the film and get it developed at Village One Hour Photo on

Westwood Blvd.;  I can shoot the slides at night, drop them off on the way

to work, and pick up the slides in time for an afternoon lecture the same

day!

-- PHOTOGRAPHING THE SCREEN --

The key here is alignment.  Draw a series of cocentric rectangles on

the screen, and use this to set up the camera.  You have to be perfectly

centered and perfectly perpendicular to the center of the screen to minimize

screen warp and distortion; focusing on this grid of rectangles will allow

you to best adjust the camera position.

Darken the room as much as possible (best to shoot at night but a few

blankets over the windows will allow daylight shooting.)  Focus carefully,

and fire away!  For your first tries, you will need to bracket your

exposures to find the best one for your setup; I have settled on  f8

(actually f2.8 with 3X extender = f8) at 1 second, using the 100 ASA 3M CRT

film discussed above.  You might start with this but as I said shoot with a

series of different aperatures or shutter speeds at first until you decide

what works best.  You will find that even 1/2 f-stop makes a difference in

the color saturation, so bracket at 1/2 f-stop increments.

It's unlikely that you would be able to use fast shutter speeds, but

just in case, remember that your shutter speed must be slower than the

screen refresh rate; stick with 1/30 second or slower to be safe.

Also, the brightness and contrast settings on the Amiga monitor have a

major effect on the slides; just set them in the middle (at the click stop)

and leave them there so that there is no variablilty from one session to the

next.

-- SOFTWARE --

I use Deluxe Paint in the hi-resolution mode (No flicker problem with

still photography!)   The 400-line resolution really makes a difference;

Atari owners will never present any competition here.

I wish you luck!  The ability to photograph Amiga screens has opened a

whole new vista for me, and has become the most important function for my

computer.  I hope that you find these hints useful.  Any suggestions for

further refinements would be welcome; leave me a message on the AmigaBoard

<EYEDOC>  or else on BIX <RGoldberg>.

File: AMIGASLIDES  7895-T

Date: 03-APR-86

Dir: 1  Sec: 7 - New Files

From: Eyedoc

Acc: 5

*** Photographing the Amiga Screen ***