Documentation Courtesy of

Jacobsen Consulting,

Fort St. John, B.C.

PRFIX

This information will help you correct some possible problems

with using spooled printers from a NOVELL file server.

The problem occurs when printing to a printer from the server

using BASIC, or a compiled BASIC program, or a program

compiled in one of several other languages.   From DOS, you

may be able to copy to a file, but the error occurs only

from within the program.

Up to three parallel printers can be supported under DOS and

NOVELL.   The port addresses used by DOS are located at

memory locations 408-40F.   Each address is two bytes long,

so as you can see, there really is room for 4 printers,

but only 3 are supported.   If you want to use 3 printers,

one of the printer ports must be on a monochrome card.

PRINTER     LOCATION      ADDRESS        ADDRESS

OF PORT       WITH           W/O

ADDRESS       MONO CARD      MONO CARD

LPT1:       0000:0408      3BC           378

LPT2:       0000:040A      378           278

LPT3:       0000:040C      278           N/A

This problem may ALSO occur if trying to print to a non-spooled

printer on the server that has some of the printers spooled.

I've noticed that NOVELL and DOS number the printers differently.

In this case, you may have to use DEBUG to modify the address

sent to a given port to solve your particular problem.   In a

recent test on a computer with 3 printers, DOS and NOVELL swapped

LPT1: and LPT2:  (One adds from right to left, the other from

left to right)   If you are using only spooled printers, then

the address entered into those locations don't really matter,

as long as they are non-zero.  (Ala the original PRFIX)

The following three programs will fix up either printer port

1, 2 or 3.   The old PRFIX.COM file only fixed printer port 1.

The assembly language listings are shown below in case you have

to modify the addresses, or combine all three into one program.

LPT1:                   LPT2:                  LPT3:

PR1FIX                  PR2FIX                 PR3FIX

PUSH   DS               PUSH   DS              PUSH   DS

MOV    AX,0000          MOV    AX,0000         MOV    AX,0000

MOV    DS,AX            MOV    DS,AX           MOV    DS,AX

MOV    BL,BC            MOV    BL,78           MOV    BL,78

MOV    [0408],BL        MOV    [040A],BL       MOV    [040C],BL

MOV    BL,03            MOV    BL,03           MOV    BL,02

MOV    [0409],BL        MOV    [040B],BL       MOV    [040D],BL

POP    DS               POP    DS              POP    DS

INT    20               INT    20              INT    20