The BPQ Switch supports a simplified subset of PK232 Hostmode Commands.

This document is intended to give a brief intorduction to Hostmode, and

define the commands supported by the switch. For full details of the

PK232 implementation, see the appropriate supplier's manuals.

With Hostmode, a single (virtual) TNC Port to the switch may support

several simultaneous conversations. Messages are passed to and from the

switch as character strings, with start and end frame delimiters. The

first byte specifies the type of data (command, status reponse, user

data, etc), and the channel number. The system allows two basic modes of

operation, polled or unpolled. With Polled, data is only sent from the

switch to the application on receipt of a poll. With unpolled, the switch

may send stuff to the application whenever it likes. Each method has its

advantages, and indeed the two BBS systems that I know of that use Hostmode

(G8UFQ and AA4RE) have chosen different modes. Unfortunately neither issues

the command which defines which mode to run in, so I have provided two

Hostmode options, PK232/AA4RE (polled), and PK232/UFQ (unpolled). There

are other minor differences between my options, which will be mentioned

when the commands that they affect are discussed.

The basic frame format is

SOH (HEX 01)  CONTROL   Data   ETB (HEX 17)

Any 01h, 10h, or 17h bytes appearing in the packet must be preceeded

by a 10h escape char, to avoid confusion with framing charaters.

The CONTROL byte has the following (hex) format:

Appl - Switch

2x		Data for channel x

4x		Command to channel x

4F		General (non channel specific) command.

Switch - Appl

3x		Data from channel x

4x		Link status (reply to CO)

4F		General command reply.

5x		Link status report from channel x

5F		Status Errors

3F is used for monitored data (RX), and 29 for Unproto stuff (TX)

The most important message formats are as follows:

TNC Command:

SOH 4F C C p p..  ETB    Where CC is the command code, and pp... params.

The Switch will respond immediately (ie without waiting for a poll) with

SOH 4F C C 00 ETB	 If OK

or	SOH 4F X X 01 ETB        If invalid.

Connect:

SOH 4x C O Callsign ETB

Note that whatever call you specify, you will just be connected to the

switch. Use switch commands from then onwards.

Session Status:

SOH 4x C O ETB

Returns

SOH 4x C O a b c d e  callsign ETB

Where a - f are hex values in range 0 - f, ORed with 30H

a = link state-1 (ie state 5 (connected) = 34h)

b = ax25 version (always 2 (32h))

c = number of unacked packets outstanding

d = number of retries (not supported - always 30h)

e = conperm flag (not supported - always 30h)

callsign = users callsign (if connected)

Session state changes (connected/disconnected) are reported as

SOH 5x 'Text string' ETB

When using polled mode the poll command is

SOH 4F G G ETB

The reply with be either SOH 4F G G 00 ETB, if nothing is available, or

one (any only one) data/monitored data/link status/data ack message.

Data is send on a connected channel as

SOH 2x (data) ETB

There is no immediate response. The system will reply with a data ack

when it is able to process another frame.

SOH 5F X X 00 ETB

The application must not send more data until this is received. Note in

polled mode, the data ack is returned in response to a poll.

Only a minimal subset of the full command set is implemented. The

following commands are processed - others are accepted, but ignored.

GG	POLL

CO	CONNECT or STATUS Request

MN	MONITOR - MN0 means monitor off, MN1 monitor on

MN2 also set MCOM on. Defaults on in UFQ mode,

off in AA4RE mode

ZM	Monitor port mask - as normal tnc MMASK command

UR	Users - set max concurrent users. In UFQ mode defaults

to 9, in AA4RE mode to zero. The maximum is 15, but as

the hostmode spec uses channel 9 for UNPROTO stuff, I

recommend a limit of 9.

DI	Disconnect. If followed by S (DIS), user is returned to

the node, instead of being fully disconnected.

John Wiseman G8BPQ

11/3/90

############################