Configuration file - BPQCFG.TXT

PORTS Section.

This section of the configuration file defines the links from the switch

to the outside world. These will normally be Radio links, but could also

be a link to another computer running another copy of the switch, or similar

software (eg NOS).

Please note that there is no relationship between the 'Virtual' COM ports

defined in the TNCPORT section, which are used by the BBS or other program

running abouve the switch, and these external links. You may use your

'real' COM1 port to connect to a KISS TNC, even if you have defined a

'Virtual' COM1 in the TNCPORT section.

There are two basic classes of links supported by the switch. One uses

normal ASYNC ports to communicate with TNCs running in KISS mode, and the

other uses special HDLC cards which fit inside the PC. ASYNC links are

supported , and interface to

the radio via either an inbuilt or external modem. Those currently supported

are the PacCom PC120, DRSI PC-PA, and Thor RLC100.

Interrupt levels may be shared by any cards with suitable hardware.

Examples are all but the earliest DRSI cards, and a MS400 ASYNC card

modified as described by AA4RE in his MBBIOS documentation. However

there is a significant overhead in using the shared interrupt feature

- try to use it on relatively low speed links, and watch out for TX

underruns/RX overuns.

Each port is defined by a set of statements of the form:

PORT

PARAM=VALUE

..

..

ENDPORT

Samples of various configurations are given later, but the full list

of PARAM values supported is given below

PORTNUM=n	; Number for use in PORTS display, connects, etc.

; Default is next sequential number.

ID=text string *; Up to 30 chars, appears on PORTS display

TYPE=ASYNC/DRSI/PC120/RLC100/QUAD/INTERNAL *

; ASYNC is for a normal ASYNC card, or a shared interrupt card

; such as the MS400

; PC120, DRSI and RLC100 are the internal HDLC cards - and

; support 2 channels.

; QUAD is the Quadram Quadport, which allows up to 5 async

; links. This version only supports a single Quadport. See

; later section for setup details.

; INTERNAL defines a 'loopback' port - anything

; sent on it will be received.Intended to allow KISS

; applications (eg TCP/IP) to talk to the switch.

PROTOCOL=HDLC/KISS/ASYNC/MULTIKISS *

; ASYNC is NETROM backend link

; MULTIKISS is for multidropped	KISS TNCs on one port, and

; needs special ROMS (JKISSP), and a diode coupler.

IOADDR=xxxH *	; in hex, H is needed

INTLEVEL=n *	; 2,3,4,5 or 7 - supports shared interrupt cards

SPEED=nnnn *	; Radio speed for DRSI, etc, async link speed for KISS

; If set to zero, the modem must supply a 1x clock.

; (TXC on RTXC pin, RXC on TRXC pin.)

CHANNEL=A/B/..*	; Selects port on DRSI/PC120, dual port KISS (eg KPC4) or

; MULTIKISS

BBSFLAG=NOBBS	; Stops direct connects to the BBS on this port

QUALITY=nnn *	; Default quality for nodes heard on this port. Set to zero

; to suppress NODES broadcasts on this port.

MAXFRAME=n *	; Maximum outstanding frames

TXDELAY=nnn *	; TX Keyup delay in milliseconds

SLOTTIME=100 *	; CSMA interval timer (milliseconds)

PERSIST=nnn *	; 'Probability to transmit' value (1-255)

FULLDUP=0/1	; currently only used by KISS driver

SOFTDCD=0/1	; not yet supported

TXTAIL=nnnn	; TXTAIL timer in ms. Used only by KISS TNCs

FRACK=nnnn *	; Level 2 timout in milliseconds

RESPTIME=nnnn *	; Level 2 delayed ack timer in milliseconds

RETRIES=n *	; Level 2 maximum retry value

PACLEN=nnn *	; default maximum packet length for this port

CWID=CALL	; Currently only supported on DRSI/PC120/RLC100

PORTCALL=CALL	; Additional callsign/alias for level 2 connects to the

PORTALIAS=ALIAS ; node for this port. Should not be the same as BBSCALL

ALIAS_IS_BBS=0/1; If set then PORTCALL and PORTALIAS give access to the BBS

; rather than the node.

VALIDCALLS=CALLS; Up to about 40 callsigns, separated by a single commas.

; If present, only stations in the list will be allowed

; to connect to you on this port. The SSID should not

; be specified, and multiple VALIDCALLS lines may be

; included. End all but he last line with commas. The

; total length of the list, including commas, as 256 bytes.

QUALADJUST=NNN	; Percentage to reduce the quality of a NODE in the NODES

; broadcast, if the best neighbour is on the same port as

; the broadcast. Designed to improve throughput if more than

; one channel is available, by discouraging routes which

; cause a node to send a packet on the same port that it

; received it on.

DIGIFLAG=0/1/255; Digipeat control flag for this port

; 0 = dont digi, 1 = digi everything, 255 = digi UI frames only

DIGIPORT=N	; Port to send digied frames on. Zero means the port it was

; received on, any other number means send on that port.

; This is intended mainly to allow a BBS on another machine,

; connected via a wire link, to send out 'MAIL FOR' messages.

USERS=N		; This limits the number of L2 sessions to the supplied

; number. Zero means no limit. The limit applies to all

; L2 sessions (even those with other nodes), but is only

; checked for incoming connects (ie you can make outgoing

; connects even when the limit is reached). Intended mainly

; to control the number of users on busy user access ports

; (especially 2m)

UNPROTO=	; Unproto destination and digi string for this port.

; Use commas between calls, and dont include the VIA.

L3ONLY=0/1	; If set, will stop users (but not HOST sessions) issuing

; downlink connects on this port.

INTERLOCK=n	; Allows groups of ports to be defined which will not transmit

; at the same time. A value of zero indicates no locking, if

; any other value is specifed the code will make sure that no

; two ports with the same value can transmit at the same time.

Those marked with a * must be included, the rest are optional. The validation

is not good, and defaults are generally zero, so if you miss out any of the

essential ones, odd things may well happen!

Sample PORT Configurations:

1. 2 Port DRSI card set for IO addr = 300h, int level 5

PORT

ID=144.650 MHz 1200 Baud

TYPE=DRSI

PROTOCOL=HDLC

IOADDR=300H

INTLEVEL=5

SPEED=1200

CHANNEL=A

QUALITY=10

MAXFRAME=1

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

SOFTDCD=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

CWID=G8BPQ

ENDPORT

;

PORT

ID=432.675 MHz 1200 Baud

TYPE=DRSI

PROTOCOL=HDLC

IOADDR=300H

INTLEVEL=5

SPEED=1200

CHANNEL=B

BBSFLAG=NOBBS			; IF MY BBS LICENCE IS ONLY FOR 2M

QUALITY=10

MAXFRAME=1

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

SOFTDCD=0

FRACK=6000

RESPTIME=1500

RETRIES=10

PACLEN=120

CWID=G8BPQ

PORTCALL=G8BPQ

PORTALIAS=NOTTM

;

ENDPORT

2. Standard KISS TNC on primary ASYNC card. (COM1). To use the secondary

ASYNC card (COM2), change IOADDR to 2F8H, and the INTLEVEL to 3.

PORT

ID=144.650 Mhz 1200 Baud

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=3F8H

INTLEVEL=4

SPEED=9600

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

The second port of a multiport TNC (Such as the KAM or KPC 4) is defined as

follows:

ID=432.675 Mhz 1200 baud

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=3F8H

INTLEVEL=4

SPEED=2400

CHANNEL=B

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

QUALADJUST=5

ENDPORT

3. Quadram Quadport.

The Quadram Quadport card supports up to 5 (yes I know it says quadport!!)

async ports. This version only supports one card - if anyone needs more,

let me know. You do not need to define all ports, but you must start with

the first, and not leave any gaps.

The card should be configured with first port at 280h, and software

selectable interrupts (3 or 4 may be used)

The card will support KISS TNCS (either normal, or KPC4 type dual port), or

NETROM links (or a combination of both).

The following sample config file is for a Dual Port KPC4 on channel 1,

a normal KISS TNC on channel 2, and a NETROM async link on channel 3.

PORT

ID=KPC4 PORT 1

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=KPC4 PORT 2

TYPE=QUAD

PROTOCOL=KISS

IOADDR=280H

INTLEVEL=3

SPEED=2400

CHANNEL=B

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NORMAL KISS

TYPE=QUAD

PROTOCOL=KISS

IOADDR=290H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

PORT

ID=NETROM BACKEND

TYPE=QUAD

PROTOCOL=NETROM

IOADDR=2A0H

INTLEVEL=3

SPEED=2400

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500		; THESE AREN'T USED, BUT I SUGGEST

SLOTTIME=100		; YOU PUT THEM IN IN CASE ONE DAY

PERSIST=64		; THEY ARE!

FULLDUP=1		;

SOFTDCD=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

######################################################################