MSYS Support for HAPN-1 and DRSI PC-Packet Adapters

(C) Copyright HAPN Hamilton and Area Packet Network, 1990

Jack Botner, VE3LNY

222 Divadale Drive

Toronto, Ontario

Canada   M4G 2P7

Aug. 15, 1990

Support for the HAPN-1 and PC-Packet adapters is available in MSYS

version 1.09 and later. This support is made up of the following

components:

1. An interface built into MSYS. Until version 1.09, MSYS would talk

only to KISS TNCs attached via serial port. An interface was added

to optionally talk to any other device using a software interrupt

vector. The alternate interface is selected by a certain specification

in the port name.

2. The use of a HAPN device driver for either the HAPN-1 adapter or the

PC-Packet adapter. These device drivers provide hardware support for

the adapters and a programming interface for applications.

3. A KISS application interface that works between the new MSYS software

interrupt interface and the HAPN device driver. The HAPN device

driver provides a simple block-oriented interface to the application

program; this module converts this interface to the KISS standard

used by MSYS.

With these components in place and the proper port specification in the

MSYS.OPT file, MSYS will use the HAPN-1 and PC-Packet adapters like

another port.

MSYS Interface

You must have version 1.09 or later. To specify a HAPN-1 or PC-Packet

adapter, you must set up a PORT statement in MSYS.OPT with a port

name beginning with an *. For example,

port 0 at $0 int 4 speed 1200 name *HAPN-1

For a system with 2 HAPN-1 adapters or 1 PC-Packet adapter,

port 0 at $0 int 4 speed 4800 name *port0

port 1 at $1 int 4 speed 1200 name *port1

Please note that the information normally used by MSYS to access a

serial port, the interrupt level, and the serial port line speed are

irrelevant. The port number is important for the reasons stated in the

MSYS documentation, and the port name beginning with * is critical to

the port being recognized as using the alternate software vector

interface; however after the * the name may be anything you like.

HAPN Device Driver

If you own a HAPN-1 adapter, you are already familiar with the device

driver HAPN1.SYS used with the software package. (If you have an old

version, you should know that the current HAPN-1 software supports

multiple connects and multiple ports. Software updates are available

for $10 US for registerd customers, and $35 US for others.)  HAPN

has also developed an 8530 device driver, called HAPNDRSI.SYS, that

works with the PC-Packet adapter and provides the identical function. You

need one or the other of these device drivers depending on which adapter

you are using. The specification for loading one of these device drivers

may be found in a separate document for each driver. Hardware-related

parameters such as adapter address, interrupt level, etc. are specified

on the command line in CONFIG.SYS. The driver will be loaded when you

boot your system. Watch your screen when you boot up. The device driver

will display a number of messages indicating that it has been loaded, or

an error message if a parameter is in error.  If the driver does not load,

the port cannot be used.

Further documentation on the HAPN-1 device driver may be found in file

HAPN-API.DOC, and on the PC-Packet driver in file HAPNDRSI.DOC.

HAPNKISS Interface Module

Finally, before running MSYS, load HAPNKISS.COM. It is a small TSR

program that searches out the HAPN-1/PC-Packet device drivers then

sets up the software interrupt for MSYS to use. The default interrupt

is D0 (hex). Loading HAPNKISS is as simple as typing HAPNKISS and

pressing enter, but the following things should be kept in mind:

1. MSYS uses vector D0 by default, and so does HAPNKISS. However if

it is necessary to use a different vector, the vector can be

specified on the command line as follows:

HAPNKISS 9E

The 2-digit hexadecimal number (upper case not necessary) will be

interpreted as the interrupt vector to use instead of D0. (Don't

ask me how to make MSYS use a different vector 'cause I don't know.)

2. HAPNKISS expects the vector (4 bytes in low memory) to contain all

zeroes. If it does not, another program could be using this vector,

or it just may not have been initialized properly. If you determine

that it is ok to use the vector even when it does not contain zeroes,

add /f to the command line, for example:

HAPNKISS /F

You will know if there is a problem because when /f is not present,

HAPNKISS will not load, and displays an error message when the vector

is not zeroes. Adding the /f parameter overrides the error and

HAPNKISS will load.

3. You cannot load HAPNKISS twice. If you try, an error message will

be displayed. One copy of HAPNKISS can support up to 4 ports made up

of HAPN-1 and PC-Packet adapters. To get rid of HAPNKISS, re-boot.

4. When it loads, HAPNKISS reports all the ports it recognizes. You

could have up to 4 ports made up of HAPN-1 and PC-Packet adapters.

The report will give you the information you need to set up the

PORT statements for MSYS.OPT. So, if you are using more than one

of these ports, set up the device drivers, boot your system, and

load HAPNKISS to see what the ports are, then use the information

to set up the MSYS PORT statements.

5. HAPNKISS uses the timer tick interrupt for timing purposes. It

takes over interrupt 8, and when it is done, calls the previous

int 8 address. If desired, HAPNKISS can be made to use interrupt

vector 1c (hex) instead, by adding the switch /a or /A on the

HAPNKISS command line. The HAPN-1 adapter manual contains more

information on the use of the timer tick interrupt by the HAPN-1

programs.

Terms of Distribution

The HAPN device drivers and the KISS interface are available from HAPN

at the above address. $5 US is required to cover the cost of diskette,

mailer, and postage.

This software is distributed under the following conditions:

1. It may be used for Amateur Radio purposes only.

2. It may be re-distributed to other amateurs for use with MSYS only.

In this case, it should be distributed as a package including this

documentation.

3. The programs are Copyright HAPN (Hamilton and Area Packet Network).

THEY MAY NOT BE USED FOR ANY PURPOSE BY ANY COMMERCIAL ORGANIZATION,

OR REDISTRIBUTED BY ANYONE FOR A PRICE. HAPN has put a lot of work

into these programs that we are distributing for free, and we don't

want anyone else making profits from them.

4. The programs are distributed "as is". HAPN will endeavor to provide

support to users experiencing problems with them, but time can only

be allocated as available. In addition, the multitude of PC

configurations in use is not available to us, so we may not be able

to reproduce your problem.

5. The HAPNDRSI.SYS driver is at Beta test level. If you have a problem,

please report it to us. Messages can be sent to VE3LNY @VE3NUU or

private mail on Compuserve ID 73327,176 or to the address above.