SB-054

DG-5 Standard Service Procedure               8-10-79

(Supplement to the DG-5 Service Manual)

Introduction

A majority of DG-1 failures are caused by nothing more than an intermittent

opening of a plated-through hole connecting top and bottom foil paths of

the PC board.

Before any component level service is attempted, it is advisable to

eliminate the possibility of an intermittent connection causing or

complicating counter failure.

Procedure:

1.   Disconnect all cables.

2.   Remove cabinet: 2 screws at the rear, 2 screws with star washers on

the bottom.

3.   Disassemble display from chassis; 4 countersunk screws on the side

rals.

4.   The display unit has 2 through holes without leads, and the Counter

main board has 82 through holes without leads passing through them.  Ideal

service would be to first remove all solder from these holdes using vacuum

desoldering devices, or WIK-IT braid.  Then, pass a length of #24 tinned

bare wire into each clean through hole, solder, and clip flush.  For 10

holes listed under IC's solder sparingly so as not to create a solder

bridge under the component.  For the remaining 72 holes, inspect each

"pinned" connection from both sides to ensure (1) no bridge and (2) a good

solder joint.

5.   Wash the display to counter Board PC board connectors (male and

female) with FRESH trichlorethelyne.  Heat dry and inspect for flux

residue.  Spray the connectorrs with silicone.

6.   Similarly, clean and coat the rear panel RCA jacks as in step 5.

7.   Reassemble, reversing steps 3,2,1.

How to solder on the DG-5 pc board.

1.   Use a low power pencil iron of 35 watts or less.

2.   Use rosin core solder, small diameter.

3.   Do Not create solder bridges or splotches.

4.   Do Not overheat or otherwise lift the foil from the board.

5.   Wash excess flux away with FRESH trichlor, and a small flux brush.

Neatness counts - it's your radio.

Through Hole Accounting

Display unit - 2

Counter unit -82

Total        -84

Locations:

A.   Under; IC2,4,13,20-25,34,  1 each

Total 10

B.   Adjacent or between; IC3,13-14,15-16,22-23 1 1 each

IC1-2,6,18              2 each

IC12                    3 each

Total 13

C.   Located between rows: IC13-18,20-25  - 5

20-25, 26-31   - 10

IC26-31, 32-37 - 15

IC32-37, 38-39 - 25

Total 55

D.   At rear panel: FS7805 regulator - 1

2SD235 transistor- 3

Total 4

E.   Display unit - 2

Additional Ponts

1.   R126 &129 are power resistors located at the left rear corner of the

PCB.  Remove the tubing from their outside legs and resolder each lead to

the top foil path.  Specific symptom for this failure is no display, and no

decimals.

2.   For counters which roll, random count, or inject hash noise back into

the TS-520/520S especially when hot, install an additional .1uF @ 12v

bypass cap across the FS7805 regulator on the foil side of the PCB from the

center ground lead to the outside leg, across C89.

3.   For complaints of hum or digital noise from the DG-5 back into the TS-

520/s both during RX and TX, tighten the mounting screws which supply

ground to the Carrier unit X50-0009-01 in the transceiver.

Please annotate your service manual.

JEB/yn

(c) 8679TKC