SB-061

TS-180S Trouble Sheet                12-18-79

ALC deflection low   Replace D21 on IF unit from 7.1 to 9.0V zener.

Readjust all ALC pots.

IF #2 OUtput low   Bridge R77 position, replace R78 with R77 470 ohms.  See

TKC service Bulletin.

RX Noise (OSC CAR leakage)   Shorten L31, L33 leads as much as possible.

Increase NB effectiveness   Replace L31, L33 for 33 uh to 27uh, with SHORT

leads.

RX noise from strong signals   Replace L30 10uh on the IF with self

shielding type (new part L40-1035-21).

TX audio distortion   Adjust T15 on the IF approx 1/2 turn back, readjust

all ALC pots up.

10 M PLL unlock   change:  R77 from 6.8 K to 4.7 K ohm, R94 from 8.2 K to

6.8 K ohm on the PLL unit.

TX audio distortion   Adjust 10 W bias for 150ma, 100w bias for 100ma.

Unlocked all bands   Check L2, (Q2 B+) in the PLL unit.

FM'ing with DFC   See service bullein of 9/10 for L12, L13 change and add

resistor and shield.

40 Mlow alc, See bulletin 9/14 change C14, 17.

JEB/yn

(c) 8979TKC