SB-811

TR-2400 RX low, INtermittent, or distorted                        2-20-80

For any of the above complaints, suspect a broken loose component in the RX

IF or audio output circuit on the RX TX unit.  Simply pull up on the leads

and resolder.

Adjacent to Q9, the audio output IC, find C49, a 1uf cap.  If broken loose

from the board, audio output may be down, while current consumption may be

up from the normal 30mA to about 100mA due to the audio output amp

oscillating.

Adjacent to L10, the discriminator, locate C33, a .033uf cap.  If this

component is broken loose, audio output may be up approximately 6 dB with

excessive high frequency audio response.

JEB/yn

(c) 22080TKC