OST Bulletin N0.60

Multichannel Television Sound Transmission and Audio

Processing Requirements for the BTSC System.

PREFACE

This bulletin contains those technical specifications for

the BTSC System as presented to the Federal Communications

Commission by the Electronics Industries Association.  These

specifications are presented herein pursuant to the Report &

Order in BC Docket 21323 adopted March 29, 1984 and are

considered guidelines for stations employing the BTSC system of

multichannel sound transmission and audio processing.

The Commission in BC Docket 21323 has adopted very general

technical rules that will allow the television aural baseband to

be used for television stereophonic sound, second language

programming and any other broadcast or non-broadcast use.

The BTSC System uses a pilot subcarrier at 15,734 Hz.  The

pilot allows receivers to recognize that transmissions are in

stereo and to switch into the stereophonic reception mode.  To

ensure compatibility and to prevent BTSC type receivers from

falsely detecting other MTS formats, Docket 21323 restricts

emissions at 15,734 Hz.  The purpose of this Bulletin is to

present the BTSC specifications which in accordance with FCC Rule

73.683(c)(3), are required to be met when a broadcast station

transmits an emission at 15,735 Hz.

A.   Definitions                                                1

B.   Transmission Standards                                     4

(a) Television Stereophonic Sound Standards                4

(b) Television Second Audio Program Standards              4

(c) Television Sound Encoding Standards                    5

(d) Television Non-Program Related Aural Subcarrier

Standards                                              7

C.   Transmission System Requirements/Multichannel Sound

Requirements                                               7

(a) Electrical Performance Standards for Stereophonic

Operation                                              7

(b) Electrical Performance Standards for Second Program

Operation                                              9

(c) Electrical Performance Standards for Video Transmission 10

(d) Electrical Performance Standards for Sound Encoding     11

D.   Modulation Levels                                           12

Page 1

Standards for the BTSC system as presented to the Federal Communications

Commission by the Electronic Industries Association.

A. Definitions

Compandoring: A noise reduction process used in the stereophonic

subchannel and the second audio program subchannel consisting of

encoding (compression) before transmission and decoding (expansion)

after reception.

Composite stereophonic baseband signal:

The stereophonic sum modulating signal, the stereophonic difference

encoded signal and the pilot subcarrier.

Crosstalk: An undesired signal occuring in one channel caused by an

electrical signal in the other channels.

Decibel ERMS value:  The exponentially time-weighted root mean

square (ERMS) value converted to dB as follows:

decibel ERMS value = 20 log

10 (ERMS value

______________

(Reference)

Where reference is the 0dB ERMS value.

Encoding:      See Compandoring.

Equivalent input separation: A method of specifying the stereophonic

separation be referring variations from ideal at the output back to

the input.  To accomplish this, an input signal which causes a non-

ideal output is varied by degrading input separation until the

output conforms to the ideal.  The amount of input separation

degradation required is the equivalent input separation.

Equivalent modulation: See equivalent input tracking.

Equivalent input tracking: A method of specifying the tracking

ability of the encoding process by referring variations from the

ideal at the output back to the input of the encoder.  To accomplish

this, an input signal which causes a non-ideal output is varied

until the output conforms to the ideal.  The amount of input

variation required is the equivalent input tracking.

Page 2

Exponentially time-weighted root mean square (ERMS) value:

The ERMS value of a waveform is obtained from the following formula:

_____________________________________________

/       t

/ 1      /`    2    -(t-u)/T

/ ---    /     S (u)e        du

\/   T    /

`

-o~o

Where S(u) is the waveform in question, a function of time, T is the

exponential time-weighting period, and t is the time at which the

ERMS value is computed.

Incidental carrier phase modulation (ICPM):

Angle modulation of the visual carrier by video signal components

which, when detected in TV receiver intercarrier circuits, cause an

audio interference known as intercarrier buzz.

Left (or right) audio signal:

The electrical output of a microphone or combination of microphones

placed so as to convey the intensity, time, and location of sounds

originating predominately to the listner's left (L) (or right (R))

of the center of the performing area.

Left (or right) stereophonic channel:

The transmission path for the left (or right) audio path.

Main Channel:

The band of frequencies from 50 to 15,000 Hz which frequency

modulates the aural carrier.

Multichannel sound:

Multiplex transmission on the TV aural carrier.

Multiplex transmission:

The simultaneous transmission of the TV program main channel audio

signal and one or more subchannel signals.  The subchannels include

a stereophonic subchannel, a second audio program subchannel, a non-

program related subchannel and a pilot subcarrier.*

Non-program related subchannel:

The subchannel for the multiplex transmission of a frequency-

modulated subcarrier for telemetry or other purposes.

Pilot subcarrier:

A subcarrier serving as the control signal for use in the reception

of stereophonic TV sound broadcasts.

* FCC NOTE:  The Commission does not require nor restrict the use of the

second audio program subchannel or the non-program related subchannel.

Page 3

Second Audio Program (SAP) broadcast:

The multiplex transmission of a second audio program utilizing the

second audio program subchannel.

Second Audio Program (SAP) subchannel:

The channel containing the frequency-modulated second audio program

subcarrier.

Second program audio signal:

The monophonic audio signal delivered to the SAP encoder.

Second program encoded signal:

The second program audio signal after encoding.

75 microsecond(75us) equivalent modulation:

The audio signal level prior to encoding that results in a stated

percentage of modulation when the encoding process is replaced by

75us pre-emphasis.

Spectral compression:

A process wherein variations in spectral content of an audio signal

are reduced by varying a frequency filtering function applied to the

signal in response to the spectral content of the signal.

Stereophonic difference audio signal:

The left audio signal minus the right audio signal (L-R).

Stereophonic difference encoded signal:

The stereophonic difference audio signal after encoding.

Stereophonic separation:

The ratio of the electrical signal caused in the right (or left)

stereophonic channel to the electrical signal caused in the left (or

right) stereophonic channel by the transmission of only a right (or

left) signal.

Stereophonic sum audio signal:

The left audio signal plus the right audio signal (L+R).

Stereophonic sum channel compensation:

A process wherin the phase and amplitude response resulting from

bandlimiting in the process of encoding the stereophonic difference

audio signal (which, if uncompensated, would detrimentaly affect

stereo separation) is compensated by an identical phase and

amplitude response applied to the stereophonic sum audio signal.

page 4

Stereophonic sum modulating signal:

The stereophonic sum audio signal after compensation, pre-emphasis

and other processing.

Stereophonic subcarrier:

A subcarrier having a frequency which is the second harmonic of the

pilot subcarrier frequency and which is employed in TV stereophonic

sound broadcasting.

Stereophonic subchannel:

The subchannel containing the sterophonic subcarrier and its

associated sidebands.

Wideband amplitude compression:

A process wherein the dynamic range of an audio signal is compressed

by simultaneously varying the gain of all audio frequencies equally.

B. Transmission Standards

(a)	Television stereophonic sound standards.

(1)   Television broadcast stations may transmit stereophonic

sound by employing a subcarrier on the aural carrier.

The main channel modulating signal shall be the

stereophonic sum modulating signal; the subcarrier

modulation shall be the stereophonic difference encoded

signal.

(2)   The subcarrier shall be the second harmonic of a pilot

signal which is transmitted at a frequency equal to the

horizontal line rate.  Note: If the station is engaged

in stereophonic sound transmission accompanied by

monochrome  picture transmission, the horizontal

scanning frequency shall be 15,734 Hz +/- 2Hz.

(3)   The subcarrier shall be double sideband amplitude

modulated with a suppressed carrier and shall be capable

of accepting a stereophonic difference encoded signal

over a range of 50 - 15,000 Hz.

(4)   The total modulation of the aural carrier, including

that caused by all sucarriers, shall comply with the

requirements of 73.1570 of the FCC Rules and

Regulations.

(b)	Television second audio program standards.

(1)   Television broadcast stations may transmit a subcarrier

carrying a second audio program.

(2)   The subcarrier frequency shall nominally be equal to the

fifth harmonic of the horizontal line rate.

Page 5

(3)   The second program encoded signal shall frequency

modulate the subcarrier to a peak deviation of +/- 10

kHz.

(4)   The second audio program subchannel shall be capable of

accepting second program encoded signals over a range of

50 - 10,000 Hz.

(5)   The modulation of the aural carrier by the second audio

program subcarrier shall comply with (D)(a)(1)(iv) of

this bulletin (+/- 15 kHz deviation).

(c)	Television sound encoding standards.

(1)   The stereophonic difference audio signal and the second

program audio signal shall be encoded prior to

modulating their respective subcarriers.*

(2)   This encoding shall have the following characteristics,

where f is expressed in kilohertz (kHz).

(i)  Fixed pre-emphasis (F(f)) whose transfer function

is as follows:

F(f) = (jf/0.408)+1   (jf/2.19)+1

____________ x ____________

(jf/5.23)+1    (jf/62.5)+1

(ii) Wideband amplitude compression wherein:

(a) The decibel gain (or loss) applied to the

audio signal during encoding is equal to minus

one times the decibel ERMS value of the

encoded signal (the result of the encoding

process), weighted by a transfer function

(P(f)) as follows:

P(f) =         (jf/0.0354)

______________________________

((jf/0.0354)+1)   ((jf/2.09)+1)

(b) The exponential time weighting period T

1

of the ERMS detector referred to above in (a)

is 34.7 ms.

(c) The zero decibel reference ERMS value for the

encoded signal referred to above in (a) is

8.99% modulation of the subcarrier at 0.300

kHz.

*   FCC NOTE: When the SAP channel is used for non-program related audio

signals, encoding is not specified.

Page 6

(iii)  Spectral compression wherein:

(a) The transfer function S(f,b) applied to the audio

signal during encoding is:

D/20

S(f,b) = 1+ (jf/F) (b+51)/(b+1)       ,  where b=-10

______________________

1+(jf/F) (1+51b)/(b+1)

F=20.1 kHz; D=decibel rms value and b is the

decibel ERMS value of the encoded signal (the

result of the encoding process) weighted according

to a frequency transfer function Q (f) as follows:

3

Q(f)=                    (jf/5.86)

________________________________________________________

2

((jf/7.66) +(jf/7.31)+1) x ((jf/26.9)+1) x ((jf/3.92)+1)

(b) The exponential time waiting period T  of the

2

ERMS detector referred to above in (iii-a) is

11.4 rms.

(c) The ERMS zero decibel reference for the encoded

signal referred to above in (iii-a) is 5.16%

modulation of the subcarrier at 8 kHz.

Note: This reference results in a +18.4 dB gain

through the encoding process at 32.0% modulation

using a 8 kHz tone, when the output bandlimiting

filter (see (iv) and (v) following) gain is +18.4

dB at 8 kHz.

(iv) Overmodulation protection which functionally follows the

functions i,ii, & iii above.

(v)  Bandlimiting to appropriately restrict bandwidth which

functionally follows the functions i, ii, & iii above.

Page 7

(d)  Television non-program related aural subcarrier standards.

(1)  Multiplexing of the aural carrier is subject to the

requirements of Section 73.682(c) of the FCC Rules and

Regulations; provided, however, that when the stereophonic

and/or second audio program subchannels are transmitted,

mulitplexing of the aural carrier by non-program related

subchannels is subject to the following changes:

(i)    The maximum modulation of the aural carrier by the

non-program related subcarrier shall comply with

the requirements of (D)(a)(1(vi) of this bulletin.

(ii)   When the stereophonic and second audio program

subcarriers are transmitted, the instantaneous

frequency of the non-program related subcarrier

shall have the average value of six and one-half

times the frequency of the horizontal scanning

frequency with a tolerance of +/- 500 Hz.

(iii)  When only the stereophonic subcarrier is

transmitted, the instantaneous frequency of the

non-program related subcarrier shall lie between 47

and 120 kHz with a tolerance of +/- 500 Hz.

(C) Transmission system requirements/Multichannel sound requirements 1/

(a)  Electrical performance standards for stereophonic operation.

(1)   The aural transmitter must operate satisfactorily with a

frequency deviation of +/- 73 kHz.

It is recommended that the transmitter operate

satisfactorily with a frequency deviation of +/- 100 kHz.

(2)   The pilot subcarrier shall be frequency locked to the

horizontal scanning frequency of the transmitted video

signal.

(3)   The requirements of section 73.687(b)(2) of the FCC Rules

and Regulations shall be complied with for both the (L+R)

main channel and (L-R) subchannel, except for pre-emphasis

as specified in Section (B)(c) of this bulletin, with the

additional requirement that the aural transmitter shall be

capable of transmitting a band of frequencies from 50 to

120,000 Hz.

1/   Unless otherwise specified, the transmission system requirements are

defined for 75us pre-emphasis (which is matched to that in the main channel in

the case of stereophonic transmission) substituted for encoding. Measurements

are made over the band of 50 to 15,000 Hz and employ 75us de-emphasis in the

measuring equipment.

Page 8

(4)   The sterophonic subcarrier, being the second harmonic of

the pilot signal, shall cross the time axis with a

positive slope simultaneously with each crossing of the

time axis by the pilot subcarrier.  The pilot subcarrier

shall cross the time axis at points located within +/- 3

degrees (approximately +/- 0.53 microseconds) of the zero

crossings of the stereophonic subcarrier.

(5)   The unmodulated stereophonic subcarrier shall be

suppressed to a level less than 0.25 kHz deviation of the

main carrier.

(6)   The combined audio frequency harmonics measured at the

output of the transmitting system  (including the sound

encoder), as defined in 73.687 (b) (3) of the FCC Rules

and Regulation, at any audio frequency from 50 - 15,000 Hz

and at modulating percentages of 25, 50 and 100%, 75us

equivalent modulation shall not exceed the rms values in

the following table:

50 to 100 Hz          3.5%

100 to 7,500 Hz        2.5%

7,500 to 15,000 Hz       3.0%

Harmonics shall be included to 30 kHz.

(7)   The ratio of peak main channel deviation to the peak

stereophonic subchannel deviation when only a steady state

left (or right) signal exists shall nominally be one half

for all levels of this signal and for all frequencies from

50 - 15,000 Hz.

(8)   The phase and amplitude characteristics of the

stereophonic sum modulating signal and the stereophonic

difference encoded signal shall be such that the minimum

equivalent input separation at 10%, 75us equivalent

modulation is as follows:

(i)       30 dB separation from 100 Hz to 8 kHz.

(ii)      Smoothly decreasing separation below 100 Hz,

from 30 dB to 26 dB at 50 Hz.

(iii)     Smoothly decreasing separation above 8 kHz,

from 30 dB to 20 dB at 15 kHz.

Page 9

Note: It is recommended that the transmission system, excluding

encoding, shall meet a 40 dB separation requirement when 75us pre-

emphasis is substituted for sound encoding.

(9)  Crosstalk into the main channel caused by a signal in the

stereophonic subchannel shall be at least 40 dB below 25 kHz main

carrier deviation.

(10)  Crosstalk into the main channel caused by a non-stereophonic

multiplex signal shall be at least 60 dB below 25 kHz aural carrier

deviation.

(11)  Crosstalk into the stereophonic subchannel caused by a signal in the

main channel shall be at least 40 dB below 50 kHz aural carrier

deviation.

(12)  Crosstalk into the stereophonic subchannel caused by another

multiplex signal shall be at least 60 dB below 50 kHz aural carrier

deviation.

(13)  The aural transmitting system output frequency modulation noise

level in band of 50 to 15,000 Hz (with de-emphasis) must be at least

58 dB below the audio level representing a frequency deviation of

+/- 25 kHz.  The frequency modulation noise level in the

stereophonic subchannel, after demodulation, in the band of 50 to

15,000 Hz (with de-emphasis) must be at least 55 dB below the audio

level representing a frequency deviation of +/- 50 kHz.

(14)  The pilot subcarrier-to-interference ration, over a bandwidth of 1

kHz centered at the pilot subcarrier, shall be at least 40 dB.

(b)  Electrical performance standards for second audio program operation

(1)   The aural transmitter frequency deviation capability must comply

with the requirements of Section C(a)(1) of this bulletin.

(2)   The aural transmitter modulation bandwidth capability shall comply

with the requirements of C(a)(3) of this bulletin.

(3)   The unmodulated subcarrier shall be frequency locked to the fifth

harmonic of horizontal line rate.  When modulated, the center shall

nominally be that of the fifth harmonic of the horizontal line

scanning frequency with a tolerance of +/- 500 Hz.

(4)   Frequency modulation of the subcarrier shall be used.

(5)   The subcarrier shall be shut off when the second audio program

subchannel is not in use.

Page 10

(6)  The combined audio frequency harmonics measured at the output of the

transmitting system (including the encoder) at any audio frequency

from 50 to 10,000 Hz and at the modulating percentages of 25, 50 and

100% 75us equivalent modulation shall not exceed the rms values in

the following table.

50 to 100 Hz          3.5%

100 to 5,000 Hz        4.0%

5,000 to 10,000 Hz       3.0%

Harmonics shall be included to 20 kHz.

(7)  Crosstalk into the SAP subchannel caused by a signal in the main

channel and/or the stereophonic subchannel shall be at least 50 dB

below the level representing full modulation of the SAP subchannel

(+/- 10 kHz deviation).

(8)  The aural transmitting system output frequency modulation noise

level after subcarrier demodulation must be at least 50 dB below the

level representing full modulation of the SAP subcarrier (+/- 10 kHz

deviation).

(9)  The aural transmitting system output amplitude modulation noise

level in the band of 63,000-94,000 Hz shall be at least 50 dB below

the level representing 100% amplitude modulation.

(c)  Electrical performance standards for video transmission.

(1)  The requirements of 73.687 (a) (1) and (2) of the FCC Rules and

Regulations shall be complied with; provided, however, that when the

station is engaged in stereophonic sound transmission, or when the

station transmits stereophonic sound and/or second audio program,

subparagraphs (1) and (2) apply, except as modified by the

following:  A sine wave of 4.5 MHz introduced at the terminals of

the transmitter which are normally fed the composite color picture

signal shall produce a radiated signal having an amplitude  (as

measured with a diode on the RF transmission line supplying power to

the antenna after the combination of visual and aural power) which

is down at least 30 dB with respect to the signal produced by a sine

wave of 200 kHz.

(2)  In the situation where stereophonic sound and/or a second audio

program is transmitted, the following requirements shall be met:

the incidental phase modulation of the visual carrier by video

signals in the frequency band between 1 and 92 kHz shall be less

than three degrees for carrier amplitude below 0.75 of the voltage

at synchronizing peaks and less than 5 degrees for carrier

amplitudes exceeding 0.75 of the voltage at synchronizing peaks.

(d)  Electrical performance standards for sound encoding.

Page 11

(1)  The equivalent input noise of the sound encoder, measured over

a 15 kHz bandwidth, shall be more than 70 dB below the 100 Hz,

100% 75us equivalent modulation level.

(2)  The tracking characteristics of the sound encoder shall be such

that the minimum equivalent input separation at modulation

percentages from 1% to 100% 75us equivalent modulation is 26 dB

from 100 Hz to 8 kHz.

D.  Modulation Level

(a)  When only a monophonic audio signal is transmitted, the

modulation of the aural carrier shall not exceed 25 kHz

deviation on peaks of frequent recurrence, unless some other

peak modulation level is specified.

(b)  For stations transmitting more than one audio program channel

the maximum modulation levels must meet the following

limitations:

(1)  TV stations transmitting stereophonic sound signals must

limit the modulation of the audio carrier by the

stereophonic sum modulating signal to 25 kHz deviation on

peaks of frequent recurrence.

(2)  TV stations transmitting stereophonic sound signals must

limit the modulation of the audio carrier by the

stereophonic sum modulating signal and stereophonic

difference encoded signal to 50 kHz deviation on peaks of

frequent reccurence.

(3)  The modulation of the aural carrier by the stereophonic

pilot signal shall be 5.0 kHz deviation with a tolerance

of +/- 0.5 kHz.

(4)  TV stations transmitting a second audio program must limit

the modulation of the aural carrier by the SAP subcarrier

to 15 kHz deviation.

(5)  TV stations transmitting multiplex signals on the aural

carrier for non-program related purposes must limit the

modulation of the aural carrier by the arithmetic sum of

all subcarriers, other than the stereophonic and second

audio program, to 3 kHz deviation.

(6)  Total modulation of the aural carrier by multichannel

sound shall not exceed 73 kHz.