NetWare 3.12 OS Patches                           (312PT5.EXE)

---------------------------------------------------------------

This package is made up of the following files:

312PT5 DOC15CB5 - This document

312PT5 EXE15CB5 - Binary self-extracting file.

---------------------------------------------------------------

NOVELL PRODUCT and VERSION: NetWare 3.12

ABSTRACT:

This file contains the latest operating system test patches for

NetWare 3.12. Included with this prerelease are all patches for

all user-versions of NetWare 3.12.  The patches included have

been system tested by Novell.

---------------------------------------------------------------

DISCLAIMER

THE ORIGIN OF THIS INFORMATION MAY BE INTERNAL OR EXTERNAL TO

NOVELL.  NOVELL MAKES EVERY EFFORT WITHIN ITS MEANS TO VERIFY

THIS INFORMATION.  HOWEVER, THE INFORMATION PROVIDED IN THIS

DOCUMENT IS FOR YOUR INFORMATION ONLY.  NOVELL MAKES NO

EXPLICIT OR IMPLIED CLAIMS TO THE VALIDITY OF THIS INFORMATION.

---------------------------------------------------------------

The change from 312PT4 to 312PT5 is that UNLDIRFX.NLM has been

removed due to a potential problem this patch may cause.

Versions of this patch prior to 9-13-94 can cause GPPE abends,

often during the process of deleting a user. The 9-13-94 or

later version of this patch which has corrected the problem is

available from the 312ITX.EXE download from the NSD forum.

---------------------------------------------------------------

BCKTTSFX NLM

============

SYMPTOM

Typically after re-booting the server and going though the

process of backing out the TTS files the server would hang with

no messages.

SOLUTION

Apply BCKTTSFX NLM       985 04-26-94   9:00a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01, V4.02

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

This Patch corrects a problem with backing out more than 263

Transaction Tracking System (TTS) files.

It is important to Load BCKTTSFX.NLM after loading PATCH

MANAGER in the STARTUP.NCF file to allow the patch to work

properly.

Installation Instructions:

NCF FILE: STARTUP.NCF

---------------------------------------------------------------

CRDIRFIX NLM

============

SYMPTOM

The following error is return if an attempt is made to create a

directory with an blank character(alt-25) as the last character

in the name.

"Unable to create directory"

SOLUTION

Apply CRDIRFIX NLM      1361 04-26-94   9:03a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Fix to allow the addition of a blank Character (alt-255) to the

end of a directory name.

3.11 allows the user to create directories for security reasons

that have a "blank"(alt-255) as the last character of the

directory name.  To allow NetWare v3.12 and v4.01 to have a

blank Character (alt-255) to the end of a directory name, apply

this patch.

example:  md test(alt-255)  This works in DOS and on a 3.11

server.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

DIRSPCFX NLM

============

SYMPTOM

Disk space can  not be accounted for.

SOLUTION

Apply DIRSPCFX NLM      1212 04-26-94   9:06a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

The DIRSPCFX.NLM file is a fix to the set parameter "Maximum

Percent of Volume Used By Directory."  The limits of this set

parameter are 5 to 50 percent default and the setting is 13

percent.

The set parameter "Maximum Percent of Volume Used By

Directory". can use double the disk space set.

Example; If the Set parameter is 13 percent, the actual space

that can be used by Extended Attributes is 26 percent.

CAUSE

The Set parameter only searched one of the mirrored tables for

extended attributes to verify that the limit had been met,

which allows the set parameter to use twice as much space as

indicated. The Table for Extended Attributes will grow;

however, it will never decrease in size.  So, after the space

is allocated for the EA table, it can not be used for any thing

other than Extended Attributes.

Installation Instructions:

NCF FILE: STARTUP.NCF

---------------------------------------------------------------

DIRSRCFX NLM

============

SYMPTOM

When running out of short term alloc memory, the server

ABEND's.

SOLUTION

Apply DIRSRCFX NLM      1087 05-09-94  11:00a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Changed procedure INWDirectorySearch to first look at the code

returned from the GetDirectoryHandle call made above before

referencing the DirHandle.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

EAACCFIX NLM

============

SYMPTOM

The following Error message: SYS0002 Access Denied is received

when trying to copy a file with Extended Attributes from a

NetWare v3.12 file server if the users only has Read and

FileScan rights.

SOLUTION

Apply EAACCFIX NLM       970 04-26-94   9:06a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

EADATFIX NLM

============

SYMPTOM

A problem exists with Extended Attributes when performing a

drag and drop COPY of a directory structure from an OS/2

client.  Which would cause the Extended Attribute for the new

Parent Directory to be corrupt.

VREPAIR returned the following error message:

Error at extended directory entry xxxx

Invalid Data error

Invalid Score card error

SOLUTION

Apply EADATFIX NLM      1181 04-26-94   4:33p

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

To install the patch, load patchman for the Operating System

which is running and the version of EADATFIX.NLM in the same

directory.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

EAREPLFX NLM

============

SYMPTOM

The following errors could be seen on a OS/2 Client:

1.   "The Drive ";,is not a valid Drive"

2.   " " " is either read-only or is being used by another

program"

This errors would be received if using the drives ICON from the

DESKTOP to view a NetWare volume and most often with a CDRom

mounted volume.  Sometimes after receiving these errors the

connection to the File Server would be lost.

SOLUTION

Apply EAREPLFX NLM      1182 04-26-94   9:00a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Also it is suggested to use NetWare Requester for OS/2 v2.10 to

help correct the problem.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

ELEVFIX  NLM

============

SYMPTOM

A window exists where two identical requests to access the same

sector on the hard disk could get placed in the elevator queue

at the same time resulting in a file server lock up.

The file server may possibly hang when a system is in the

process of re-mirroring and when there is extremely high I/O

traffic on the bus.

SOLUTION

Apply ELEVFIX  NLM      1717 04-26-94   9:08a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

GLOCKFIX NLM

============

SYMPTOM

NWScanPhysicalLocksByFile fails if more than 23 locks on a

file.

NWScanPhysicalLocksByFile is an assembly call.  It is making

the NCP 23 238. The lanalyzer shows that garbage is returned by

this NCP when more than 23 locks are placed on a file.

SOLUTION

Apply GLOCKFIX NLM      1080 03-09-94   8:30a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

GNSFIX   NLM

============

SYMPTOM

This patch fixes a problem found when re-booting and attempting

to get a connection to the file server.  If all of the

connections are in use, (e.g. 20 for the 20-user version, 50

for the 50-user version, etc.), and one of the users re-boots,

the user will not be able to log back in to the server. This

problem will only occur if there is only one network.  If the

user is on an internet, a re-connection will be possible.

The problem is that once a user re-boots, his connection is not

terminated at the server until a time out limit is reached

(usually 15 minutes).

SOLUTION

Apply GNSFIX   NLM      1099 03-28-94   8:47a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

This patch corrects the problem by checking the source node

address of the incoming packet to the  node address of all

workstations the server currently thinks it is attached to.  If

a match is found the server responds to the workstation, thus

re-creating a connection in the same slot the workstation had

prior to its reboot.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

IPXRCVFX NLM

============

SYMPTOM

These patches fix a problem with receiving a GPPE ABEND because

of the generation of an NCP request that IPX.386 did not handle

correctly, thus causing an invalid fragment count.

A problem has been encountered with the NetWare SFT III

operating system that is produced by Running ARCServe 4.0 on

one of the mirrored NetWare SFT III servers.  If ARCServe is

backing up a file to the tape and the secondary server restarts

for any reason, upon re-synchronizing the memory the operating

system will ABEND with a General Protection Processor Exception

(GPPE) error.

This error only occurs if a file is being copied to the tape at

the time.

If ARCServe is loaded but not in the process of copying a file

to  tape, then the problem is not apparent.

CAUSE

The problem is that an NCP request is generated that IPX.386

did not handle correctly, causing an invalid fragment count

that would bring down the primary server.  Because the primary

server is being re-mirrored to the secondary server, the

secondary server dies as well.

SOLUTION

Apply IPXRCVFX NLM       976 04-26-94   9:04a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V3.11 SFT III

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: MSAUTO.NCF, IOAUTO.NCF

The patch needs to be loaded in both the MSEngine and IOEngine.

1. Copy the IIPXRCFX.NLM and PM311IO.NLM files to the NetWare

boot directory.

2. Copy the IPXRCVFX.NLM and PATCHMAN.NLM files to the

SYS:SYSTEM directory.

3. Edit the IOSTART.NCF file to load the PM311IO.NLM and

IIPXRCFX.NLM files.

4. Edit the MSSTART.NCF file to load PATCHMAN.NLM and

IPXRCVFX.NLM files.

---------------------------------------------------------------

MMACCFIX NLM

============

SYMPTOM

The workstation will hang while performing reads from the file

server.  High (100%) utilization at the server console.  By

typing "scan for new devices" at the console prompt the high

utilization drops but will climb back up over time.

SOLUTION

Apply MMACCFIX NLM      1006 04-26-94   8:54a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

MMACCFIX.NLM fixes this condition by patching the media manager

accelerator module of the NetWare 4.01 OS code.  This patch is

recommended for general application on all 4.01 NetWare

servers.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

PBWANFIX NLM

============

SYMPTOM

WAN links saturated with packets.

SOLUTION

Apply PBWANFIX NLM      3028 04-19-94   9:41a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Description of problem addressed by PBWANFIX.NLM.  Due to the

nature of packet burst, it is very easy to saturate WANS with

packets that have already been sent and arrived correctly.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

PM312    NLM

============

SYMPTOM

SOLUTION

Apply PM312    NLM      9221 03-24-94  11:06a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

This README file accompanies PM312.NLM.  The following files

are included for this revision:

PM312.NLM replaces any and all previous versions of PM312.NLM.

PM312.NLM is the Patch Manager which tracks and manages all

official patches for NetWare 386 v3.12.

Patches for NetWare 386 v3.12 consist of three types:

DYNAMIC -- Dynamic patches are implemented as .NLM files that

can be loaded/unloaded while the server is running.  Unloading

a dynamic patch will restore the Operating System to its

original "un-patched" state.

SEMI-STATIC -- Semi-static patches can also be loaded while the

server is running, but they cannot be unloaded.  It is not

possible to undo the effects of a semi-static patch without

first downing the server and bringing it back up without

loading the semi-static patch.

STATIC -- A static patch is a DOS executable program that

modifies the SERVER.EXE file.  This type of patch is applied

once and its effects are permanent.  It is suggested that a

backup of the original SERVER.EXE file be made before applying

a static patch.  (In other words, do not patch the SERVER.EXE

on your original distribution diskette])

Dynamic and semi-static patches modify the Operating System in

memory, not on the disk.  This means that dynamic and semi-

static patches must be loaded each time the Operating System is

brought up in order for any 'fixes' to take effect. It is

recommended that the command "LOAD <patch>" be placed in the

file server's AUTOEXEC.NCF file to ensure that the patch is

always in effect as soon as the server is brought up.

In order to see which patches are currently loaded on the

system, type "PATCHES" at the file server command line.  The

patches will then be grouped and displayed according to their

type (i.e. - STATIC, SEMI-STATIC, or DYNAMIC).  Typing

"MODULES" at the server console will show which dynamic patches

have been loaded, but will not show semi-static or static

patches.

To install PM312.NLM, log into the server as supervisor or

supervisor equivalent and copy PM312.NLM into the SYS:SYSTEM

subdirectory.  The Patch Manager (PM312.NLM) must be loaded

before any dynamic or semi-static patches can be loaded.  If

PM312 is not already loaded, the <patch>.NLM will attempt to

locate PM312 and, if found, will load it automatically.  PM312

only needs to be loaded once; all NLM patches can then be

loaded.  PM312 cannot be unloaded until all patches that rely

on it are first unloaded.

---------------------------------------------------------------

REALMFIX NLM

============

SYMPTOM

This patch deals with applications that directly control

hardware devices through NetWare (i.e. Jukeboxes, worms, tapes,

etc.).  If an error condition occurs such as reaching the end

of a tape, an error code is returned to the calling

application.  The error code would be scrambled, resulting in

the wrong error passed to the application.  The result, as far

as the Server (i.e. ABEND, hang, etc.) would depend on the

specific application.   The patch applies to applications which

use a DAI interface to communicate with the server's hardware.

Check with the vendor of the application to verify the use of

the DAI interface and apply the patch if necessary.

Application requests made to a device driver using the device

IOCTL and DeviceBlockingIOCTL API's are aborted if NetWare is

in the process of switching to real mode.

Application requests made to a device driver using the

DeviceIOCTL and DeviceBlockingIOCTL API's are aborted if

NetWare is in the process of switching to real mode (i.e. the

DOS Mode semaphore is set).  The API's should instead delay the

execution of the request.

FIXED 2 DAI emulation bugs. Fixed the device IOCTL routine to

return the driver specified error code instead of the media

manager code.  Fixed the IOCTL routines to postpone the

execution of the IOCTL if the DOSDiskSemaphore is set instead

of returning with an error.

SOLUTION

Apply REALMFIX NLM      3207 02-08-94   9:14a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

RKIELFIX NLM

============

SYMPTOM

Message Correction for the following ABEND Message: "Richard

Kiel memorial ABEND #27"

SOLUTION

Apply RKIELFIX NLM      1044 05-04-94   2:38p

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

The new ABEND message is "Invalid entry in message list.".

In the OS there is a process that checks the consistency of the

message list. When this list is found to be invalid the OS will

return this ABEND message.

Follow the Guidelines for troubleshooting in the System

Messages Manual Appendix "A".

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

RTCHGFIX NLM

============

SYMPTOM

This patch fixes a problem with packet signing after a switch

over occurs on either an SFT III server or a router switch over

(i.e. networks with redundant routes).  When the switch over

occurs the packet size would be renegotiated between the server

and the workstation.  The server in would send unsigned packets

to the workstation during the renegotiation process.  The

workstation, expecting signed packets will discard all of the

packets from the server attempting the renegotiation.  The

workstation would eventually timeout or appear to be hung.  The

key to the patch is that packet signing would have to be

enabled or order for this problem to happen.

SOLUTION

Apply RTCHGFIX NLM       984 12-21-93  11:30a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

RTRTIME  NLM

============

SYMPTOM

The speed of the network link is slow enough to require

increasing timeout values to more than the default value.

SOLUTION

Apply RTRTIME  NLM      2054 02-23-94   4:19p

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

This patch provides a new server console command "ROUTER

TIMEOUT"  which adjusts the RIP and SAP information timeout

value.  This patch would be used on a network where the speed

of the network link is slow enough to require increasing

timeout values to more than the default value.

The default RIP/SAP timeout value is 4 minutes, which means

that the server router will keep Server or Network information

for 4 minutes without that information being updated.  If the

server does not get updated RIP/SAP information within the time

specified by the timeout value, the RIP/SAP information is

removed from the server's tables.

This new console command allows the router timeout value to be

changed in the range of 4 to 255 minutes.

SYNTAX:

ROUTER TIMEOUT 100

This command would change the timeout default from 4 to 100

minutes.

RTRTIME.NLM is intended for 386 v3.12 and is a dynamic patch

which is loaded at the file server console as a NetWare

Loadable Module.  RTRTIME.NLM requires the latest version of

PATCHMAN.NLM in order to load.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

SPXDDFIX NLM

============

SYMPTOM

This fixes a problem with the way SPX handles retried connect

requests.  If the connection had already transitioned into

SESSION_TERMINATED when the retry came in, it was treated as a

new connection request.

Also fixes a problem where SPX was still acknowledging data

packets that arrived after SPXTerminateConnection was called.

This could lead to incorrect handling of the data by the

application, leaving both endpoints with different information

about the state and completeness of the data. NOTE: This SPX

problem is masked by the SPXS.NLM that ships with NMS 2.0 and

SPXS.NLM included in STRTL2.EXE.  The fix for this specific

issue is addressed by SPXDDFIX.NLM.

SOLUTION

Apply SPXDDFIX NLM      1148 03-10-94   9:48a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Before the patch, data that arrived after the application

initiated a disconnect by calling SPXTerminateConnection, was

acknowledged and delivered to the application.  Depending on

how the application was written this could give the

transmitting side an invalid picture of the guaranteed delivery

of the data.  The patch causes all inbound packets after

SPXTerminateConnection has been called to just be dropped.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

SPXFIX2  NLM

============

SYMPTOM

Fix SPX code in the case where an SPX terminate ack is received

on a non-existent session.  When the terminate ack is received

and the session does not exist, this fix returns the ECB to IPX

rather than re-posting it back to the application socket.

SOLUTION

Apply SPXFIX2  NLM      1035 11-23-93   9:13a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

This bug is a side effect of SPXFIX2 v2.00 (which is native to

NetWare v3.12) that leaves SPX terminate ack packets in system

ECB's rather than copying them into application ECB's.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

SPXNSFIX NLM

============

SYMPTOM

If the session did not exist, SPX failed to interpret terminate

ACKs as system packets.

SOLUTION

Apply SPXNSFIX NLM      1044 05-26-94   8:45a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.11, V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

When an application errors out because the session does not

exist, SPX posts the system buffer to the socket rather than

returning it to the Link Support Layer (LSL).

The SPXNSFIX.NLM for NetWare 4.01, 3.12, and 3.11 fixex the way

that SPX handles the Terminate Acknowledgement packets on

sessions that do not exist.

Installation Instructions:

NCF File:

---------------------------------------------------------------

SYNCMFIX NLM

============

SYMPTOM

One drive off of one HBA is mirrored to two drives both of

which are on the other HBA.  (A total of 3  drives mirroring

the same data).  The single drive off of the first HBA is in

sync and the other two drives off of the second HBA are

mirrored to the first drive.  The server is downed while the

two secondary mirrored drives are still out of sync, (install

NLM shows them still to be in the mirroring process).  When

SERVER.EXE is invoked, INSTALL.NLM shows the primary drive as

In Sync and one of the secondary drives as In Sync and the

other as Out Of Sync.  The two secondary drives are the only

ones showing disk activity as if they are mirroring each other

and ignoring the actual in sync primary drive.

SOLUTION

Apply SYNCMFIX NLM      1025 04-26-94   8:58a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12, V4.01

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

The problem is resolved by loading the SYNCMFIX.NLM patch. You

must load the patch manager and the SYNCMFIX.NLM in the

startup.ncf before the disk driver or move the disk driver load

statements to the AUTOEXEC.NCF and have them loading after the

patch manager and the SYNCMFIX.NLM patch.

Installation Instructions:

NCF FILE: STARTUP.NCF

---------------------------------------------------------------

UNLDIRFX NLM

============

SYMPTOM

ABEND "UnLockDirectoryEntry called with entry that was already

unlocked" happens in INWPurgeTrustee when UnLockDirectoryEntry

is called with TNode->TFileEntryNumber which sometimes gets

overwritten by the call to ImmediateReuseGetDirectoryEntry.

Whenever the directory structure for the file entry isn't in

RAM, ImmediateReuseGetDirectoryEntry reads in a directory block

over the top of the current block and invalidates the TNode

pointer. NOTE:Locking the file entry doesn't guarantee the

TNode is locked.

SOLUTION

Apply the current UNLDIRFX.NLM (9-13-94 or later), currently

available in the 312ITX.EXE download file on the NSD forum.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

UNLOADFX NLM

============

SYMPTOM

NLMs dynamically importing can ABEND if not unloaded in same

order.

The OS needs a patch to check for a NULL pointer on unload.

SOLUTION

Apply UNLOADFX NLM       992 11-11-93   2:32p

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

WSMSGFIX NLM

============

SYMPTOM

This patch fixes a problem with WS's  when sending double byte

broadcast messages.  The double byte characters would get cut

in half.

To reproduce the problem:

1) Login from 2 workstations.

2) From one of the workstation send an double-byte message

which is longer than 55 bytes to the other workstation. send

"XXXXXXXXX..........." to STATION_ID XXXXXXXXXX.... is a double

byte message longer than 55 bytes.

3) At the recipient workstation press Ctrl and ENTER key to

clear the first part of the message.

4) The second part of the message appears on the recipient, but

the message is collapsed.

This is not a utility problem, but rather a server problem.

When the message is too long to be displayed on the screen, the

server divides the message without checking to see whether-or-

not the character is double-byte.  If the division takes place

on the second half of a double-byte character, the characters

displayed are changed.  Double-byte characters must not be

split.

SOLUTION

Apply WSMSGFIX NLM      1132 12-16-93  10:00a

Solution Specifics:

Different versions of this patch are available for the

following NetWare Operating System's:

V3.12

Example: v3.11 is in 311ITx.EXE or 311PTx.EXE,

v3.12 is in 312ITx.EXE or 312PTx.EXE,

v4.01 is in 401ITx.EXE or 401PTx.EXE,

v4.02 is in 402ITx.EXE or 402PTx.EXE.

v3.11 SFT is in SFTITx.EXE.

Installation Instructions:

NCF FILE: AUTOEXEC.NCF

---------------------------------------------------------------

Self-Extracting File Name:  312PT5.EXE     Revision:  A

Files Included     Size     Date      Time

\

312PT5.TXT         (This File)

PM312.NLM       9221   03-24-94   11:06a

GNSFIX.NLM       1099   03-28-94    8:47a

ELEVFIX.NLM       1717   04-26-94    9:08a

RTRTIME.NLM       2054   02-23-94    4:19p

SPXFIX2.NLM       1035   11-23-93    9:13a

BCKTTSFX.NLM        985   04-26-94    9:00a

CRDIRFIX.NLM       1361   04-26-94    9:03a

DIRSPCFX.NLM       1212   04-26-94    9:06a

DIRSRCFX.NLM       1087   05-09-94   11:00a

EAACCFIX.NLM        970   04-26-94    9:06a

EADATFIX.NLM       1181   04-26-94    4:33p

EAREPLFX.NLM       1182   04-26-94    9:00a

GLOCKFIX.NLM       1080   03-09-94    8:30a

IPXRCVFX.NLM        976   04-26-94    9:04a

MASTERTD.TXT       1370   08-17-94    3:14p

MMACCFIX.NLM       1006   04-26-94    8:54a

PBWANFIX.NLM       3028   04-19-94    9:41a

REALMFIX.NLM       3207   02-08-94    9:14a

RKIELFIX.NLM       1044   05-04-94    2:38p

RTCHGFIX.NLM        984   12-21-93   11:30a

SPXDDFIX.NLM       1148   03-10-94    9:48a

SPXNSFIX.NLM       1044   05-26-94    8:45a

SYNCMFIX.NLM       1025   04-26-94    8:58a

UNLOADFX.NLM        992   11-11-93    2:32p

WSMSGFIX.NLM       1132   12-16-93   10:00a

---------------------------------------------------------------

Any trademarks referenced in this document are the property of

their respective owners.  Consult your product manuals for

complete trademark information.

---------------------------------------------------------------