[ PROTOCOLS:MSTD-1782-TEST.DOC ]                             [ 4/88 ]

DCEC PROTOCOL LABORATORY

TELNET PROTOCOL (TELNET)

MIL-STD-1782

CERTIFICATION TESTS INDEX

This Certification Tests Index provides information on the

derivation, organization, and function of tests specified for

TELNET within the DCEC Protocol Laboratory.

This document is divided into five sections:

TELNET TRACEABILITY INDEX;

USER TELNET TESTS INDEX;

SERVER TELNET TESTS INDEX;,

TELNET TEST SCENARIOS INDEX;

and TELNET SCENARIOS AND TESTS DESCRIPTIONS.

TELNET TRACEABILITY INDEX: TELNET TEST NUMBERS versus TELNET

MIL-STD-1782 References...

The table shows, for MIL-STD-1782, the TELNET Test Numbers which

may be regarded as the "principal tests" of each required

function, operation, option, mode, response, or state.

----------

NOTE...

Although the Test Numbers in the USER scenarios replicate the

Test Numbers in the SERVER scenarios a common Test Number does

not indicate equivalent, complementary, or corresponding tests;

---------

USER TELNET TESTS INDEX: TELNET TEST NUMBERS versus USER TELNET

Commands/Primitives/Options/Modes...

The table shows the TELNET Test Numbers which may be regarded as

the "principal tests" of each USER TELNET Command or Primitive

and/or Option or Mode.

----------

SERVER TELNET TESTS INDEX: TELNET TEST NUMBERS versus SERVER

TELNET Response Codes...

The table shows the TELNET Test Numbers which may be regarded as

the "principal tests" of each SERVER TELNET Response (or State,

etc.) to the indicated TELNET Commands or Primitives.

TELNET TEST SCENARIOS INDEX: TELNET TEST SCENARIO FILES versus

TELNET TEST NUMBERS...

The table shows, for each TELNET Test Number, the UNIX filenames

of the TELNET test Scenario Files in which it may be regarded as

a "principal test objective,.

---------

TELNET SCENARIOS AND TESTS DESCRIPTIONS...

This section provides a brief narrative of the scope and

objectives of each TELNET test Scenario File and a narrative or

graphic operational description of each TELNET Test Number.

TELNET TRACEABILITY INDEX

TELNET TEST NUMBERS

versus

TELNET MIL-STD-1782 References

The following table shows, for MIL-STD-1782, the TELNET Test

Numbers which may be regarded as the "principal tests" of each

required function, operation, option, mode, response, or state.

Test Numbers not indicated here may be used to establish

necessary predecessor conditions for these tests.

Although the Test Numbers in the USER scenarios replicate the

Test Numbers in the SERVER scenarios a common Test Number does

not indicate equivalent, complementary, or corresponding tests.

Reference                                   User Test Number

---------                                   ----------------

4.2.2                                       1, 2, 3, 4, 5, 6

4.2.3                                       1, 2, 3, 4, 5, 6

4.2.4                                       18, 20

4.3                                         0, 14

4.3.1.2                                     0

4.3.2                                       19

4.4.1                                       17

4.4.2                                       11

4.4.3                                       8

4.4.4                                       9

4.4.5                                       10

4.5                                         7

4.6.1.1                                     15, 16

4.6.1.3.2                                   13

4.7.1.b                                     12

4.8                                         0

APPENDIX A                                  3

APPENDIX B                                  1

APPENDIX C                                  2

APPENDIX D                                  6

APPENDIX E                                  4

APPENDIX F                                  5

TELNET TRACEABILITY INDEX

Continued

Reference                                   Server Test Number

---------                                   --------------------

4.2.2                                       1, 2, 3, 4, 5, 6

4.2.3                                       1, 2, 3, 4, 5, 6

4.3                                         0, 11

4.3.1.2                                     0

4.4.2                                       7

4.5                                         7

4.5.1                                       15

4.6.1.1                                     11, 12

4.6.1.3.3                                   17

4.6.1.3.4                                   16

4.6.1.3.5                                   8

4.6.1.3.6                                   9

4.6.1.3.7                                   10

4.7.1.b                                     14

4.8                                         0

APPENDIX A                                  3

APPENDIX B                                  1

APPENDIX C                                  2

APPENDIX D                                  6

APPENDIX E                                  4

APPENDIX F                                  5

USER TELNET TESTS INDEX

USER TEST NUMBERS

versus

USER TELNET Commands/Primitives/Options/Modes

The following table shows the TELNET Test Numbers which may be

regarded as the "principal tests" of each U8ER TELNET Command or

Primitive and/or Option or Mode.

Test Numbers not indicated here may be used to establish

necessary predecessor conditions for these tests.

Although the Test Numbers in the USER scenarios replicate the

Test Numbers in the SERVER scenarios a common Test Number does

not indicate equivalent, complementary, or corresponding tests.

Command or Primitive     Option              User Test Number

--------------------     ------              --------------------

Open connection                              0

Network Virtual Terminal Go Ahead status     0

Network Virtual Terminal Echo status         0

Response to DO option request

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to DO option request for enabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to DON'T option request for disabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to WILL option request for enabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to WONT option--announcement of disabling

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to request to enable option peer has enabled

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Correct implementation of enabled option

Remote echo         1

GoAhead             2

Binary              3

Status              6

Correct implementation after disabling option

Remote echo         1

GoAhead             2

Binary              3

Status              6

Correct implementation of option when both sides enabled

GoAhead             2

Binary              3

Status              6

Generation of synch embedding Data Mark (DM) 7

Generation of Are You There command (AYT)    8

Generation of Erase Character command (EC)   9

Generation of Erase Line command (EL)        10

Generation of Abort Output command (AO)      11

Generation of No Operation (NOP)             12

Generation of Break command (BRK)            13

Transmission of ASCII printable characters   14

Transmission and receipt of Newline (CRLF)   15

Transmission and receipt of Carriage Return (CR NULL) 16

Generation of Interrupt Process command (IP) 17

Non-transmission of request for previously refused option

Binary              18

Generation of GoAhead command (GA)           19

Non-transmission of subnegotiation for disabled option

Status              20

Close connection                             21

SERVER TELNET TESTS INDEX

TELNET TEST NUMBERS

versus

SERVER TELNET Responses to each SERVER TELNET Command or

Primitive.

The following table shows the TELNET Test Numbers which may be

regarded as the "principal tests" of each SERVER TELNET Response

(or State, etc.) to the indicated SERVER TELNET Commands or

Primitives.

Test Numbers not indicated here may be used to establish

necessary predecessor conditions for these tests.

Although the Test Numbers in the USER scenarios replicate the

Test Numbers in the SERVER scenarios a common Test Number does

not indicate equivalent, complementary, or corresponding tests.

Command or Primitive     Option              Server Test Number

--------------------     ------              --------------------

Open connection                              0

Network Virtual Terminal go ahead status     0

Network Virtual Terminal echo status         0

Response to DO option request

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to DO option request for enabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to DON'T option request for disabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to WILL option request for enabled option

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to WONT option--announcement of disabling

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Response to request to enable option peer has enabled

Remote echo         1

GoAhead             2

Binary              3

Timing Mark         4

Extended Options    5

Status              6

Correct implementation of enabled option

Remote echo         1

GoAhead             2

Binary              3

Status              6

Correct implementation after disabling option

Remote echo         1

GoAhead             2

Binary              3

Status              6

Correct implementation of option when both sides enabled

GoAhead             2

Binary              3

Status              6

Response to synch                            7

Response to Are You There (AYT)              8

Response to Erase Character (EC)             9

Response to Erase Line (EL)                  10

Receipt of ASCII printable characters        11

Transmission and receipt of newline (CRLF)   12

Transmission and receipt of carriage return (CR NULL) 13

Response to No Operation (NOP)               14

Response to Data Mark (DM) with no TCP Urgent 15

Response to Abort Output (AO)                16

Response to Interrupt Process (IP)           17

Close connection                             18

TELNET TEST SCENARIOS INDEX

TELNET TEST SCENARIO FILES

versus

TELNET TEST NUMBERS

Within the PROTOLAB VAX-11/750 Test Master System, all TELNET

test Scenario Files may be found in the following UNIX directory:

/usr/users/TSL/test/telnet

The following table shows, for each TELNET Test Number, the UNIX

filenames of the TELNET test Scenario Files in which it may be

regarded as a "principal test objective".

A Test Number may be used, to establish necessary predecessor

conditions for other Test Numbers, in Scenario Files not

indicated here.

Although the Test Numbers in the USER scenarios replicate the

Test Numbers in the SERVER scenarios a common Test Number does

not indicate equivalent, complementary, or corresponding tests.

Test Number              User Scenario File

--------------------     ----------------------------------------

USER_BASIC

0

1

2

3

4

5

6

14

15

16

18

20

21

USER_OPTIONS1

8

9

10

17

19

USER_OPTIONS2

11

13

USER_SYNCH

7

TELNET TEST SCENARIOS INDEX

Continued

Test Number Server       Scenario File

--------------------     ----------------------------------------

SERVER_BASIC

0

1

2

3

4

5

6

11

12

13

14

15

18

SERVER_OPTIONS1

8

9

10

17

SERVER_OPTIONS2

16

SERVER_SYNCH

7

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File USER_BASIC Tests Network Virtual Terminal (NVT)

characteristics of TELNET implementation.

------------------------------------

Test 0  Open, initial NVT GoAHead and Echo status

a. Open

- Can a connection be opened?

- Action: IUT directed to send an open command.

- Verification: Check that message "SERVER TELNET"

received from REF server implementation.

SUCCESS: Connection opened.

FAILURE: Connection not opened.

b. NVT GoAhead status

- Is the user TELNET waiting for the GoAhead before

sending data?

- Action: Send a data string from the IUT to the REF

peer.

- Verification: Check that data not received by REF

peer until the REF has sent a GoAhead.

SUCCESS: Data sent within 60 seconds of receiving a

GoAhead.

FAILURE: Data sent without waiting for GoAhead or

not sent within 60 seconds of receiving GoAhead-

c. NVT echo status

- Is the user not remotely echoing?

- Action: Send a data string REF to the IUT.

- Verification: Check that the data string the REF

has issued is not returned back over the network

connection to the REF.

SUCCESS: Data string not received back by REF.

therefore IUT not remotely echoing.

FAILURE: Data string received back by REF - IUT

remotely echoing.

Test 12  NoOperation (NOP) command

- Can the user TELNET generate a NOP?

- Action: IUT sends a NOP command.

- Verification: Check that REF reports that it has

received the NOP command.

SUCCESS: NOP command received by REF.

FAILURE: NOP command not received by REF.

Test 14  Correct generation of ASCII printable character set

- Can the user TELNET correctly generate the ASCII

printable character set?

- Action: The IUT sends a string containing all the

ASCII printable characters to the REF.

- Verification: Check that the REF receives the

character string sent by the IUT with all characters

correctly transmitted.

SUCCESS: REF receives a correctly transmitted character

string of all the ASCII printable characters.

FAILURE: REF does not correctly receive a character

string of all ASCII printable characters.

Test 15  Transmission and reception of newline character.

- Does the IUT correctly send and receive the Carriage

Return Line-Feed <CR LF> combination?

- Action: The IUT sends a data string with newline

characters embedded. The REF sends a data string with

newline characters embedded to the IUT.

- Verification: Check that the REF receives the data

string with the newlines in the correct places. Check

that the IUT receives the data string with the newlines

in the correct places.

SUCCESS: The IUT user TELNET correctly sends the

<CR LF> combination for newline and correctly

interprets <CR LF> as newline when it is received.

FAILURE: The IUT does not correctly send and/or

interpret the newline character.

Test 16  Transmission and reception of Carriage Return

- Does the IUT correctly transmit a Carriage Return as

<CR NULL> and interpret <CR NULL> as Carriage Return

when it is received?

- Action: IUT sends a data string with a carriage

return embedded in it. The REF sends a data string

with a carriage return embedded in it.

- Verification: Check that the REF reports that it has

received a <CR NULL> from the IUT. Check that the IUT

has received a data string from the REF contains a

carriage return.

SUCCESS: The IUT correctly transmits a <CR NULL> for a

carriage return and can correctly interpret a <CR NULL>

as a carriage return.

FAILURE: The IUT does not correctly transmit <CR NULL>

for carriage return and/or interpret <CR NULL> as

carriage return.

Test 1  Negotiation and Implementation of Remote Echo option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the remote echo option

it has agreed to enable?

Does the IUT correctly disable the remote echo option

when negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to the request

to do the the option, responding to a request for an

option when it is already enabled, responding to a

request to disable the option, responding to a peer's

announcement that it wishes to do an option, and the

peer's refusal to do the option.

If the IUT agrees to enable the remote echo, the REF

sends a data string to the IUT to determine if the

string is remotely echoed back. When the option is

disabled, the REF sends a data string to the IUT to

determine whether the IUT is still remotely echoing.

- Verification: Check the option replies reported as

received by the REF for correctness. If testing

correct implementation of the remote echo, check if the

REF has received back the transmitted data string.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

remote echo. The data string is received by the REF

when the IUT is in remote echo mode; it is not

received when the option is disabled.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Remote Echo option.

Test 2  Negotiation and Implementation of Suppress GoAhead option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the GoAhead option it

has agreed to enable?

Does the IUT correctly disable the GoAhead option

when negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Suppress GoAhead

option, the IUT sends a data string to the REF; no

GoAheads are being generated by the server REF. When

the option is disabled, the REF is inhibited from

sending GoAheads and the IUT attempts to send a data

string to the REF.

- Verification: Check the IUT option replies reported

by the REF for correctness. If testing correct

implementation of the Suppress GoAhead, check if the

REF has received the transmitted data string although

no GoAheads are being generated. If the option is

disabled, check if the IUT waited for the GoAhead to

send data and if the data string was sent within 60

seconds of receiving the GoAhead.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

Suppress GoAhead option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Suppress GoAhead option.

Test 3  Negotiation and Implementation of Binary option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the Binary option it

has agreed to enable?

Does the IUT correctly disable the Binary option when

negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Binary option, the

IUT sends a data string of non-printable ASCII

characters , including one TELNET IAC(377), to the REF;

When the option is disabled, the IUT sends the same

data string to the IUT. When the IUT has agreed that

the REF can transmit in binary, a data string

containing ASCII unprintable characters is sent from

the REF to the IUT.

- Verification: Check the IUT option replies reported

by the REF for correctness. When testing

implementation of the enabled Binary option, check if

the REF has reported receiving two IACs from the IUT.

When the option has been disabled, check the REF to

determine that two IACs have not been transmitted.

When the IUT has agreed that the REF may transmit

Binary, check that the transmitted data string was

correctly interpreted by the IUT.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

Binary option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Binary option.

Test 4  Negotiation of Timing Mark option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

- Verification: Check the IUT option replies reported

by the REF for correctness.

SUCCESS: The IUT correctly responds to the negotiation

sequences.

FAILURE: The IUT does not correctly respond to the

negotiation sequences.

Test 5  Negotiation of Extend Option List option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

- Verification: Check the IUT option replies reported

by the REF for correctness.

SUCCESS: The IUT correctly responds to the negotiation

sequences.

FAILURE: The IUT does not correctly respond to the

negotiation sequences.

Test 6  Negotiation and Implementation of Status option

- Does the IUT user TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the Status option it

has agreed to enable?

Does the IUT correctly disable the Status option when

negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Status option, the

REF sends the subnegotiation command Status Send to the

IUT.

- Verification: Check the IUT option replies reported

by the REF for correctness. When testing

implementation of the enabled Status option, check if

the REF has reported receiving a Status is

subnegotiation reply from the IUT correctly listing the

status of the negotiated options.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables the Status

option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable the Status option.

Test 18  Non transmission of request for a refused option until

a new connection has been made or the status of another option changed.

- Does the IUT user TELNET correctly not send a request

for a previously refused option?

- Action: REF is placed in a state where it refuses

all options. The IUT requests the REF to Do Binary and

is refused. The IUT is directed to again request the

option.

- Verification: Check to see if the REF reports

having received the second option request.

SUCCESS: The IUT correctly did not send the second

option request to the REF.

FAILURE: The IUT sends the second option request to

the REF.

Test 20  Subnegotiation command not sent for an unnegotiated option

- Does the IUT user TELNET correctly not send a

subnegotiation request for an unnegotiated option?

- Action: Status option is not enabled. IUT is

directed to send the subnegotiation command Status Send

to the REF.

- Verification: Check to see if the REF reports

having received the subnegotiation request.

SUCCESS: The IUT correctly did not send the

subnegotiation request to the REF.

FAILURE: The IUT sends the subnegotiation request to

the REF.

Test 21  Close the Telnet Connection (Not an explicit part of

MIL STD 1782)

- Can the IUT user TELNET close the Telnet connection?

- Action: The IUT is directed to close the connection.

- Verification: Check to see if the REF reports

having received the Close.

SUCCESS: The IUT correctly closes the Telnet

connection.

FAILURE: The REF never receives a Close from the IUT.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File USER_OPTIONS1 Tests commonly implemented optional

TELNET commands: Are You There; Erase Character; Erase Line; Go

Ahead; and Interrupt Process.

------------------------------------

Test 8   Are You There (AYT) command

- Can the user TELNET generate an AYT?

- Action: IUT sends a AYT command.

- Verification: Check that REF reports that it has

received the AYT command.

SUCCESS: AYT command received by REF.

FAILURE: AYT command not received by REF.

Test 9   Erase Character (EC) command

- Can the user TELNET generate an EC?

- Action: IUT sends a EC command.

- Verification: Check that REF reports that it has

received the EC command.

SUCCESS: EC command received by REF.

FAILURE: EC command not received by REF.

Test 10  Erase Line (EL) command

- Can the user TELNET generate an EL?

- Action: IUT sends a EL command.

- Verification: Check that REF reports that it has

received the EL command.

SUCCESS: EL command received by REF.

FAILURE: EL command not received by REF.

Test 19  GoAhead (GA) command

- Can the user TELNET generate a GA?

- Action: IUT sends a GA command.

- Verification: Check that REF reports that it has

received the GA command.

SUCCESS: GA command received by REF.

FAILURE: GA command not received by REF.

Test 20  Interrupt Process (IP) command

- Can the user TELNET generate an IP?

- Action: IUT sends a IP command.

- Verification: Check that REF reports that it has

received the IP command.

SUCCESS: IP command received by REF.

FAILURE: IP command not received by REF.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File USER_OPTIONS2 Tests less commonly implemented

optional TELNET commands: Break and Abort Output.

------------------------------------

Test 13  Break (BRK) command

- Can the user TELNET generate a BRK?

- Action: IUT sends a BRK command.

- Verification: Check that REF reports that it has

received the BRK command.

SUCCESS: BRK command received by REF.

FAILURE: BRK command not received by REF.

Test 11  Abort Output (AO) command

- Can the user TELNET generate an AO?

- Action: IUT sends an AO command.

- Verification: Check that REF reports that it has

received the AO command.

SUCCESS: AO command received by REF.

FAILURE: AO command not received by REF.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File USER_SYNCH Tests less commonly implemented optional

TELNET command: Synch.

------------------------------------

Test 7   Synch command (TCP urgent with Data Mark (DM) embedded in

data stream).

- Can the user TELNET generate a Synch?

- Action: IUT sends a Synch command embedded in a data

string.

- Verification: Check that REF reports that it has

received the TCP Urgent and the DM. Check that the DM

is placed in the proper position in the data stream.

SUCCESS: The TCP Urgent and the correctly placed DM

received by REF.

FAILURE: The TCP Urgent and/or the correctly placed DM

not received by REF.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File SERVER_BASIC Tests Network Virtual Terminal (NVT)

characteristics of TELNET implementation.

------------------------------------

test 0  Open, initial NVT GoAHead and Echo status

a. Open

- Can a connection be opened?

- Action: REF sends an open command.

- Verification: Check that message "login" received

from REF user implementation.

SUCCESS: Connection opened.

FAILURE: Connection not opened.

b. NVT GoAhead status

- Is the server TELNET generating the GoAhead?

- Action: Opening the connection should result in

the server Telnet sending a GoAhead to user Telnet.

- Verification: Check that the REF reports having

received a GoAhead.

SUCCESS: A GoAhead generated by the IUT.

FAILURE: IUT not generating GoAhead.

c. NVT echo status

- Is the IUT server remotely echoing?

- Action: Send a data string from the REF to the IUT.

- Verification: Check that the data string the REF

has issued is not returned back over the network

connection to the REF.

SUCCESS: Data string not received back by REF

therefore IUT not remotely echoing.

FAILURE: Data string received back by REF - IUT

remotely echoing.

Test 11  Correct interpretation of ASCII printable character set

- Can the server TELNET correctly interpret the ASCII

printable character set?

- Action: The REF sends a string containing all the

ASCII printable characters to the IUT.

- Verification: Check that the IUT returns to the

Central Driver the same data string sent by the REF.

SUCCESS: IUT correctly interprets the character string

of all the ASCII printable characters.

FAILURE: IUT does not correctly interpret a character

string of all ASCII printable characters.

Test 12  Transmission of newline character.

- Does the IUT correctly send the Carriage-Return

Line-Feed <CR LF> combination?

- Action: The IUT is directed to send a data string

with newline characters embedded.

- Verification: Check that the REF receives the data

string with the newlines in the correct places and

specified with a <CR><LF>(octal 15 followed by octal 12)

SUCCESS: The IUT user TELNET correctly sends the

<CR LF> combination for newline.

FAILURE: The IUT does not correctly send newline

character.

Test 13  Transmission and reception of Carriage Return

- Does the IUT correctly transmit a Carriage Return as

<CR NULL> and interpret <CR NULL> as Carriage Return

when it is received?

- Action: IUT sends a data string with a carriage

return embedded in it. The REF sends a data string

with a carriage return embedded in it.

- Verification: Check that the REF has received a data

string from the IUT containing a <CR> (an octal 15).

Check that the IUT has received a data string from the

REF containing a carriage return.

SUCCESS: The IUT correctly transmits a <CR NULL> for a

carriage return and can correctly interpret a <CR NULL>

as a carriage return.

FAILURE: The IUT does not correctly transmit <CR NULL>

for carriage return and/or interpret <CR NULL> as

carriage return.

Test 14  NoOperation (NOP) command

- Can the server TELNET correctly handle a NOP in the

data stream?

- Action: REF sends a NOP command embedded in a data

string to the IUT .

- Verification: Check that IUT returns to the Central

Driver the data string with the NOP removed and having

had no effect on the data string.

SUCCESS: NOP command correctly handled by IUT.

FAILURE: NOP command not correctly handled by the IUT.

Test 15  Data Mark (DM) command with no corresponding TCP Urgent.

- Can the server TELNET correctly handle a DM in the

data stream?

- Action: REF sends a DM command embedded in a data

string to the IUT. No TCP Urgent is sent.

- Verification: Check that IUT returns to the Central

Driver the data string with the DM removed and having

had no effect on the data string.

SUCCESS: Unaccompanied DM command correctly handled by IUT.n

FAILURE: Unaccompanied DM command not correctly handled

by the IUT.

Test 1  Negotiation and Implementation of Remote Echo option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the remote echo option

it has agreed to enable?

Does the IUT correctly disable the remote echo option

when negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to the request

to do the the option, responding to a request for an

option when it is already enabled, responding to a

request to disable the option, responding to a peer's

announcement that it wishes to do an option, and the

peer's refusal to do the option.

If the IUT agrees to enable the remote echo, the REF

sends a data string to the IUT to determine if the

string is remotely echoed back. When the option is

disabled, the REF sends a data string to the IUT to

determine whether the IUT is still remotely echoing.

- Verification: Check the option replies reported as

received by the REF for correctness. If testing

correct implementation of the remote echo, check if the

REF has received back the transmitted data string.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

remote echo. The data string is received by the REF

when the IUT is in remote echo mode; it is not

received when the option is disabled.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Remote Echo option.

Test 2 Negotiation and Implementation of Suppress GoAhead option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the GoAhead option it

has agreed to enable?

Does the IUT correctly disable the GoAhead option

when negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Suppress GoAhead

option, the IUT sends a data string to the REF; no

GoAheads are being generated by the server REF. When

the option is disabled, the REF is inhibited from

sending GoAheads and the IUT attempts to send a data

string to the REF.

- Verification: Check the IUT option replies reported

by the REF for correctness. If testing correct

implementation of the Suppress GoAhead, check if the

REF has received the transmitted data string although

no GoAheads are being generated. If the option is

disabled, check if the IUT waited for the GoAhead to

send data and if the data string was sent within 60

seconds of receiving the GoAhead.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

Suppress GoAhead option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Suppress GoAhead option.

Test 3  Negotiation and Implementation of Binary option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the Binary option it

has agreed to enable?

Does the IUT correctly disable the Binary option when

negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Binary option, the

IUT sends a data string of non-printable ASCII

characters , including one TELNET IAC(377), to the REF;

When the option is disabled, the IUT sends the same

data string to the IUT. When the IUT has agreed that

the REF can transmit in binary, a data string

containing ASCII unprintable characters is sent from

the REF to the IUT.

- Verification: Check the IUT option replies reported

by the REF for correctness. When testing

implementation of the enabled Binary option, check if

the REF has reported receiving two IACs from the IUT.

When the option has been disabled, check the REF to

determine that two IACs have not been transmitted.

When the IUT has agreed that the REF may transmit

Binary, check that the transmitted data string was

correctly interpreted by the IUT.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables and disables the

Binary option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable and disable the Binary option.

Test 4  Negotiation of Timing Mark option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

- Verification: Check the IUT option replies reported

by the REF for correctness.

SUCCESS: The IUT correctly responds to the negotiation

sequences.

FAILURE: The IUT does not correctly respond to the

negotiation sequences.

Test 5  Negotiation of Extend Option List option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

- Verification: Check the IUT option replies reported

by the REF for correctness.

SUCCESS: The IUT correctly responds to the negotiation

sequences.

FAILURE: The IUT does not correctly respond to the

negotiation sequences.

Test 6  Negotiation and Implementation of Status option

- Does the IUT server TELNET correctly respond to

negotiation requests and denials?

Does the IUT correctly enable the Status option it

has agreed to enable?

Does the IUT correctly disable the Status option when

negotiated?

- Action: REF initiates negotiation sequence and takes

the IUT through the states of responding to a request

to do the option, responding to a request for the

option when it is already enabled, responding to a

command to disable the option, responding to its peer's

announcement that is wishes to enable the option, and

responding to the peer's command to disable the option.

If the IUT agrees to enable the Status option, the

REF sends the subnegotiation command Status Send to the IUT.

- Verification: Check the IUT option replies reported

by the REF for correctness. When testing

implementation of the enabled STATUS, check if the REF

has reported receiving a Status is subnegotiation reply

from the IUT correctly listing the status of the

negotiated options.

SUCCESS: The IUT correctly responds to the negotiation

sequences. The IUT correctly enables the Status

option.

FAILURE: The IUT does not correctly respond to the

negotiation sequences and/or the IUT does not correctly

enable the Status option.

Test 18  Close the Telnet Connection (Not an explicit part of

MIL STD 1782)

- Does the IUT server TELNET respond correctly when the

user TELNET closes the connection.

- Action: The REF sends a command to close the

connection.

- Verification: Check to see if the REF reports the

connection closed.

SUCCESS: The IUT correctly responds to a close of the

Telnet connection.

FAILURE: The IUT does not correctly respond to a close.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File SERVER_OPTIONS1 Tests commonly implemented optional

TELNET commands: Are You There; Erase Character; Erase Line;

GoAhead; and Interrupt Process.

Test 8  Are You There (AYT) command

- Can the server TELNET correctly respond to an AYT?

- Action: REF sends an AYT command.

- Verification: Check that REF reports that it has

received a visible response from the IUT.

SUCCESS: Receipt of AYT command causes IUT to respond

with a visible response.

FAILURE: No visible response returned by IUT after

receiving an AYT command.

Test 9  Erase Character (EC) command

- Can the server TELNET correctly respond to an EC?

- Action: REF sends a EC command embedded in a data

string.

- Verification: Check that IUT sends to the Central

Driver the data string with the character immediately

preceding the EC deleted.

SUCCESS: IUT correctly handles EC command by deleting

immediately preceding character.

FAILURE: IUT does not correctly handle EC command -

the character immediately preceding the EC is not

deleted.

Test 10  Erase Line (EL) command

- Can the user TELNET correctly respond to an EL?

- Action: REF sends a EL command embedded in a string

string containing three lines..

- Verification: Check that IUT returns to the Central

Driver the received data string with the line

immediately preceding the EL command deleted.

SUCCESS: IUT correctly handles EL by deleting

immediately preceding line.

FAILURE: IUT does not correctly handled EL -- the

correct line was not deleted.

Test 17  Interrupt Process (IP) command

- Can the server TELNET correctly respond to an IP?

- Action: REF sends a IP command.

- Verification: Check that REF reports that it has

received the IUT cursor indicating that the process

being run has been interrupted. (The process being run

on the IUT was the server Remote Driver.)

SUCCESS: IUT correctly responds to IP by interrupting

its running process.

FAILURE: IUT does not correctly respond to IP command.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File SERVER_OPTIONS2 Tests optional TELNET command:

Abort Output.

------------------------------------

Test 11  Abort Output (AO) command

- Does the server TELNET respond correctly to AO?

- Action: REF sends an AO command.

- Verification: Check that REF reports that it has

received a Synch. Then the IUT is directed to send a

data string and the REF checked to see if the data

string is received. It should not be received.

SUCCESS: The IUT responds to an AO by sending the REF a

Synch and not sending any further output data across

the TELNET connection.

FAILURE: The IUT either does not respond to an AO with

a Synch and/or continues to send output data to the

peer protocol.

TELNET SCENARIOS AND TESTS DESCRIPTIONS

Scenario File SERVER_SYNCH Tests optional TELNET command: Synch.

------------------------------------

Test 7  Synch command (TCP urgent with Data Mark (DM) embedded in

data stream).

- Does the server TELNET correctly respond to a Synch?

- Action: REF sends a series of Synch commands

embedded in a very long data string.

- Verification: Check the data string returned to the

Central Driver by the IUT to determine if any data

immediately preceding a Synch has been lost. The loss

of this data indicates that the IUT has responded

correctly to the Synch. However, if no data is lost

the IUT may also have handled the Synch correctly.

SUCCESS: Data immediately preceding a Synch was lost or

no data was lost indicates successful handling of the

Synch by the IUT.

FAILURE: If data transmitted after the last Synch was

lost this indicates that the Synch is not being

correctly handled by the IUT.