[ PROTOCOLS:MSTD-1778-TESTS.DOC ]                           [ 4/88 ]

UNISYS Corporation

6 February 1987                   -1-                       TM-WD-8801/206/00

DCEC PROTOCOL LABORATORY

TRANSMISSION CONTROL PROTOCOL (TCP)

MIL-STD-1778

TRACEABILITY MATRIX

This Traceability Index provides information on the derivation,

organization, and function of tests specified for TCP within the DCEC

Protocol Laboratory.

This document is divided into five sections:

TCP TRACEABILITY INDEX;

TCP TESTS INDEX;

TCP TEST SCENARIOS INDEX;

TCP SCENARIOS AND TESTS DESCRIPTIONS.

-------------

TCP TRACEABILITY INDEX: TCP TEST NUMBERS VERSUS TCP MIL-STD-1778

References...

The table indicates the cross reference between the Test Scenarios and

the applicable section in MIL-STD-1778 regarding each required

function, operation, option, mode, response, or state.

-------------

TCP TESTS INDEX: TCP TEST NUMBERS versus TCP

Commands/Primitives/Options/Modes...

The table shows the TCP Test Numbers which may be regarded as the

"principle tests" of each TCP Command or Primitive and/or Option or

Mode.

-------------

TCP TEST SCENARIOS INDEX: TCP TEST SCENARIO FILES versus TCP TEST

NUMBERS...

The table shows, for each TCP Test Number, the UNIX filenames of the

TCP Test Scenario Files in which it appears.

-------------

TCP SCENARIOS AND TESTS DESCRIPTIONS...

This section provides a brief narrative of the scope and objectives of

each TCP Test Scenario File and a narrative or graphic operational

description of each TCP Test Number.

UNISYS Corporation

6 February 1987                   -2-                       TM-WD-8801/206/00

TCP TRACEABILITY MATRIX

TCP TEST NUMBERS

versus

TCP MIL-STD-1778 References

The table indicates the cross reference between the TCP tests and the

applicable section in MIL-STD 1778. The TCP Test Numbers may be

regarded as the "principal tests" of the applicable section in MIL

STD-1778 regarding each required function, operation, option, mode,

response, or state.

Reference                               Test Number

---------                               ----------

TCP Service Request Primitives

------------------------------

6.4.1 Unspecified Passive Open          1, 39, 57

6.4.2 Fully Specified Passive Open      9, 10, 40

6.4.3 Active Open                       2, 38, 55

6.4.4 Active Open with Data             11, 12

6.4.5 Send                              3, 14, 15, 16, 56

6.4.6 Allocate                          51

6.4.7 Close                             6, 7, 14, 15, 16

6.4.8 Abort                             17, 18, 19, 20

6.4.9 Status                            24

TCP Service Response Primitives

-------------------------------

6.4.10.1 Open Id                        1, 2, 9, 10, 11

6.4.10.2 Open Failure                   35, 371, 41, 42, 49, 63

6.4.10.3 Open Success                   1, 2, 66

6.4.10.4 Deliver                        3, 6

6.4.10.5 Closing                        6, 7

6.4.10.6 Terminate                      6, 7, 17, 18, 19, 20,

21, 44, 45, 46, 50, 55

56, 57, 58, 67

6.4.10.7 Status Response                24

6.4.10.8 Error                          4, 5, 31, 38, 40, 73

TCP Mechanisms Tested

---------------------

9.2.3 Flow Control Window               59, 61

9.2.4 Duplicate/Order Detection         25, 26, 27, 29, 32

9.2.5 Acks and Retransmission           27, 52, 53, 54

9.2.6 Checksum                          28

9.2.7 Push                              63, 64

9.2.8 Urgent                            60, 61, 62

9.2.9 ULP Timeout/Action                55, 56, 57, 58

9.2.10 Security                         38, 39, 40, 41, 42, 43

44, 45, 46, 47, 48, 49, 50

UNISYS Corporation

6 February 1987                   -3-                       TM-WD-8801/206/00

9.2.11 Precedence Level                 21, 22, 23

9.2.12 Multiplexing                     30, 31, 32, 33, 34, 36

9.2.13 Connection Opening               35, 37

9.2.14 Connection Closing               14, 15, 16, 17

9.2.15 Resets                           35, 50, 65, 66, 67, 68,

69, 70, 71, 72

9.3.1 Source Port Address Range         13

9.3.2 Destination Port

Address Range                     13

TCP Options Tested

------------------

9.3.11.1.3 Maximum Segment Size 52

UNISYS Corporation

6 February 1987                   -4-                       TM-WD-8801/206/00

TCP TESTS INDEX

TCP TEST NUMBERS versus TCP Commands/Primitives/Options/Modes...

The table shows the TCP Test Numbers which may be regarded as the

"principle tests" of each TCP Command or Primitive and/or Option or

Mode.

TEST NUMBER     PURPOSE

---- ------     ------

1         Unspecified Passive Open Request

2         Active Open Request

3         Basic Data Transfer

4         Remote Driver Interpretation of command LCN

5         Determine IUT Standard Send Buffer

6         Closing Handshake - IUT initiates close

7         Closing Handshake - IUT peer initiates close

8         Ability to reconnect Remote Driver command channel

9         Fully Specified Passive Open Request

10         Illegal Fully Specified Passive Open Request

11         Active Open with Data

12         Active Open with Data

13         Port Number Range

14         Graceful Closing - completion of data transfer after ULP close

15         Graceful Closing - data transfer after receipt of peer's FIN

16         Graceful Closing - peer data transfer after IUT initiates close

17         ULP Abort

18         Peer Abort

19         ULP Abort - data queued for sending

20         Peer Abort - data queued for sending

21         Precedence - mismatched

22         Precedence - matched

23         Precedence Negotiation

24         Status

25         Out of order data

26         Overlapping data

27         Lost data

28         Bad Checksum detection

29         Sequence number wraparound

30         Multiplexing - two connections with unique 4-tuple

IUT opens passively

31         Multiplexing - common destination port in 4-tuple common

IUT port

32         Multiplexing - common destination port in 4-tuple common

REF port

33         Multiplexing - two connections with unique 4-tuple IUT

opens actively

34         Multiplexing - three connections with common IUT port

in 4-tuple

UNISYS Corporation

6 February 1987                   -5-                       TM-WD-8801/206/00

TEST NUMBER      PURPOSE

---- ------      -------

35         Duplicate connection attempt - IUT passive

36         Multiplexing - Same sequence numbers on two connections.

37         Duplicate connection attempt - IUT active

38         Setting security in Active Open

39         Setting security in Passive Open

40         Setting security in Fully Specified Passive Open

41         Secure IUT rejecting connection to unsecured peer

42         Secure IUT rejecting connection from unsecured peer

43         Security option placement in sending data

44         Response to data with mismatched security class

45         Response to data with mismatched security protection authority

46         Response to data with extra protection authority

47         Use of security option for unclassified connections

48         Recognition of UNCLASS and GENSER as unsecured

49         Unsecured IUT response to connection attempt by secured host

50         Unsecured IUT response to data marked with classified security

51         Alloc

52         Maximum segment size option

53         Retransmission after acknowledgment of data

54         Retransmission after acknowledgment of SYN and FIN

55         ULP timeout service in Active Open

56         ULP timeout service in Send

57         ULP timeout service in Passive Open

58         ULP timeout notify action tested.

59         TCP in window mechanism

60         Urgent service

61         Urgent service when peer has zero window

62         Urgent data delivery

63         Push Service - Service not requested

64         Push service - Service requested

65         Reset - as response to connection refusal

66         Reset - partial reset prior to connection establishment

67         Reset - response to reset received while sending data

68         Reset segment format on receipt of Active Open with no

listening port

69         Reset segment format on receipt of Active Open with

data with no listening port

70         Reset segment format on receipt of invalid segment with ACK set

71         Reset segment format on receipt of invalid segment with

SYN and ACK set

72         Reset - no reset sent on receipt of segment with bad

acknowledgment number

73         Determine number of connections resources will allow

UNISYS Corporation

6 February 1987                   -6-                       TM-WD-8801/206/00

TCP TEST SCENARIOS INDEX

TCP TEST SCENARIO FILES versus TCP TEST NUMBERS...

The table shows, for each TCP Test Number, the UNIX filenames of the

TCP test Scenario Files in which it appears.

TEST NUMBER             SCENARIO NAME

-----------             -------------

1                      INIT1

2                      INIT1

3                      INIT1

4                      INIT1

5                      INIT1

6                      INIT1

7                      INIT1

8                      INIT1

9                      BASIC1

10                      BASIC1

11                      BASIC1

12                      BASIC1

13                      BASIC1

14                      BASIC2

15                      BASIC2

16                      BASIC2

17                      BASIC2

18                      BASIC2

19                      BASIC2

20                      BASIC2

21                      BASIC2

22                      BASIC2

23                      BASIC2

24                      BASIC2

25                      BASIC3

26                      BASIC3

27                      BASIC3

28                      BASIC3

29                      BASIC3

UNISYS Corporation

6 February 1987                   -7-                       TM-WD-8801/206/00

TEST NUMBER               SCENARIO NAME

-----------               -------------

30                      BASIC4

31                      BASIC4

32                      BASIC4

33                      BASIC4

34                      BASIC4

35                      BASIC4

36                      BASIC4

37                      BASIC4

38                      BASIC5

39                      BASIC5

40                      BASIC5

41                      BASIC5

42                      BASIC5

43                      BASIC5

44                      BASIC5

45                      BASIC5

46                      BASIC5

47                      BASIC5

48                      BASIC5

49                      BASIC5

50                      BASIC5

51                      BASIC6

52                      FULL1

53                      FULL1

54                      FULL1

55                      FULL1

56                      FULL1

57                      FULL1

58                      FULL1

59                      FULL2

60                      FULL2

61                      FULL2

62                      FULL2

63                      FULL2

64                      FULL2

65                      FULL3

66                      FULL3

67                      FULL3

68                      FULL3

69                      FULL3

70                      FULL3

71                      FULL3

72                      FULL3

73                      QUAL1

UNISYS Corporation

6 February 1987                   -8-                       TM-WD-8801/206/00

TCP SCENARIOS AND TESTS DESCRIPTIONS

This section provides a brief narrative of the scope and objectives of

each TCP test Scenario File and a narrative of each individual test in

that scenario.

========================================================================

Scenario INIT1

--------------

Scenario INIT1 is the first test of a test session. This scenario

tests the most basic TCP functions of the TCP Implementation Under

Test (IUT) and the compliance of the IUT Remote Driver to the DCA

Protocol Laboratory TCP Remote Driver Specification. If the IUT and

the Remote Driver do not receive good results on the first run of

this scenario, further testing should be abandoned until the problems

exhibited are corrected.

INIT1 tests the TCP implementation for its ability to perform the most

basic TCP functions: Active Open, Passive Open, transfer of data and

closing. The scenario determines that the Remote Driver interprets

Central Driver commands correctly, acknowledges the receipt of a

command from the Central Driver and correctly formats IUT responses

that it sends to the Central Driver.

____________________________________

Test 1:  UNSPECIFIED PASSIVE OPEN

------

- Does the Implementation Under Test (IUT) implement a Passive

Open which accepts an Active Open request?

- Action: Remote Driver (RD) performs a Specified Passive

Open. Laboratory Slave Driver (LSD) does an Active

Open to it.

- Verification: Determine that a connection is made by finding

an OPEN SUCCESS response from the LSD. Ensure that

the RD acknowledges all commands received from the

Central Driver (CD). Determine that all RD

responses are correctly formatted.

Success: The connection is made. The RD correctly interprets

CD commands, acknowledges CD commands and formats

IUT responses.

Failure: Connection is not made or the RD does not correctly

interpret CD commands, acknowledge CD commands, or

format IUT responses.

UNISYS Corporation

6 February 1987                   -9-                       TM-WD-8801/206/00

Test 2:  ACTIVE OPEN

------

- Can the Implementation Under Test (IUT) implement an Active

Open?

- Action: Laboratory Slave Driver (LSD) performs a Passive

Open. The Remote Driver (RD) does an Active Open

to the listening port.

- Verification: Determine that a connection is made by finding

an OPEN SUCCESS response from the RD. Ensure that

the RD acknowledges all commands received from the

Central Driver. Determine that all RD responses

are correctly formatted.

Success: The connection is made. The RD correctly

interprets CD commands, acknowledges CD commands

and formats IUT responses.

Failure: Connection is not made or the RD does not correctly

interpret CD commands, acknowledge CD commands, or

format IUT responses.

Test 3:  BASIC DATA TRANSFER

------

- Does the Implementation Under Test (IUT) send and deliver data

correctly?

- Action: The Laboratory Slave Driver (LSD) sends data to the

Remote Driver (RD). The RD sends data to the LSD.

- Verification: Ensure that all the data the IUT is directed to

send to the LSD is received by the Laboratory

Reference Implementation (REF). Check that all the

data sent by the LSD is delivered to the RD by the

IUT. Ensure that the RD acknowledges all commands

received from the Central Driver. Determine that

all RD responses are correctly formatted.

Success: The IUT sends all the data it is requested to send

and delivers all the data it receives. The RD

correctly interprets CD commands, acknowledges CD

commands and formats IUT responses.

Failure: The IUT does not send all the data it is requested

to send or does not deliver all the data it

receives. Also that the RD does not correctly

interpret CD commands, acknowledges CD commands or

format IUT responses.

UNISYS Corporation

6 February 1987                   -10-                      TM-WD-8801/206/00

Test 4:  SIGNIFICANCE OF COMMAND LCN TO REMOTE DRIVER AND IUT

------

- Do the Implementation Under Test (IUT) and the IUT Remote

Driver (RD) correctly interpret the local connection name

(lcn) specified in the commands the RD receives from the

Central Driver (CD).

- Action: The Central Driver (CD) sends a Send command to the

Remote Driver (RD) which contains an lcn for a

connection that is not established. The RD

requests the IUT to send data to the LSD over this

connection.

- Verification: Ensure that the RD reports in correct format

that the IUT reponds to the incorrect lcn with TCP

ERROR: CONNECTION DOES NOT EXIST.

Success: The IUT recognizes an invalid lcn. The RD

correctly interprets CD commands, acknowledges CD

commands and formats IUT responses.

Failure: The IUT does not recognize an invalid lcn. Also,

the RD does not correctly interpret CD commands,

acknowledge CD commands or format IUT responses.

Test 5:  DETERMINE IUT STANDARD SEND BUFFER FOR TESTING PURPOSES

------

- Determine the standard size buffer that the

Implementation Under Test (IUT) requires to guarantee that

the IUT will output at least two segments without delay.

- Action: The Remote Driver issues a series of Send commands.

each command requests a larger increment of data to

be sent.  After each Send command, the LSD waits

for the data to be delivered. Once the data is

delivered, check the TCP segments collected by the

Laboratory Reference Implementation (REF) to

determine if more than one segment was required to

send the data. When more than one segment is used

to send the data or the response TCP ERROR:

INSUFFICIENT RESOURCES is returned by the IUT, the

test is ended.

- Verification: When a Send command data length requires the

IUT to send the data in more than one segment, that

data length is noted. If the response TCP ERROR:

INSUFFICIENT RESOURCES has been reported, the data

length of the Send command immediately preceding

the command that caused that response is noted.

- Observation: The IUT standard send buffer is noted for the

test operator.

UNISYS Corporation

6 February 1987                   -11-                      TM-WD-8801/206/00

Test 6:  CLOSING HANDSHAKE WHEN IUT INITIATES CLOSE

------

- Does the Implementation Under Test (IUT) correctly perform

the closing handshake when it initiates closing.

- Action: The Remote Driver (RD) performs a close. When the

Laboratory Slave Driver (LSD) receives the

indication of peer closing from the Laboratory

ReferenceImplementation (REF), it performs a close.

- Verification: Ensure that the RD reports TERMINATE:

CONNECTION CLOSED or TERMINATE: ULP CLOSE when the

IUT indicates the connection closed. Check the TCP

segments collected by the REF to determine that the

IUT acknowledged the FIN from the REF before it

terminated.

Success: The IUT acknowledges the FIN of the REF. The IUT

reports its connection closed. The RD correctly

interprets Central Driver (CD) commands,

acknowledges CD commands and formats IUT responses.

Failure: The IUT does not acknowledge the FIN of the REF or

the IUT does not report its connection closed. The

IUT reports its connection closed before the REF

closes. The RD does not correctly interpret CD

commands, acknowledge CD commands or format IUT

responses.

Test 7:  CLOSING HANDSHAKE WHEN IUT PEER INITIATES CLOSE

------

- Does the Implementation Under Test (IUT) correctly perform

the closing handshake when its peer initiates closing.

- Action: The Laboratory Slave Driver (LSD) performs a close.

When the IUT reports receiving the Laboratory

Reference Implementation (REF) closing, it performs

a close.

- Verification: Ensure that the RP correctly reports CLOSING

and TERMINATE: CONNECTION CLOSED when it receives

these responses from the IUT for the REF's closing

and the final closing of the connection. Check the

TCP segments collected by the REF to determine that

the IUT did not send its FIN before being

instructed to close by the RD. Also check that the

IUT sends a FIN to the REF once it has been

instructed to close.

UNISYS Corporation

6 February 1987                   -12-                      TM-WD-8801/206/00

Success: The IUT waits to be instructed to close before it

sends a FIN. It sends a FIN when it is instructed

to close. The IUT correctly reports its peer's

closing and the closing of the connection. The RD

correctly interprets Central Driver (CD) commands,

acknowledges CD commands and formats IUT

responses.

Failure: The IUT sends a FIN in response to a peer's FIN

before it is instructed to close or the IUT does

not send a FIN when it is instructed to close.

Failure also occurs when the IUT does not report a

peer's closing or the closing of the connection.

Other reasons for failure are when the RD does not

correctly interpret CD commands, acknowledge CD

commands or format IUT responses.

Test 8:  ABILITY TO RECONNECT TO COMMAND CHANNEL AFTER CLOSURE

------

- Does the Implementation Under Test Remote Driver remain

listening after the Central Driver closes the command

channel?

- Action: The Central Driver instructs the Remote Driver to

kill itself. After waiting five seconds to allow

the connection to be cleared, the Central Driver

attempts to reconnect the command channel to the

Remote Driver.

- Verification: If the scenario aborts when the Central Driver

attempts to reconnect the command channel to the

Remote Driver, the Remote Driver is no longer

listening. If the Central Driver is able to

reconnect the command channel, a Remote Driver is

left listening after the command channel is closed.

Success: A Remote Driver is left listening when the Central

Driver (CD) closes the command channel to the

Remote Driver, and reconnection to the Central

Driver occurs.

Failure: A Remote Driver is not left listening when the

Central Driver closes the command channel to the

Remote Driver. The Remote Driver does not

correctly interpret CD commands, acknowledge CD

commands or format IUT responses.

UNISYS Corporation

6 February 1987                   -13-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC1

---------------

Scenario BASIC1 tests the TCP implementation of Fully Specified

Passive Open, and Active Open with Data.

_____________________________________

Test 9:  FULLY SPECIFIED PASSIVE OPEN

------

- Does the Implementation Under Test (IUT) implement a passive

open which accepts an Active Open request only from a

specified addresss

- Action: Remote Driver (RD) performs a fully Specified

Passive Open. Laboratory Slave Driver (LSD) does

an Active Open to it from a socket bound with the

specified address.

- Verification: Determine that a connection was made by

finding an OPEN SUCCESS response from the LSD.

Success: The connection was made.

Failure: Connection was not made.

Test 10:  FULLY SPECIFIED PASSIVE OPEN

-------

- Will the the Implementation Under Test (IUT) fully Specified

passive Open accept an Active Open request from an

unspecified port?

- Action: Remote Driver (RD) performs a fully Specified

Passive Open. Laboratory Slave Driver (LSD) does

an Active Open to it from a socket with a different

port number than the one specified in the fully

Specified Passive Open.

- Verification: Determine that a connection was made by

checking that the Laboratory Reference

Implementation (REF) sent an OPEN FAILURE

response.

Success: Connection was not made.

Failure: Connection was made.

UNISYS Corporation

6 February 1987                   -14-                      TM-WD-8801/206/00

Test 11:  ACTIVE OPEN WITH DATA

-------

- Does the Implementation Under Test (IUT) send data on its SYN

segment on an Active Open with Data.

- Action: Remote Driver (RD) performs an Active 0pen with Data.

- Verification: Check the IUT's SYN segment to see if data

was sent on that segment.

Success: The IUT sends data on its SYN segment.

Failure: The IUT does not send data on its SYN segment.

Test 12: ACTIVE OPEN WITH DATA

-------

- Does the Implementation Under Test (IUT) acknowledge

data received on the SYN segment in its SYN ACK (before

connection establishment)s

- Action: Laboratory Slave Driver (LSD) performs an Active

Open with Data.

- Verification: Check the IUT SYN ACK segment to ensure that it

does not acknowledge data.

Success: Data is not acknowledged on the IUT SYN ACK segment.

Failure: Data is acknowledged on the IUT SYN ACK segment.

Test 13: PORT NUMBER RANGE

-------

- Can the Implementation Under Test (IUT) assign a range of

local port numbers?

- Action: Remote Driver (RD) does two Passive Open requests:

one specifies source port 1 and the other specifies

source port 65535. The RD also does an Active Open

to destination port 65534.

- Verification: Determine whether the IUT returns SYSTEM ERROR:

REQUESTED SOURCE PORT NOT PERMITTED or the OPEN ID

from the Passive Open requests. Ensure that

a connection was made when destination port 65534

was specified in the Active Open.

Observation: Result is to indicate the range of port numbers

that the IUT allows to be assigned.

UNISYS Corporation

6 February 1987                   -15-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC2

---------------

Scenario BASIC2 tests the TCP implementation of graceful closings, its

handling of aborts, precedence, and status.

_______________________________________

TEST 14: DATA TRANSFER IN CLOSING STATES

-------

- Does the Implementation Under Test (IUT) provide graceful

closing by completing data transfer after its Upper Level

Protocol has initiated closings

- Action: The Remote Driver (RD) sends a large amount of

data and immediately closes.

- Verification: Determine that the Laboratory Reference

Implementation (REF) receives all the data the IUT

was requested to send.

Success: All expected data was received by the REF.

Failure: All expected data not received by REF.

TEST 15: DATA TRANSFER IN CLOSING STATES

-------

- Does the Implementation Under Test (IUT) provide graceful

closing by continuing to send data after receiving peer's

FIN segments

- Action: The IUT is instructed to send data after it has

received its peer's closing FIN segment.

- Verification: Determine that the IUT sends the data by

checking that all expected data is received by the

Laboratory Reference Implementation (REF).

Success: REF receives all data IUT was asked to send.

Failure: REF does not receive all data IUT was asked to send.

UNISYS Corporation

6 February 1987                   -16-                      TM-WD-8801/206/00

TEST 16: DATA TRANSFER IN CLOSING STATES

-------

- Does the Implementation Under Test (IUT) deliver data sent

after it has initiated closings

- Action: Remote Driver tells IUT to close. On receiving the

IUT's close, the Laboratory Slave Driver (LSD)

requests the Laboratory Reference Implementation

(REF) to send data to the IUT.

- Verification: Determine that the IUT delivers all the data

sent by the REF.

Success: IUT delivers all data sent by REF.

Failure: IUT does not deliver all data sent by REF.

TEST 17: UPPER LAYER PROTOCOL ABORT

-------

- Does the Implementation Under Test (IUT) perform an abort by

sending a correct reset segments

- Action: Remote Driver (RD) aborts the connection.

- Verification: Check that the Laboratory Slave Driver (LSD)

reports a TERMINATE: REMOTE ABORT response and the

RD reports TERMINATE: ULP ABORT or TERMINATE: USER

ABORT.

Success: The IUT sends a reset segment and the correct

terminate response.

Failure: The IUT does not send a reset segment or correct

terminate response.

Test 18: PEER ABORT

-------

- Can the Implementation Under Test (IUT) detect its peer's aborts

- Action: The Laboratory Slave Driver (LSD) instructs its

Laboratory Reference Implementation (REF) to abort

connection.

- Verification: Check that the IUT reports TERMINATE: REMOTE ABORT.

Success: IUT correctly reports REMOTE ABORT.

Failure: IUT does not correctly report its peer's abort.

UNISYS Corporation

6 February 1987                   -17-                      TM-WD-8801/206/00

TEST 19: UPPER LAYER PROCESS ABORT

-------

- Does the Implementation Under Test (IUT) discard data queued

for sending when it performs a ULP aborts

- Action: Remote Driver (RD) sends data to its peer and then

immediately aborts the connection.

- Verification: Check that the Laboratory Slave Driver (LSD)

reports a TERMINATE: REMOTE ABORT response and the

RD reports TERMINATE: ULP ABORT. Examine the IUT

reset segment for correctness and determine if all

data sent by the RD was received by the LSD.

Success: The IUT sends the correct terminate response, its

reset segment is correctly formatted, and not all

data sent by the IUT is received by the

Laboratory Reference Implementation (REF).

Failure: The IUT sends an incorrect terminate response or an

incorrect reset segment. If all data is received

by the LSD, this particular result is inconclusive

as the data may already be sent before the Central

Driver command to abort is received by the IUT.

Test 20: PEER ABORT

-------

- Does the Implementation Under Test (IUT) detect its peer's

abort and then discard data queued for sendings

- Action: The Laboratory Slave Driver (LSD) instructs its

Laboratory Reference Implementation (REF) to abort

while the IUT is sending data.

- Verification: Check that the IUT reports TERMINATE: REMOTE

ABORT and that the IUT stops sending data after

receiving the peer's abort.

Success: IUT correctly reports REMOTE ABORT and stops sending

data after receiving its peer's reset.

Failure: IUT does not correctly report its peer's abort. If

all the data sent is received by the LSD, that

particular result is inconclusive as all the data

may be delivered before the Central Driver

command to abort is received.

UNISYS Corporation

6 February 1987                   -18-                      TM-WD-8801/206/00

Test 21: MISMATCHED PRECEDENCE

-------

- Does the Implementation Under Test (IUT) provide the option

to set a precedence value for the connection and make correct

hecks of the precedence of incoming segments to the

connections

- Action: Remote Driver (RD) passively opens a connection

with maximum precedence. Laboratory Slave Driver

(LSD) actively opens at a less than maximum

precedence and Laboratory Reference Implementation

(REF) does not raise its precedence.

- Verification: Determine that IUT sets the precedence option

by checking the segments output by the IUT. Ensure

that IUT aborts the connection when REF does not

match precedence by looking for TERMINATE: REMOTE

ABORT from the LSD or an OPEN FAILURE from the LSD

connection.

Success: IUT sets precedence option and aborts connection

when peer's precedence does not match.

Failure: IUT does not set precedence option or makes

connection when precedence does not match.

Test 22: PRECEDENCE

-------

- Does the Implementation Under Test (IUT) provide the option

to set precedence for the connection and make correct checks

on the incoming segments to the connection?

- Action: Remote Driver (RD) passively opens a connection

with maximum precedence. Laboratory Slave Driver

(LSD) actively opens at a less than maximum

precedence. The Laboratory Reference Implementation

(REF) raises its precedence during the opening

handshake.

- Verification: Determine that the connection is made by

checking that the LSD reports an OPEN SUCCESS.

Check the IUT segments to ensure that precedence

was set.

Success: The IUT makes the connection and sets-precedence.

Failure: The IUT does not set precedence or does not make

the connection even though the REF matched its

precedence.

UNISYS Corporation

6 February 1987                   -19-                      TM-WD-8801/206/00

Test 23: PRECEDENCE NEGOTIATION

-------

- Does the Implementation Under Test (IUT) provide the option

to set precedence for the connection and raise its precedence

to match its peer's precedences

- Action: Laboratory Slave Driver (LSD) passively opens a

connection with a given precedence. The Remote

Driver opens with a zero precedence.

- Verification: Determine that the connection is made by

checking that the LSD reports an OPEN SUCCESS.

Check the IUT segments to ensure that precedence

was raised.

Success: The IUT makes the connection and sets precedence.

Failure: The IUT does not raise precedence to match the

precedence of the REF.

Test 24: STATUS

-------

- Does the Implementation Under Test (IUT) in its status

response correctly report a connection's source port,

destination port, destination address, window,

precedence and states (These fields were selected to

represent both static and dynamic internal information).

- Action: The Remote Driver (RD) asks the IUT to give the

status of a connection.

- Verification: Ensure that the status response given matches

the true status of the connection.

Success: The IUT reports the status of the connection

correctly.

Failure: The IUT does not report the status of the connection

correctly.

UNISYS Corporation

6 February 1987                   -20-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC3

---------------

Scenario BASIC3 tests the TCP implementation's ability to maintain

data integrity: out of order data, overlapping data, lost data,

segments with bad checksums and segments with sequence number

wraparound.

________________________________________

Test 25: OUT OF ORDER DATA

- Can the Implementation Under Test (IUT) correctly handle

segments which arrive out of order, as indicated by their

sequence number?

- Action: The Laboratory Slave Driver (LSD) sends data to the

IUT. The Laboratory Reference Implementation (REF)

divides the data into segments and outputs them out

of order.

- Verification: Determine if the IUT delivers the data in the

correct order.

Success: The IUT correctly reorders the data.

Failure: The IUT does not correctly reorder the data.

Test 26: OVERLAPPING DATA

-------

- Can the Implementation Under Test (IUT) clean up overlapping

datas

- Action: The Laboratory Slave Driver (LAB) sends data to the

IUT. The Laboratory Reference Implementation (REF)

repackages the data for retransmission so that some

segments contain both new data and data already

sent.

- Verification: Determine that IUT accepts the data and

correctly delivers it.

Success: The IUT is able to accept the overlapping data and

deliver it correctly.

Failure: The IUT does not deliver the data on segments after

the first arrival or delivers the data incorrectly.

UNISYS Corporation

6 February 1987                   -21-                      TM-WD-8801/206/00

Test 27: LOST DATA

-------

- Can the Implementation Under Test (IUT) detect that data is losts

- Action: The Laboratory Slave Driver (LSD) sends data to the

Remote Driver. The Laboratory Reference

Implementation (REF) divides the data into several

segments and sends data, omitting a segment. The

missing segment is not sent until the last segment

sent is retransmitted several times.

- Verification: Determine that the IUT does not acknowledge

anything sent after this missing segment until the

missing segment is transmitted.

Success: The IUT does not acknowledge any data received after

the last correctly ordered segment, until the

missing segment is sent.

Failure: The IUT acknowledges data sent after the missing

segment before the missing segment is sent.

TEST 28: CHECKSUM

-------

- Does the Implementation Under Test (IUT) detect a segment

with a bad checksum and discard it? -

- Action: Laboratory Slave Driver (LSD) sends data to the

IUT. The Reference Implementation (REF) sends out

a segment with a bad checksum and retransmits it

incorrectly several times before transmitting it

correctly.

Verification: Determine that the IUT acknowledges only the

segments sent with a good checksum by checking

segment data collected by the REF.

Success: The IUT only acknowledges segments sent with good

checksums.

Failure: REF acknowledges segments sent with bad checksums.

UNlSYS Corporation

6 February 1987                   -22-                      TM-WD-8801/206/00

Test 29: SEQUENCE NUMBER WRAPAROUND

-------

- Does the Implementation Under Test (IUT) use correct module

arithmetic when comparing sequence numberss

- Action: The Laboratory Slave Driver (LSD) sends data to the

IUT. The Laboratory Reference Implementation (REF)

uses as its initial sequence number a number

guaranteed to cause wraparound from 2**32-1 to 0

on the data segments.

- Verification: Determine if the IUT acknowledges all data sent

by the REF.

Success: The IUT acknowledges the data sent by the REF.

Failure: The IUT does not acknowledge any data sent by the

REF after the sequence number wraparound occurs.

UNISYS Corporation

6 February 1987                   -23-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC4

---------------

Scenario basic 4 test how well the TCP implementation can establish

multiple connections and demultiplex data sent over these connections.

A TCP connection is defined by the source port, source address,

destination port and destination address of the connection. This four

element identification is known as a 4-tuple. This scenario tests by

opening multiple connections with different combinations in the 4

tuple (from the viewpoint of the IUT).

_________________________________________

Test 30: MULTIPLEX DATA OVER 2 CONNECTIONS WITH UNIQUE 4-TUPLES.

-------

- Can the Implementation Under Test (IUT) correctly deliver

data sent over two connections it actively opened with no

4-tuple element in common?

- Action: The Remote Driver (RD) opens two connections to the

Laboratory Slave Driver(LSD). The LSD then sends

unique data over each connection.

- Verification: Determine that the IUT keeps the data sent on

each connection separate and delivers it correctly.

Success: The IUT delivers the data sent over each connection

correctly.

Failure: The IUT does not deliver the data sent over each

connection correctly.

UNISYS Corporation

6 February 1987                   -24-                      TM-WD-8801/206/00

Test 31: MULTIPLEX OVER 2 CONNECTIONS WITH COMMON DEST.  PORT IN 4-TUPLE.

-------

- Can the Implementation Under Test (IUT) multiplex data over

two connections which share a common destination port in

their 4-tuples.

- Action: Two connections are opened which have the same

destination port on the Laboratory Reference

Implementation (REF). The LSD sends unique data

over each connection.

- Verification: Determine that the IUT keeps the data received

over connection separate and delivers it correctly.

Success: The IUT delivers the data sent over each connection

correctly.

Failure: The IUT does not deliver the data sent over each

connection correctly.

Test 32: MULTIPLEX WITH 2 CONNECTIONS HAVING SAME REF SRC PORT IN 4-TUPLES.

-------

- Can the Implementation Under Test multiplex data over two

connections that have the same destination port in their

4-tuples?

- Action: The Laboratory Slave Driver (LSD) actively opens

two connections from the same source port to

distinct listening ports on the IUT. The LSD then

sends unique data to the Remote Driver (RD) over

each,connection.

- Verification: Determine that all the data sent by the LSD is

delivered correctly to the RD.

Success: All sent data is delivered correctly to the RD.

Failure: Data is not delivered correctly to the RD.

Test 33: MULTIPLEX DATA OVER 2 CONNECTIONS WITH UNIQUE 4-TUPLES.

-------

- Can the Implementation Under Test (IUT) correctly deliver

data sent over two connections that the IUT passively opens,

that have no 4-tuple elements in commons

- Action: The Laboratory Slave Driver (LSD) opens two

connections to the Remote Driver (RD). The LSD

then sends unique data data over each connection.

UNISYS Corporation

6 February 1987                   -25-                      TM-WD-8801/206/00

- Verification: Determine that the IUT keeps the data sent on

each connection separate and delivers it correctly.

Success: The IUT delivers the data sent over each connection

correctly.

Failure: The IUT does not deliver the data sent over each

connection correctly.

TEST 34: MULTIPLEX DATA OVER 3 CONNECTIONS WITH SAME SRC PORT IN 4-TUPLE

-------

- Can the Implementation Under Test (IUT) correctly establish

connections and demultiplex data when the same IUT source

port is in the 4-tuple of 3 connectionss

- Action: The Laboratory Slave Driver (LSD) actively opens 3

connections to the same IUT port. The LSD then

sends unique data to the Remote Driver (RD) over

each connection.

- Verification: Determine that all three connections are opened

and that the all the data sent over each connection

is correctly delivered.

Success: All the data sent over the 3 connections is

correctly delivered.

Failure: The data sent over the 3 connections is not

correctly delivered.

TEST 35: IUT REJECTS DUPLICATE CONNECTION

-------

- Does the Implementation Under Test (IUT) reject an attempt to

make a duplicate connection to its listening ports

- Action: The Laboratory Slave Driver (LSD) actively opens 2

connections to the IUT from different ports. LSD

then attempts to make a third connection using the

same source and destination port used in the second

connection.

- Verification: Determine that IUT resets the original of the

duplicate connections, refuses to make the

duplicate connection (OPEN FAILURE response

received by LSD) and does not enter into the

opening handshake for the duplicate connection.

Success: IUT rejects the duplicate connection and does not

enter into the opening handshake for that

connection.

Failure: The IUT fails to reject the duplicate connection.

UNISYS Corporation

6 February 1987                   -26-                      TM-WD-8801/206/00

TEST 36: MULTIPLEX DATA OVER CONNECTIONS USING THE SAME SEQUENCE NUMBERS

-------

- Can the Implementation Under Test (IUT) multiplex data when

the REF uses the same sequence number on two connections s

- Action: The Laboratory Slave Driver opens three connections

to the IUT and sends unique data over the three

connections to the Remote Driver (RD). The

Laborory Reference Implementation (REF) ensures

that the the TCP segments on two of the connections

have the same sequence numbers.

- Verification: Determine that all the data sent over each

connection is correctly delivered to the RD.

Success: All the data sent over each connection is correctly

delivered.

Failure: The data sent over each connection is not correctly

delivered.

Test 37: IUT'S REFUSAL TO MAKE DUPLICATE CONNECTION

-------

- Does the Implementation Under Test refuse to make a

duplicate connection when it is actively opening?

- Action: The Remote Driver (RD) makes a connection to the

Laboratory Slave Driver (LSD). RD then attempts to

open a second connection with the same 4-tuple

(same source and destination ports).

- Verification: Verify that the IUT refuses to make the

duplicate connection by responding with TCP ERROR:

CONNECTION ALREADY EXISTS or SYSTEM ERROR:

REQUESTED SOURCE PORT IN USE.

Success: IUT does not make a duplicate connection and does

not allocate resources for a duplicate connection.

Failure: IUT allocates resources for a duplicate connection

or does not give proper response to the attempt to

open the duplicate connection.

UNISYS Corporation

6 February 1987                   -27-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC5

---------------

Scenario BASIC5 tests the TCP Implementation Under Test (IUT) for

basic and default security. It tests that the IUT:

- allows the security of the connection to be set by the use

of parameters in the open service requests

- rejects a connection request with a wrong security level,

- aborts the connection if a segment arrives with any mismatch

of the security option.

Tests 38 through 46 test basic security. Tests 47 through 50 test

default security. Default security checks are those checks which both

classified and unclassified hosts should perform. If the IUT does not

pass Test 38 or Test 39 (the setting of security in the IUT Active

Open and the IUT Passive Open), tests 40 through 46 are not executed.

Test 38: IUT'S ABILITY TO SET SECURITY IN ITS ACTIVE OPEN

-------

- Is the Implementation Under Test (IUT) able to set security

in its Active Open s

- Action: The Remote Driver (RD) opens a connection with the

classification and protection parameters supplied

for the IUT Active Open. The Laboratory Slave

Driver (LSD) opens with the same security in its

Passive Open.

- Verification: Determine if the connection is opened success

fully. If the connection is opened, determine that

the security parameters have been set correctly.

If the open service request does not return an OPEN

ID, the response TCP ERROR: SEC/PREC NOT ALLOWED or

SYSTEM ERROR: REQUESTED PARAMETER NOT IMPLEMENTED

must be found.

Success: Connection is successfully opened showing that the

security parameters have been set correctly. If the

connection is not established the response TCE

ERROR: SEC/PREC NOT ALLOWED or SYSTEM ERROR:

REQUESTED PARAMETER NOT IMPLEMENTED is found.

Failure: Connection is not established and neither the

response TCP ERROR: SEC/PREC NOT ALLOWED nor SYSTEM

ERROR: REQUESTED PARAMETER NOT IMPLEMENTED is

returned from the IUT. Connection is established

but the parameters have not been set correctly.

UNISYS Corporation

6 February 1987                   -28-                      TM-WD-8801/206/00

Test 39: IUT'S ABILITY TO SET SECURITY PARAMETERS IN ITS PASSIVE OPEN

-------

- Is the Implementation Under Test (IUT) able to set security

in its Passive Open ?

- Action: The Remote Driver (RD) uses the classification

and protection parameters that it is given for an

IUT Passive Open. The Laboratory Slave Driver

(LSD) sets the same parameters in its Active Open

and attempts to open a connection.

- Verification: Determine that the connection is established.

If a connection is opened successfully, then

security fields in the TCP segments are checked to

make sure that the IUT is setting the security from

its input parameter. The Laboratory Reference

Implementation (REF) collects these segments. If a

connection is not established, check that the IUT

open service response returned the response TCP

ERROR: SEC/PREC NOT ALLOWED or SYSTEM ERROR:

REQUESTED PARAMETER NOT IMPLEMENTED.

Success: The connection is established. Analysis determines

that the security parameters were set in the IUT

Passive Open. Also, the connection is not

established but the IUT returns either TCP ERROR:

SEC/PREC NOT ALLOWED or SYSTEM ERROR: REQUESTED

PARAMETER NOT IMPLEMENTED in response to the open

service request.

Failure: Although the connection is established, analysis

reveals that the IUT is not setting security but

merely matching peer's security. Also, the

connection is not established but no proper security

related response is received to the open service request.

Test 40: IUT'S ABILITY TO SET SECURITY IN ITS SPECIFIED PASSIVE OPEN

-------

- Is the Implementation Under Test (IUT) able to set security

in its Specified Passive Opens

- Action: The Remote Driver (RD) uses the classification

CONFID and protection DIA as the parameters for an

IUT Passive Open. The Laboratory Slave Driver

(LSD) sets the same parameters in its Active Open

and attempts to open a connection.

- Verification: Determine that the connection is established.

If a connection is opened successfully, the

security fields in the TCP segments are checked to

make sure that the IUT is setting security from its

input parameter. The Laboratory Reference

Implementation (REF) collects these segments. If a

connection is not established, check that the IUT

UNISYS Corporation

6 February 1987                   -29-                      TM-WD-8801/206/00

open service reponse returned the response TCP

ERROR: SEC/PREC NOT ALLOWED or SYSTEM ERROR:

REQUESTED PARAMETER NOT IMPLEMENTED.

Success: The connection is established and analysis deter

mines that the security parameters were set in the

IUT specified passive open. Also, the connection

is not established but the IUT returns either TCP

ERROR: SEC/PREC NOT ALLOWED or SYSTEM ERROR:

REQUESTED PARAMETER NOT IMPLEMENTED in response to

the open service request.

Failure: Connection is established but analysis reveals that

the IUT is not setting security but merely matching

peer's security. Also, connection is not

established and no proper security related response

is received to the open service request.

Test 41: SECURE IUT REJECTS CONNECTION TO UNSECURED PEER

-------

- Does the secured Implementation Under Test (IUT) reject a

connection to an unsecured peers

- Action: The Remote Driver (RD) with security set in its

Active Open opens to the Laboratory Reference

Implementation (REF). The REF has passively opened

without security set.

- Verification: Check that the IUT returns an OPEN EAILURE and

that the connection is not established.

Success: The IUT successfully gets an OPEN FAILURE.

Failure: The IUT-fails to get an OPEN FAILURE.

Test 42: SECURE IUT REJECTS CONNECTION ATTEMPT OF UNSECURED PEER

-------

- Does the secured Implementation Under Test (IUT) reject

connection attempt of an unsecured host ?

- Action: Laboratory Slave Driver (LSP) with no security

actively opens to the IUT. The IUT has passively

opened with security set.

- Verification: Determine that no connection is established by

checking for an OPEN FAILURE response from the

Laboratory Reference Implementation (REF).

Success: The REF gets an OPEN FAILURE or the IUT can not set

security parameters.

Failure: The connection is established.

UNISYS Corporation

6 February 1987                   -30-                      TM-WD-8801/206/00

Test 43: SECURITY OPTION PLACEMENT IN DATA SEGMENTS

-------

- Is the Implementation Under Test (IUT) consistent in its

security option placement when sending datas

- Action: A secure connection is opened. Both the Laboratory

Slave Driver (LSD) and the Remote Driver (RD) send data.

- Verification: Determine that all the data sent by the LSD and

the RD is correctly delivered by the other peer.

Check the segment information collected by the

Laboratory Reference Implementation (REF) to ensure

that the correct security was placed on every

segment.

Success: All data is delivered and the security is correctly

placed on each TCP segment.

Failure: All data is not correctly delivered or the security

fields are not correctly placed on every TCP segment.

TEST 44: RESPONSE TO DATA WITH MISMATCHED SECURITY CLASS

-------

- Does the Implementation Under Test (IUT) reset a connection

on receiving data with a mismatched security class?

- Action: A secure connection is established. The Laboratory

Slave Driver (LSD) sends data to the Remote Driver

(RD). The Laboratory Reference Implementation

(REF) places a wrong security class on the data

segments.

- Verification: Determine that the IUT resets the connection by

checking that the REF reports REMOTE ABORT. Check

that the IUT reports SEC/PREC MISMATCH.

Success: The IUT resets the connection and reports SEC/PREC

MISMATCH.

Failure: IUT fails to reset connection or does not report

SEC/PREC MISMATCH.

UNISYS Corporation

6 February 1987                   -31-                      TM-WD-8801/206/00

TEST 45: RESPONSE TO DATA WITH MISMATCHED PROTECTION AUTHORITY

-------

- Does the Implementation Under Test (IUT) reset a connection

on receiving data with a mismatched security authority?

- Action: A secure connection is established. The Laboratory

Slave Driver (LSD) sends data to the Remote Driver

(RD). The Laboratory Reference Implementation

(REF) places an incorrect protection authority on

the data segments.

- Verification: Determine that the IUT resets the connection by

checking that the REF reports REMOTE ABORT. Check

that the IUT reports SEC/PREC MISMATCH.

Success: The IUT resets the connection and reports SEC/PREC

MIS MATCH.

Failure: IUT fails to reset connection or does not report

SEC/PREC MISMATCH.

TEST 46: RESPONSE TO DATA WITH AN EXTRA PROTECTION AUTHORITY

-------

- Does the Implementation Under Test (IUT) reset a connection

on receiving data with an extra security protection

authority?

- Action: A secure connection is established. The Laboratory

Slave Driver (LSD) sends data to the Remote Driver

(RD). The Laboratory Reference Implementation

(REF) places an extra protection authority on the

data segments.

- Verification: Determine that the IUT resets the connection by

checking that the REF reports REMOTE ABORT. Check

that the IUT reports SEC/PREC MISMATCH.

Success: The IUT resets the connection and reports SEC/PREC

MISMATCH.

Failure: IUT fails to reset connection or does not report

SEC/PREC MISMATCH.

UNISYS Corporation

6 February 1987                   -32-                      TM-WD-8801/206/00

TEST 47: USE OF SECURITY OPTION FOR UNCLASSIFIED CONNECTIONS

-------

- Does the Implementation Under Test (IUT) use the security

option for unclassified connection ?

- Action: Laboratory Slave Driver (LSD) does a Passive Open

with security set. The Remote Driver (RD) does an

Active Open with no security set.

- Verification: Check TCP segments collected by the Laboratory

Reference Implementation (REF) to determine if IUT

uses the security option at a default level for an

unsecured connection. Ensure that the connection

is not opened by looking for the OPEN FAILURE

response from the IUT.

Observation: IUT does or does not use the security option for

unclassified connections.

Failure: Test connection opened.

TEST 48: RECOGNITION OF UNCLASS AND GENSER AS EQUIVALENT TO UNSECURE

-------

- Does the unsecured Implementation Under Test (IUT) connect

with peer with security of UNCLASS and GENSERs

- Action: Laboratory Slave Driver (LSD) does a passive open

with security set to UNCLASS and GENSER. The

Remote Driver (RD) does an active open with no

security setting.

- Verification: Determine that the connection is opened by

looking for OPEN SUCCESS response from the IUT.

Success: Connection established.

Failure: Connection not established.

UNISYS Corporation

6 February 1987                   -33-                      TM-WD-8801/206/00

TEST 49: UNSECURED IUT RESPONSE TO CONNECTION ATTEMPT BY SECURE HOST

-------

- Does the IUT with no security reject an Active Open from a

peer with security sets

- Action: Remote Driver (RD) does a Passive Open with no

security set. The Laboratory Slave Driver (LSD)

does an Active Open with security set to CONFID and DIA.

- Verification: Determine that the IUT rejects the connection

by searching for OPEN FAILURE response reported by

LSD.

Success: Connection is not established.

Failure: Connection established.

TEST 50: UNSECURED IUT RESPONSE TO DATA MARKED WITH CLASSIFIED SECURITY

-------

- Does the Implementation Under Test (IUT) reset connection

when finding data marked with security higher than unclassifieds

- Actions: A connection is established with no security

settings. The Laboratory Slave Driver (LSD) sends

data to the Remote Driver (RD). The Laboratory

Reference Implementation (REF) places a classified

security option on a data segment.

- Verification: Determine that the IUT resets the connection

and reports SEC/PREC MISMATCH to the Remote Driver.

Success: The IUT correctly resets connection on finding

incorrectly marked security on data.

Failure: The IUT fails to recognize wrong security marking

on data and does not reset the connection.

UNISYS Corporation

6 February 1987                   -34-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario BASIC6

---------------

Scenario BASIC6 tests the TCP implementation for its ability to

perform the ALLOC service request correctly in the EXPLICIT mode under

control of the Central Driver.

________________________________________

TEST 51: ALLOC

-------

- Can the Implementation Under Test (IUT) perform the ALLOC

service request correctly ?

- Action: Remote Driver (RD) and Laboratory Slave Driver

(LSD) establish a connection. The RD instructs the

IUT it has allocated a specified buffer size for

data. Then the LSD sends data equal to twice that

buffer size to the RD.

- Verification: Determine that the amount of data delivered to

the RD by the IUT is not greater than that

specified in the ALLOC request. If all the sent

data is acknowledged prior to the RD sending

another ALLOC, the amount of data delivered must be

equal to or less than the buffer size specified in

the ALLOC. If it is necessary for the RD to send a

second ALLOC for all the sent data to be

acknowledged, then results are determined by

examining the TCP segments collected by the

Laboratory Reference Implementation (REF). The

segments are analyzed to make sure that no more

data than was specified in the ALLOC was

acknowledged before the IUT receive window was set

to zero.

Success: The IUT delivers data equal to or less than the

ALLOC request.

Failure: The IUT delivers more data than ALLOC specifies.

Inconclusive: Analysis cannot be done to determine IUT's performance.

UNISYS Corporation

6 February 1987                   -35-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario FULL1

--------------

Scenario FULL1 tests the TCP implementation of maximum segment size

option, basic retransmission and ULP timeout.

________________________________________

Test 52: MAXIMUM SEGMENT SIZE OPTION

-------

- Does the Implementation Under Test (IUT) use the maximum

segment size (MSS) option and respond correctly to its peer's

maximum transmission units

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The Laboratory

Reference Implementation (REF) places a low MSS on

its opening segment. The RD sends data.

- Verification: Check segment data collected by the REF to see

if IUT uses the MSS option. Examine the segment

data to see if the IUT sends any data segment with

a length greater than the MSS specified by the REF.

Success: The IUT sends segments which do not exceed the

maximum segment size specified by the REF.

Failure: IUT sends segments which exceed the maximum segment

size specified by the REF.

Observation: Observation is made as to whether the IUT uses

the MSS-option.

TEST 53: RETRANSMISSION AFTER ACKNOWLEDGMENT OF DATA.

-------

- Does the Implementation Under Test (IUT) stop retransmitting

promptly after receiving acknowledgment of datas

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

data. The Laboratory Slave Driver (LSD) withholds

acknowledgments until it has received several

transmissions of the oldest data.

- Verification: Examine the TCP segments collected by the REF.

Count the number of retransmissions of data sent

after the data is acknowledged. Ensure that the

REF receives no more than two retransmissions after

it has acknowledged data.

Success: The IUT does not retransmit after data is acknowledged.

UNISYS Corporation

6 February 1987                   -36-                      TM-WD-8801/206/00

Failure: IUT does not stop retransmitting promptly after

data is acknowledged.

Test 54: RETRANSMISSION AFTER ACKNOWLEDGMENT OF SYN AND FIN

-------

- Does the Implementation Under Test (IUT) stop retransmitting

promptly after its SYN and FIN are acknowledgeds

- Action: The Laboratory Slave Driver (LSD) actively opens a

connection to the Remote Driver (RD). The

Laboratory Reference Implementation (REF) withholds

acknowledgments of the IUT's SYN segment until it

is transmitted several times. Then the RD closes

the connection. The REF withholds retransmissions

until the IUT's FIN segment is transmitted several times.

- Verification: Examine the TCP segments collected by the REF.

Count the number of times the IUT SYN segment is

retransmitted after it is acknowledged. Count the

number of times the IUT FIN segment is retrans

mitted after it is acknowledged. Ensure that the

REF receives no more than two retransmissions after

it acknowledges SYN or FIN.

Success: The IUT does not retransmit its SYN and FIN

segments after they are acknowledged.

Failure: The IUT does not stop retransmitting its SYN and

FIN segments promptly after they are acknowledged.

TEST 55: IMPLEMENTATION OF ULP TIMEOUT SERVICE IN ACTIVE OPEN

-------

- Does the Implementation Under Test (IUT) implement ULP

timeout when timeout is specified in its Active Open ?

- Action: Find the number of retransmissions by the IUT when

the IUT's default ULP timeout occurs or when the

default timeout of two minutes suggested by

MIL-STD 1778 is reached. This is done by opening a

connection and having the Remote Driver (RD) send

data. The Laboratory Reference Implementation (REF)

does not acknowledge the data. When the connection

is aborted (or after 2 minutes), check the TCE

segments collected by the REF and count the number

of retransmissions for one data element. This

becomes the default number of retransmissions. The

Remote Driver (RD) does an Active Open with a small

ULP timeout and a timeout action of terminate set.

The RD then sends data. The Laboratory Reference

Implementation (REF) withholds acknowledgments.

The test continues until the connection terminates

or 2 minutes pass.

UNISYS Corporation

6 February 1987                   -37-                      TM-WD-8801/206/00

- Verification: Check the TCP segments collected by the REF.

Count the number of times a data element is

retransmitted. The number of retransmissions must

be significantly smaller than the default number of

retransmissions.

Success: IUT implements ULP timeout if it resets the

connection and the number of data retransmissions

is significantly smaller than the default number of

retransmissions.

Failure: IUT does not implement ULP timeout. The IUT does

not reset the connection or, if the connection is

reset, the number of IUT retransmissions indicate

that the abort was caused by the TCP default rather

than the prescribed ULP timeout.

TEST 56: IMPLEMENTATION OF ULP TIMEOUT SERVICE IN SEND

-------

- Does the Implementation Under Test (IUT) implement ULP

timeout when timeout specified in its Sends

- Action: The Remote Driver (RD) and the Laboratory Slave

Driver (LSD) establish a connection. The RD sends

data with a small ULP timeout and a timeout action

of terminate set. The Laboratory Reference

Implementation (REF) withholds acknowledgments.

The test continues until the connection terminates

or 2 minutes pass.

- Verification: Check the TCP segments collected by the REF.

Count the number of times a data element is

retransmitted. The number of retransmissions must

be significantly smaller than the default number

of retransmissions (determined in Test 55).

Success: IUT implements ULP timeout if it resets the

connections and the number of data retransmissions

is significantly smaller than the default number of

retransmissions.

Failure: IUT does not implement ULP timeout. The IUT does

not reset the connection or, if the connection is

reset, the number of IUT retransmissions indicate

that the abort was caused by the TCP default rather

than the prescribed ULP timeout.

UNISYS Corporation

6 February 1987                   -38-                      TM-WD-8801/206/00

TEST 57: IMPLEMENTATION OF ULP TIMEOUT SERVICE IN PASSIVE OPEN

-------

- Does the Implementation Under Test (IUT) implement ULP

timeout when timeout specified in its Passive Opens

- Action: The Remote Driver (RD) does a Passive Open with a

ULP timeout set. The Laboratory Slave Driver (LSD)

performs an Active Open and a connection is

established. The RD then sends data. The

Laboratory Reference Implementation (REF) withholds

acknowledgments. The test continues until the

connection terminates or 2 minutes pass.

- Verification: Check the TCP segments collected by the REF.

Count the number of times a data element is

retransmitted. The number of data retransmissions

must be significantly smaller than the default

number of retransmissions (determined in Test 55).

Success: IUT implements ULP timeout if it resets the

connections and the number of data retransmissions

is significantly smaller than the default number of

retransmissions.

Failure: IUT does not implement ULP timeout. The IUT does

not reset the connection or, if the connection is

reset, the number of IUT retransmissions indicate

that the abort was caused by the TCP default rather

than the prescribed ULP timeout.

TEST 58: IMPLEMENTATION OF ULP TIMEOUT NOTIFY SERVICE

-------

- Does the Implementation Under Test (IUT) implement a ULP

timeout where the timeout action is notifys

- Action: The Remote Driver (RD) does an Active Open with an

ULP notify timeout set and establishes a

connection with the Laboratory Slave Driver (LSD).

The RD sends data. The Laboratory Reference

Implementation (REF) withholds acknowledgments on

the data. The test continues until the connection

terminates or 2 minutes pass.

- Verification: Check that the RD reports the TCP ERROR message

ULP NOTIFY

Success: IUT implements ULP notify.

Failure: IUT does not implements ULP notify.

UNISYS Corporation

6 February 1987                   -39-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario FULL2

--------------

Tests the TCP implementation of zero window, urgent data, and pushed

data.

------------------------------------------

Test 59: RESPONSE TO ZERO WINDOW

-------

- Does the Implementation Under Test (IUT) correctly respond to

a peer's zero windows

- Action: A connection is established between the Laboratory

Slave Driver (LSD) and the Remote Driver (RD).

The RD sends data to the LSD. The Laboratory

Reference Implementation (REF) acknowledges the

first data segment but announces a zero window in

this acknowledgment.

- Verification: Check the TCP segments collected by the REF

for the amount of data sent by the IUT while the

REF had a zero window. The IUT should not send

more than one byte of data while the REF has a

zero window.

Success: The IUT sends no segment of-length greater than 1

while its peer has a zero window.

Failure: The IUT sends a segment of length greater than 1

while its peer has a zero window.

TEST 60: IMPLEMENTATION OF URGENT SERVICE

-------

- Does the Implementation Under Test (IUT) set the urgent flag

and urgent pointer when requested to send urgent datas

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

urgent data to the LSD.

- Verification: Check the TCP segment information collected by

the Laboratory Reference Implementation (REF) to

ensure that the IUT sets the urgent flag on every

segment of the data. Also check that the value of

the urgent pointer on these segments equals the

amount of urgent data that exists between the

first sequence number of the segment to the end of

the urgent data.

UNISYS Corporation

6 February 1987                   -40-                      TM-WD-8801/206/00

Success: The IUT correctly sets the urgent flag and urgent

pointer on urgent data.

Failure: The IUT does not set the urgent flag or urgent

pointer correctly on urgent data.

Test 61: URGENT SERVICE WHEN PEER HAS ZERO WINDOW

-------

- Does the Implementation Under Test (IUT) handle urgent

correctly when its peer has a zero window s

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

normal data and then urgent data to the LSD. The

Laboratory Reference Implementation (REF) sets its

receive window to zero when it acknowledges the

first IUT segment. The REF opens its window again

later in the data transfer.

- Verification: Check the TCP segments collected by the REF.

Ensure that IUT sends segment while the REF is

showing a zero window with the urgent flag set,

the value of the urgent pointer set to the number

of bytes of urgent data to be sent, and a length

no longer than 1.

Success: The IUT sends one byte probe segmensts and sets

the urgent flag and urgent pointer correctly in them.

Failure: The IUT does not send one byte probe segments

although urgent data is present, or it does not

set the urgent flag or urgent pointer correctly in

the probe segments it sends.

Test 62: DIFFERENTIATION BETWEEN URGENT AND NON-URGENT DATA

-------

- Is the Implementation Under Test (IUT) able to deliver urgent data?

- Action: To make sure that the IUT can differentiate and

deliver urgent and non-urgent data. The Laboratory

Slave Driver (LSD) and the Remote Driver (RD)

establish a connection. The LSD sends 1 byte of

urgent data to the RD. It then sends the RD some

bytes of non-urgent data.

- Verification: Evaluate the DELIVER responses from the RD.

The urgent data must be delivered in a separate

DELIVER from the non-urgent data.

UNISYS Corporation

6 February 1987                   -41-                      TM-WD- 801/206/00

Success: The RD DELIVER responses show the 1 byte of urgent

data delivered separately from the subsequent non

urgent data.

Failure: The RD DELIVER responses do not show the 1 byte of

urgent data delivered separately from the

subsequent non-urgent data.

Test 63: PUSH SERVICE WHEN NOT REQUESTED

-------

- Does the Implementation Under Test (IUT) push data when not

requested to do so?

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

data to the LSD. lt does not request that the

data be pushed.

- Verification: Examine the TCP segments collected by the REF.

The Push flag should not be set on any segments

sent by the IUT except possibly the last data

segment. (It is permissible for the IUT to set a

push flag on the very last data segment sent).

Success: The IUT correctly does not push any data or IUT

only pushes the last data segment.

Failure: The IUT incorrectly sets push flags on data not

requested to be pushed.

Test 64: PUSH SERVICE ON REQUEST

-------

- Is the Implementation Under Test (IUT) able to push data on

requests

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

some bytes of data with a push indication. It

then sends data without the push indication.

- Verification: Examine the TCP segments collected by the

Laboratory Reference Implementation (REF). The

first data segment the IUT sends should have the

push flag set and should contain at least the

number of bytes of data sent with the push

indication. The only other segment which could

have a valid push flag is the last data segment.

Success: The IUT correctly sets push flag only on pushed

data and possibly the last data segment.

Failure: The IUT fails to set push flag on pushed data or

sets push flag on unpushed data.

UNISYS Corporation

6 February 1987                   -42-                      TM-WD-8801/206/00

Observation: The IUT allows it Upper Level Protocol to

request rather than command push service. This

observation is made when the segment with the push

flag carries more than the pushed data.

UNISYS Corporation

6 February 1987                   -43-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario FULL3

--------------

Tests whether the TCP implementation does correct reset processing.

It tests the common reset conditions.

________________________________________

Test 65: RESPONSE TO CONNECTION REFUSAL

-------

- Does the Implementation Under Test (IUT) respond correctly to

connection refusals

- Action: The Remote Driver (RD) actively opens. There is

no listening process at the destination port of

its Active Open.

- Verification: Determine that the RD receives an OPEN FAILURE

response from the IUT.

Success: The IUT reports an OPEN FAILURE.

Failure: The IUT does not report an OPEN FAILURE .

Test 66: PARTIAL RESET PRIOR TO CONNECTION ESTABLISHMENT

-------

- Does the Implementation Under Test (IUT) continue listening

after receiving a reset during connection

establishments

- Action: The Remote Driver (RD) initiates a Passive Open.

The Laboratory Slave Driver (LSD) actively opens.

On receipt of the IUT's SYN ACK, the Laboratory

Reference Implementation (REF) resets the

connection. The LSD attempts to open the

connection again.

- Verification: Determine that the LSD reports an OPEN SUCCESS

response from the REF on the second connection.

Success: The second connection is established.

Failure: The second connection cannot be established.

UNISYS Corporation

6 February 1987                   -44-                      TM-WD-8801/206/00

Test 67: RESPONSE TO RESET RCVD WHILE SENDING DATA OVER CONNECTION

-------

- Does the IUT abort with the correct responses when its peer

aborts while the IUT is sending data over a connections

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The RD sends

data. When the Laboratory Reference Implementa

tion (REF) receives the first IUT data segment,

the REF resets the connection.

- Verification: Determine that after the reset, the RD reports

the response TERMINATE: REMOTE ABORT from the IUT

and the LSD reports the response TERMINATE:

SERVICE FAILURE from the REF.

Success: The correct TERMINATE responses are reported from

the IUT and the REF.

Failure: The correct TERMINATE responses are not received

from the IUT and the REF.

TEST 68: RESET FORMAT RESPONDING TO ACTIVE OPEN WITHOUT LISTENING PORT

-------

- Does the Implementation Under Test (IUT) send a correct reset

segment on receipt of a SYN for a non-existent ports

- Action: The Laboratory Slave Driver (LSD) initiates an

Active Open to a port on the IUT without a

listening process.

- Verification: Check the TCP segments collected by the

Laboratory Reference Implementation (REF). The

IUT reset segment must have the format: sequence

number = 0, acknowledgment number = the sequence

number of the REF's SYN(segment<s+1>s),and the ACK and

RESET flags set.

Success: IUT uses correct reset segment format.

Failure: The IUT uses incorrect reset segment format.

UNISYS Corporation

6 February 1987                   -45-                      TM-WD-8801/206/00

TEST 69: RESET FORMAT RESPONDING TO ACTIVE OPEN WITH DATA WITHOUT

-------  LISTENING PORT

- Does the Implementation Under Test (IUT) send a correct reset

when it receives the SYN of an Active Open with Data for a

non-existent ports

- Action: The Laboratory Slave Driver (LSD) initiates an

Active Open with Data to a port on the IUT

without a list process.

- Verification: Check the TCP segments collected by the

Laboratory Reference Implementation (REF). The

IUT reset segment must have the format: sequence

number = 0; acknowledgment number = the sequence

number of the last byte of data on the REF's syn

segment, and the ACK and RESET flags set. If the

IUT has not passed the Active Open with Data test,

the reset segment must have the expected format of

Test 68.

Success: IUT uses correct reset segment format.

Failure: IUT uses incorrect reset segment format.

TEST 70: RESET FORMAT ON RECEIPT OF INVALID SEGMENT WITH ACK SET

-------

- Does the Implementation Under Test (IUT) send the correct

reset format on receiving a segment with ACK set for a

non-existent port?

- Action: The Laboratory Slave Driver (LSD) initiates an

Active Open to a port on the IUT without a

listening process. The Laboratory Reference

Implementation (REF) omits the SYN from its

initial segment but sets the acknowledgment flag

and puts a value in the acknowledgment number.

- Verification: Check the TCP segments collected by the

Laboratory Reference Implementation. The IUT's

reset must have the format: sequence number =

acknowledgment number on the REF's initial

segment, and the RESET flag set. The ACK flag

must not be set.

Success: IUT uses correct reset segment format.

Failure: The IUT uses incorrect reset segment format.

UNISYS Corporation

6 February 1987                   -46-                      TM-WD-8801/206/00

TEST 71: RESET FORMAT ON RECEIPT OF INVALID SEGMENT WITH SYN AND ACK SET

-------

- Does the Implementation Under Test (IUT) send a correct reset

on receiving a SYN segment with ACK set for a port in LISTEN

state?

- Action: The Remote Driver (RD) does a Passive Open. The

Laboratory Slave Driver (LSD) does an Active Open.

The Laboratory Reference Implementation (REF)

places an acknowledgment on its SYN segment.

- Verification: Determine that no connection is established.

Check that an OPEN FAILURE is reported from the

REF. If the connection attempt correctly fails,

check the TCP segments collected by the REF. The

IUT reset must have the following format: sequence

number = acknowledgment number on the REF's

initial segment, and the RESET flag set. The ACK

flag must not be set.

Success: IUT resets and uses correct reset format.

Failure: The IUT establishes connection after receiving

invalid SYN segment or the IUT uses incorrect

reset segment format.

Test 72: NO RESET ON RECEIPT OF SEGMENT WITH BAD ACK

-------

- Does the Implementation Under Test (IUT) erroneously perform

a reset on receiving a segment with a bad acknowledgment number?

- Action: The Laboratory Slave Driver (LSD) and the Remote

Driver (RD) establish a connection. The LSD sends

data to the the RD. The Laboratory Reference

Implementation (REF) places an incorrect

acknowledgment number on an outgoing segment. The

REF retransmits the bad segment three times and

then corrects it.

- Verification: Determine that the IUT does not terminate the

connection. If the connection is not terminated,

check the TCP segments collected by the REF to

determine that the IUT has transmitted empty

acknowledgments. These acknowledgments must have

the format: sequence number = IUT's current

segment sequence number, acknowledgment number =

the sequence number of the REF's segment (not

incremented to acknowledge any of the data on the

bad segment) and the ACK flag set.

UNISYS Corporation

6 February 1987                   -47-                      TM-WD-8801/206/00

Success: The IUT sends empty acknowledgments until it

receives the corrected segment. The connection is

not reset.

Failure: The IUT resets the connection on receiving the bad

data segment or the IUT acknowledges data segments

with bad acknowledgment number.

UNISYS Corporation

6 February 1987                   -48-                      TM-WD-8801/206/00

TCP SCENARIOS AND TEST DESCRIPTIONS

Scenario QUAL1

--------------

QUAL1 tests the TCP implementation to determine how many TCE

it is able to provide. The scenario tests up to 145

____________________________

Test 73:  NUMBER OF TCP CONNECTIONS RESOURCES CAN SUPPORT

-------

- Determine the maximum number of TCP connections provided by

the Implementation Under Test(IUT).

- Action: The Laboratory Slave Driver (LSD) performs 12

passive opens. The Remote Driver (RD) is

instructed to consecutively open 12 active

connections to each of these ports until it runs

out of resources.

- Verification: Count every connection where an OPEN SUCCESS

is the response received on the IUT Active Open

Request. When the response TCP ERROR:

INSUFFICIENT RESOURCES is found or 144 connections

are opened, the test is ended. The total number

of connections the IUT can support equals the

number of connections the RD has opened plus the

connection for the command channel.

Observation: The total number of connections the IUT is able

to support is noted. If the IUT is able to

support more than 145 connections (it does not run

out of resources), this observation is noted.