PROGRAMMERS MANUAL FOR IMAGE DISPLAY AND I/O ROUTINES

IMDISP is written in a modular way so that it is easy to change

it to work with different display devices or on other computers.

The code can also easily be adapted for use in other application

programs.  There are device dependent image display modules,

general image display subroutines, image I/O routines, and image label

and command line parsing routines.  The general purpose display routines

can draw lines, write text with different fonts, manipulate a cursor,

interactively adjust the palette, display histograms, etc.

The IMDISP system is written in the "C" computer langauge.

Model of Device Independent Image Display

The image display is modeled as having coordinates ranging from 1 to

"dispnl" from top to bottom and from 1 to "dispns" from left to right

(i.e. a line-sample space).  There is one image plane that can take

on "numDN" values.  There is assumed to be a lookup table to convert

those DN values into voltages for the red, green, and blue guns in the

monitor.  There are "numshades" shades of each primary color for a total

of numshades**3 different colors.  The palette is a table of numDN

entries, with each entry containing the three primary color intensities

on a scale from 0 to 255.

There are five global variables that define the display

characteristics and are set in the DISPDRIV module:  dispnl, dispns,

numDN, numshades, OneScreen.  There are two related contants:

MAXDISPNL and MAXDISPNS.

The cursor, text, and font global variables are in the DISPUTIL module.

The image control block constants and variables are in the IMAGEIO module.

Compiling and Linking Routines

This document describes the "C" language version of the IMDISP

system.  The routines are written in Microsoft C version 5.1 on an

IBM AT compatible computer.  The large memory module is used.  There

is one module (dispsub.asm) written in 8086 assembly language for reasons

of execution speed.

The following commands are are used to compile and link the subroutines

and programs.  They are contained in the COMPLINK.BAT file.

cl /AL /c dispio.c disputil.c fileio.c fileutil.c imageio.c imagutil.c

imdisp.c labutil.c memutil.c

masm dispsub.asm;

link/NOE imdisp+dispsub+dispio+disputil+fileio+filesel+imagutil+

imageio+labutil+memutil/stack:8000/exepack;

LIST OF PROGRAMS

IMDISP.C     - Main program, help, command drivers.

DISPIO.C     - Display on/off, read/write lines, pixels, palette.

DISPUTIL.C   - Font, cursor, text and palette routines.

DISPSUB.ASM  - Assembly language routines for EGA/PGA display.

FILEIO.C     - MSDOS and CDROM file input/output routines.

FILEUTIL.C   - File menu and wild card routines.

IMAGEIO.C    - Image open/close/read routines.

IMAGUTIL.C   - Palette, histogram, profile and stretch routines.

LABUTIL.C    - Label parsing routines.

MEMUTIL.C    - Special memory allocation routines (malloc/free).

-----------------------------------------------------------------

Routines for image display

--------------------------

Module: DISPIO

DisplayOn

DisplayOff

WritePalette (palette)

WritePixel (line, sample, DNvalue)

ReadPixel (line, sample, DNvalue)

DisplayLine (buffer, line, samp, ns)

ClearDisplay (DNvalue)

PutRefresh (buffer, line, ss, ns)

GetRefresh (buffer, line, ss, ns)

Module: DISPSUB

_putmem

_getmem

WritePixelEGA (line, samp, DN)

DisplayLineEGA (buffer, line, samp, ns)

int ClearDisplayEGA (DN,dispnl)

_PGAsend

_EGAinit        (initializes special 480 line EGA mode).

Module: DISPUTIL

InitDisplay

ReadPalette (palette)

DrawLine (line1, sample1, line2, sample2, DNvalue)

DrawBox (line, samp, linesize, sampsize, DNvalue)

RemoveCursor

PlaceCursor (line, sample, DNvalue)

MoveCursor (line, sample)

Font (FontType)

DrawText (text, line, sample, height, angle, DNvalue)

LengthText (text, height, TextLength)

TypeText (text)

TypeTextLine (text)

AcceptText (string)

WriteText (text)

Module: IMAGUTIL

DiddlePalette (numcolors)

DisplayHistogram (histbuffer, numbins, minDN, maxDN)

Stretch (DNlow, DNhigh : Integer);

Profile

Routines for image and file Input/Output

----------------------------------------

Module: FILEUTIL

FileSel (selFilename)

Wildexp (argc, argv)

Module: IMAGEIO

OpenImage (filename, unit, IOmode, nl, ns, bitsperpix, status)

ReadLine (unit, buffer, line, ss, ns, status)

WriteLine (unit, buffer, line, ss, ns, status)

CloseImage (unit, status)

CheckStatus (status)

ConvertLine (inbuffer, outbuffer, inbits, outbits, ns, status)

Module: LABUTIL

GetLabelInteger (LabelBuf, Len, Keyword, default, value, flag)

GetLabelReal (LabelBuf, Len, Keyword, default, value, flag)

GetLabelString (LabelBuf, Len, Keyword, default, value, flag)

InterpretLabel (LabelBuf, Len, labelsize, nline, nsamp, bitsperpix,

reclen, lineheadbytes, DetachedFilename)

MakeLabel (LabelBuf, labelsize, nline, nsamp, bitsperpix, reclen)

GetCommand (CommandString, CommandList)

GetKeywordInteger (CommandString, Keyword, default, value, flag)

GetKeywordReal (CommandString, Keyword, default, value, flag)

GetKeywordString (CommandString, Keyword, default, value, flag)

Module: FILEIO

OpenFile (filename, unit, IOmode, blocksize, status)

ReadBlocks (unit, buffer, StartBlock, NumBlocks, BlocksRead, status)

WriteBlocks (unit, buffer, StartBlock, NumBlocks, status)

CloseFile (unit, status)

InitializeCD (status)

ReadBlocksCD (buffer, StartBlock, NumBlocks, status)

SUMMARY AND USE OF SUBROUTINES

/***    Module:    DISPIO

DISPIO contains the device dependent display routines for

the Color Graphics Adapter (CGA), the Enhanced Graphics Adapter (EGA),

the Professional Graphics Adapter (PGA), and the Video Graphics Array

(VGA).  The global variables that define the device are allocated and

initialized here.

***/

DisplayOn()

/***  DisplayOn figures out which display device is on the system and

then initializes for that device.  The device is turned on.

The global variables that define the display characteristics

and the default color palette are initialized.  The refresh

buffer is zeroed.

***/

DisplayOff()

/***  DisplayOff turns off the display device returning the screen

to text mode.

***/

WritePalette (coltab)

/***  WritePalette updates the display device palette with the

palette passed in coltab.  coltab must be of type Color.

***/

WritePixel (line, sample, DN)

/***  WritePixel writes a pixel on the display screen.

Parameter   type       description

line     integer     The line coordinate of the pixel

sample   integer     The sample coordinate of the pixel

DN       integer     The DN value of the pixel

***/

ReadPixel (line, sample, p_DN)

/***  ReadPixel read a pixel value from the display screen.

Parameter   type       description

line     integer     The line coordinate of the pixel

sample   integer     The sample coordinate of the pixel

p_DN     int ptr     The DN value of the pixel is returned

***/

DisplayLine (buffer, line, sample, ns)

/***  DisplayLine writes a line of pixels on the display screen.

Parameter   type       description

buffer   char ptr    The array of pixel values

line     integer     The line coordinate of the first pixel

sample   integer     The sample coordinate of the first pixel

ns       integer     The number of pixels to display

***/

ClearDisplay (DN)

/***  ClearDisplay sets the whole display to a particular value.

Parameter   type       description

DN       integer     The DN value to set (usually 0)

***/

PutRefresh (buffer, line, ss, ns)

/***  PutRefresh stores a line of pixels in the refresh buffer.

Parameter   type       description

buffer   char ptr    The array of pixel values

line     integer     The line coordinate of the first pixel

sample   integer     The sample coordinate of the first pixel

ns       integer     The number of pixels to store

***/

GetRefresh (buffer, line, ss, ns)

/***  GetRefresh reads a line of pixels from the refresh buffer.

Parameter   type       description

buffer   char ptr    The receiving array of pixel values

line     integer     The line coordinate of the first pixel

sample   integer     The sample coordinate of the first pixel

ns       integer     The number of pixels to read

***/

--------------------------------------------------------------------------

/***  Module: DISPUTIL

Device Independant General Purpose Graphics Routines

Includes line drawing, text display, and cursor routines.

***/

InitDisplay()

/*** InitDisplay turns the display device on, loads the default palette,

initializes the global variables, and loads the font arrays.

***/

ReadPalette (coltab)

/***  ReadPalette returns the current color palette in coltab.

coltab is of type Color.

***/

DrawLine (x1, y1, x2, y2, color)

/***  DrawLine draws a line of pixels from the first point to the

second point with the desired DN value.  Clipping is not performed.

Parameter  Type   Description

x1       int    the line coordinate of the first point

y1       int    the sample coordinate of the first point

x2       int    the line coordinate of the second point

y2       int    the sample coordinate of the second point

color    int    the DN value of the drawn pixels

***/

DrawBox (line, samp, lsize, ssize, color)

/*** DrawBox draws a solid rectangle on the screen.

Used for erasing parts of the screen.

Parameter   Type     Description

line       int      starting line coordinate of rectangle

samp       int      starting sample coordinate of rectangle

lsize      int      number of lines in rectangle

ssize      int      number of samples in rectangle

color      int      the DN value to draw the box

***/

Font (FontType)

/***  Font reads in the font file for type FontType and fills the

font arrays appropriately.  If FontType = 0 the internal font file

is used instead.  FontType = 5 reads in font file 005.FNT, etc.

The global variable thefont is set to the font number.

The format of a font file is a sequence of bytes.  The first byte

is the number of characters defined in the font.  For each

character there is the following:  the ascii code, the number of

line strokes, the character width, and then the list of line strokes.

Each line stroke consists of three bytes: the pen motion, and the

x and y coordinate.  The width, x, and y values are converted

back into character coordinates by subtracting 50 and dividing

by 100.  The pen is 2 for pen down, and 3 for pen up.

***/

DrawText (text, line, sample, height, angle, DNvalue)

/***  DrawText draws a text string on the display using the

current font.

Parameter  Type     Description

text     char ptr  text string to display

line     int       line coordinate of lower left of first char

sample   int       sample coordinate of lower left of first char

height   int       height of characters in pixels

angle    int       angle in degrees ccw from positive sample direction

DNvalue  int       the DN value to draw the pixels in with

***/

LengthText (text, height, p_TextLength)

/*** LengthText finds the length in pixels of a given text string.

Parameter       Type     Description

text          char ptr   text string to display

height         int       height of characters in pixels

p_TextLength  int ptr    where to return length of string

***/

TypeText (TypeString)

/***  TypeText displays a line of text on the screen at the current

text location with a height of 15 pixels.  The area is first erased.

The text position is advanced to the next position after the last

character.

Parameter       Type     Description

TypeString    char ptr   text string to display

***/

TypeTextLine (TypeString)

/***  TypeTextLine displays a line of text on the screen using TypeText.

The text position is advanced to the beginning of the next line.

Parameter       Type     Description

TypeString    char ptr   text string to display

***/

AcceptText (AcceptString)

/***  AcceptString gets a string from the keyboard.  The characters

are echoed on the display screen using DrawText.  The string is

terminated with a return (ascii 13).  Backspace deleting is supported.

The text position is left at the beginning of the next line.

Parameter       Type      Description

AcceptString    char ptr  returned text string from user

***/

WriteText (text)

/***  WriteText writes out a line of text to the user.  If there

is only one screen then the text is written on the image display.

Otherwise it is written to the terminal.

Parameter     Type     Description

text        char ptr   text string to display

***/

RemoveCursor()

/***  RemoveCursor removes the cursor, if it is on, from the display

screen, replacing the original pixel values.

***/

PlaceCursor (line, sample, cursDN)

/***  PlaceCursor puts the cursor, if it is not already on, on the

screen, while storing the original pixel values.

Parameter     Type     Description

line         int      the line coordinate to put the cursor at

sample       int      the sample coordinate to put the cursor at

cursDN       int      the DN value of the cursor

***/

MoveCursor (p_line, p_sample)

/***  MoveCursor performs the cursor mode operation.

The user moves the cursor around using the numeric keypad.

The cursor moves in discrete steps whose size can be changed.

The final cursor position is returned.

Parameter     Type      Description

p_line     int ptr     the cursor line coordinate

p_sample   int ptr     the cursor sample coordinate

***/

--------------------------------------------------------------------------

/***  Module: IMAGUTIL

Special Purpose Device Independant Display Routines

Contains high level display routines such as interactive palette

adjustment, histogram display, and image profile drawing.

***/

DiddlePalette (numdiddle)

/***  DiddlePalette performs interactive adjustment of the color

palette.  A wedge of numdiddle DN values, from 0 to the maximum,

is displayed at the bottom of the screen, and the palette is

changed to have only numdiddle distinct shades.  Then a loop

for user input is entered.  The active color is the one in the

wedge with the small black box inside it.

The following is a list of commands:

Character      Action

4 (6)      move to next lower (higher) color in wedge

r (R)      decrease (increase) amount of red one notch

g (G)      decrease (increase) amount of green one notch

b (B)      decrease (increase) amount of blue one notch

2 (8)      decrease (increase) amount of all colors one notch

.          exit from palette diddle mode

The wedge is erased after the user exits.

Parameter   Type   Description

numdiddle   int   number of colors in the wedge and resulting palette

***/

DisplayHistogram (histbuf, numbins, minDN, maxDN)

/***  DisplayHistogram diplays the passed histogram on the screen.

The third highest bin in the histogram is used to scale the

plot.  The axes of the plot are labeled and annotated.  The

number in each bin is indicated by the length of the line

drawn vertically from the axis.

Parameter  Type            Description

histbuf   long int array  the array of count values for each bin

numbins   int             the number of bins in the histogram

minDN     int             the minimum DN value in the histogram

maxDN     int             the maximum DN value in the histogram

***/

Stretch (DNlow, DNhigh)

/***  Stretch applies a linear gray scale ramp to the palette

starting with 0 at Dnlow and proceeding to 255 at DNhigh.

Parameter  Type   Description

DNlow     int    the starting DN value of the linear ramp

DNhigh    int    the ending DN value of the linear ramp

***/

Profile()

/***  Profile plots a profile of the pixel values between two points

on the screen.  Cursor mode is used to select the two endpoints.

A line is drawn between the endpoints and a plot of DN value

versus distance along the line is made.

***/

--------------------------------------------------------------------------

/***  Module:  IMAGEIO

Image I/O routines for labeled images

Includes routines for opening images, reading and writing

image lines, and converting pixel formats.

***/

OpenImage (filename, unit, IOmode, p_nline, p_nsamp, p_bitsperpix, status)

/***  OpenImage opens an image file either for reading or writing.

The size of the image and pixel format are returned if the file

is opened for reading.  If the file is opened for writing

then the passed number of lines and samples and pixel format are

used for the new image.  The unit number is used to refer to the

file in subsequent operations.  If the file is opened for input

then the label processing routines will parse Vicar2 and PDS

labels and will prompt for user info if the image is unlabeled.

PDS detached labels are handled transparently in this routine.

If the file is for writing then a PDS label will be output.

An error is returned if there is insufficient room to dynamically

allocate the image buffer, or if there is a file error.

For images with pixel sizes less than a byte, each line is assumed

to start with a new byte.

Parameter    Type    In/out  Description

filename  char ptr   in     Name of image file to open

unit        int      in     Unit number of image (0,1,2...)

IOmode    char ptr   in     Access mode of file

("r..." for reading, "w..." for writing)

p_nline   int ptr  in/out   The number of lines in the image

p_nsamp   int ptr  in/out   The number of samples in the image

p_bitsperpix int ptr  in/out   The number of bits in a pixel (pixel format)

(16 for integer, 8 for byte,

4 for nibble,  1 for binary)

status    char ptr  out     Return string with error message

(0 length string if no error)

***/

ReadLine (unit, buffer, line, ss, ns, status)

/***  ReadLine reads a line of pixels from the image into the user's

buffer.  The reading is completely random access, the lines may

be read in any order; it is more efficient, however, to read lines

sequentially.  Partial image lines may be read.  The output data

is in the same format as the file (the convert routine may be

used to convert pixel formats).

Parameter Type   In/out  Description

unit     int      in     Unit number of image (same as in open)

buffer  char ptr  out    Buffer to receive pixels

line     int      in     The number of the image line to read (1 is first)

ss       int      in     The starting sample in the line

ns       int      in     The number of samples to read into buffer

status  char ptr  out    Return string with error message

(0 length string if no error)

***/

WriteLine (unit, buffer, line, ss, ns, status)

/***  WriteLine writes a line of pixels from the user buffer to the image.

Although the writing is buffered, completely random access will not

work; the image lines should be written sequentially.

Partial image lines may be written.  The input data must be in the

same format as the file (no conversion is performed).

Parameter Type   In/out  Description

unit     int      in     Unit number of image (same as in open)

buffer  char ptr  in     Buffer of image pixels

line     int      in     The number of the image line to write

ss       int      in     The starting sample in the line

ns       int      in     The number of samples to write from buffer

status  char ptr  out    Return string with error message

(0 length string if no error)

***/

CloseImage (unit, status)

/***  CloseImage closes the image file.  If the image is opened for

writing then the image buffer is first flushed.

Parameter Type   In/out  Description

unit     int      in     Unit number of image (same as in open)

status  char ptr  out    Return string with error message

(0 length string if no error)

***/

CheckStatus (status)

/***  CheckStatus checks the status string passed in.  If there

is no error the routine returns, otherwise the error message

is printed to the terminal and the program is aborted.

***/

ConvertLine (inbuffer, outbuffer, inbits, outbits, ns, status)

/***  ConvertLine converts a line of pixels into a different format.

Parameter     Type   In/out  Description

inbuffer   char ptr  in    The input buffer of pixels

outbuffer  char ptr  out   The output buffer of pixels

inbits      int      in    The pixel format of the input buffer

outbits     int      in    The pixel format of the output buffer

ns          int      in    The number of samples to convert

status     char ptr  out   Return string with error message

Conversions supported :        outbits

1  4  8  16

inbits    1  .  -  .  .      . means supported

4  -  .  .  .

8  .  .  .  .

16  .  .  .  .

***/

--------------------------------------------------------------------------

/***  Module: LABUTIL

Label Processing Routines

***/

GetLabelInteger (LabelBuf, LabelBufLen, Keyword, defaul, p_value, p_flag)

/***  GetLabelInteger returns the integer value following the keyword in the

label buffer.  If the keyword is not there then the default value

is returned.  The flag indicates whether the keyword is there.

Parameter    Type    In/out   Description

LabelBuf  char ptr   in      The buffer containing the label

LabelBufLen  int     in      The length in bytes of the label buffer

Keyword   char ptr   in      The keyword string to match

defaul       int     in      The value to return if no keyword or value

p_value    int ptr   out     The returned value

p_flag     int ptr   out     The flag: -1 if no keyword match

0 if illegal value, 1 if keyword and value

***/

GetLabelReal (LabelBuf, LabelBufLen, Keyword, defaul, p_value, p_flag)

/***  GetLabelReal returns the real value following the keyword in the

label buffer.  If the keyword is not there then the default value

is returned.  The flag indicates whether the keyword is there.

Parameter    Type    In/out   Description

LabelBuf  char ptr   in      The buffer containing the label

LabelBufLen  int     in      The length in bytes of the label buffer

Keyword   char ptr   in      The keyword string to match

defaul      float    in      The value to return if no keyword or value

p_value   float ptr  out     The returned value

p_flag     int ptr   out     The flag: -1 if no keyword match

0 if illegal value, 1 if keyword and value

***/

GetLabelString (LabelBuf, LabelBufLen, Keyword, defaul, value, p_flag)

/***  GetLabelString returns the string value following the keyword in the

label buffer.  If the keyword is not there then the default value

is returned.  The flag indicates whether the keyword is there.

Parameter    Type    In/out   Description

LabelBuf  char ptr   in      The buffer containing the label

LabelBufLen  int     in      The length in bytes of the label buffer

Keyword   char ptr   in      The keyword string to match

defaul    char ptr   in      The value to return if no keyword or value

p_value   char ptr   out     The returned value

p_flag     int ptr   out     The flag: -1 if no keyword match

0 if illegal value, 1 if keyword and value

***/

InterpretLabel (buf, len, p_labelsize, p_nline, p_nsamp,

p_bitsperpix, p_reclen, p_lineheadbytes,

DetachedFileName)

/***  InterpretLabel interprets the label in the buffer to extract

the image information.  First something is identified to determine

whether the image has a PDS label or a Vicar2 label or no label.

If the image is labeled then the appropriate keyword values are

extracted, otherwise the user is prompted for the into.

Parameter    Type   In/out  Description

buf        char ptr  in    The label buffer

len          int     in    The length in bytes of the label buffer

p_labelsize   int ptr  out   The size of the label or header to skip

p_nline       int ptr  out   The number of lines in the image

p_nsamp       int ptr  out   The number of samples in the image

p_bitsperpix  int ptr  out   The pixel format (16,8,4,1)

p_reclen      int ptr  out   The record length in bytes

p_lineheadbytes int ptr  out   The header bytes at beginning of each line

DetachedFileName char ptr out  The name of the file of data

null string if no detached label

***/

MakeLabel (Buf, p_labelsize, nline, nsamp, bitsperpix, reclen)

/***  MakeLabel creates a PDS label in the buffer, having the appropriate

image parameters.

Parameter    Type   In/out  Description

Buf        char ptr  out   The label buffer

p_labelsize  int ptr   out   The size of the label

nline       int      out   The number of lines in the image

nsamp       int      out   The number of samples in the image

bitsperpix  int      out   The pixel format (16,8,4,1)

reclen      int      out   The record length in bytes

***/

int GetCommand (CommandString, CommandList)

/***  GetCommand is an integer function that returns the number of

the command that is in the beginning of the CommandString parameter.

The command is assigned a number according to the list of

commands in the CommandList string parameter.  The list of commands

should be in uppercase and have spaces between the commmand.

***/

GetKeywordInteger (CommandString, Keyword, defaul, p_value, p_flag)

/***  GetKeywordInteger scans the CommandString for the Keyword

and returns the value of the following integer.

Parameter       Type      In/out   Description

CommandString  char ptr    in      The command string to scan thru

Keyword        char ptr    in      The keyword string to match

defaul          int        in      The default value to return if

no keyword or illegal value

p_value        int ptr     out     The returned value

p_flag         int ptr     out     The flag:  -1 for no keyword match

0 for keyword with no/bad value

1 for keyword and good value

***/

GetKeywordReal (CommandString, Keyword, defaul, p_value, p_flag)

/***  GetKeywordReal scans the CommandString for the Keyword

and returns the value of the following real.

Parameter       Type      In/out   Description

CommandString  char ptr    in      The command string to scan thru

Keyword        char ptr    in      The keyword string to match

defaul          float      in      The default value to return if

no keyword or illegal value

p_value        float ptr   out     The returned value

p_flag         int ptr     out     The flag:  -1 for no keyword match

0 for keyword with no/bad value

1 for keyword and good value

***/

GetKeywordString (CommandString, Keyword, defaul, value, p_flag)

/***  GetKeywordString scans the CommandString for the Keyword

and returns the value of the following string.

Parameter       Type      In/out   Description

CommandString  char ptr    in      The command string to scan thru

Keyword        char ptr    in      The keyword string to match

defaul         char ptr    in      The default value to return if

no keyword or illegal value

p_value        char ptr    out     The returned value

p_flag         int ptr     out     The flag:  -1 for no keyword match

0 for keyword with no/bad value

1 for keyword and good value

***/

--------------------------------------------------------------------------

/***  Module:  FILEIO

Contains block level I/O routines for MS-DOS files.

***/

OpenFile (filename, unit, IOmode, p_blocksize, status)

/***  OpenFile opens a file with the given file name.

The file may be open for reading or writing.

If the file name begins with "CD:" then it is assumed to be an

absolute CD-ROM file on a non-standard CD-ROM.  The file name is

of the form "CD:mm:ss:bb", where mm is the minute of the starting

sector, ss is the second, and bb is the block number within the second.

The logical blocksize to be used in the image I/O routines

is returned.

Parameter    Type    In/out   Description

filename   char ptr  in      File name string

unit        int      in      Unit number (0,1,2...)

IOmode     char ptr  in      "r..." for reading, 'w...' for writing

p_blocksize  int ptr  out     The returned logical blocksize to use

status     char ptr  out     The error message string

(0 length for no error)

***/

ReadBlocks (unit, buffer, startblock, numblocks, p_blocksread, status)

/***  ReadBlocks reads blocks from the file into the buffer.

Parameter    Type    In/out   Description

unit        int      in      Unit number from open

buffer     char ptr  out     The output buffer of data

startblock long int  in      The starting logical block

numblocks   int      in      The number of blocks to read

p_blocksread  int ptr   out     The number of blocks actually read

status     char ptr  out     The error message string

(0 length for no error)

***/

WriteBlocks (unit, buffer, startblock, numblocks, status)

/***  WriteBlocks writes blocks from the buffer to the file.

Parameter    Type    In/out   Description

unit        int      in      Unit number from open

buffer     char ptr  in      The input buffer of data

startblock long int  in      The starting logical block

numblocks   int      in      The number of blocks to write

status     char ptr  out     The error message string

(0 length for no error)

***/

CloseFile (unit, status)

/*** CloseFile closes the file.

Parameter    Type    In/out   Description

unit        int      in      Unit number from open

status     char ptr  out     The error message string

(0 length for no error)

***/

--------------------------------------------------------------------------

InitializeCD (p_PB_size, status)

/***  InitializeCD initializes the CD player and returns the

physical block size.

Parameter   Type      Description

p_PB_size   int ptr   The size of the physical blocks for this driver.

(in this case 2048 bytes)

status      char ptr  Error message string (0 length if no error)

***/

ReadBlocksCD (buffer, StartPB, NumPB, status)

/***  ReadBlocksCD reads blocks from the CD-ROM to the user's buffer.

Parameter    Type      Description

buffer     char ptr   Buffer to receive data

StartPB    long int   The starting absolute physical block

NumPB      int        The number of physical blocks to read

status     char ptr   Error message string (0 length if no error)

***/

--------------------------------------------------------------------------

Low Level Block I/O with MS-DOS for non-standard CD-ROM's

InitializeCD (p_PB_size, status)

/***  InitializeCD initializes the CD player and returns the

physical block size.

Parameter   Type      Description

p_PB_size   int ptr   The size of the physical blocks for this driver.

(in this case 2048 bytes)

status      char ptr  Error message string (0 length if no error)

***/

ReadBlocksCD (buffer, StartPB, NumPB, status)

/***  ReadBlocksCD reads blocks from the CD-ROM to the user's buffer.

Parameter    Type      Description

buffer     char ptr   Buffer to receive data

StartPB    long int   The starting absolute physical block

NumPB      int        The number of physical blocks to read

status     char ptr   Error message string (0 length if no error)

***/

--------------------------------------------------------------------

/***  Module:  FILEUTIL

Contains file selection and wildcard expansion routines.

***/

int FileSel(selFilename)

/*** filesel -- allow user to browse directories and select image

files for display.  Returns selected file name.

Parameter    Type       Description

selFilename char ptr   filename selected for processing.

***/

wildexp(oargcp, oargvp)

/*** wildexp -- expand wildcard file name.  Returns array of pointers

to filenames meeting wildcard specifications.

Parameter    Type       Description

oargcp    int ptr      pointer to number of values in oargvp

oargvp char ptr ptr ptr pointers to text strings containing

file names meeting wild card specification.

***/

Memory Allocation Procedures

Willard Gersbacher  - CompuServe User ID: (76117,2611)

The procedures included in this archive provide functionally equivalent

memory allocation procedures for the main memory procedures used in C.

These procedures were written primarily because of a need to minimize

memory usage for a large program under development.  The MSC 5.1

library procedures were not satisfactory for this application because

they did not release memory back to the system when it was 'free'ed.

Additionally, they tended to allocate too much memory under certain

circumstances which lead to a reduction in space available for other

programs and data which were currently active at the time.

These procedures should only be used with large data model programs

(i.e., compact, large, and huge memory models).  No provision has been

made for small or medium model programs.

/* The following routines from the ALLOC.ARC distribution file

have all been incorporated in MEMUTIL.C */

free.c       -  Functional equivalent to free.  Included in this

file is an additional procedure _ffree which calls

free.

malloc.c     -  Functional equivalent of malloc.  This file also

contains the memory structures which are used to

keep track of the blocks of memory allocated.  In

addition, the procedure _fmalloc is defined which

just makes a call to malloc.

memutil.c    -  This file contains a set of procedures which are

used by the main memory allocation procedures to

maintain the memory structure used to keep track of

the user allocations and frees.

reduceal.c   -  This procedure may be called to reduce the memory

allocated to a program.  It attempts to release back

to DOS the memory not currently being used.

reducedd.c   -  This procedure may be called to reduce the default

data segment to the minimum size required.  Normally

this should be called before any calls to memory

allocation procedures.  Included in this file are

dummy procedures for _nmalloc and _nfree.  These

latter procedures were included because some

of the MSC library uses the near heap for memory

allocation (even though you may be using large

data model programs).  Since this procedure does

away with any space which might be used for near

heap allocation, this forces the procedures which

call them to use the 'far' heap.

###########################################