The TOMS program reads and displays the ozone data on this CD-ROM. It is

divided into three major parts:

Processing Daily Gridded Data:

-----------------------------

The following options are available:

a) Read the daily gridded data for a specific day. The user is prompted for

a date and the daily gridded data for that day is read into a gridded

array.  The dimensions of the array are 180 rows by 288 columns.  Each

grid cell in the array covers a 1.0 degree latitude by a 1.25 degree

longitude area, and therefore the array covers the entire world.  The

latitudes range from -90.0 to 90.0 and the longitudes range from -180.0

to 180.0.  Zero values in the array denote flagged data (bad or missing

data).

b) Display the current daily gridded data in table format using one of

three resolutions:  full resolution (every value in the gridded array is

displayed), half resolution (every other value in the gridded array is

displayed), or quarter resolution (every fourth value in the gridded

array is displayed).  The user can move through the table using the

movement keys (press F1 for help after this option has been chosen).

c) Calculate and display statistical values for the current day.  The user

is prompted for the latitude range (and possibly the longitude range) of

the zone to be processed and the following values are displayed for this

zone:  minimum ozone value and its location, maximum ozone value and its

location, the area weighted zonal mean (each one-degree latitude zone

within this zone has a different weight), the standard deviation, and

the percentage of valid (not flagged) data points present within this

zone (flagged points are points that were set to 0 in the data due to

bad or missing data).  Only valid points are included in the calculation

of the zonal mean and the standard deviation.

d) Display the ozone value at a specific location for the current day. The

user is prompted for the latitude and longitude of the location to be

processed and the grid points are interpolated to produce the ozone

value at that location.  If the grid points can not be interpolated due

to flagged data, the latitude and longitude of the grid cell

corresponding to the location are displayed instead of the latitude and

longitude entered by the user.

Processing Daily or Monthly Statistical Data:

--------------------------------------------

The user is prompted for the type of data to be processed (daily or

monthly) and for the date (day or month).  Then, the pre-computed

statistical data for this day or month is read in and the following values

are displayed: global minimum ozone value, global maximum ozone value, and

the latitude range, minimum value, maximum value, area weighted zonal mean,

and standard deviation for each of the 18 ten-degree latitude zones (each

one-degree latitude zone within the ten-degree latitude zone has a

different weight). When these zonal means and standard deviations were

calculated, if there were not enough valid data points present within the

ten-degree latitude zone (85% or more) or enough valid (non-zero) daily

zonal means or standard deviations for the month (20 days or more), the

daily or monthly zonal means and standard deviations were set to 0.0.

Processing Time Series Data:

---------------------------

The following options are available:

a) Read and display daily or monthly pre-computed statistical data. The

user has the option to display all zonal statistics for a specific zone

for a range of days or months and a single year, or to display one

statistical value for a range of days or months and multiple years.  The

user is prompted for the year(s) to be displayed, the type of statistics

to be displayed (if appropriate), the zone number (if appropriate --

these are 10-degree latitude zones), and the first and last days or

months of the period to be processed.  If displaying a single year, the

following statistical values are displayed for one zone: the minimum

ozone value, the maximum ozone value, the area weighted zonal mean, and

the standard deviation.  If displaying multiple years, one of the

following statistical values is displayed: global minimum ozone value,

global maximum ozone value, area weighted zonal mean for a specific

zone, standard deviation for a specific zone, minimum ozone value in a

specific zone, maximum ozone value in a specific zone.

b) Display the ozone values at a specific location for any range of days

(could include the entire database).  The user is prompted for the

latitude and longitude of the location and for the first and last days

of the period to be processed.  Due to the amount of time that it takes

to read the daily gridded data for one day, this part of the program

might take a VERY LONG time to execute if the user chooses to process

lots of days.

c) Display the mean for a specific zone for any range of days (could

include the entire database).  The user is prompted for the latitude and

longitude ranges of the zone and for the first and last days of the

period to be processed.  Due to the amount of time that it takes to read

the daily gridded data for one day, this part of the program might take

a VERY LONG time to execute if the user chooses to process lots of days.