GEOPHYSICS OF NORTH AMERICA                                    etopo5b.doc

- 5 MINUTE TOPOGRAPHY/BATHYMETRY CONTOURS

This directory contains the contours of the 5 minute topography and

bathymetry data set.

In order to understand how the data are arranged, one must understand

something about the process of generating these data.  The contouring was

done in 100 regions each 15 by 15 degrees.  Each contiguous section of each

contour line is called a segment and each segment is made up of some number

of line fragments.  Each segment starts with an implicit move command and

the fragments are the endpoints of a series of draw commands.  The data

storage scheme therefore has three basic units: regions, segments, and

points (where points are the ends of fragments).

The regionalization of the data is expressed in the file structure.  The

data are stored in 72 files, each called ETOPO5.NUM, where NUM is the region

number.  Each data file has the longitude and latitude pairs of the

endpoints of the contour lines for all of the contours within the given

region.  These values are stored in two 4-byte floating point numbers.

Parameters for ETOPO5 Contour Regions:

Region   Minimum  Minimum   Maximum  Maximum Number of Number of

Longitude Latitude Longitude Latitude  Segments    Points

0       170   75.083   184.917   89.674        58      9050

1       170   60.083   184.917   75.167       242      9463

2       170   45.083   184.917   60.167       523     28953

3       170   30.083   184.917   45.167       937     29778

4       170   15.083   184.917   30.167      1419     33312

5       170    0.083   184.917   15.167       993     24397

6   184.833   75.083   199.917   89.821        81     12537

7   184.833   60.083   199.917   75.167       166      4925

8   184.833   45.083   199.917   60.167       340     19195

9   184.833   30.083   199.917   45.167       782     16017

10   184.833   15.083   199.917   30.167      1569     39950

11   184.833    0.083   199.917   15.167      1608     30427

12   199.833   75.083   214.917   89.866        72     10786

13   199.833   60.083   214.917   75.167       553     19750

14   199.833   45.083   214.917   60.167       709     19572

15   199.833   30.083   214.917   45.167       950     15433

16   199.833   15.083   214.917   30.167      1313     25058

17   199.833    0.083   214.917   15.167      1123     20232

18   214.833   75.083   229.917   89.866        49      6937

19   214.833   60.083   229.917   75.167       746     20892

20   214.833   45.083   229.917   60.167      1564     28256

21   214.833   30.083   229.917   45.167      1406     20005

22   214.833   15.083   229.917   30.167      1005     15816

23   214.833    0.083   229.917   15.167       406      7947

24   229.833   75.083   244.917   89.833        76     12642

25   229.833   60.083   244.917   75.167       608     13877

26   229.833   45.083   244.917   60.167      2748     45774

27   229.833   30.083   244.917   45.167      1872     38753

28   229.833   15.083   244.917   30.167      1489     22037

29   229.833    0.083   244.917   15.167       474      8678

30   244.833   75.083   259.917   89.926       106     11038

31   244.833   60.083   259.917   75.167       159      5634

32   244.833   45.083   259.917   60.167       548     11254

33   244.833   30.083   259.917   45.167      1314     30710

34   244.833   15.083   259.917   30.167      2032     43493

35   244.833    0.083   259.917   15.167       846     14853

36   259.833   75.083   274.917   89.955       215     15071

37   259.833   60.083   274.917   75.167       220      6923

38   259.833   45.083   274.917   60.167       120      3383

39   259.833   30.083   274.917   45.167       198      4804

40   259.833   15.083   274.917   30.167       832     30647

41   259.833    0.083   274.917   15.167       988     24543

42   274.833   75.083   289.917   89.962       321     22769

43   274.833   60.083   289.917   75.167       408     12129

44   274.833   45.083   289.917   60.167       280      5756

45   274.833   30.083   289.917   45.167       337     12307

46   274.833   15.083   289.917   30.167      1100     45348

47   274.833    0.083   289.917   15.167      1039     38914

48   289.833   75.083   304.917   89.961       172     15964

49   289.833   60.083   304.917   75.167       421     16172

50   289.833   45.083   304.917   60.167       581     13936

51   289.833   30.083   304.917   45.167       749     19516

52   289.833   15.083   304.917   30.167       941     29602

53   289.833    0.083   304.917   15.167       504     21259

54   304.833   75.083   319.917   89.929        90     11163

55   304.833   60.083   319.917   75.167       701     19842

56   304.833   45.083   319.917   60.167       265     13996

57   304.833   30.083   319.917   45.167      1226     26121

58   304.833   15.083   319.917   30.167      2106     42257

59   304.833    0.083   319.917   15.167       788     22293

60   319.833   75.083   334.917   89.698       110     11292

61   319.833   60.083   334.917   75.167       330     17288

62   319.833   45.083   334.917   60.167       979     21708

63   319.833   30.083   334.917   45.167      1510     31812

64   319.833   15.083   334.917   30.167      1678     30754

65   319.833    0.083   334.917   15.167      1372     29441

66   334.833   75.083   349.917   89.289       177     13894

67   334.833   60.083   349.917   75.167       570     20739

68   334.833   45.083   349.917   60.167       503     20608

69   334.833   30.083   349.917   45.167      1076     32388

70   334.833   15.083   349.917   30.167       451     16682

71   334.833    0.083   349.917   15.167       573     19404

The index file (ETOPO5.INX) contains information about the segments.  There

is one 20 byte record for each segment.  Each record contains the contour

level for the segment (2 byte integer), the minimum longitude (degrees

east), minimum latitude, maximum longitude (degrees east), maximum latitude

of the box surrounding the segment (each a 4 byte floating point number),

and the number of points in the segment (2 byte integer).  The relationship

between the index file and the data files is known through the byte offset

to the beginning of each data set in the index file.  These byte offsets can

be calculated from the number of segments in each data set (20 * cumulative

number of segments).

The contour level is a relative value related to the actual data value and

contour interval as follows:

Contour interval = 200 meters

First contour level   =  1   Contour data value = -8200 meters

Zero  contour level   = 42   Contour data value =     0 meters

Maximum contour level = 66   Contour data value =  4800 meters

These are the offsets in bytes to the first segment of

each region in the ETOPO5 contour index file:

Reg  Offset  Reg  Offset  Reg  Offset  Reg  Offset  Reg  Offset

0       0    1    1160    2    6000    3   16460    4   35200

5   63580    6   83440    7   85060    8   88380    9   95180

10  110820   11  142200   12  174360   13  175800   14  186860

15  201040   16  220040   17  246300   18  268760   19  269740

20  284660   21  315940   22  344060   23  364160   24  372280

25  373800   26  385960   27  440920   28  478360   29  508140

30  517620   31  519740   32  522920   33  533880   34  560160

35  600800   36  617720   37  622020   38  626420   39  628820

40  632780   41  649420   42  669180   43  675600   44  683760

45  689360   46  696100   47  718100   48  738880   49  742320

50  750740   51  762360   52  777340   53  796160   54  806240

55  808040   56  822060   57  827360   58  851880   59  894000

60  909760   61  911960   62  918560   63  938140   64  968340

65 1001900   66 1029340   67 1032880   68 1044280   69 1054340

70 1075860   71 1084880

The region parameters and the byte offsets are contained in the file

contour.h which is on your program disk.

The programs listindx and listdata (on the accompanying diskette) read the

index and data files respectively and output the contents to standard output

as ASCII.