GEOPHYSICS OF NORTH AMERICA - MAGSAT CONTOURS                 magsat.doc

This directory contains the Magsat contours.

In order to understand how the data are arranged, one must understand

something about the process of generating these data.  The contouring was

done in 3 regions each 59 degrees wide.  Each contiguous section of each

contour line is called a segment and each segment is made up of some number

of line fragments.  Each segment starts with an implicit move command and

the fragments are the endpoints of a series of draw commands.  The data

storage scheme therefore has three basic units: regions, segments, and

points (where points are the ends of fragments).

The regionalization of the data is expressed in the file structure.  The

data are stored in 3 files, each called MAGSAT.NUM, where NUM is the region

number.  Each data file has the longitude and latitude pairs of the

endpoints of the contour lines for all of the contours within the given

region.  These values are stored in two 4-byte floating point numbers.

Parameters for MAGSAT contour Regions:

Region   Minimum  Minimum   Maximum  Maximum Number of Number of

Longitude Latitude Longitude Latitude  Segments    Points

0       170        0       229       50        98     12188

1       229        0       289       50       149     19737

2       289        0       349       50       121     14783

The index file (MAGSAT.INX) contains information about the segments.  There

is one 20 byte record for each segment.  Each record contains the contour

level for the segment (2 byte integer), the minimum longitude (degrees

east), minimum latitude, maximum longitude (degrees east), maximum latitude

of the box surrounding the segment (each a 4 byte floating point number),

and the number of points in the segment (2 byte integer).  The relationship

between the index file and the data files is known through the byte offset

to the beginning of each data set in the index file.  These byte offsets can

be calculated from the number of segments in each data set (20 * cumulative

number of segments).

The contour level is a relative value related to the actual data value and

contour interval as follows:

Contour interval = 1 nanoTessla

First contour level   =  1   Contour data value = -10 nanoTesslas

Zero  contour level   = 11   Contour data value =   0 nanoTesslas

Maximum contour level = 25   Contour data value =  14 nanoTesslas

These are the offsets in bytes to the first segment of each region in the

MAGSAT contour index file

Region Offset Region Offset Region Offset

0      0      1   1960      2   4940

The region parameters and the byte offsets are contained in the file

contour.h which is on your program disk.

The programs listindx and listdata (on the accompanying diskette) read the

index and data files respectively and output the contents to standard output

as ASCII.