GEOPHYSICS OF NORTH AMERICA - AVHRR IMAGE DATA            avhrr.doc

Eight files of Advanced Very High Resolution Radiometer (AVHRR) data are

provided in image format as follows:

JUN86.CH1:  Channel 1 for June 1986

JUN86.CH2:  Channel 2 for June 1986

JUN86.CH5:  Channel 5 for June 1986

JUN86.NVI:  Normalized Vegetation Index for June 1986

NOV86.CH1:  Channel 1 for Nov. 1986

NOV86.CH2:  Channel 2 for Nov. 1986

NOV86.CH5:  Channel 5 for Nov. 1986

NOV86.NVI:  Normalized Vegetation Index for Nov. 1986

Each file is a one week composite of the Global Coverage (GAC).  The two

time periods chosen for inclusion were selected by George Stephens

(NOAA/NESDIS/Office of Satellite Data Production and Distribution, In

teractive Processing Branch, (E/SP22), Washington DC 20233) for best

cloud-free coverage.  The days used in these composites are listed below:

June, 1986:  Julian days 162-168

Nov., 1986:  Julian days 316-322

The Normalized Vegetation Index (NVI) is computed as:  NVI = (ch2 - Ch1)

/ (ch2 + ch1).  For a detailed description of the NVI, refer to the

"Global Vegetation Index Users' Guide" available from:

NOAA/NCDC/SDSD/ECC6

Satellite Data Services Division

World Weather Building, Room 100

Washington, DC  20233

Tel: (301) 763-8111

Note:  the values in the NVI and channel 5 are numerically reversed in

the SDSD format).

All files are stored as 2-byte binary image files with values ranging

from 0 to 255, increasing numerically with brightness/emittance

("positive" images).  Each grid has 1080 columns by 451 rows.  Each row

exactly fills the space between longitude 170-deg. E and 10-deg. W.  The

first row is oriented with the lower-left corner of the first pixel at

longitude 170-deg. E, and latitude 75-deg. N.  The last row therefore is

oriented with the lower-left corner of the first pixel at longitude 170-

deg. E and latitude 0-deg. N.  The right hand edge of the last pixel of

each row is at longitude 10-deg. W.

The following format information can be used for display:

image title       : (all 8 AVHRR image files)

data type         : integer

file type         : binary

rows              : 451

columns           : 1080

minimum           : 0

maximum           : 255

cell width x      : .16666660000

cell width y      : .16666660000

Processing of AVHRR data:

Source files:  The SDSD "Plate Carree" tapes were used as source data in

latitude/longitude projection on a world grid of 2500 columns by 904

rows.  The Plate Carree grid began at Longitude 180-deg. W and Latitude

75-deg. N, and ends at Longitude 180-deg. E and Latitude 55-deg. S.

Sequencing was from West to East, and by latitude row from North to

South.

Re-sampling to 10-min grid:  The "Plate Carree" files were re-sampled

using a simple algorithm which aggregated the original pixels into their

corresponding 10-min by 10-min cells within the North American window

(170-deg. E to 10-deg. W, and 75-deg. N to 0-deg. N).  The brightness

values for Channels 1,2 and 5 were aggregated by averaging.  The Normal

ized Vegetation Index values were aggregated by choosing the maximum

value of those which mapped directly into each new grid cell.  Because

both input and output grids were the same projection, and because the

output grid was only slightly larger than the input, most output values

represent a single input value, however there can be up to four with a

lesser frequency throughout the grid.  No input point can be represented

in more than one output value, however.