topo30.doc

Topography for the U.S. - These 30-second grid average elevation

values were compiled by the U.S. Geological Survey, the National

Geodetic Survey, the Defense Mapping Agency, and the National

Geophysical Data Center.

The original Digital Terrain Data Base (3-second point data) was

compiled by the Defense Mapping Agency.  The data were generated

by digitizing 1-deg. x 2-deg. (1:250,000) scale maps.  This was

done by selecting every 10th point in the   3-second Digital

Terrain Data.  The Defense Mapping Agency selected every 10th

point from the Digital Terrain Data Base to create the 30-second

point data set.  These data were rounded to the nearest 20 feet

for every 30 seconds of latitude and longitude (approximately

2700 ft. on the surface of the Earth).  A copy of the data was

then transferred to the National Geodetic Survey (NGS), who

reformatted and corrected the data, rounding to the nearest 10

meters.  NGS sent a copy of the 30-second data to the USGS, who

averaged the data by using the four corners of each cell.

To complement the 30-second land data, coastal and waterway

bathymetry were developed by the National Geophysical Data Center

from data archived by the National Ocean Survey (NOS).  Trackline

data were averaged into 15-second cells and then averaged into

30-second cells.  In the process of averaging these data a code

was used to determine how many 15-second cells were used to

develop the 30-second values (and this code is part of the

archive record).

One of the problems with the 30-second land data is that much of

the offshore data were coded with a no data value of "1" which

represents a valid land elevation.  This created many problems in

representing coastlines accurately, especially in regions such as

southern Florida and the Mississippi Delta.  To solve this

problem we decided to merge the NOS bathymetry data with the land

data.  This was done by first developing a mask to delete the

offshore data from the land data set, and then adding the coastal

bathymetry data.  The techniques used were judged to be suitable

for displaying the data; however, caution is recommended for

those doing detail coastal studies.

Included in the coastal data are some inland waterways.  Some of

these regions are on datum that are other than sea level.  For

examples:

Lake Superior..................600 feet

Lake Michigan..................577 feet

Lake Huron.....................577 feet

Lake St. Clare.................572 feet

Lake Erie......................569 feet

Lake Ontario...................243 feet

The above values represent 1955 datum.  As the datum changes with

time and the surveys used in the averages represented values

collected over a long time,     these datum values should be

considered as an approximation whose accuracy is less than 2

feet.