MCX11 NOTES

The two S-record object files on this diskette, SYSTEM.S19 and TEST.S19,

were created using the AS11 assembler. SYSTEM.S19 is the object file for

the MCX11 kernel, system tables, and clock driver consistent with the needs

of TEST.AS.

The TEST.AS file contains four tasks (tasks 2-5, task 1 is the clock driver)

which are designed to execute all of the functions in MCX11. They do not

necessarily test all possibilities for each function. It will operate on an

EVM board having as little as 256 bytes of RAM. TEST.AS does not do anything

important so don't expect earthshaking results or outputs.

The command lines used for assembling the files are as follows:

For MCX11 kernel - Object is SYSTEM.S19

AS11 SYSTEM.AS MCX.AS CLKDRIVR.AS VECTORS.AS -L CRE C > MCX.LST

For TEST program - Object is TEST.S19

AS11 TEST.AS -L CRE C > TEST.LST

MCX.LST and TEST.LST are the listing files for the two assemblies. Both

listings include the cycle count and produce a cross reference listing. If

either is not desired, the command lines can be changed accordingly.

It would be possible to join all elements into a single assembly and produce

only one object and listing file.

As a time saver (and paper saver), listing suppression directives could be

inserted into the source code. A most likely candidate would be the code in

the MCX.AS file. Since it is the main part of the kernel, it is unlikely to be

changing as often as the application (TEST.AS) program.

A batch file, ASSEMBLE.BAT, is also included for a quick construction of the

test environment.