USING THE SURVEY DRAFTER

INTRODUCTION

The survey drafter is a menu driven software program that

converts user input raw survey data into an AutoCAD script file

format. Once the data is converted, the user begins a new AutoCAD

drawing and plots the survey data by executing the script file

created by the SURVEY DRAFTER program. Once the survey points are

plotted the drawing can be further edited as desired by the

AutoCAD user. The SURVEY DRAFTER also creates an ASCI format X Y

Z output file from the survey data that can be used as the

database for several different contour generating software

packages.

HARDWARE REQUIREMENTS

To use the SURVEY DRAFTER program you must have an IBM or

compatible computer with either two floopy disk drives or a hard

drive and 256K RAM. You must have the file SVY.CFG located in the

same subdirectory of your disk as the SURVEY.EXE main program.

SURVEY DRAFTER DATA FILES

The SURVEY DRAFTER stores survey data in either of two type of

files. A survey data points file or a traverse points data file.

The data file type depends on what type of survey you are working

with.

A survey data point file contains all survey points "shot" from

one particular transit setup. Each different transit setup used

on a particular survey must be entered into a separate survey

data points file. Each file would be given a distinct name.

A traverse points data file would be created to "tie" all

separate transit setup points used on one particular survey

together to create one survey plot. If your survey has only one

transit setup point, you do not need to create a traverse points

file. Only one data file need be started that will contain all of

the survey data points "shot" at the one transit setup point.

EXAMPLE SURVEY

Say a team of surveyors set out to survey a tract of land the

will require three different transit setups in order to cover the

entire site.  They initial setup at a given point they call

"PT.A". From this setup they turn angles and distances and record

elevations of 25 points.  They then determine that their second

transit setup point will be 125 feet at an angle of 45 degrees

from their first setup. They move their transit to the second

point which they call "PT.B". They take a backsite to point

"PT.A" and call this their zero line. They then turn angles

distances and record elevations for 25 points. They determine

that their third and last transit setup point will be 100 feet at

an angle of 180 degrees from their second setup point. They move

to the third point and setup again and call this point "PT.C".

They again turn angle, distances and record elevations for 25

points.

USING SURVEY DRAFTER TO PLOT EXAMPLE SURVEY

THE MAIN MENU

1) ENTER Data to an EXISTING file

2) REVIEW/EDIT existing data file

3) CREATE AutoCAD script + XYZ

4) START a new survey data file

5) PRINT a survey data file

6) DISPLAY output files

7) CONFIGURE Operating Parameters

8) EXIT

All functions of the SURVEY DRAFTER are accessed thou the Main

menu. The following is a description of each menu option.

Before any survey data can be entered, the user must start a new

survey data files. To do this, choose option 4) from the main

menu. Upon choosing this option you will be asked if this is

really what you want to do. If is proceed, if not you will be

returned to the main menu.

You can start either of the two types of survey data files. Since

the example survey had more than one transit setup point, you

will need a separate survey data point file for each different

transit setup point and a traverse point file to "tie" all three

transit setup points together. You would start three (3) survey

data files named; EXAM-A, EXAM-B, EXAM-C and a traverse points

data file named; EXAMPLE. The names can be any valid DOS file

names. NO EXTENSIONS are necessary! The program will add the .SDT

extension for survey point data files and .TRV extension for

traverse files.

1) ENTER data to an EXISTING file

After all the necessary data files have been started you may

proceed with entering the data. First you will want to enter the

random distances, angles, elevations and any notes recorded for

each of the three different transit setup points. In the example

survey the surveyors took 25 "shots" at each setup point.

Choose option 1) to enter data to an existing survey data file.

The data input consists of a DISTANCE to a point in decimal

format representing feet (125.5, 1015.25, 25.2). You must enter a

distance. You must enter an ANGLE to the point in degrees,

minutes and seconds format. The format must be like (125d15'25",

44d12', 179d): You may enter an ELEVATION at the point in decimal

format (101.5, 1215.25, 45): and you may enter a SURVEY NOTE

(4"tree, corner of parking, cl road, fire hydrant). The survey

note may have up to 16 characters with spaces. The point

elevation and survey note input are optional. If you do not have

an elevation or note for a particular point, just press <RETURN>

without entering any data. To exit the data entry sequence just

type the letter Q to Quit.

As you enter the data for each point it is displayed on the

survey data screen. If you enter incorrect data you can edit it

by entering the letter "E" at the distance prompt. You will then

be asked which data point to edit. Choose the data point number

that you wish to edit. You will then be prompted for new data for

the DISTANCE, ANGLE, ELEVATION and NOTE. To retain the existing

value for a given prompt, just press <RETURN> in leu of re-

entering the data. If you wish to enter revised data, type it at

the prompt and press <RETURN> to enter.

After entering the survey point data for each of the three survey

setup points, you should enter the survey traverse point data.

The traverse point data file will contain the distances, angles,

backsite-to, and side shot file name data.

For the example survey, the traverse file would contain the

following entries:

TRAVERSE POINTS

POINT #    DISTANCE        ANGLE        BS-PT        SS FILE

=================================================================

1        0              0               0            EXAM-A

2        125            45d             1            EXAM-B

3        100            180d            2            EXAM-C

Since their first setup point #1 was a random or other

established point in the field, the DISTANCE, ANGLE, BACK-SITE

POINT (BS-PT) are all 0. The name of the  file containing all the

survey points shot around setup #1, (or "PT.A" as the surveyors

called it) are in a survey data point file named EXAM-A.

Survey setup point #2 was 125 feet at an angle of 45 degrees from

point number #1. Thus the BS-PT would be 1. The SIDE SHOT FILE

(SS FILE) that contains all of the survey points shot around set

up #2, (or "PT.B")  are in the survey data point file named EXAM-

B. The third setup point was 100 feet at an angle of 180 degrees

from point #2 or "PT.B". The BS-PT was point #2. The file

containing the side shots for this point are in a file named

EXAM-C.

2) REVIEW/EDIT existing data file

Choose this option from the Main Menu when you wish to review or

edit data that has previously been entered. When you choose this

option, you will be asked whether you wish to edit a survey data

point file or a traverse point data file and the name of the

file. Data editing is exactly the same as editing data while

entering it. You select the point # to be edited an either enter

new data or press <RETURN> to keep the current value.

3) CREATE AutoCAD script + XYZ

Choose this option when you are ready to convert the raw survey

data into an AutoCAD script file format. You must enter the

following information to create an AutoCAD script file.

NUMBER OF TRANSIT SET-UPS------------------->>3

Enter the total number of different transit set-ups used in your

survey. For the example survey you would enter three (3). If your

survey only had one, enter 1.

NAME OF TRAVERSE POINT FILE----------------->>EXAMPLE

Enter the name of the traverse point file that describes the

distances, angles, back-sites and side shot file names to use in

your survey. For the example survey, the name of the traverse

file was example. If you have only one survey setup point, you

will not be asked for a traverse point file name.

DO YOU WANT A LINE ON TRAVERSE (Y/N)-------->>Y

If you answer "Y" AutoCAD will draw a line connecting each of the

different transit setup points. If you answer "N", no line will

be drawn. If you have only one survey setup point, you will not

be asked whether you wish to plot a traverse line.

DO YOU WANT SIDE SHOT FILES PLOTTED--------->>Y

Normally you will want to plot the side shot files. These files

are the survey points shot around each of the different transit

setup points. If you answer "N", then only the points where the

transits were setup would be plotted. If you have only one survey

setup point you will not be asked if your wish to plot side

shots.

NAME FOR THE AutoCAD SCRIPT FILE------------>>EXAMPLE

Enter the name you would like to give the output AutoCAD script

file created from the survey data. NO EXTENSION is necessary! The

program adds the .SCR extension which is the default for an

AutoCAD script file.

X VALUE FOR INITIAL TRANSIT POINT----------->>5000

Enter a value to use as the base X coordinate for calculating all

other survey points. The value can be any number, but if it best

if it large enough to avoid getting calculated negative values.

Y VALUE FOR INITIAL TRANSIT POINT----------->>5000

Enter a value to use as the base Y coordinate for calculating all

other survey points. The value can be any number, but if it best

if it large enough to avoid getting calculated negative values.

INITIAL SURVEY ANGLE ----------------------->>0d

This angle is a correction angle to rotate the entire survey when

it is plotted in AutoCAD. AutoCAD always assumes that the zero 0

degree reference is to the right side of your screen, unless you

configure it differently. The survey angle you enter, rotates the

entire survey by adding the degrees you enter to the default

AutoCAD zero angle. The survey itself is not altered, only it's

orientation or (north) in the AutoCAD drawing. The angle you

enter must be in the same format as other angles entered in the

program.

HEIGHT FOR AutoCAD TEXT -------------------->>10

This is the height that you wish to assign to all text plotted in

AutoCAD. Your default text style in AutoCAD must accept a text

height input. If it does not, your will get an error when running

the script file.

WHAT THE SURVEY DRAFTER PROGRAM DOES.....

The conversion routine reads the point distance and angle from

the survey data file. It then calculates the X and Y location

of that point. The calculation utilizes the laws of Sin and

Cosine for right triangles when it solves the X and Y distances

from the survey starting point. The routine then writes the X and

Y coordinates of the point to the user-named script file along

with the AutoCAD "point" command. The point number, elevation and

survey note for that point are written to the script file along

with the AutoCAD "text" command. A command sequence to change the

current layer to a "TEXT" layer is written to the script file

prior to the text command so that all text will be on a separate

layer. A command to change the current layer back to the "0"

layer is written to the script file before the next point is

calculated. Once the data is converted to a script file, the user

is ready to EXIT the program and begin a new AutoCAD drawing.

5) PRINT a survey data file

Choose this option from the main menu when you wish to print the

contents of either a survey data points file or a traverse points

file on a line printer. You must have a printer connected to your

parallel port to use this option.

6) DISPLAY output files

Choose this option from the main menu when you wish to display

the output files created by the program. The following display

menu will be shown. Choose the option that suits your wishes.

Display Output files

===================================

1) Display AutoCAD Script file

2) Display X-Y-Z-Note output file

3) LIST Survey Data (.SDT) files

4) LIST Script output (.SCR) files

5) LIST XYZ output (.XYZ) files

6) RETURN to Main Menu

===================================

7) CONFIGURE Operating Parameters

Choose this option when you wish to set or change the default

settings of the SURVEY DRAFTER programs. The following variables

are available for your customization.

SURVEY DRAFTER CONFIGURATION

=================================================

Number of Survey Data point in a file ---->>250

Create an AutoCAD Script file (Y/N) ------>>Y

Create an X Y Z Output file (Y/N) -------->>Y

AutoCAD text Height ---------------------->>10

Initial X coordinate for calculations ---->>10000

Initial Y coordinate for calculations ---->>10000

Do you want point plotted in 3D (Y/N) ---->>N

Initial Survey Correction angle (North)--->>0d

==================================================

The AutoCAD text height, X and Y initial coordinates, and survey

correction angle can be changed any time you create a output

file. Pressing <RETURN> at any prompt will accept the default

values that you enter in this configuration menu.

8) EXIT

Choose this option to exit the SURVEY DRAFTER program and return

to DOS.

PLOTTING THE SURVEY

To plot the converted survey data a user must take the following

steps:

1) Copy the script file containing the converted survey data to

the AutoCAD sub-directory of the hard disk.

2) Begin a new AutoCAD drawing

3) Set the new drawing units to be decimal format

4) Set the angle format to be in degrees, minutes and seconds.

5) Set the drawing limits to be large enough to contain the

entire scope of the survey

6) Set the AutoCAD variable "pdmode" = 3 and "pdsize" = 3

(This setting is suggested for best results, it is not

mandatory)

7) Create a layer called "TEXT" (if not already existing)

The CURRENT TEXT STYLE MUST ACCEPT both text height and angle

input.

8) Set current layer to be "0"

9) ZOOM ALL

10)Enter the SCRIPT command and respond with the name of the

script file you created.

AutoCAD will plot the survey points and note them appropriately.

Once the survey data has been plotted the user may change the

unit format, drawing scale, limits, etc. to meet their specific

needs.

USING A LISP ROUTINE TO INTERPOLATE

Users wishing to take the plotted survey data a step further

might want to use a lisp routine to interpolate between random

survey points. In the example lisp program,"INTERP.LSP", the user

must load the routine and then enter the command "INTERP". The

routine prompts the user to select two (2) survey points.  He/she

uses a pointing device to select two survey points to inter-

polate between. The lowest elevation point must be selected

first. The lisp routine then asks for the known elevation of

each point, as well as the elevation of the first interpolated

point. The user must enter this data from the keyboard. The lisp

program then proceeds to plot the interpolated points between the

two user selected points. Each point is noted with the

appropriate point elevation, based on the users original input.

=================================================================

The SURVEY DRAFTER ....................

IT'S EASY TO USE!   Put done those old drafting tools.

Survey Drafter only requires an IBM

computer and AutoCAD software.

IT'S FAST!          Survey Drafter can plot hundreds of

survey points in a matter of minutes.

IT'S PRECISE        Survey Drafter takes advantage of the

power of AutoCAD to produce precision

survey drawings.

IT'S INEXPENSIVE    Survey drafter is a fraction of the

cost of other coordinate geometry

software.

IT WILL SAVE YOU    Using Survey drafter just once, will

TIME and MONEY!     pay for the shareware asking price!

���������������������Ŀ

� TDM Software        �

� 3620 L Street       �

� Lincoln, Ne. 68510  �

�����������������������