Subject: TS940 control program (TS940.ASC)

From: WK2S at WB2COY packet radio BBS in Poughkeepsie, NY

Date: September 10, 1990

TS940.ASC is a BASIC language program for controlling a Kenwood

TS-940S and various other Kenwood transceivers (TS440, TS711, TS811 and

maybe some others.)

For use with the TS-940S, you need to install an IF-10B interface

unit in the 940, and you need an IF-232C level converter between

your radio and your computer.

TS-940S

+----------+                             IBM PC

|   IF-10B | <---------> IF-232C <-----> serial

+----------+                             port

The program is run as any other BASIC program on your PC.  Serial

communications port 1 is used by the program;  if you use port 2,

change the port number in line 20.

Program functions

-----------------

The program displays a representation of the TS-940 tuning dial

and shows the frequency and mode in the upper left corner.  The

status of the RIT and XIT is displayed in the middle of the top

line along with the offset value.

F1  key - sets VFO A (enter the desired frequency in MHz)

F2  key - sets VFO B

F3  key - refreshes the tuning dial display (this is often necessary

when switching between the LIST and tuning dial display.)

F5  key - loads a file of frequencies which you can set into the

current VFO (see F9 LIST function).

F9  key - display a LIST of frequencies from a frequency file.

When the list is displayed, use the Up and Down arrow keys

to position the cursor (reverse lettering) and press Enter

to load the highlighted frequency into the current VFO.

The Esc key is used to return you to the frequency dial

display.

F10 key - saves the current frequency in the LIST of frequencies

that can be selected by the F9 key.  When you press F10,

you are prompted for a "Description ===>" that you can fill

in to remind you of why you've saved a particular frequency.

Frequency LIST file

-------------------

You can create a file containing a list of frequencies.  The F5 key

allows you to load the file, and the F9 key displays the list

from which you can select a desired frequency.

Each line in the file should begin with the frequency (in MHz) followed

by one (or more) spaces and an optional comment.  For example,

10.000  WWV in Colorado

A sample frequency file, FREQ.SWL, is supplied in this package.

Program description

-------------------

The program is somewhat useful "as is"; however, I encourage you to

modify it to suit your needs.

Line 100 - tells BASIC that the subroutine at line 5000 should be

called for each character received from your radio.

Line 5000 - is the beginning of the subroutine that get control for

each character from the TS-940.  The TS-940 sends lines of data to

your program.  Each line is ended by a semi-colon (;).  This subroutine

looks for a semicolon and sets a variable, L, when the semi-colon is

found.

Line 1000 - is the beginning of the programming loop where the

program spends most of its time.

Line 1005 sends the 'AI;' command to the TS-940; this command tells

the radio to send the computer a status line whenever the frequency

or mode of the radio changes.

When the end of a line from the TS-940 is detected (L=1 in line 1100),

the subroutines at 6000 and 9000 are called.

Line 6000 - is the beginning of the subroutine that extracts information

from the data received from the TS-940

Line 9000 - displays the data received from the TS-940 and the

tuning dial.