THE WA8BXN MSYS PACKET BULLETIN BOARD SYSTEM

USER'S MANUAL

By Steve Wolf, NO8M @ NO8M.OH.USA.NA

April, 1993

Version 1.14

Please copy it and pass it around!

(Keep the credits and trademark notice!)

Additions, corrections and suggestions:

Steve Wolf, NO8M

27132 Butternut Ridge Road

North Olmsted, OH  44070-4417

NO8M @ NO8M.#NEOH.OH.USA.NA

MSYS is written by Mike Pechura, WA8BXN, and is copyright 1993 by

HUB COMPUTERS, INC.  It is distributed free for amateur use and

can be obtained by various means including the Cleveland Hamnet

BBS in Cleveland, Ohio

(216-942-7516 - 9600/2400/1200 baud - RBBS).

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 1

THIS MANUAL

This  manual is  intended  to be  a  reference to  the  many

commands available in the MSYS PBBS. It will not teach you how to

operate the board efficiently,  you will have to experiment  with

it. It will give you the tools to experiment.

COMMON ERRORS

The  misspelling  of a  file  name  you  are  attempting  to

download is one of the most often observed errors.   The improper

use of subdirectories is a close second.

The use  of the wrong command  is another "most often  made".  If

you find yourself  in a  bind, check the  command description  in

this manual.

Another  common  problem  is  attempting to  use  the  nodes

without  giving a port  number.  You  must give a  port number to

connect from the nodes.

Any  traffic moving off the board should be signed with your

call and your home PBBS, like this:  "KA8ZVV @ NO8M.OH.USA.NA"

A send to a station must  contain the exact callsign of  the

person you want the  message to go to.  If you  send a message to

KA8ZZV rather  than  KA8ZVV,  your  station will  never  get  the

message.

The  sysop is  more  than happy  to  answer your  questions.

Please don't  hesitate to leave a message to the call of the PBBS

asking about your problem!

PRINTING THE MANUAL

The manual can be printed by using the DOS command

TYPE USERMAN.DOC > PRN

KA-Node, KAM and KPC-4 are trademarks of Kantronics, Inc.

NET/ROM is a trademark of Software 2000.

IBM is a trademark of International Business Machines Corp.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 2

STRUCTURE

MULTI-USER/MULTI-TASKING

The  board is  a  multi-user system.  Many  stations can  be

connected at once.  It will support many connects on many bands.

This is normally transparent  to the user.   However, during

prime  time, you  might observe  some slowing.  Even if  you hear

nothing on your frequency, the  board might be up to its  ears in

tasks  on other frequencies. Disk reads and writes will also slow

a response.

HOUSECLEANING

The board  will automatically kill messages  after a certain

time period. Decisions about what is kept  and what is killed are

made by  the sysops. Should you be out  of town for a long period

of time, you might ask the sysop to hold your mail.

BACKUPS

Every message gets copied to a backup file. Should the board

crash, the messages can be  restored. If a message that you  have

already read  and killed  suddenly  reappears, it  may have  been

restored after a crash.

The  board  is  very  stable  if  run  with  a  conservative

attitude.  Normally  sysops will  tweak and  tune to  get optimum

performance. They will experiment with other ports. They will not

act conservatively.

SECTIONS

There are four basic sections. There is a message section, a

download section,  a TCP/IP section and a  user services section.

The message section is  capable of delivering mail and  bulletins

almost  anywhere  in the  world.  The  download section  contains

information held for reference  and training. The service section

provides a node  and conference  that enhances the  value of  the

software.   Send a message to  the callsign of the  PBBS for more

information on the TCP/IP section.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 3

THE SERVICES SECTION

The Conference

K-Node

Network Node

THE CONFERENCE

Normally, only two amateurs can connect via packet radio and

hold a conversation. Three or more can talk  in the converse mode

but will be forced to disregard all other traffic on the channel.

MSYS provides a means  by which any number, depending  on channel

congestion, can talk.

To  enter or  start a  conference, you  must connect  to the

PBBS. From there  you issue the conference  command. Every packet

you send will be received and acknowledged by the  board. It will

then  send your  packet  and receive  acknowledgement from  every

other user.

The  first packet received from  a user will  be prefaced by

the user's  callsign. If the  next packet  sent is from  the same

person, the callsign is not included.

Overhead  (channel usage)  is  very high  for a  conference.

However, if the local  area network (LAN) is on  a clear channel,

it will support many users.

You might be asked to join in  a conference.  Do it!  If you

get a  message that says  that someone would  like you to  join a

conference, type the letter C.

THE K-NODE

MSYS  provides the  user  with  two  types  of  nodes.    An

explanation of what a node is and how it operates can be found in

most  download sections. For  now, suffice it to  say that a node

acts somewhat  like a  repeater, extending  the range  of another

packet station.

One MSYS node  is called a K-Node and operates much like the

Kantronics KA-Node. They may also provide digipeating and gateway

service.

To use  the K-node, you  must first connect  to it.  It will

have a different callsign than the PBBS and will normally use  an

alias.   The ID command, described later,   will let you know the

call of the K-Node.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 4

The following K-Node commands are available:

Bye ------- This is used to disconnect from the K-node.

Connect --- This command allows the user to connect  to

another station.   Two  additional  items, a  port

number and a callsign  are necessary. They MUST be

included.  The ID  command, described  later, will

tell  you what  port  number is  attached to  what

frequency.

C#   call -- Connect on port  # to call, PORT NUMBER IS

REQUIRED!

Help ----- Help.

Just Heard  ----- What  stations the K-node  has heard.

This list is much  more limited than the   command

on  the PBBS.  It  will also take  an operand port

number.

Node -----  This lists K-Nodes  heard and  will take  a

port number operand.

THE NET NODE

The NET  NODE is different from the  K-Node in that the node

listens to  the frequency and  logs what it  hears.  If  it hears

another node, it logs that node as  a "route", a node that it can

directly connect to.  It also logs the nodes that route can hear.

It compares the  other nodes with what is in  its log and figures

out the best path to a destination.

NET  NODES  only talk  to  other NET  NODES,  NET/ROM nodes,

TheNet nodes  and other versions of the network nodes.  They will

typically have  a callsign  with SSID  and an  alias.  Many  have

rather strange numbers  as alias names.   This number  represents

the node's identifier when used with the TCP/IP protocol.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 5

The following network node commands are available:

BBS --------- Connect to the bulletin board.

B<ye> ------- Disconnect

C#  ---------- Connect on port  # to call  (# is unnecessary

for known nodes)

C<onnect> --- name or call --- for known nodes

H<elp> ------ Help

I<nfo> ------ Basic information about the node

J<ustheard> - What has been heard (a limited output)

JK - Kantronics nodes that have been heard

JN - Network Nodes that have been heard

N call ------ Information about a specific node

Q<uality> --- The quality of the  link, about 192 for a GOOD

1200  baud  two-meter-type  connect but  this

value can be adjusted  by the sysop.   OBS --

Obsolescence, decremented by one  each period

that the node is not heard.  Port -- Port the

node is on

P<orts> ----- The port numbers of the node and the PBBS

R<outes> ---- To nodes that can be directly connected to

R call ------ Routes from a particular node

T<alk> ------ Page the Sysop

U<sers>  ----- Info on the  users and node  connected to the

MSYS node.

THE MESSAGE SECTION

This is  the meat of the  board. Messages from  user to user

and  bulletins  appear  here.  The board  is  capable  of routing

messages  to almost  anywhere in the  world. The  bulletins carry

information that  you may not  otherwise see until  the magazines

come out.

CITY, STATE and ZIP

If  you are  entering a  message for  another party  and the

route has not been proven by a number of other messages that have

made the  trip, please put  the City, State  and Zip code  in the

title line  of the message.   It will really help  the routing of

your message should it get hung up somewhere.

HIERARCHIAL FORWARDING  (Routing Messages)

This  is not near  as complicated as you  might think! As an

example, let's say that your friend in Tahiti,  French Polynesia,

wants to send you a  message. The local PBBS might not  know that

NO8M is  in Ohio. It might not even know  how to get traffic into

Ohio.  However if  the message  is sent  like this:  SP WB8RNI  @

NO8M.OH.USA.NA the message should have no problem.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 6

The local  board will look at  the @PBBS line and  see if it

knows how to  route to NO8M. If it does not,  if will look at the

OH to  see if  it knows  how to  get traffic  into Ohio. If  that

fails,  it  will see  if  it knows  about the  United  States. If

nothing else, it had better know about North America!

See, nothing to  it! Your PBBS knows all  about the PBBSs in

your  state. In  fact, you  can verify  in-state traffic  will be

routed by using the PF command.  The board will come back and say

to where  your traffic will be  sent. In the United  States, your

PBBS  should recognize a PBBS callsign and respond with a routing

like KY.USA.NA.

If  it does not, and you are SURE  that the @PBBS given is a

forwarding  PBBS, then  you may  send it  by adding a  two letter

state abbreviation after the  @PBBS call. In this way,  MSYS will

send the  traffic to a forwarding PBBS in that state and let that

board figure out what to do with it.

For  international traffic that you are  not sure of, please

send a message to the call of the PBBS and inquiries will be made

in order to obtain the proper routing.

DATABASE

MSYS  has  the facilities  to  maintain a  number  of simple

databases. Adding to a database is done like this:

Line entered                       Explanation

============================       ============================

SP MSYS @ thatbbs                  A message to the PBBS.

DBA 3                              (title  line) Add  to database

#3.

wind damage, Mentor, Ohio \        The "\" character

large tree down at 615 & 20 \           causes one line after

reported by Mentor PD \                 it to be listed

A no injuries                           with the original line.

/EX                                "/ex" to end.

Now if a search (Grep) on database number three were used on

the string "Mentor, Ohio", all four lines of the message would be

sent.   Grep  can  be used  on  all files  ...  PBBS lists,  club

rosters, hazardous materials lists, etc.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 7

MSYS COMMANDS

Abort ------- This command stops the PBBS  from what it is doing.

For  example, you  have asked  it to list  all the

messages.  After about 300 messages you decide you

have had enough!   Issue  the Abort  command.   It

also works with reading messages, downloads, etc.

Bye --------- This disconnects you from the PBBS.  It resets your

statistics,  too.   If you  just disconnect  or go

away  without disconnecting, the  PBBS will assume

you ran into trouble and log the disconnect.   For

example, on  Monday you check-in and  read all the

new  messages.  You  then disconnect without using

the  Bye.   Then, you  check in  on Tuesday.   You

issue an L (list  since you were last on).   Since

the PBBS ignored your Monday connect, you will get

the  messages since  you last  checked in  and the

messages you read on Monday. Now, had you used Bye

on Monday,  your Tuesday check-in would have given

you just the new messages.

Conference -- The conference  allows as many users as  desired to

connect  with  each  other.     The  best  way  to

understand it is  to try  it with a  friend.   The

first time you  talk, your line that is  sent will

be prefaced with your  callsign, like this:  NO8M:

If the next line does not have a callsign in front

of it, then it came from the same station.  Inside

the conference you can use the following  commands

(send  a control-Z  by  pressing  and holding  the

CONTROL key while tapping the Z.  Release both.

^ZH ---- Help

^ZQ ---- To return to the PBBS

^ZU ---- List users and their channels

^ZA # -- Ask user on channel # to join conference

Download  ---- To download a file.   You must enter the full name

and  directory  that you  obtained  from  the What

command  or  from  a   list  describing  what   is

available  that  is   sometimes  available.     To

download  a file  in a  subdirectory, use  this: D

subdirectory/filename.   So, to  download the file

APPLICAT.INF  in  the NCARC  directory,  you would

issue the command

"D NCARC/APPLICAT.INF".

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 8

Grep --------  This command searches  a file for  a given set  of

characters.  First just enter a G.  The board will

ask for a filename.  It is asking for something in

the download  section, that you get  with the What

command.   It will then ask you for the string you

want  to  find.   Let's  say  you want  to  search

NCBYLAWS.DOC    for    the   string    "election":

G(ENTER)NCBYLAWS.DOC(ENTER)election(ENTER).    The

board  will  then send  you  all  lines that  have

"election"  in   them.    The   command  does  not

recognize wildcards (*/?).   If you do not specify

a file, the default of DATABASE.0 will be used.

Help -------- A short list of commands.

?x --- A longer explanation of the command, use ?x

where  x is the name of  the command. I would

use the command ?L to get help on LIST.

Info -------- This gives information on the system's hardware.

ID  ----------  This gives  you  the callsigns,  ports  and other

information connected.

Just heard -- This gives information about stations that the PBBS

has  heard.   If the  call is  a funny  alias, the

sponsor's call is given inside the curly brackets.

If you want to connect to someone like that, don't

use the  sponsor's call, use  the alias.   Use the

Path  command  to  determine  how   stations  were

connected  to the  PBBS.  You  can also  limit the

output with  a channel number, like this: JK 0 ...

this lists all KA nodes heard on port 0.

JB -- BBSs

JD -- Digipeaters

JG -- Gateways

JK -- KA nodes

JM -- Other MSYS PBBSs

JN -- NET/ROM nodes

JT -- TCP/IP protocol stations

Kill  -------- This deletes  messages.  You may  kill a number of

messages at the same time:  K 2 34 556.

KM -- Kills messages addressed to you.

KT -- Kills NTS traffic you are going to deliver.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 9

List --------  List  is  used to  give  you the  headers  of  the

messages.   Used alone, it will  list all messages

since you  last logged  off with the  Bye command.

The  PBBS has  a  function to  force all  personal

messages to end up  as private messages.  If  this

is  on,  any  message  sent except  bulletins  are

changed to private.

LM --------- List mine, messages addressed to you.

LN --------- Lists messages not yet read.

LT --------- Lists NTS traffic messages.

LL --------- List the last message entered.

LL # ------- Last # messages.

LO yymmdd -- List OLDER than yymmdd.

LU --------- Lists unread messages to you.

LY --------- Lists messages that have been read

LW --------- Lists weather messages (type W$).

L? --------- Lists messages that will not forward.

L< call ---- Lists messages from call (or alias).

L> call ---- Lists messages to call (or alias).

L@ call ---- Lists messages with call as an @BBS.

L x -------- Lists  messages since a given number.

To see them all, use L 0.

L x y ------ List numbers between numbers x and y.

L"string"  --  Lists messages  with  the character

string  you  specify in  the title.

The quote marks are required.  This

is  case insensitive,  "Yaesu" will

match "YAESU", "yaesu", etc.

L'string' --  Lists  messages with  the  character

string  you  specify in  the title.

The   single    quote   marks   are

required.  This is  case sensitive,

"Yaesu" will not match "yaesu".

L$ --------- Lists type $, list bulletins.

Message ----- This will send the message of the day. This message

contains information  on happenings.   It will  be

sent  on  the initial  check  in  UNLESS you  have

selected the  eXpert mode.  If you  have, you must

use M to  read it.   Expert users  DO NOT get  the

message of the day!

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 10

Name -------- This command is used to register with the PBBS.

N --- Your first name  with only the first  letter

capitalized.

NQ -- Your QTH, for example:  North Olmsted, OH.

NZ -- Your zip code (only the five digit code).

NH  -- Your home PBBS,  where you want  to pick up

mail  addressed  to  you.   Traffic  may

arrive to  you on  other boards  in your

area, you may want to declare your  home

PBBS on those.  If you change this, make

sure you  do it  on all boards  you have

checked into  And only  declare one PBBS

as  your  home  PBBS.   Otherwise,  your

message may  end up  as the potato  in a

game   of   hot-potato.      One   board

forwarding  to  another and  back again.

Declare only  one home PBBS.   NH should

be a full service PBBS, not a TNC PBBS.

Path -------- This command will tell you if the PBBS knows how to

route traffic to a  PBBS or if the PBBS  knows the

amateur.

P call -- If the person is known, information that

was entered  is displayed.    This is  a

good way  to  find out  someone's  name,

their  last  connect  and   where  their

traffic  will be sent.   A response that

indicates that the person did  not enter

a home board and has not checked-in  for

a  long time  would  indicate a  message

left for them might not be received.

PF call -  This is  used to indicate  if the  PBBS

knows  how to  route traffic  to another

PBBS.  If this PBBS does not know how to

route to  the PBBS of interest,  you can

insure  routing by  adding a  two letter

state abbreviation  to  the end  of  the

PBBS call:    KA8ZVV.OH.    If  you  are

routing traffic to  a PBBS  in Ohio  and

this  PBBS  does  not know  it,  leave a

message for the sysop.

PC call -  If there is a callbook  (REQQTH) server

enabled at the PBBS, this command can be

used  to  obtain  information   about  a

callsign from the callbook.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 11

Read  -------- This is used  to read messages  on the board. Many

messages may be strung together:  R 1 45 68.

RE  ------ The  read for export.   This  reads the

message in a format where it can then be

easily  uploaded  into  a  PBBS  or mail

drop.

RH ------ The  read with headers.  This  shows the

forwarding  headers  that  are added  at

each PBBS  as the message is  sent.  The

routes are not the same at all times and

it can  be fun to see how  a message got

from tither to yon.  If a message author

did not give the  home pbbs for a return

reply, use this  to determine where  the

message originated.  Normally, that PBBS

can be tried for a response.

RN ------ This reads only  the text of the message

and is  a  good way  to save  time on  a

marginal path.

RM ------ Reads unread messages addressed to you.

RP ------  Reads a  message without marking  it as

having been read  so it  will be  listed

the next time you check in.

Send -------- This is used  to send a message.  The  basic format

is  Sx call @PBBS.  Sx is  a send with the type of

message it  is.  The  @PBBS is the  location where

the addressee  picks up their  mail.  Call  is the

exact callsign of  the recipient.   If you make  a

mistake on  the addressee's call, the message will

probably end up in limbo, never to be received.

(control-A) -- Aborts the message; to start over.

CC  callsign -- This causes a copy of a message to

be  sent  to  the  calls  following  the

carbon  copy command.   The CALLSIGN can

t  a  k  e       t  h  e      f  o  r  m

CALLSIGN1@PBBS;CALLSIGN2@PBBS   etc.  to

send the same message to many users.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 12

REPLY  #  --  Allows  the  PBBS  to  automatically

address  a  message replying  to another

message.  If you  add a period after the

message number, like  this:  "REPLY #.",

the PBBS will also automatically enter a

title for you.

SP -------  A personal  message listed or  read by

addressee and author.

ST ------- NTS Traffic.   Most boards have a  help

file -- READ IT!

SB  ------- Send  a bulletin.   Without  an @PBBS,

these will stay on  your PBBS and may be

addressed in whatever way you wish.  For

example, in northern Ohio,  when sending

a bulletin, the  @PBBS field may  be set

to the following:

ALLBBS   --  To  quote  Jim,  WK8G,  messages

addressed to ALLBBS are, "intergalactic,

omnipresent,  transuniversal All  Points

Bulletins"!   This  message would  go to

every PBBS  in the  world.   PLEASE  USE

THIS  WITH CAUTION.   The packet network

in Guam does not want to know about your

wattmeter for sale!.  Some areas use WW.

ALLUS --  This would go to every  PBBS in the

U.S.  USE WITH  CAUTION as your for sale

would  normally  NOT  be  in  the proper

forum  if  sent in  this  manner!   Some

areas use "USA".

OKIPN  -- (Or  your  wide  area flood  route)

Ohio,  Kentucky and  Indiana.   Again, a

message to NEOH or ALLOH is better!

ALLOH -- All Ohio (or ALLMI, ALLHI, etc.).

NEOH --  (Or  your local  area  flood  route)

Northeast Ohio, but better  described as

north  of Columbus.    This is  the best

routing for the "for sale" messages.

PBBSCALL -- For all at a PBBS.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 13

Talk -------- Page the sysop.

Users  -------  Gives the  current users  of  the system  and the

number  of messages.    If  you  want to  talk  to

someone using the board,  you may want to start  a

Conference and invite them in.

Version ----- The  version of  the software and  the date it  was

last started.

What --------  This gives  a  list of  what is  available in  the

download section.

W   subdirectoryname -- Gives a list of what is in

the  subdirectory.  W  NCARC or W AMSAT/BULLS

are examples.

Xpert  ------- This  sets and resets  your expert status.   As an

expert, you get  a short command prompt and do not

receive the message of the day.

X   x -- Where x  is a number, sets  the number of

lines you  receive before the PBBS  stops and

asks you  if you want "More?".   Setting this

to zero  defeats the "More?".   Answering the

"More?" with a C defeats it for that message.

XF -- Sets  the PBBS to  send you multiple  lines.

Use this with a good route.

XS --  Sets the  PBBS to  send you  one line  at a

time.  Use this with a poor route.

Yapp  -------- This command allow  you to download  a binary file

from  the YAPP directory.  You  may also upload if

you have been authorized.

YW -- What is in the YAPP directory.

YU -- Upload a file to the YAPP directory.

YD -- Download a file from the YAPP directory.

*  ------------ This makes the line after the asterisk a comment.

It can be used to answer the sysop after receiving

a "Message from sysop" line.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 14

KEEPING LINE LENGTHS LESS THAN 80 CHARACTERS

There are a number  of reasons you want to keep  the lengths

of your lines less than 80 characters.  This will explain why you

need to do this.

Depending on  your computer, you may see 80, 60 or even less

characters on your screen.   This is not the line length.  A line

ends in a carriage return (where you hit the  ENTER key).  If you

only  have carriage returns at  the end of  your paragraphs, then

your line length is the length of the paragraph.

YOUR TNC

Your  TNC will  affect how  your message  looks after  it is

uploaded.  When  you send information to  your TNC, a circuit  in

the unit will format it into packets to be sent over the air.  It

will  use  the  value set  in  the PACLEN  command  to  break the

information up.  PACLEN  is the maximum number of  characters per

packet.

If the TNC  encounters a carriage return prior to  getting to the

number  of characters in  PACLEN, it knows  to stop  and send the

packet.   If you send a very  long line, one with more characters

than the value that is in PACLEN, the TNC will do the sending for

you.

Lets say  your PACLEN value is  255 characters.  You  send a

very  long  paragraph with  a carriage  return  only at  the end.

Let's  say the paragraph contains 1000 characters.  Your TNC will

accept 255 characters, stop,  add a carriage return and  send the

packet.  This carriage return  is added regardless of what  is at

the  255th character.  It may  be added right in  the middle of a

word.

This is the reason you see so many bulletins that look all broken

up.

Here is  an example of a  message sent with a  TNC PACLEN of

45.  The operator sent lines much longer:

THE YACKAMONGA HAM RADIO CLUB WISHES TO ANNO

UNCE THAT WE ARE HAVING OUR THREE HUNDRETH Q

UADANNUAL HAMFEST IN THE GREAT STATE OF YACK

A.  YACKAMONGA IS EASY TO GET TO.  TAKE INTE

RSTATE 322S TO EXIT 435.  TURN RIGHT, TURN L

EFT, THEN RIGHT UNTIL ...

You can not get around this by setting you PACLEN  less than

80 characters.  The  TNC will break the lines  up willy-nilly and

you will get all sorts of weird looking lines.  You need to enter

a carriage  return before the  value you set  in PACLEN.   As you

will see, use a carriage return before the 80th character.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 15

If  your screen does not show an 80 character line, there is

no problem sending messages  formatted with smaller line lengths.

A good example is with the Tandy TRS-80 Model 100 laptop.  It has

a  screen length of 40 characters.   You might see a message like

this from a Model 100:

The Tandy TRS-80 Model 100 is a nice

computer for both at home and portable

packet use.  It is battery powered and

has an RS-232 communications port.

But, there is no storage (at least

no easy storage).  There is also a

problem with the 40 character screen.

Many of the 80 column messages will

be hard to read with this screen.

TRANSPORT

If you  use a node to  transport your packet to  a PBBS, you

might get another  surprise.   There are additional  things in  a

packet  used  by  nodes to  help  the  nodes  route your  packet.

Because  of  this, a  node will  only  accept 236  characters per

packet.

If you had  sent 255  characters from your  TNC, the  packet

will be  sent out of the  node as two  packets.  One will  be 236

characters  (plus the routing information) and  the other will be

19 characters.  This slows your flow of information down.

There will be no additional carriage returns added to your packet

in  this case.   It  will  still look  busted up  because of  the

carriage returns added by your TNC.

PBBS EDITORS

Let's say you  enter a message  into the PBBS  but you  have

made  a mistake.  You  signed your message  in the proper manner,

with  the full  hierarchial return  address, like  this:   NO8M @

NO8M.OH.USA.NA.  But this time, you  made a mistake.  You entered

your  home call wrong, like this:  NO8M@NM8O.OH.USA.NA.  In these

cases  your sysop can edit  your message and  correct your error.

It is easier than sending the message back to you with a note.

However, the  editor on the MSYS  PBBS is limited.   It will

work with  only 80 character  lines.  This  is to limit  the code

that  is  required for  the  editor.    A  better editor  can  be

installed  but  it  would  take  space   better  suited  to  PBBS

functions.

If  your line lengths  are over 80  characters, your message

would have  to be taken to  another program and edited.   You are

more likely to get it sent back.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 16

SOLUTIONS

USING A TERMINAL PROGRAM

If you are entering a message from a terminal program, hit a

carriage return (the ENTER key) before you send 80 characters.

USING A WORD PROCESSOR

Many  word processors will  allow you to  enter your message

without  regard  to  line-length.    This  makes  editing,  spell

checking  and  other functions  easier.   They  will then  have a

special function to allow the file to be saved in an 80 character

per line format.

This file  was done in  the WordPerfect  word processor.   It was

saved with carriage returns using the "DOS SAVE" function.

It might help if you can set your word processor to show carriage

returns on the screen.  Consult your manual.

DON'T GET FANCY

Many newsletter editors watch  packet messages for ones they

would like  to print.  Do  not add fancy stuff  the messages that

you  might want  to see printed.   It  causes additional  work to

reformat  them into  usable  text.   Do  not justify  your  text.

Justification adds spaces between words to make both margins come

out straight.   It is very  difficult to edit these  extra spaces

out.

Do  not indent your whole message.  These indentations are a

pain to take out.  Add two spaces between sentences.   After each

period, question mark, etc., there should be two spaces.

Single space your messages.  This saves network time as the extra

carriage return does  not need to  be sent over  and over as  the

message gets sent to other boards.  Again, the newsletter editors

will not have to remove the extra carriage returns.

The use of capital  letters is a questionable practice.   Studies

have been done showing  that messages in all capital  letters are

more difficult to read.

APLINK

Many  times, the AMTOR PBBSs, called APLINK, will be used to

send traffic.   AMTOR has a very restricted character set.  There

are no lower case letters.  Many characters, such as "~", "^" and

"_" will be ignored.

PROOFREAD YOUR MESSAGES!

After  sending a bulletin, read  it and see  if that is what

you meant it to say and how  you meant it to look.  After sending

it, use the  command "LL" (list last) to get  the number and read

it.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 17

EXPERIMENT

You can send messages to yourself.   You can then read  them

back to see if what you have done is what you wanted to do.

NTS BASICS

WHY THE GOOFY FORMAT?

Your message may not stay on  the packet network.  It may be

forwarded by nets using RTTY, CW, phone or whatever.  These other

modes are  NOT  error free.    The preamble  and format  of  your

message will help  standardize and check your  message for better

handling.

NTS IS PICKY

Danged right, they are!   NTS operators want to  insure that

the  system works now, before the disaster.   If they can get you

using the standard now, you will not be a problem later.

Right now you are handling messages that might not mean much

if  they don't get to their destination.   After a disaster, your

message  content  may  have  much  more  meaning.    You  may  be

transferring messages  about critical supply needs, about medical

supplies  that will save someone's life.  Information to a family

who's relatives assume they are dead.

If you do something wrong while handling NTS traffic, expect

to get corrected.  You will be one heck of an asset in a disaster

if  you know  what you  are doing.   You  will be  one heck  of a

liability if you don't.

A TYPICAL MESSAGE

|    ST 44070 @ NTSOH

|    TITLE:  North Olmsted, OH  (216)777

|    Enter message--End with /EX or CTRL-Z

|    NR 15 R HXG N8GNJ 4 Seattle, WA 9-18-89 0045Z

|    To:  Steve Wolf

|         North Olmsted, OH

|         216-777-1177

|   -bt-

|    Request home PBBS call

|    -bt-

|    Steve N8GNJ, Seattle, WA

|    Operator's note:  Reply to

|    N8GNJ@N8GNJ.WA.USA.NA  Seattle, WA

Let's examine the above message line by line:

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 18

|    ST 44070 @ NTSOH

When you  send a piece of NTS traffic, it has a special send

command.  The traffic indicator will help it on its way.  It will

also (sometimes)  cause the last  PBBS in the line  to generate a

message back to you telling you who took it off the packet system

and where.  In many cases, this might not be  at its destination.

An  operator in Iowa might pull an  Ohio message because he knows

he can get  it there quickly.  He might be  getting on a net that

has  an operator in  the town that  the message  is destined for.

ALWAYS use a  ST zip @NTSxx  command as all the  forwarding PBBSs

will recognize that without interpretation.

|    North Olmsted, OH  216-777-1177

This  title  line tells  that there  is  one message  in the

packet message and that it is destined for Ohio.  If NOT disaster

related,  there may be many  messages in the  packet messages, as

long as they are destined  for the same location.   The inclusion

of  an area  code (AC 216)  is sometimes  used but  does not mean

diddley  in many areas.   If the precedence  (explained below) is

EMERGENCY or P, then  that could be added to the title line.  The

title line is limited to 37 characters!

|    NR 15 R HXG N8GNJ 4 Seattle, WA 9-18-89 0045Z

This is the preamble.  NR 15 is the number of the message at

the sender's station.  On January 1,  you start with message one.

You assign each message going out of your station a number.

"R"  is ROUTINE.  Other  precedents are "W"elfare, "P"riority and

"EMERGENCY".   EMERGENCY is always  spelled out!   Any precedents

other than routine should appear in the title line.

"EMERGENCY" deals with information related to the safety of  life

and  property.   "Priority"  is  everything  that  is  almost  an

emergency.  It  includes important messages having  a time limit,

notices  of death  and  injury  in  a  disaster  area  and  press

dispatches.   "Welfare" is for  health and welfare  inquiries and

responses  that indicate all  is well.   If all is  not well, the

message  should be  sent as  Priority.   "Routine"  is everything

else.

The  HX- is the  handling instructions  (if any).   Handling

instructions are rarely used.

HXA# -- Collect call within # miles (or unlimited if no #).

HXB# -- Cancel in # hours/notify originator.

HXC  -- Report time and date of delivery.

HXD  -- Report time, date, method and call delivering.

HXE  -- Obtain and send reply.

HXF# -- Hold message until # (date).

HXG  -- Mail or toll call delivery not required.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 19

The number five is the check of the message.  It is how many

words you are sending.  The stop character, the "x", is counted.

The rest  of the line is  self-explanatory.  For gosh  sakes, use

UTC!  There are other people in the world and  they don't all use

EDST!

|    To:  Steve Wolf

|         North Olmsted, OH

|         216-777-1177

This is the address that the message is to go to.  The phone

number  is almost  always mandatory.   It  will probably  just be

serviced back without it.

|    -

|    -bt-

|    Request home PBBS call

|    -bt-

The meat, the message.   Preceded and followed by a  -bt- to

set  it  off, the  message should  be  no longer  than  19 words.

Rambling messages do not make it!

|    Steve N8GNJ, Seattle, WA

|    Operator's note:  Reply to

|    N8GNJ@N8GNJ.WA.USA.NA  Seattle, WA

This  is  the  signature,  to  whom  a  response  is  to  be

forwarded.  It  is a real good idea to add your packet address to

the end.

CHECKS--COUNTING THE WORDS

If you  are going to run into trouble, this is where it will

be.   Probably the best available advise  is to not banter on the

air about disagreements over the  check.  If they know more  than

you, they are right.  If they know less, they are wrong.

Here is a quick list of some sticky check situations:

Alfred E. Neumann   3 words

Grand Cayman        2 words

NY                  1 word

New York City       3 words

ARL SIXTY THREE     3 words

Fifty Six           2 words

6146B               1 word

You can change an improper check.  DON'T CHANGE THE WORDING!

If you do change the check, use a slant bar  between the old

and new values.  For example, if you received a check of five and

the check should be seven, note it 5/7.

WA8BXN MSYS PBBS User's Manual

Version 1.14       April, 1993

Page 20

ARRL NUMBERED RADIOGRAMS

These are "standard" messages.   The text of the  message is

simply ARL followed by the number  spelled out:  ARL SIXTY EIGHT.

It allows the operator  to quickly make up standard "are  you OK"

and the like type messages.  There should be a file on your board

describing all the different ARL numbers.

The one  seen in a disaster situation is ARL NINETEEN.  Here

is an example:

|    NR 132 W NO8M ARL 2 N. OLMSTED, OH AUG 5

|    JANE DOE

|    1234 WEST 5TH

|    GARY, IN

|    PHONE 123-456-7890

|    ARL NINETEEN

|    JOHN SMITH 987-654-3210

|    OR REPLY NO8M@NO8M.OH.USA.NA

(Note that this message  could be sent either to Jane Doe, a

person in the disaster area, or  to an agency who could follow it

up.  For example, it could be addressed to the American Red Cross

agency  in  the area  with  the text  requesting  a check  on the

person.)

DO NOT BOOK DISASTER TRAFFIC!

Booking traffic  is sending a  number of  messages with  the

same information in one message.  The stations in a disaster area

do not have the time to separate them.

WA8BXN MSYS PACKET BBS COMMAND SUMMARY ... Version 1.14

=======================================================

Abort..........Stop current action      Path...........Path to a user

Bye............Disconnect                    PF........Path to a PBBS

Conference.....Multi-user\DX Node            PC........Request a callsign

^ZH.......Conference help          Read...........Read a message

^ZQ.......Return to PBBS                RM........Read Mine

^ZU.......List users                    RE........Read for export

^ZA #.....Invite user on #              RH........Read with headers

Download.......Download from FILES           RN........Read only text

Grep...........String search file            RP........W/O mark as read

Help...........Help                     Send...........Send a message

Information....PBBS hardware                ^A.........Abort message

ID.............Port definitions             CC.........Carbon copy

Just Heard.....Log                          REPly #....Reply auto title

JB........BBS Stations                 SP.........Send personal

JD........Digipeters                   ST.........Send traffic

JG........Gateways                     SB.........Send bulletin

JK........K and KA nodes           Talk...........Page sysop

JM........MSYS PBBSs               Users..........Users

JN........Net/Rom                  Version........Version

JT........TCP/IP protocol          What...........Download list

Kill...........Kill a message                W x.......What in x=subdir

KM........Kill mine                eXpert.........Toggle expert

KT........Kill traffic                  X #.......Lines per page

List...........Message headers               XF........Multiple lines

LM........List mine                     XS........1 line per reply

LN........List not read            Yapp...........Down and uploads

LT........List traffic                  YW........YAPP Directory

LL........List last message             YD........Download a file

LL x......List last x                   YU........Upload a file

LO x......List older x=yymmdd      * .............Comment to sysop

LU........List unread to you

LY........List read to you         NETWORK NODE COMMANDS

LW........List weather             BBS............Connect to the PBBS

L?........List ? forward           Bye............Disconnect

L<< x.....List fm x=callsign       Connect CALL...Connect to call

L>> x.....List to x=callsign            C# CALL...C on port # to call

L@ PBBS...Messages to PBBS         Help...........Help

L #.......List since #             Info...........Basic Information

LL #......List last #              Justheard......Limited output JH

L # #.....List from # to #         K-Nodes........K-Node output.....

L"x"......List with x=string       Nodes..........Nodes heard

L'x'......List with x=sTrInG          Nodes CALL..Info on node

L$........List bulletins           Ports..........Port information

Message........Message of the day       Routes.........Direct connect nodes

Name...........First name                  Route CALL..Other node

NH........Home board               Talk...........Page Sysop

NQ........QTH=City, State          Users..........Users/nodes

NZ........Zip code

Version 1.14     March 1993