�¿ � � �Ŀ    �Ŀ �Ŀ �Ŀ �¿ � � �Ŀ �Ŀ �Ŀ    � � �Ŀ �Ŀ � � �Ŀ

�  �Ĵ ��     �Ŀ � � ��   �  ��� �Ĵ ��� ��     ��� � � ��� ��  �Ŀ

�  � � ���    ��� ��� �    �  ��� � � � � ���    ��� ��� � � � � ���

24 Hours a Day, 7 Days a Week

Fax Number +61-3-589-3574

How to a Telebit WorldBlazer (Australia Version) with D'Bridge v1.50

First off I must thank Roy Austen for his help with this.

Step 1.

You will notice that I have setup the WorldBlazer LOCKED at 38400 Baud, this is

fine for must systems. Please note you will need 16550 UARTS.

Please run the install program from AUSTMCFx.ZIP (Version 7 or Higher)

Step 2.

Unzip a File called T3500WBO.ZIP and print out T3500.DOC

Step 3.

Goto your comms setup and change your modem type to T3500AUST.MCF

Step 4.

Add the following into the Specific dial strings.

PEP ATS50=255S115=0D

Step 5.

Goto Advanced Security and add the following in the External Reciever window

ERROR|C:\DB\T3500.BAT

This will fix the 10 call out to one number with-in a set time period.

Step 6.

Have fun.

Rgds

Simon Gronow

The Software Works

Fidonet   -:   3:632/998.0

Worldnet  -:  61:9420/100

DBNET     -: 201:300/314

Internet  -: Simon.Gronow@n998.f632.z3.fidonet.org