This TOKODI.DOC accompanies DOSUP8.EXE which includes DOS ODI

drivers for IBM Token-Ring and Lan Support NetWare Clients.  The

following file(s) are included:

TOKEN    COM    27381  4-12-93  6:27p

LANSUP   COM    21943  2-02-93 11:54a

TOKODI   DOC     (this file)

Token.com and lansup.com have been enhanced to include 'Least

Significant Bit' first (LSB) node addresses per the 'IEEE canonical

form' when sending frames above the Data-Link layer.   Previously,

token.com and lansup.com drivers provided 'Most Significant Bit'

first (MSB) only when sending MAC header node address information

above the Data-Link layer.  This LSB ability provides node ID

bit-reversal compatibility with IBM 8209 Bridges. Bit-reversals

take place on all source and destination node addresses

when 8209 bridges transfer frames from ethernet segments to

token-ring segments unless "IPX Support" has been enabled on the

8209.  The 8209 will not bit-reverse IPX (E0E003/AAAA03/8137) node

addresses when "IPX Support" has been enabled.  The new LSB feature

of lansup.com and token.com allow compatibility with the 8209

without "IPX Support" enabled.  It is advised only to use the LSB

parameter on the Token- Ring segments with 8209 bit-reversal

enabled.  (see paragraphs below on Bit-Reversals)

Additionally, these drivers provide full compatibility with the

latest IBM locally administered node address standard for

Token-Ring adapters.  In the past, IBM recommended that locally

administered addresses for Token-Ring adapters fall within a range

between and including the following addresses:

4000 0000 0000  to  4000 7FFF FFFF  (HEX)

IBM has expanded the range of recommended locally administered

addresses to the following:

4000 0000 0000  to  7FFF FFFF  (HEX)

There are some restrictions that apply to the above changes.  Some

devices and/or programs may only be able to use decimal addresses.

In this case, the following locally administered address range

would be appropriate:

4000 0000 0000  to  7999 9999 9999  (HEX)

It is recommended that 'NSSS FLASH N1005' be obtained from IBM for

all details pertaining to the above LAA changes and associated

restrictions.

********* USAGE **********

The following NET.CFG file demonstrates the enhanced abilities and

usage of lansup.com and token.com.    Please refer to the Novell

ODI shell for DOS manual (part #100-000871-001) for further

information on individual parameters.  Additional information

regarding implementation of token.com and lansup.com is available

in DOSODIWS.DOC found in DOSUP8.EXE also available on NetWire.

Sample NET.CFG

IPX SOCKETS = 20

IPX RETRY COUNT = 20

CACHE BUFFERS = 5

LONG MACHINE TYPE = IBM_PC

PROTOCOL ipx

BIND Token (or Lansup)

LINK DRIVER Token (or Lansup)

Node Address 444400002D43

Slot 2

Frame TOKEN-RING_SNAP LSB

SAPS 2

Link Stations 2

ALTERNATE

Max Frame Size = 2168

*********** BIT-REVERSALS ********

Bit-reversals are necessary because of the difference between

ethernet adapters and token-ring adapters:

Ethernet adapters transmit data onto the wire by transmitting one

byte of data at a time in the following manner:    Each byte of

data is bit-wise right-shifted one bit at a time onto the wire

until all 8 bits of the byte are transmitted onto the wire.  The

next byte of data is then loaded into the transmit register and the

process repeats itself.  The least significant bit, therefore, goes

onto the wire first with ethernet adapters.

Token-Ring adapters do the opposite.  They take one byte of data

and do a bit-wise left-shift one bit at a time onto the wire until

all 8 bits of the byte are transmitted onto the wire.  Another byte

of data is then loaded into the transmit register for transmission.

The most significant bit, therefore, goes onto the wire first.

Therefore, the byte 'AA (hex)' on an ethernet wire would appear as

the byte '55 (hex)' on a Token-Ring wire after being bit-reversed

by an IBM 8209 Ethernet to Token-Ring Bridge.

************* TOKODI MODIFICATION HISTORY **************

3-12-91:   Initial release of TOKODIFT.ZIP beta drivers.

6-3-92:    Lansup.com has been updated to allow frame sizes above

1K during RPL.  A new "OPEN" parameter for the NET.CFG

has been added.  This feature allows lansup.com to open

the token-ring adapter and set the frame size.  If

dxmt0mod.sys is also loading, one of two actions must be

performed:  (1) set "o=n" on the dxmt0mod.sys command

argument line in the config.sys and use the "OPEN"

parameter for lansup.com in the net.cfg. -or-  (2) Set

"o=y" in dxmt0mod.sys command line allowing

dxmt0mod.sys to open the adapter and set the frame

size, and exclude lansup.com's "OPEN" parameter from the

net.cfg.

Use the following net.cfg syntax when using "OPEN":

LINK DRIVER LANSUP

OPEN

<add other parameters as needed>

9/14/92:  Lansup.com has been updated to be compatible with

DXME0MOD.sys from IBM Lan Support program v1.30.

Token.com has also been updated to work with Odinsup.com

where DXME0MOD.SYS V1.30 is involved.

Files included in the previous revision: (Included in

TOKODI.ZIP)

LANSUP   COM    21524  6-03-92  9:47a

TOKODIFT DOC     (this file)

Files also in a previous revision:

TOKEN    COM    23564  8-05-92  2:55p

LANSUP   COM    21588  8-05-92  2:56p

TOKODI   DOC     (This File)

************* END OF MODIFICATION HISTORY *********************

***************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

***************************************************************