{

������������������������������������������������������������������������������

Visionix RealTime Alarm Clock Functions Unit (VALARM)

Copyright 1991,92,93 Visionix

ALL RIGHTS RESERVED

������������������������������������������������������������������������������

Revision history in reverse chronological order:

Initials  Date      Comment

��������  ��������  ����������������������������������������������������������

lpg       03/11/93  Source Code Documentation Complete

mep       02/11/93  Cleaned up code for beta release

jrt       02/08/93  Sync with beta 0.12 release

jrt       12/07/92  Sync with beta 0.11 release

lpg       12/04/92  Everything functional and tested. (works except CallFlag)

lpg       12/02/92  Added RealTime Clock System Time Functions.

Get/SetSysTime,Get/SetSysDate,SystemTime/Date,

Sync,SyncSystemTime/Date,Get/SetDayLightSavings

lpg       11/30/92  Created Alarm Unit

������������������������������������������������������������������������������

Caveats/Known Bugs

Register C - Alarm Call Flag is Invalid.  Seems to only be briefly

set between interrupts.  Need to keep own flags to cover for this.

AFFECTS: AlarmCallFlag, UnsetAlarmCallFlag.

������������������������������������������������������������������������������

}

(*-

NOTES on the RTC Registers:

Register    Meaning

----------- -----------------------------------

0(00h) ....... Current Second

1(01h) ....... Alarm Second

2(02h) ....... Current Minute

3(03h) ....... Alarm Minute

4(04h) ....... Current Hour

5(05h) ....... Alarm Hour

6(06h) ....... Day of the Week

7(07h) ....... Number of Day

8(08h) ....... Month

9(09h) ....... Year

10(0Ah) ....... Clock Status Register A

11(0Bh) ....... Clock Status Register B

12(0Ch) ....... Clock Status Register C

13(0Dh) ....... Clock Status Register D

14(0Eh) ....... Diagnostic Byte

50(32h) ....... Year Hundreds

---------------------------

STATUS REGISTER A (BYTE)

76543210 (Bits) = BYTE

Bit

7   - VIP 0=Time Not Actualized, 1=Time Actualized

6-4 - Time Frequency

3-0 - Interrupt Frequency

---------------------------

STATUS REGISTER B

76543210 (Bits) = BYTE

Bit

7 - Actualized Time                         (0=yes,1=no)

6 - Call Periodic Interrupt                 (0=no, 1=yes)

5 - Call Alarm Interrupt                    (0=no,1=yes)

4 - Call Interrupt after time Actualization (0=no,1=yes)

3 - Block Generator                         (0=off,1=on)

2 - Time and date Format                    (0=BCD,1=binary)

1 - 24/12 Hour Format                       (0=12-hour,1=24hour)

0 - DayLight Savings Time                   (0=yes,1=no)

---------------------------

STATUS REGISTER C

76543210 (Bits) = BYTE

Bits:

7 - UNUSED

6 - Alarm Time Reached

5 - Periodic Interrupt Call

4 - Close Time Actualization

3 - UNUSED

2 - UNUSED

1 - UNUSED

0 - UNUSED

Generally indicates the reason for this Time Interrupt.

Bit 6: 1=Reason for Interrupt was for Alarm Time Reached

Bit 5: 1=Reason was for was a routine Clock Update Call.

Bit 4: 1=Reason was that Time Actuation was Close (Restarted?)

---------------------------

STATUS REGISTER D

76543210 (Bits) = BYTE

Bits:

7 - Battery Status (1=battery ok,0=Battery low/needs replacement)

6 - ?

5 - ?

4 - ?

3 - ?

2 - ?

1 - ?

---------------------------

DIAGNOSTIC BYTE

Bits

7 - Battery (0=Battery is OK, Battery Dead or almost Dead)

6 - Checksum (0=Checksum in memory location 46 and 47 OK,

1=Checksum in memory location 46 and 47 is FALSE)

5 - Config (0=Configuration in Memory Location 20 is OK,

1=Another Configuration found during booting)

4 - Memory (0=Mem Size in Memory Locations 21-24

1=Other Memory Size Determined during Booting)

3 - Hard Disk Controller (0=HD & Controller OK

1=Hard Disk not Present or Not Functional)

2 - RESERVED

1 - RESERVED

0 - RESERVED

--------------------------

-*)

Unit VAlarm;

Uses

DOS,

VDates,

VGen;

{����������������������������������������������������������������������������}

Var

AlarmCalled : BOOLEAN;     { Was the Alarm Called? }

AlarmMsg    : STRING;      { Area Where Alarm INT can pass data }

{----------------------------------}

{ Primary Alarm Interrupt Function }

{----------------------------------}

Function  SetAlarmProc(         ProcPtr        : POINTER   ) : BYTE;

Function  ClearAlarmProc                                     : BYTE;

{---------------------------}

{ Low-Level Alarm Functions }

{---------------------------}

Function  GetRegA                                            : BYTE;

Function  GetRegB                                            : BYTE;

Function  GetRegC                                            : BYTE;

Function  GetRegD                                            : BYTE;

Function  AlarmSetFlag(     Var BcdHour        : BYTE;

Var BcdMin         : BYTE;

Var BcdSec         : BYTE      ) : BOOLEAN;

Procedure UnsetAlarmSetFlag;

{-------------------------------------------------------}

{NOTE: Call Flag is Invalid. Also Can't Set or Reset it!}

Function  AlarmCallFlag                                      : BOOLEAN;

Procedure UnsetAlarmCallFlag;

{-------------------------------------------------------}

Function  SetAlarmTime(         BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      ) : BYTE;

Function  ResetAlarmTime(       BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      ) : BYTE;

Function  UnsetAlarmTime                                     : BYTE;

{---------------------------}

{ Mid-Level Alarm Functions }

{---------------------------}

Function  SetAlarmOn(           Time           : TTime     ) : BOOLEAN;

Function  SetAlarmOff                                        : BOOLEAN;

Function  AlarmIsSet                                         : BOOLEAN;

Function  AlarmWasCalled                                     : BOOLEAN;

(*  **************** MOVED TO VALARMHI.PAS ***************

{----------------------------}

{ High-Level Alarm Functions }

{----------------------------}

{ keeps sorted alarm list and manages alarm calls }

Function  AddAlarm() : BOOLEAN;

Function  SubAlarm() : BOOLEAN;

Function  CallAlarm(             Var ProcItem  : TProc;

ProcPtr   : POINTER      ) : BOOLEAN;

Function  NewAlarm(              Var ProcItem  : TProc;

ProcPtr   : POINTER;

Time      : TTime        ) : BOOLEAN;

Function  DisposeAlarm(              ProcItem  : TProc        ) : BOOLEAN;

*)

{---------------------------------}

{ Low-Level System Time Functions }

{---------------------------------}

Procedure GetSysTime(       Var BcdHour        : BYTE;

Var BcdMin         : BYTE;

Var BcdSec         : BYTE      );

Procedure SetSysTime(           BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      );

Procedure GetSysDate(       Var BcdDay         : BYTE;

Var BcdMonth       : BYTE;

Var BcdYear        : BYTE;

Var BcdCent        : BYTE;

Var BcdDOW         : BYTE      );

Procedure SetSysDate(           BcdDay         : BYTE;

BcdMonth       : BYTE;

BcdYear        : BYTE;

BcdCent        : BYTE;

BcdDOW         : BYTE      );

{----------------------------------}

{ High-Level System Time Functions }

{----------------------------------}

Procedure Sync;

Procedure AccuSync;

Function  GetSystemTime                                      : TTime;

Function  GetSystemDate                                      : TDate;

Procedure SyncSystemTime(       NewTime        : TTime     );

Procedure SyncSystemDate(       NewDate        : TDate     );

Function  GetDayLightSavings                                 : BOOLEAN;

Procedure SetDayLightSavings(   DLS            : BOOLEAN   );

{����������������������������������������������������������������������������}

������������������������������������������������������������������������������

[FUNCTION]

Function SetAlarmProc(            ProcPtr        : POINTER   ) : BYTE;

[PARAMETERS]

ProcPtr     Far Pointer to User Provided Alarm Procedure

[RETURNS]

Status Code ($00 = Successful)

[DESCRIPTION]

Installs a Provided Alarm Function into the Alarm Interrupt Routine.

Now, whenever the RTC Alarm Call Interrupt is made, this function is

called.

WARNING: This Function MUST be kept very simple as some std functions

don't actually return and will lock up the System!

Sets the User Alarm Procedure by Storing the Original Alarm Proc

Vector and Replacing it the User Provided Vector.

NOTE: All the following Alarm Clock Functions require that the Caller

perform this action prior to any Activity, otherwise they will all

report Errors.

IMPORTANT: As this function is an Interrupt (!!!) it absolutely MUST

be kept simple and Obey all rules of Interrupt Procedures.  Watch out

for Nesting Interrupts and other Nasties as they WILL Lock the System!

If you need to pass data to or from this Procedure, store it and look

for it at a predetermined location.  It should also be noted by the

Programmer that some std functions don't properly call RETURN when

complete (Even WriteLn!) and will Randomly Lock up the System when

called.

[SEE-ALSO]

ClearAlarmProc

[EXAMPLE]

Procedure AlarmProc; FAR;

BEGIN

Flag := TRUE;

END;

VAR

Flag : BOOLEAN;

BEGIN

Flag := FALSE;

SetAlarmProc( @AlarmProc );

SetAlarmOn( AddTime( CurrTime, 0, 0, 5 ) );  { Alarm 5 sec from Now! }

While NOT KeyPressed Do;

WriteLn( 'Alarm Called = ',Flag );

ClearAlarmProc;

END;

������������������������������������������������������������������������������

[FUNCTION]

Function ClearAlarmProc                                        : BYTE;

[PARAMETERS]

(None)

[RETURNS]

Status Code ($00 = Successful)

[DESCRIPTION]

Removes the User Alarm Procedure by Restoring the Original

System INT 4Ah Vector.

NOTE: Calling this Function will Disable all the Alarm Clock

functions provided here.  This Function MUST be called Prior

to Terminating the Host Application.

[SEE-ALSO]

SetAlarmProc

[EXAMPLE]

Procedure MyAlarmProc;  FAR;

BEGIN

END;

BEGIN

If SetAlarmProc( @MyAlarmProc )=$00 Then

WriteLn( 'Alarm Procedure Cleared : ',ClearAlarmProc=$00 )

Else

WriteLn( 'Alarm Procedure was NEVER ACCEPTED!' );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetRegA                                               : BYTE;

[PARAMETERS]

(None)

[RETURNS]

The RealTime Clock Status Register A Flags

[DESCRIPTION]

Reads and returns the RealTime Clock Status Register A

Flags are as follows:

Bits

7   = Time Actualization (0=NO, 1=YES)

6-4 = Time Frequency (010b)

3-0 = Interrupt Frequency (0110b)

[SEE-ALSO]

GetRegB

GetRegC

GetRegD

[EXAMPLE]

Uses VGen;

BEGIN

WriteLn( 'Register A = ', ByteToBin( GetRegA ) );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetRegB                                               : BYTE;

[PARAMETERS]

(None)

[RETURNS]

The RealTime Clock Status Register B Flags

[DESCRIPTION]

Reads and returns the RealTime Clock Status Register B

Flags are as follows:

Bits

7 = Actualized Time (0=YES, 1=NO)

6 = Call Periodic Interrupt (0=NO, 1=YES)

5 = Call Alarm Interrupt (0=NO, 1=YES)

4 = Call Intr after Time Actualization (0=NO, 1=YES)

3 = Block Generator (0=OFF, 1=ON)

2 = Time & Date Format (0=BCD, 1=Binary)

1 = 12/24 Hour Format (0=12 hour, 1=24 hour)

0 = Daylight Savings Time (0=YES, 1=NO)

[SEE-ALSO]

GetRegA

GetRegC

GetRegD

[EXAMPLE]

Uses VGen;

BEGIN

WriteLn( 'Register B = ', ByteToBin( GetRegB ) );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetRegC                                               : BYTE;

[PARAMETERS]

(None)

[RETURNS]

The RealTime Clock Status Register C Flags

[DESCRIPTION]

Reads and returns the RealTime Clock Status Register C.  The Flags

int this Register represent the reasons for the Interrupt Call.

NOTE: Most probably they are cleared during each Intr and therefore

only useful data if the INT 70h Function were being trapped.  Use

extreme care in doing so.

Flags are as follows:

Bits

7   - UNKNOWN

6   - 1=Alarm Time Reached (seems between Intr ONLY!)

5   - 1=Periodic Interrupt Call

4   - 1=Close Time Actualization

3-0 - UNKNOWN

[SEE-ALSO]

GetRegA

GetRegB

GetRegD

[EXAMPLE]

Uses VGen;

BEGIN

WriteLn( 'Register C = ', ByteToBin( GetRegC ) );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetRegD                                               : BYTE;

[PARAMETERS]

(None)

[RETURNS]

The RealTime Clock Status Register D Flags

[DESCRIPTION]

Reads and returns the RealTime Clock Status Register D

Flags are as follows:

Bits

7   = Battery Status (0=Dead/Dying, 1=OK)

6-0 = UNKNOWN

[SEE-ALSO]

GetRegA

GetRegB

GetRegC

[EXAMPLE]

Uses VGen;

BEGIN

WriteLn( 'Register D = ',ByteToBin( GetRegD ) );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function AlarmSetFlag(        Var BcdHour        : BYTE;

Var BcdMin         : BYTE;

Var BcdSec         : BYTE      ) : BOOLEAN;

[PARAMETERS]

BcdHour     VAR Returned Previous Alarm Hour in BCD

BcdMin      VAR Returned Previous Alarm Minutes in BCD

BcdSec      VAR Returned Previous Alarm Seconds in BCD

[RETURNS]

(Function : Is the Alarm Set Flag Currently Set?)

(VAR      : [BcdHour] Returned Previous Set Hour)

(VAR      : [BcdMin] Returned Previous Set Minute)

(VAR      : [BcdSec] Returned Previous Set Second)

[DESCRIPTION]

Low Level Function that Returns whether or not the Alarm Clock

is Active as well as the stored Previous Alarm Time.

This is the most direct way to turn off the Alarm Clock.  Without

knowing what other actions are taken by the Standard Interrupt

Function, it would probably be better to use the Std Int Func over

this unless you intend to turn it back on manually also.

[SEE-ALSO]

UnsetAlarmSetFlag

[EXAMPLE]

VAR

Hr,Min,Sec : BYTE;

Active     : BOOLEAN;

BEGIN

Active := AlarmSetFlag( Hr, Min, Sec) );

WriteLn( 'Alarm is Active :, Active );

WriteLn( 'Current/Previous Alarm Time is : Hr=',Hr,

' Min=',Min,

' Sec=',Sec );

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure UnsetAlarmSetFlag;

[PARAMETERS]

(None)

[RETURNS]

(None)

[DESCRIPTION]

Turns the Alarm Trigger Flag to OFF, so that the Alarm Clock is

not called.

This is the most direct way to turn off the Alarm Clock.  Without

knowing what other actions are taken by the Standard Interrupt

Function, it would probably be better to use the Std Int Func over

this unless you intend to turn it back on manually also.

[SEE-ALSO]

AlarmSetFlag

[EXAMPLE]

BEGIN

UnsetAlarmSetFlag;

{ Alarm should not be turned Off }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function AlarmCallFlag                                         : BOOLEAN;

[PARAMETERS]

(None)

[RETURNS]

Is the Alarm Called Flag Set?

[DESCRIPTION]

Returns whether or not the Alarm has already been called.

NOTE: While it is perfectly valid to Read the Flag, it seems of

little use as even when the alarm is called, this flag is never

set.  Thus this data is of little use to the applications programmer.

This will be corrected in future versions.

[SEE-ALSO]

SetAlarmOn

SetAlarmOff

AlarmIsSet

AlarmWasCalled

[EXAMPLE]

BEGIN

WriteLn( 'AlarmCallFlag Set : ', AlarmCallFlag );

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure UnsetAlarmCallFlag;

[PARAMETERS]

(None)

[RETURNS]

(None)

[DESCRIPTION]

Turns the Alarm Called Status to FALSE, so that the Alarm Clock

is not considered to have been called already.

NOTE: While it is perfectly valid to Read the Flag, it seems of

little use as even when the alarm is called, this flag is never

set.  Thus this data is of little use to the applications programmer.

This will be corrected in future versions.

[SEE-ALSO]

AlarmCallFlag

SetAlarmOn

SetAlarmOff

AlarmIsSet

AlarmWasCalled

[EXAMPLE]

BEGIN

UnsetAlarmCallFlag;

{ Alarm Called Flag should not be FALSE (unset) }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function SetAlarmTime(            BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      ) : BYTE;

[PARAMETERS]

BcdHour     Desired Alarm Hours in BCD Format

BcdMin      Desired Alarm Minutes in BCD Format

BcdSec      Desired Alarm Seconds in BCD Format

[RETURNS]

Status Code ($00 = Success)

[DESCRIPTION]

This function loads the provided Alarm Time and turns on the System

Alarm Clock.

NOTE: This Requires the caller to have Installed an Alarm Clock

Interrupt Procedure (Thru SetAlarmProc) otherwise it will return

an Error.

[SEE-ALSO]

ResetAlarmTime

UnSetAlarmTime

[EXAMPLE]

VAR

Active : BOOLEAN;

BEGIN

Active := SetAlarmTime( DecToBCD(14), DecToBCD(15), DecToBCD(30) ) = $00;

WriteLn( 'Alarm is Set to 2:15:30PM : ', Active );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function ResetAlarmTime(          BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      ) : BYTE;

[PARAMETERS]

BcdHour     Desired Alarm Hours in BCD Format

BcdMin      Desired Alarm Minutes in BCD Format

BcdSec      Desired Alarm Seconds in BCD Format

[RETURNS]

Status Code ($00 = Success)

[DESCRIPTION]

Resets a previously Set Alarm Time to a New Alarm Time.  Once Set,

You Must use this Method to Reset all New Alarm Times (Unless the

Alarm has been Called).  This removes the Old Time before attempting

to Set the New Time.

NOTE: This Requires the caller to have Installed an Alarm Clock

Interrupt Procedure (Thru SetAlarmProc) otherwise it will return

an Error.

[SEE-ALSO]

SetAlarmTime

UnSetAlarmTime

[EXAMPLE]

VAR

Active : BOOLEAN;

BEGIN

Active := SetAlarmTime( DecToBCD(14), DecToBCD(15), DecToBCD(30) ) = $00;

{ If Active, Alarm Time is: 2:15:30 PM }

If Active Then

BEGIN

Active := ResetAlarmTime( DecToBCD(8),

DecToBCD(30),

DecToBCD(0)  ) = $00;

WriteLn( 'Alarm Set to New Time : ', Alarm );

{ If Active, Alarm Time is now: 8:30:00 AM }

END  { If Active }

Else

WriteLn( 'Alarm Never Accepted 1st Alarm Time!' );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function UnSetAlarmTime                                        : BYTE;

[PARAMETERS]

(None)

[RETURNS]

Status Code ($00 = Success)

[DESCRIPTION]

Low-Level Function to turn the System Alarm Timer Off.

NOTE: This Requires the caller to have Installed an Alarm Clock

Interrupt Procedure (Thru SetAlarmProc) otherwise it will return

an Error.

[SEE-ALSO]

SetAlarmTime

ResetAlarmTime

[EXAMPLE]

VAR

Active : BOOLEAN;

BEGIN

Active := SetAlarmTime( DecToBCD(14), DecToBCD(15), DecToBCD(30) ) = $00;

If Active Then

BEGIN

Active := UnSetAlarmTime = $00;

WriteLn( 'Alarm Time is Active : ',Active )

WriteLn( 'The alarm will now go off at 2:15:30PM' );

END  { If Active }

Else

WriteLn( 'Alarm was Never Activated!' );

END;

������������������������������������������������������������������������������

[FUNCTION]

Function SetAlarmOn(              Time           : TTime     ) : BOOLEAN;

[PARAMETERS]

Time        Time to Set the Alarm To

[RETURNS]

Was the Alarm Time Set?

[DESCRIPTION]

Turns on the RealTime Alarm Clock based upon the time passed in.

This operation will Overwrite any previously set time.

[SEE-ALSO]

AlarmCallFlag

SetAlarmOff

AlarmIsSet

AlarmWasCalled

[EXAMPLE]

VAR

Time : TTime;

BEGIN

Time := HMStoTime( 14, 15, 30 );

If SetAlarmOn( Time ) Then

WriteLn( 'Alarm is ACTIVE' )

Else

WriteLn( 'Alarm FAILED to Activate!' );

{ If Successful, Alarm Time is now set for 2:15:30 PM }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function SetAlarmOff                                           : BOOLEAN;

[PARAMETERS]

(None)

[RETURNS]

Was the Alarm Turned Off Successfully?

[DESCRIPTION]

Turns off any current RealTime Alarm Clock alarm Times.

[SEE-ALSO]

AlarmCallFlag

SetAlarmOn

AlarmIsSet

AlarmWasCalled

[EXAMPLE]

BEGIN

WriteLn( SetAlarmOff );

{ If RTC Alarm Disabled successfully, Displays 'TRUE' }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function AlarmIsSet                                            : BOOLEAN;

[PARAMETERS]

(None)

[RETURNS]

Is the Alarm Clock Currently Set?

[DESCRIPTION]

Returns whether or not the Alarm Clock is currently set.  This function

isolates the caler from the Previous Time data information.

[SEE-ALSO]

AlarmCallFlag

SetAlarmOn

SetAlarmOff

AlarmWasCalled

[EXAMPLE]

BEGIN

If SetAlarmOn Then;

WriteLn( AlarmIsSet );

{ If RTC Alarm Set successfully, Diplays 'TRUE' }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function AlarmWasCalled                                        : BOOLEAN;

[PARAMETERS]

(None)

[RETURNS]

Has the Alarm been Called?

[DESCRIPTION]

[SEE-ALSO]

AlarmCallFlag

SetAlarmOn

SetAlarmOff

AlarmIsSet

[EXAMPLE]

BEGIN

WriteLn( AlarmWasCalled );

{ If Alarm Called Flag was set, Displays 'TRUE' else 'FALSE' }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure GetSysTime(         Var BcdHour        : BYTE;

Var BcdMin         : BYTE;

Var BcdSec         : BYTE      );

[PARAMETERS]

BcdHour     VAR Returned RealTime Clock Hours

BcdMin      VAR Returned RealTime Clock Minutes

BcdSec      VAR Returned RealTime Clock Seconds

[RETURNS]

(Function : None)

(VAR      : [BcdHour] RealTime Clock Hours)

(VAR      : [BcdMin] RealTime Clock Minutes)

(VAR      : [BcdSec] RealTime Clock Seconds)

[DESCRIPTION]

Returns the System Time as reported by the Battery powered

Motorola MC-146818 processor chip.  May not always jibe with

DOS's System Time.

[SEE-ALSO]

SetSysTime

GetSysDate

SetSysDate

[EXAMPLE]

VAR

Hr,Min,Sec : BYTE;

BEGIN

GetSysTime( Hr, Min, Sec );

WriteLn( BCDtoDec(Hr), ':', BCDtoDec(Min), ':', BCDtoDec(Sec) );

{ Displays the Date as according to the RTC }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure SetSysTime(             BcdHour        : BYTE;

BcdMin         : BYTE;

BcdSec         : BYTE      );

[PARAMETERS]

BcdHour     Hour to set RTC to

BcdMin      Minute to set RTC to

BcdSec      Second to set RTC to

[RETURNS]

(None)

[DESCRIPTION]

Returns the System Time as reported by the Battery powered

Motorola MC-146818 processor chip.  May not always jibe with

DOS's System Time.

[SEE-ALSO]

GetSysTime

GetSysDate

SetSysDate

[EXAMPLE]

BEGIN

SetSysTime( DecToBCD( 14 ), DecToBCD( 15 ), DecToBCD( 30 ) );

{ Sets the RTC Time to 2:15:30 PM }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure GetSysDate(         Var BcdDay         : BYTE;

Var BcdMonth       : BYTE;

Var BcdYear        : BYTE;

Var BcdCent        : BYTE;

Var BcdDOW         : BYTE      );

[PARAMETERS]

BcdDay   (BYTE) Returning BCD Day

BcdMonth (BYTE) Returning BCD Month

BcdYear  (BYTE) Returning BCD Year (MOD 100)

BcdCent  (BYTE) Returning BCD Century (DIV 100)

BcdDOW   (BYTE) Returning BCD DayOfWeek (base=1)

[RETURNS]

(Function : None)

(VAR      : [BcdDay] Returning BCD Day               )

(VAR      : [BcdMonth] Returning BCD Month           )

(VAR      : [BcdYear] Returning BCD Year (MOD 100)   )

(VAR      : [BcdCent] Returning BCD Century (DIV 100))

(VAR      : [BcdDOW] Returning BCD DayOfWeek (base=1))

[DESCRIPTION]

Returns the System Time as reported by the Battery powered

Motorola MC-146818 processor chip.  May not always jibe with

DOS's System Time.

[SEE-ALSO]

GetSysTime

SetSysTime

SetSysDate

[EXAMPLE]

VAR

Day,Mon,Yr,

Cent,DOW   : BYTE;

BEGIN

GetSysDate( Day, Mon, Yr, Cent, DOW );

WriteLn( 'Day=',BCDtoDec(Day),

'Mon=',BCDtoDec(Mon),

'Yr =',BCDtoDec(Cent)*100 + BCDtoDec(Yr) );

'DayOfWeek=',BCDtoDec(DOW) );

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure SetSysDate(             BcdDay         : BYTE;

BcdMonth       : BYTE;

BcdYear        : BYTE;

BcdCent        : BYTE;

BcdDOW         : BYTE      );

[PARAMETERS]

BcdDay      New Day in BCD Format

BcdMonth    New Month in BCD Format

BcdYear     New Year (MOD 100) in BCD Format

BcdCent     New Century (DIV 100) in BCD Format

BcdDOW      New DayOfWeek (base=1) in BCD Format

[RETURNS]

(None)

[DESCRIPTION]

Sets the System Time of the Battery powered Motorola MC-146818

processor chip.

There is NO Range or Error Checking so Caller MUST ensure the

Date/DayOfWeek is Valid prior to Calling.

[SEE-ALSO]

GetSysTime

SetSysTime

GetSysDate

[EXAMPLE]

BEGIN

SetSysDate( DecToBCD( 1 ), DecToBCD( 2 ), DecToBCD( 00 ),

DecToBCD( 19 ), DevToBCD( 0 ) );

{ Will Set the RTC Date to 1 Feb 1990, SUN (whether or not it is valid) }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetSystemTime                                         : TTime;

[PARAMETERS]

(None)

[RETURNS]

RealTime Clock Current System Time

[DESCRIPTION]

Returns the System Time as reported by the Battery powered

Motorola MC-146818 processor chip.  May not always jibe with

DOS's System Time.

[SEE-ALSO]

GetSystemDate

SyncSystemTime

SyncSystemDate

[EXAMPLE]

BEGIN

WriteLn( TimeToStr( GetSystemTime ) );

{ Will Display the Time as according to the RTC }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function GetSystemDate                                         : TDate;

[PARAMETERS]

(None)

[RETURNS]

RealTime Clock Current System Date

[DESCRIPTION]

Returns the System Date as reported by the Battery powered

Motorola MC-146818 processor chip.  May not always jibe with

DOS's System Date.

[SEE-ALSO]

GetSystemTime

SyncSystemTime

SyncSystemDate

[EXAMPLE]

BEGIN

WriteLn( DateToStr( GetSystemData ) );

{ Will Display the Date as according to the RTC }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure Sync;

[PARAMETERS]

(None)

[RETURNS]

(None)

[DESCRIPTION]

This Procedure Synchronizes the Battery System Clock with (what

I consider to be the more Reliable) DOS System Clock.  It is

100% Assembly to get the Synchronization as accurate as possible.

The tolerance is within One (1) Second which should be enough for

most applications.

Also, In order to truly Sync the Clocks, you must call this routine

at the exact instant of the 1st tick of the new second.  In other

words, both Clocks should be Synchronized at the exact beginning of

the same Second to ensure a nearly exact time synchronization.

NOTE: Only Sychronizes the Clock Time not Date.

[SEE-ALSO]

AccuSync

[EXAMPLE]

BEGIN

Sync;

{ RTC Times should be within a Second of Synchronization with DOS Times }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure AccuSync;

[PARAMETERS]

(None)

[RETURNS]

(None)

[DESCRIPTION]

In order to truly Sync the Clocks, you must call the Sync routine

at the exact instant of the 1st tick of the new second.  In other

words, both Clocks should be Synchronized at the exact beginning

of the same Second to ensure a nearly exact time synchronization.

[SEE-ALSO]

Sync

[EXAMPLE]

BEGIN

AccuSync;

{ RTC Times should be in Synchronization with the DOS Times }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure SyncSystemTime(         NewTime        : TTime     );

[PARAMETERS]

NewTime     New Time to Set RTC Time to

[RETURNS]

(None)

[DESCRIPTION]

This Procedure Synchronizes the Battery System Clock with (what I

consider to be the more Reliable) DOS System Clock.  Synchronization

as accurate is within One (1) Second which should be enough for most

applications.

[SEE-ALSO]

GetSystemTime

GetSystemDate

SyncSystemDate

[EXAMPLE]

VAR

Time : TTime;

BEGIN

Time := HMStoTime( 14, 15, 30 );

SyncSystemTime( Time );

{ The New RTC Time will be 2:15:30PM }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure SyncSystemDate(         NewDate        : TDate     );

[PARAMETERS]

NewTime     New Time To Set RTC Time To

[RETURNS]

(None)

[DESCRIPTION]

This Procedure Synchronizes the Battery System Clock with (what I

consider to be the more Reliable) DOS System Clock.

[SEE-ALSO]

GetSystemTime

GetSystemDate

SyncSystemTime

[EXAMPLE]

VAR

Date : TDate;

BEGIN

Date := DMYtoDate( 1,2,1990 );

SyncSystemDate( Date );

{ The New RTC Date will be set to 2 Feb 1990 }

END;

������������������������������������������������������������������������������

[FUNCTION]

Function  GetDayLightSavings                                   : BOOLEAN;

[PARAMETERS]

(None)

[RETURNS]

Is DayLight Savings Set?

[DESCRIPTION]

Returns whether or not the RealTime Clock DayLight Savings is

Activated.

[SEE-ALSO]

SetDayLightSavings

[EXAMPLE]

BEGIN

WriteLn( 'RTC Daylight Savings is Active : ', GetDayLightSavings );

{ If DLS is Active in the RTC Register then reports TRUE else FALSE }

END;

������������������������������������������������������������������������������

[FUNCTION]

Procedure SetDayLightSavings(     DLS            : BOOLEAN   );

[PARAMETERS]

DLS         Whether DayLight Savings is to be Set

[RETURNS]

(None)

[DESCRIPTION]

Sets the RealTime Clock Day Light Savings according to the Flag

you pass in.

[SEE-ALSO]

GetDayLightSavings

[EXAMPLE]

BEGIN

SetDayLightSavings( TRUE );

{ DayLight Savings Time is Now Valid in the RTC registers }

END;