WHY NOT DUPLEX?

Duplex printing is not recommended on simplex HP LaserJet printers.

A simplex printer prints on one side of the paper.  A duplex printer

prints on both sides of the paper.  The HP LaserJet Series II, IIP,

III and IIIP are simplex printers.  The LaserJet IID and IIID are

duplex printers.

It is not recommended that you print on both sides of the paper by

turning over the paper and running it through the printer on the

reverse side.  The results of two-side (duplex) printing with

simplex printers are unsatisfactory.

FRICTION BETWEEN SHEETS

Paper is taken from the tray and fed into the printer by the pick-

up roller.  The pick-up roller is composed of a soft rubber.  This

rubber pick-up roller pulls paper into the printer by applying

pressure to the surface of the top sheet of paper in the paper tray

and sliding this sheet off the stack of paper in the tray.  This

results in friction between the top sheet and the second sheet of

paper in the stack.

SMEAR ON THE SECOND SIDE PRINTED

During simplex printing, unprinted paper contacts only unprinted

paper as it is drawn from the paper tray.  If you were to attempt

two-sided printing in a simplex printer, the printed page would

have to be placed print side down in the paper tray.  Now, as the

page is drawn into the printer, the face-down printed surface is in

contact with the sheet below it.  As the pick-up roller slides this

sheet into the printer, friction between the printed surface and

the sheet beneath it causes the printed surface to smear toner on

the lower sheet.  The lower sheet will show a smear when it exits

the printer.  This smear is identifiable because it's always

centered on the leading end of the second side printed.

TONER SMUDGES

In addition, two-sided printing in a simplex printer will deposit

some toner on the separation pad.  The separation pad is on the

underside of the paper as it feeds into the printer.  The

separation pad is contacted by the paper only when paper is

initially fed.  This type of toner contamination produces the same

type of smudging:  on the leading end of the second side printed.

PAPER JAMS

Attempting to perform two-sided printing in a simplex printer may

also lead to excessive jamming.  This is because a slight curl may

be ironed into the paper during the first printing.  Paper curl

makes it difficult for the pick-up roller to feed paper into the

printer.  A duplex printer uses the pick-up roller only when the

paper is initially fed into the printer so this problem is avoided.

For trouble-free duplex printing, the HP LaserJet IID, LaserJet

IIID or the LaserJet 2000 printers are the recommended solutions.