P C   F L T   P L A N   1.1�

Computer Flight Planning for PC's

D O C U M E N T A T I O N

Note:  This is a beta test version of the program, but as it is

late such, one should not expect any great changes in the final

version 1.1. A program like this can always be improved however, so

there will sure be later versions. Registrations will be valid for

future versions.

-------------------------------------------------------------------

1.   DISCLAIMER

----------

THIS PROGRAM MAY ONLY BE USED IF THE USER AGREES TO THE FOLLOWING

TERMS:

There is no warranty of any kind. In no event shall the author be

liable for any damages, including any lost profits or other

incidental or consequential damages arising out of the use of or

inability to use this program, even if the author has been advised

of the possibility of such damages. The author shall not be liable

for any possible wrong calculations made by the program.

2.   REGISTRATION

------------

This program is "shareware", which means that the program package

can be freely distributed in its unmodified form and tested. If you

find the program useful, a registration of USD 25,- would be

appreciated. You will then receive a registration number to be

inserted under the configuration option. The serial no. will appear

in your program, showing that you have registered the copy. There

are no crippled features without a serial no., - the only differ-

ence is that the information screen will not be shown at startup.

A registration will be valid for future upgrades of the program.

This program must NOT be used by COMMERCIAL-, MILITARY- or

GOVERNMENTAL operators without a special licence agreement with the

author. Professional aviators may wonder how this should be app-

lied, so I'll try to clarify: If for example you are a military

pilot and you use this program to plan your flights on your home

computer, or even have it on your private laptop PC, I consider it

as personal use. But if you install the program on the squadron's

PC, a special agreement will be required.

You may print out the file MAILER and use the form to register the

program, or just mail me your name, adress and payment.

------------------------------------------------------------------

Geir A. Granviken            May be contacted via:

Godheimvn. 85                -Thunderball Cave BBS (PCBoard)

N-3440 Roeyken                Oslo, Norway.  (+47) 2 567018/567017

Norway                       -RelayNet: Aviation conference

User name: Geir Granviken

------------------------------------------------------------------

3.   DISTRIBUTION

------------

The program package can be freely distributed, but all files must

be included and they must not be altered in any way. The regist-

ration number is private for one user and must not be given away.

The configuration file FPL.CFG, which is created by the program,

shall NOT be included. No fees must be charged, except for diskette

costs etc.

The package consists of the following files:

- MAILER       - Registration form

- FPLINFO.TXT  - This file, information on installing and usage

- FPL.EXE      - Main program file

- FPLSETUP.EXE - Program file (normally called from main program)

- FPL.DTA      - Data file

- FPL2.DTA     - Data file

- FPLFLY.DTA   - Data file, aircraft data

- FPLFAC.DTA   - Data file, nav database

- FPLFAC.IDX   - Data file, nav database

- FPLFAC.TXT   - Nav database in ASCII text format

- ********.RT* - Examples, saved routes

4.   INTRODUCTION

------------

4.1  The program is used to make operational flight plans for VFR- or

IFR flights, - fixed wings or helicopters. (VFR = Visual Flight

Rules, IFR = Instrument Flight Rules.)

The author is an airline pilot with a background as a military

helicopter pilot. But this program is not made with airline operat-

ions in mind. Most such carriers use big main frame systems fed

with wind prognosis for large parts of the globe from met centers

several times a day....  This program should be quite useful for

the pilot flying smaller airplanes or helicopters and who have

access to an IBM compatible PC.

This is not a complete program manual (One day maybe I'll get

finished with it!), so the small details, like basic keyboard

usage, are not covered.  But the program should hopefully be easy

to use, and if you take a close look at the sample routes, it

should be possible to make some sense of it (I hope).

4.2  A flight can be defined to the destination and to one or two

alternates. In addition, a list of other divertion fields may be

created.

4.3  The program has a nav-database with positions, magnetic variations

and frequencies for airfields, navigational aids, intersections

etc. The data included are examples only (they are correct though),

but the user can enter his own waypoints into the database as he

creates or edits a route, and in that way build up a database.

4.4  The program also provides a small register where some key data for

the different aircraft used may be stored.

4.5  The routes created may be saved on disk, so that a collection of

frequently used routes may be compiled.

4.6  There are 4 different flight plan print-outs to be selected from.

One of these is a kneeboard size flight plan.

4.7  Hardware requirements:

- IBM PC/XT/AT/PS2 or compatible.

- Minimum 512 kb RAM.

- Any display type.

- Harddisk not required, but the program will work much faster. On

a floppy disk system, the drive should be at least 720 kb.

- A disk cache program is recommended but not required.

SMARTDRV.SYS (or .EXE) which is included with Windows and DOS 5.0

is such a cache program, and works very well with PC FLt Plan.

- To be able to print out all four flight plan variants, the

printer should be able to print IBM's graphic characters (line

drawing characters).

5.   INSTALLING

----------

If you have a harddisk, make a directory and copy the required

files into this. The program may also be installed on a floppy

disk.  Even if you have a harddisk, it might be convenient to also

have the program on a floppy which you can bring with you.

The files required to run the program are:

- FPL.EXE

- FPLSETUP.EXE

- FPL.DTA

- FPL2.DTA

- FPLFLY.DTA

- FPLFAC.DTA

- FPLFAC.IDX

The two EXE-files work fine if compressed with PKLite(tm), which is

a shareware program that makes certain EXE- and COM files signifi-

cantly smaller. Program files compressed with this product will

self expand in memory when run. The programs do not load and start

any slower either, rather the contrary; since the files are

smaller, they are loaded faster from the disk. If not already used

on your copies of the EXE files in this package, they will occupy

about half the disk space if compressed with PKLite. Recommended!

6.   STARTING PC FLT PLAN

--------------------

Make the disk and directory where the program is located the

current directory (i.e. CD C:\FPL). Start the program with the

command FPL. FPLSETUP.EXE may be run directly, but is normally

called from the PC Flt Plan main menu. If you go through the

CONFIGURE option in the SETUP menu, a configuration file, FPL.CFG,

will be created. Do not delete this.

7.   PREPARING THE PROGRAM FOR USE

-----------------------------

7.1  CONFIGURING THE PROGRAM:

The first time the program is run, you should go through the

CONFIGURE option in the SETUP menu. If you have selected this

option but do not want to change anything, just press ESC until you

are back in the SETUP menu.

7.2  AIRCRAFT DATA:

Then you should enter data for the aircraft you will use. This is

done with the option AIRCRAFT DATA in the SETUP menu. You may also

specify aircraft data when making a flight plan, but it is most

convenient to have them in this little database.

The aircraft data that can be stored are:

- AIRCRAFT - You may do this two ways: You can store the AIRCRAFT

TYPE, i.e. Cessna 172. Or, if you will operate several

different aircraft of the same type, but with slightly

different Dry Operating Weights for example, you may

have one entry for each aircraft and instead enter the

AIRCRAFT REGISTRATION here.

- REMARKS  - The data here are not used in the flight plans so far,

but the entry will be shown when you select an air-

craft when making a flight plan. If you have stored

the aircraft by registration, you may put the type and

configuration here.

- ICAO FLIGHT PLAN NAME      - Not used by the program so far, but

the entry is reserved for the air-

craft code  used in ICAO flight

plans.

- WEIGHT UNIT       - K=Kg/L=lbs. All weight data for an aircraft

must be in the unit you enter here.

- DRY OPERATING WT  - Aircraft weight, with full oiltanks,

equipment, catering and crew. Includes every-

thing except Payload and Fuel.

- MAX ZERO FUEL WT  - If applicable. If not, enter 0.

- MAX TAKEOFF WT    - Yes, just that.

- MAX LANDING WT    - If applicable. If not, enter 0.

- START/TAXI FUEL   - Fuel normally used for startup and taxi

before takeoff.

- RESERVE FUEL FLOW - Fuel consumption rate normally used when

calculating the reserve required by the

flight rules you operate under.

- HOLDING FUEL FLOW - Fuel consumption rate normally used when

calculating extra holding, extra reserves,

company fuel etc.

- MAX FUEL CAPASITY - Max tank capasity. Not mandatory.

- UNUSABLE FUEL     - On some aircraft, the last rest of fuel in

the tanks is considered not useable, and can

therefore not be counted as available fuel.

- ACTUAL FUEL LOAD  - Typical Fuel Load in the same unit as the

fuel-data above . If Fuel Load varies, you

may enter 0, and enter the number when making

the flight plan.

- ACTUAL FUEL WT    - The WEIGHT of ACTUAL FUEL LOAD above. Must be

in the same unit as the other weight data. If

ACTUAL FUEL LOAD and ACTUAL FUEL WT are in

the same unit, i.e. lbs, you may enter 0.

If FUEL FLOW-data and WEIGHT-data are in different units, i.e.

gallons and pounds, PC Flt Plan must know the relationship between

these to be able to calculate landing weights etc. This relation is

found when the program is given ACTUAL FUEL LOAD and ACTUAL FUEL

WEIGHT in their respective units.

7.3  NAV-DATABASE:

If you want to enter airfields, navaids etc., you can select NAV-

DATABASE from the SETUP menu. But the records may also be entered

as you create a route. When you enter a waypoint in the route

editor, the program tries to look up the identifier in the nav-

database, if it can't find it, the necessary data will be asked

for, and the program then asks if you want to save it in the

database.

On how to edit the nav-database, select the NAV-DATABASE option in

the SETUP menu. If you play around a little here, most things

should hopefully be pretty straightforward.

8.   HOW TO MAKE A FLIGHT PLAN

-------------------------

8.1  The flight plan is made from two sets of information:

- ROUTE DESCRIPTION is the route to be flown from point of depar-

ture, through several waypoints to the destination, and if

required, to one or two alternates and/or a list of divertion

fields. Altitude/flight level, winds, true airspeed and fuel flow

are entered for each leg. For IFR flights, special legs like

SID's, approaches etc. may be entered.

This is the the information that is stored when you save a route.

When you load a route, winds, TAS etc. may be changed quickly for

the entire flight without having to edit each leg.

- FLIGHT PLAN DATA are aircraft (and it's data), reserves, fuel

load and payload. These data are not saved together with the

route, but are contained between program sessions.

8.2  CREATE/EDIT A ROUTE:

Select FLIGHT PLAN from the MAIN MENU. To clear the route in the

work area, if any, select NEW ROUTE. Select ROUTE EDITOR and then

NEW LEG to start entering legs.

Leg# 0 is the point of departure. No further data are entered for

this leg.

To-/From:

---------

In the TO-/FROM fields you enter the identifier for the airfield/

navaid/fix. The program searches the nav-database, and if the entry

is not found, you may enter lat-/long positions, name, magnetic

variation and frequency manually. You will then also be asked if

you want to save the waypoint in the nav-database.

You do not have to enter lat-/long positions if you want to enter

corse and distance manually instead of letting the program calc-

ulate these. This may be useful if you plan a trip where you just

pick landmarks on the map as waypoints and measure course/distance

with a ruler.

A star (*) as the first character in the FROM-field denotes a

special leg, which is used for SID's, STAR's, approaches etc. They

are also used together with codes to separate the route to destina-

tion, to the alternate(s), and the divertion list:

*xxxx - A "*" and up to 4 characters is used when you want to enter

a SID, STAR, or approach instead of a straight leg. "xxxx"

can be anything (i.e. "SID", "ILS", "APPR" etc.) except for

the codes below. In the To-field enter the point where the

leg ends. For these legs you will be asked for estimated

average ground speed, time or distance. You only have to

enter 2 of the values. For example if you want to reserve

10 minutes of fuel for an approach at the destination, you

can enter 10 minutes and the average GS for the approach.

The program will then calculate the distance this

represents and add it to the total trip distance.

*ALTx - Means that the route after this leg describes flight to the

ALTERNATE(s). No further data are entered in a leg with

this mark, it is used as a separation line. "x" may be

anything, for example "1" or "2". This code may be used

twice in a route description. If used, the route will be

divided into the following sections:

- Before any *ALTx  : From DEP.FIELD to DESTINATION

- After first *ALTx : From DESTINATION to ALTERNATE 1

- After second *ALTx: From ALTERNATE 1 to ALTERNATE 2

(The reason you can have two alternates is that under some

regulations, you can file a flight plan to an airfield

which is below weather minima, - provided that you have two

alternates. The first alternate must then have landing

minima, the second must be above alternate minima.)

*DIV -  Means that the following legs make up a DIVERSION list.

These legs are shown on the flight plan as a list of

optional alternates. They show time and fuel used for

direct flight from DESTINATION to the various DIVERSION-

fields, plus extra time for approach or extra route

distance, which may be entered for these legs.

Automatic or Manual calculation of Course/Dist:

-----------------------------------------------

For straight legs you enter "A" or "M" for Auto-/Manual course &

distance. If you select "A", course & dist will be calculated from

the lat-/long positions and magnetic variations for the waypoints.

If you find that the course differs a degree or so from what is

found on your IFR-chart, you may select "M" and enter the same

values as in the chart manually.

Flight Data:

------------

When FROM and TO for a leg are defined, you enter the following

data:

- FL/ALT- Flight Level or Altitude. These data are not used by any

calculations, but are shown as information about planned-

or minimum altitudes on the flight plan. It is recom-

mended to enter the values as in ATC flight plans; for

example "F090" for Flight Level 90, "A050" for 5000 feet.

Or you may use "CLMB", "DESC", "VFR" or what you like.

- TAS   - True AirSpeed in knots.

- WIND  - Wind Direction (what direction the wind comes FROM), and

Wind Speed in knots.

- F.FLOW- Fuel consumption per hour.

8.3  FLIGHT PLAN DATA

When the route has been defined, you select PRINT-/SHOW FLIGHT PLAN

from the FLIGHT PLAN MENU. You will then be taken to an input

screen, where the Flight Plan Data are entered. The following data

may be entered or changed here:

- DATE     - Date of flight.

- FLTNO/CS - Flight Number or CallSign.

- AIRCRAFT - The aircraft picked from the aircraft database, or one

that you specify here.

- SKED DEP - Scheduled time of departure (block).

- SKED ARR - Scheduled time of arrival.

- FINAL RESERVE     - Reserve required by the flight regulations

you operate under. Enter minutes and fuel

flow.

- ROUTE RESERVE     - Additional reserve in PERCENT. If used, the

amount will be calculated as the specified

percent of fuel required to destination and

to the alternate(s), if any.

- EXTRA HOLDING     - May be used for expected holding, company

fuel etc. Enter minutes and fuel flow.

- START/TAXI        - Fuel used for startup and taxi, before take-

off. The minutes are estimated time from off

block to airborne. Will be counted when ETA

is calculated.

- UNUSABLE FUEL     - On some aircraft, the last rest of fuel in

the tanks is considered not useable, and can

therefore not be counted as available fuel.

- ACTUAL RAMP FUEL  - Planned fuel load. Amount must be in the same

unit as the fuel flow figures above. Weight

must be in the same unit as the other weight

data. If Fuel LOAD and WEIGHT are in the same

unit, just enter 0 in the weight field.

- DRY OPERATING WT  - Aircraft weight, with full oiltanks, equip-

ment, catering and crew. Includes everything

except payload and fuel.

- ACTUAL PAYLOAD    - Planned payload.

8.4  PRINT OR SHOW THE FLIGHT PLAN

When you have entered the Flight Plan Data, a screen with a summary

of the flight will be shown. From here you may print out the flight

plan or view it on the screen. Explanation of some of the items

showed on the screen:

- TIME  - Airborne time (hr:mins) to destination.

- BURN  - Start/Taxi-fuel + tripfuel to destination.

- DIST  - Distanse (NM) to destination. Includes approaches.

- AVG.WC- Average wind component to destination (plus=tailwind,

minus=headwind).

- DEP   - Sheduled Time of Departure.

- ETA   - Estimated Time of Arrival at destination. Taxi time at

the departure field is counted in.

- NO WIND  - Burn/Time to destination if no winds.

- REQUIRED FUEL     - Total fuel required, considering flight to

destination, alternates(s) (if any), winds,

reserves and start/taxi.

- EXTRA FUEL        - The difference between Required fuel and

Actual fuel load. The time shown is the app-

roximate flight time this represents, calc-

ulated from the average consumtion to dest-

ination and alternate(s).

If you want to print out a flight plan in kneepad-size, you must

have a printer that accepts the same codes for condensed pitch as

IBM's dot matrix printers, and it must be capable of printing the

IBM line drawing characters. If it doesn't work, you may try

another solution: Select PRINT FLIGHT PLAN TO FILE. The file

written is plain ASCII, and may be imported into your word proces-

sor and printed from here. Select a font with a pitch of around 17

characters/inch and normal line height.

The two A4-variants send plain ASCII-text to the printer, and

should therefore be usable on most printers.

9.   ABOUT THE FLIGHT PLAN

---------------------

9.1  LEG TIMES

In the flight plan produced, the time for each leg is rounded up or

down to the nearest whole minute. But the small difference between

the accurate leg time and the whole minute is added or subtracted

to the next leg time. This is the same method used in professional

flight planning systems used by airlines. The reason for this is

that on a trip with many legs, the total time may become several

minutes wrong if for example most of the legs happen to be rounded

down.

9.2  FUEL

The fuel amount shown in the last column of the flight plan is the

fuel you would have left over the checkpoint if you departed with

the TOTAL FUEL REQUIRED. This way you can see if you have all your

specified reserves intact along the flight. If you have less fuel,

you have started to consume your reserve, route reserve or extra

holding fuel.

9.3  CORRECTIONS

The correction factors shown on the flight plan are fuel and time

to destination, and total fuel required if the conditions were:

- NO WIND  - No winds at all.

- +20KT    - 20 knots more TAILWIND than the average wind component

you get from the winds you have planned for.

- -20KT   -  20 knots more HEADWIND than the average wind component

you get from the winds you have planned for.

Example: If you get an average wind component of +7 knots (tail-

wind), the "+20" will be the figures for 27 knots tailwind, while

the "-20" will be the figures for 13 knots headwind. The "no wind"

will be for a wind component of 0 no matter what the planned winds

are.

9.4  HOW TO USE THE ETO/RTO/ATO FIELDS:

After takeoff, you start filling out all the ETO-times all the way

down to destination, using the leg times given. Usually, you will

only put down the minutes, except when starting on a new hour.

When you pass a waypoint, you write down the time in the ATO

column, then calculate the new estimated time for the next way-

point, and put this in the RTO column for that point. This way, you

will have a fresh estimate for the next reporting point for ATC

(and yourself!).

As you now pass the waypoints, it will be very easy to see if you

are ahead- or behind the flight plan by comparing the ATO time and

the ETO time (the latter was calculated in relation to the takeoff

time).

10.  TERMS & ABBREVIATIONS USED IN THE PROGRAM

-----------------------------------------

Acc     - Accumulated time

AMT     - Average Magnetic Track

ATC     - Air Traffic Control

ATO     - Actual Time Overhead (when passing a waypoint)

ATT     - Average True Track

Alt     - Altitude

Arr     - Arrival

Burn    - Fuel Burn

CS      - Call Sign

Dist    - Distance in nautical miles

DOW     - Dry Operating Weight. Weight of everything except payload

(cargo/passengers) and fuel

Dep     - Departure

Dest    - Destination

ETA     - Estimated Time of Arrival

ETD     - Estimated Time of Departure

ETO     - Estimated Time Overhead

FF      - Fuel Flow (Fuel consumption per hour)

FL      - Flight Level

Flt No  - Flight Number

GS      - Ground Speed. Speed in relation to the ground after wind

effect is taken into account

Kts     - Knots, = speed in nautical miles per hour

LWT     - Landing Weight

MH      - Magnetic Heading

MLWT    - Max Landing Weight

MT      - Magnetic Track

MTOW    - Max Takeoff Weight

MZFW    - Max Zero Fuel Weight

NM      - Nautical Miles (1 NM = 1.852 meters)

RTO     - Revised Time Overhead. Means Revised ETO

Rte Res - Route Reserve. In percent of fuel used to destination and

alternate(s)

TAS     - True AirSpeed

TOW     - Takeoff Weight

WC      - Wind Component (Headwind or tailwind)

ZFW     - Zero Fuel Weight. Weight of everything except fuel.

Larger aircraft usually has a max limit on this.