To start off, the xprzedzap.library is just a slightly modified

xprzmodem.library. The xprzedzap.library uses the 2.10 version of the

xprzmodem.library as its base and uses the 2.50 version for the 32bit

CRC routines. The reason the 2.50 version is not used as a base is

that it does not compile correctly under SAS/C 5.10a. After analysis

on the source code to both versions, they appear to be identical ex-

cept for the 32bit CRC routines.

It appears, that there might be a bug in the xprzmodem.library

when the ZFIN is not acknowledged by the receiver when one is sending

a file, but I believe that this is not caused by the library, and is

instead caused by the comm. program accessing the library.

The changes from zmodem to zedzap are as follows. 1) The maximum

packet size can now be set with a maximum/default at 8192. This will

vary when sending and will be static when receiving. When sending

the maximum packet size will be baud rate dependant, and the size is

calculated with the formula MAX_PACKET = (BPS_RATE * 8192 / 9600).

You can specify a limit for the maximum packet size with the M op-

tion, but it only influence the packet size if it is smaller than

MAX_PACKER or if one is receiving a file (NOTE: It should allways be

set to 8192 if one is receiving a file... But the option in there to

limit it to less). Also, the IO Buffer for reading/writing to/from

the disk must be equal to twice the maximum packet size (or the ma-

ximum packet size will automatically be decreased). 2) One can start

and end a transfer if there are no files to be sent. If this option

in used and no files are sent, then most BBS programs will report

that the upload was unsuccessful (experienced this when testing the

library).

All options must be in the following order (to be sent by one's comm.

program): T(YN?C),O(YNRS),B(num),F(num),E(num),A(YN),D(YN),K(YN),

S(YN),R(YN),C(num),N(YN),M(num),P(string)

new options:

N = Send if there are no files                            Def: Y

M = Set maximum packet size ((M <= 8192 )&&(B >= M * 2))  Def: 8192

C = Set Pseudo BPS Rate (if you are have you BPS rate locked, this

will make the library use this BPS rate to calculate the maxi-

mum packet size). I it is set below 300 or above 56800 then it

will be ignored. If it is left out, it will also be ignored.

This could be used in the future by Welmat if the BPS rate is

locked and a connection of 2400BPS is made with the serial port

at 19200.

-=> Please see Codes.Doc for changes to the result messages! <=-

Example: (for coorect operation on Fidonet)

TN,ON,B16,F0,E30,AN,DY,KN,SN,RN,C2400,NY,M8192,Pmail:inbound

Features:

-Send in binary mode

-Append .dup[n] to duplicate files

-File IO buffer set to 16k (should be M * 2 = 8192 * 2 = 16384 = 16k)

-Do Not Use Frames

-Allow a maximum of 30 errors to occur during transfer

-Do not download automatically

-Do not keep partial files (because of option O)

-Do not send full path

-Do not use received path

-We are using the maximum packet size that is associated with 2400BPS

(in this case 2048, so the M8192 would not really make a difference.

This should be set after connecting. If this is not specified, then

the normal BPS rate be used in the calculation of the maximum packet

size)

-Start transfer even if there are no files to be sent

-Set maximum packet size to 8192 (this might be set lower if the BPS

rate is below 9600 or if the file IO buffer is smaller than 16k. This

should be kept at 8192 at all times, but the option is there to set

it to a lower level)

-Receive all files into the directory mail:inbound/

Problems:

-If file requests are received and are not replied to in such a way

that the *.REQ file is deleted (or renamed), then the node that sent

the *.REQ file will not be able to send the file again with that

name, as it will automatically be named *.REQ.DUPn by the protocol,

or original *.REQ file will be overwritten. This is all specified

with the 'O' option ('OS' would make the *.REQ file get skipped all-

together).

-The same goes with *.OUT,*.HUT, and *.CUT files (but since these are

barely used in the first place...)

This XPR library has been tested with VLT and Welmat.

Credits:

-Chuck Frosberg for the original ZMODEM specs

-Rick Huebner for the original xprzmodem.library (--> v2.10)

-William M. Perkins for the 32bit CRC improvements (2.50)

-Yves Konigshofer for the slight changes to allow ZedZap-type transfers

Address Comments To:

Yves Konigshofer

1:109/456.0@FidoNet

(301)469-YVES

[9837]

----

Updates - Russell McOrmond

-----------------------------------------------------------------------

Bug Fixes - Yves Konighofer

20 Nov. 1992

Welmat [rwm- current beta's, not a valid concern] does not use v->Baud

so a packet size of 2400 was assumed in the

zgethdr() routine. This works just fine for 2400bps transfers, BUT at

high speeds with large 8192 byte blocks it would be almost fatal for an

error to occur that messes up the header since only the next 2400 bytes

would be checked. This has been fixed so that 8192*2 characters will be

read instead of 2400 before the "garbage count" becomes too large. This

is because we CANNOT assume that the sending will use a smaller packet

than 8192 bytes (even at only 1200bps). Also, be allowing 2*8192 bytes

to be read, we increase the likelyhood of hitting the next header if

the fist is messed up and becomes longer due to line noise (thus one

less garbage count error would be reported). Alas, both ways will re-

quire a resync to be performed.

The order of the 'case' statements in the rzfile() routine has been

changed slightly so that it will not send two Attention sequences when

a bad header is followed be a bad packet.

A few result codes for errors have been changed slightly. If you speci-

fy that an already existant file should be skipped, the procheader()

routine will report ZSKIP instead of ERROR. ZDCO has also been updated

to show that the carrier has been dropped. While none of the XPR inter-

faces that I know support a carrier check, this result has been pro-

vided. [rwm - All XPR interfaces that I know of return (-1) from

an xpr_sread() to indicate a carrier drop.  I know that both TERM

and Welmat support this properly]

-----------------------------------------------------------------------