Warning: this is my first attempt at real-time midi programming, so use this

program at your own risk!

The program requires a soundblaster (or compatible, I guess), EGA or VGA and

a Microsoft compatible mouse.  It will allow you to control two midi

controllers in real time.  The EGAVGA.BGI file is used by Borland's graphics

routines, and it must be present on the same directory as MIDICTRL.EXE.

The program will prompt you for:

1. the transmit channel,

2. the controller number for the horizonal movement of the mouse,

3. the controller number for the vertical movement of the mouse, and

4. a delay time in milliseconds.  I suggest using 50-100.

Some controllers:

1 modulation

2 breath

4 foot

5 portamento time

7 main volume

8 balance

10 pan

11 expression

The parameters are obtained from the mouse position, with (0,0) being the

upper left hand corner of the box.  The maximum is (127,127), so this

will only work well for 7-bit controllers.

The delay time is necessary to let the instrument catch up, since most PCs

are capable of sending info faster than most midi devices are capable of

using it (this is my experience at least--I'm no midi expert).

The data are sent out the SoundBlaster's midi port.  I have no way of knowing

if it will work with other midi ports.  The program assumes that the addresses

for the SB registers start at 0x220.

To exit the program, just hit any key on the keyboard.

I use this program to control an ART SGX-T2 guitar effects processor

with my feet!  It takes a little practice...

If you want to source code to make your own modifications, it's in Turbo C++.

Just leave me a note.

Btw, is anyone else out there interested in computer generated music (e.g. by

neural nets, rule based algorithms, etc.)?

David Eubanks

70034,557