***   MIDI   ***

MIDI is a diagnostic/universal-librarian utility.  It can be used to

identify midi communication problems with instruments or computers.  It can

also save midi data dumps that are in standard System Exclusive format.

Except for the Real Time commands page in Send Mode, MIDI uses the hex

numbering system.  All midi data displayed on the screen is in hex, and all

midi data entered at the keyboard must be done in hex.  The program is not

case sensitive.  Most information in synth manuals about sys-ex messages is

given in hex but be careful, some manufacturers have been known to mix hex

and decimal in their charts.  There are several memory resident programs on

the market that convert between hex, decimal, and binary, which can be

useful while working with MIDI.  Below is a conversion chart that can be

used also.

When using a memory resident program while in MIDI, it may be necessary to

press a key when returning to MIDI.  This is due to the way some resident

programs handle the keyboard interrupt (INT 16h).  If the midi interface

gets confused while in Monitor Mode, the Esc key will reset it.

MIDI checks to see if it has a Roland MPU-401 or equivalent interface.  If

it doesn't find one, it will inform you and give you the option of running

the program anyway.  If you run the program without an interface, the only

thing that doesn't work is receiving and transmitting data (although it will

appear to transmit).

MONITOR MODE

Monitor Mode is used to look at the midi data coming from an instrument.

The Mask status bytes option enables you to have the program ignore and not

display data you don't want to see.  To remove or enter a mask, press the

"/" key.  The arrow keys can be used to move the cursor to an open mask or

to a specific mask to delete.  Delete by pressing "X" or Esc.  You can mask

up to five different bytes.  When a masked status byte is received, it and

any associated data bytes are not displayed.  This is very useful for

"uncluttering" the display with information you don't want to see.  Pressing

"E" or "8" is a shorthand way of entering "FE" or "F8". If you make a

mistake in entering the first digit of the byte, the Backspace key can be

used to start again.  If the mistake is made on the second digit, you will

have to press the "/" key, arrow over to the byte, and then reenter it.

To clear all masks, press "C".

Note:  The "/" key is called "?" on the program screen - you don't have to

press Shift when hitting this key.

The "Esc" key resets the midi interface.

To make the display of data more readable, any byte higher than 7F is

displayed in reverse video or a different color (to identify it as a status

byte), and 2 spaces are inserted before it.  Pressing "L" will display a

list of midi status bytes and what they mean.  The "K" key clears the screen

and also clears the history buffer.

Pressing "H" calls up the history buffer, which holds 30,000 bytes.  After

the buffer is filled, it is still updated, so it always holds the last

30,000 bytes.  The History Buffer can be written to disk as an ASCII file

which can then be printed if desired.

Note:  Do not begin any file names with "MIDI".  You will not be allowed to

if you try.  This is done to prevent accidentally overwriting the program or

this .DOC file.

SEND MODE

Send Mode is used to find out how equipment will respond to various midi

data.  The loop repeat can be used to constantly send a small sequence for

trouble shooting purposes.  Use spaces for delays.  Each space is about

1/40th of a sec.  You can enter up to two lines of data. The "G" key retypes

the last line(s) that were entered.  The "U" key allows you to send a "dump

request" and then view the response on the monitor screen.  Just type in the

request, complete with F0 at the beginning and F7 at the end, and then press

"U".  The request will be sent, and you will be immediately put in Monitor

Mode to view the response, with the first two masks set to FE and F8.  (This

is because many synths & drum machines keep sending these bytes while doing

a bulk dump).  The response can then be viewed in the history buffer and

saved to disk if desired.

It takes two hex characters to make a complete byte.  If an odd number of

hex characters is entered, the last single character won't be sent.  If you

press "U" with an odd number of characters, the entry area will be blanked

and you won't be put in Monitor Mode.  If this happens, press "G" to retype

the entry, correct it, and then press "G" again.

The Real Time commands screen has more options for sending data, including

midi clock and associated commands.  The instructions on the screen are self

explanatory.  This screen is particularly useful for checking sequencers and

drum machines for proper operation and finding out what commands they

support.  The midi interface sends midi clock bytes (F8) while in this

screen only.  The Start & Continue commands turn on the audible metronome.

Start is without accents and Continue is with.

Note:  If you're using Song Position Pointer, send the pointer, and then a

"continue" command (not start).  The start command always means "start at

the beginning".

PATCH MODE

Patch Mode consists of several subsections for dealing with saving and

transmitting bulk patch or sequence data that is in the standard "System

Exclusive" format.  When you first enter Patch Mode the screen shows an

alphabetical listing, by extension or name, of all the files on the disk or

current subdirectory.  The maximum number of files that can be displayed on

a single page is 114.  The maximum number of pages allowed is four (456

files). If there are more, they will not be displayed and the "number of

pages" area at the upper right corner of the screen will have a star (*).

As files are added, deleted, or renamed, the directory will be updated. You

will notice that the names of programs that start with "MIDI" (like this

one) are not displayed.  This is done on purpose to save space for voice

bank listings.

The "Receive" option will save System Exclusive dumps from midi instruments.

When you press "R", you are given the choice of initiating the dump from the

instrument or sending a "dump request".  Some common dump requests are

listed on the screen along with instructions for entering the request. Since

it takes two hex characters to make a byte, if you type an odd number of

digits and press Enter, the request will not be sent, and you will be

instructed to try again.  If you elect not to send a dump request, the

computer is immediately put in sys-ex receive mode and is waiting for data.

As data comes in, small musical notes will appear between the braces in the

upper left portion of the screen.  When the notes are gone, the "W" key

should be pressed to finish writing the data to the disk.  This is a

separate operation so that separate bulk dumps from one or several

instruments can be saved to one file.  Just keep initiating bulk dumps until

you are done, then press the "W" key.  MIDI saves the data to disk as it is

being received, so available disk space is the only limiting factor.

A switch in the SPX-90 has to be changed which makes the "thru" jack an

"out" jack in order to use the special SPX-90 dump request.  See the

documentation at the end of this doc file.

The "Transmit" option transmits the system exclusive files back to the

instruments.  You have a "block-at-a-time" choice here in case you have

saved several instruments to one file and need time between each "send" to

connect the proper instrument to the computer.  If the instruments are

different, they can ALL be fed the file non stop.  Each instrument will

respond only to it's particular sys-ex message.

Note:  MIDI checks files before they are transmitted.  If the file is not

in standard "system-exclusive" format, an error message will be displayed.

The "Rename" and "Delete" options are self explanatory.

Once again, you will not be able to name any files "MIDI...".

The "File Size" command lists sizes of sys-ex dumps of various instruments.

On some instruments, like the Yamaha DX7II, you have the option of sending

from as little as one bank to as much as "everything". The numbers on this

screen are for "everything".

Pressing "X" changes the file sort order.  The sort order is for MIDI's

display only - the files are not permanently sorted.

The (V)iew command has two different meanings.  If the highlight is over a

file, pressing "V" will display the contents of the file in "debug" format.

This is useful for looking at voice names in a bulk dump file, or just being

curious about what's there.  At the bottom right corner of the display is a

"counter" that indicates the byte number of the last byte displayed on the

screen.

Any filenames with an extension of "*s*" are subdirectories.  Pressing "V"

when a subdirectory is highlighted will take you to that subdirectory.  This

is useful for keeping patches for different instruments separate.  The

(B)ack command simply takes you "back" one subdirectory level.

Note:  If you name a subdirectory with an extension, MIDI will not display

it.  This doesn't hurt anything, you just won't be able to move to it.

The (C)hange command allows you to change disk drives.  If you select an

invalid drive, there will be no error message - you just won't change

drives.

DOS COMMAND LINE ARGUMENTS

MIDI supports command line arguments for Monitor (M), Send (S), Patch (P),

black & white (B), and display sorted by name instead of extension (X).

For example:  To immediately enter Patch Mode, type "MIDI P" at the DOS

prompt.  For black & white display, files sorted by name, and initially

entering Monitor Mode, type "MIDI BXM".  The parameters can be in any order

and invalid parameters are ignored.

MIDI is user-supported shareware.  If you find it useful, send a check or

money-order for $15.00 or more, and you will receive the latest version.

Please include the version number you currently have.  The version number

is on the first screen when you enter MIDI without any options (other than

"B").  If you have the latest, you will get the next version when it is

available.

If you give MIDI to a friend, or upload it to a BBS, be sure & give them

the self-extracting MIDIZIP.EXE - that way, they will get this documentation

along with the program.  This version is 3a.

Eddy Reynolds

11214 Olde Mint House Lane

Tomball TX 77375

NUMBER CONVERSION

~~~~~~~~~~~~~~~~~

******   Status Bytes   ******

~~~~~~ ~~~~~

HEX  DECIMAL  BINARY        HEX  DECIMAL  BINARY       HEX  DECIMAL  BINARY

~~~  ~~~~~~~  ~~~~~~        ~~~  ~~~~~~~  ~~~~~~       ~~~  ~~~~~~~  ~~~~~~

80   128   10000000         AB   171   10101011        D6   214   11010110

81   129   10000001         AC   172   10101100        D7   215   11010111

82   130   10000010         AD   173   10101101        D8   216   11011000

83   131   10000011         AE   174   10101110        D9   217   11011001

84   132   10000100         AF   175   10101111        DA   218   11011010

85   133   10000101         B0   176   10110000        DB   219   11011011

86   134   10000110         B1   177   10110001        DC   220   11011100

87   135   10000111         B2   178   10110010        DD   221   11011101

88   136   10001000         B3   179   10110011        DE   222   11011110

89   137   10001001         B4   180   10110100        DF   223   11011111

8A   138   10001010         B5   181   10110101        E0   224   11100000

8B   139   10001011         B6   182   10110110        E1   225   11100001

8C   140   10001100         B7   183   10110111        E2   226   11100010

8D   141   10001101         B8   184   10111000        E3   227   11100011

8E   142   10001110         B9   185   10111001        E4   228   11100100

8F   143   10001111         BA   186   10111010        E5   229   11100101

90   144   10010000         BB   187   10111011        E6   230   11100110

91   145   10010001         BC   188   10111100        E7   231   11100111

92   146   10010010         BD   189   10111101        E8   232   11101000

93   147   10010011         BE   190   10111110        E9   233   11101001

94   148   10010100         BF   191   10111111        EA   234   11101010

95   149   10010101         C0   192   11000000        EB   235   11101011

96   150   10010110         C1   193   11000001        EC   236   11101100

97   151   10010111         C2   194   11000010        ED   237   11101101

98   152   10011000         C3   195   11000011        EE   238   11101110

99   153   10011001         C4   196   11000100        EF   239   11101111

9A   154   10011010         C5   197   11000101        F0   240   11110000

9B   155   10011011         C6   198   11000110        F1   241   11110001

9C   156   10011100         C7   199   11000111        F2   242   11110010

9D   157   10011101         C8   200   11001000        F3   243   11110011

9E   158   10011110         C9   201   11001001        F4   244   11110100

9F   159   10011111         CA   202   11001010        F5   245   11110101

A0   160   10100000         CB   203   11001011        F6   246   11110110

A1   161   10100001         CC   204   11001100        F7   247   11110111

A2   162   10100010         CD   205   11001101        F8   248   11111000

A3   163   10100011         CE   206   11001110        F9   249   11111001

A4   164   10100100         CF   207   11001111        FA   250   11111010

A5   165   10100101         D0   208   11010000        FB   251   11111011

A6   166   10100110         D1   209   11010001        FC   252   11111100

A7   167   10100111         D2   210   11010010        FD   253   11111101

A8   168   10101000         D3   211   11010011        FE   254   11111110

A9   169   10101001         D4   212   11010100        FF   255   11111111

AA   170   10101010         D5   213   11010101

******   Data Bytes   ******

~~~~ ~~~~~

HEX  DECIMAL  BINARY        HEX  DECIMAL  BINARY       HEX  DECIMAL  BINARY

~~~  ~~~~~~~  ~~~~~~        ~~~  ~~~~~~~  ~~~~~~       ~~~  ~~~~~~~  ~~~~~~

0     0   00000000         2B    43   00101011        56    86   01010110

1     1   00000001         2C    44   00101100        57    87   01010111

2     2   00000010         2D    45   00101101        58    88   01011000

3     3   00000011         2E    46   00101110        59    89   01011001

4     4   00000100         2F    47   00101111        5A    90   01011010

5     5   00000101         30    48   00110000        5B    91   01011011

6     6   00000110         31    49   00110001        5C    92   01011100

7     7   00000111         32    50   00110010        5D    93   01011101

8     8   00001000         33    51   00110011        5E    94   01011110

9     9   00001001         34    52   00110100        5F    95   01011111

A    10   00001010         35    53   00110101        60    96   01100000

B    11   00001011         36    54   00110110        61    97   01100001

C    12   00001100         37    55   00110111        62    98   01100010

D    13   00001101         38    56   00111000        63    99   01100011

E    14   00001110         39    57   00111001        64   100   01100100

F    15   00001111         3A    58   00111010        65   101   01100101

10    16   00010000         3B    59   00111011        66   102   01100110

11    17   00010001         3C    60   00111100        67   103   01100111

12    18   00010010         3D    61   00111101        68   104   01101000

13    19   00010011         3E    62   00111110        69   105   01101001

14    20   00010100         3F    63   00111111        6A   106   01101010

15    21   00010101         40    64   01000000        6B   107   01101011

16    22   00010110         41    65   01000001        6C   108   01101100

17    23   00010111         42    66   01000010        6D   109   01101101

18    24   00011000         43    67   01000011        6E   110   01101110

19    25   00011001         44    68   01000100        6F   111   01101111

1A    26   00011010         45    69   01000101        70   112   01110000

1B    27   00011011         46    70   01000110        71   113   01110001

1C    28   00011100         47    71   01000111        72   114   01110010

1D    29   00011101         48    72   01001000        73   115   01110011

1E    30   00011110         49    73   01001001        74   116   01110100

1F    31   00011111         4A    74   01001010        75   117   01110101

20    32   00100000         4B    75   01001011        76   118   01110110

21    33   00100001         4C    76   01001100        77   119   01110111

22    34   00100010         4D    77   01001101        78   120   01111000

23    35   00100011         4E    78   01001110        79   121   01111001

24    36   00100100         4F    79   01001111        7A   122   01111010

25    37   00100101         50    80   01010000        7B   123   01111011

26    38   00100110         51    81   01010001        7C   124   01111100

27    39   00100111         52    82   01010010        7D   125   01111101

28    40   00101000         53    83   01010011        7E   126   01111110

29    41   00101001         54    84   01010100        7F   127   01111111

2A    42   00101010         55    85   01010101

YAMAHA SPX90 SYSTEM EXCLUSIVE

In order to make the Yamaha SPX-90 send system exclusive messages, it is

necessary to change the midi thru jack into a midi out jack.  This is

accomplished by removing the cover of the unit (2 screws on each side, 2

screws on the front top, and 1 screw on the upper back) and changing switch

SW105.  This switch is located on the circuit board about 2 inches toward

the front of the unit from the midi in jack.  It will be in the "N"

position.  Change it to the "T" position.  This modification only changes

the midi thru jack to an out jack so it will respond to bulk dump requests.

It does not affect the SPX-90 in any other way.  Below is the format of the

bulk dump.

BULK DUMP FORMAT (Memory Parameter)

Byte#       Data(Hex)      Comments

1            F0           Status

2            43           Yamaha ID

3            0n           n - Receive Channel (0 hex to F hex)

4            7E           Format No.

5            00

6            58           Byte count (88 bytes)

--------------------------------------------------------

7            4C           "L"

8            4D           "M"

9            20           " "

10            20           " "

11            38           "8"

12            33           "3"

13            33           "3"

14            32           "2"

15            4D           "M"

16            xx           Patch Number

---------------------------------------------------------------

17           (02)

18           (00)          " "          First Character of Title

19           (02)

20           (00)          " "

21           (02)                     The Title is 16 characters long.

22           (00)          " "        Each character uses two bytes.

23           (04)

24           (05)          "E"

25           (05)                     Only the least significant nybble

26           (08)          "X"            of each byte is used.

27           (04)

28           (01)          "A"

29           (04)

30           (0D)          "M"

31           (05)

32           (00)          "P"

33           (04)

34           (0C)          "L"

35           (04)

36           (05)          "E"

37           (02)

38           (00)          " "

39           (02)

40           (00)          " "

41           (02)

42           (00)          " "

43           (02)

44           (00)          " "

45           (02)

46           (00)          " "

47           (02)

48           (00)          " "

-------------------------------------------------------

49                         Balance Data

50

51

52

-------------------------------------------------------

53                         Output Level Data

54

55

56

---------------------------------------------------------

57                         Parameter 1

58

59

60

---------------------------------------------------------

61                         Parameter 2

62

63

64

--------------------------------------------------------

65                         Parameter 3

66

67

68

---------------------------------------------------------

69                         Parameter 4

70

71

72

----------------------------------------------------------

73                         Parameter 5

74

75

76

----------------------------------------------------------

77                         Parameter 6

78

79

80

----------------------------------------------------------

81                         Parameter 7

82

83

84

----------------------------------------------------------

85                         Parameter 8

86

87

88

----------------------------------------------------------

89                         Parameter 9

90

91

92

----------------------------------------------------------

93            tt           Program Type Code (01 to 1E)

94            0b           b=1: Bypass on    b=0: Bypass off

95            ee           Checksum

96            F7           EOX

Balance, Output Level, and Parameter use 4 bytes each.

Once again, only the least significant nybble is used.

If a parameter is not used, the data is 00 00 00 00

The program type code refers to the original program (1 thru 30)

that this program came from.

BULK DUMP FORMAT (Relative program number and memory number)

(This is the map of which program is called when

a particular patch change is received via midi)

Byte #      Data(Hex)      Comments

1            F0           Status

2            43           Yamaha ID

3            0n           n - Receive Channel

4            7E           Format No.

5            01

6            0B           Byte count (139 bytes)

--------------------------------------------------------

7            4C           "L"

8            4D           "M"

9            20           " "

10            20           " "

11            38           "8"

12            33           "3"

13            33           "3"

14            32           "2"

15            54           "T"

16            xx           Bank No. (01 to 04)  (A to D)

---------------------------------------------------------------

17           (08)          Patch 1          Memory No. (Ex. 8)

18           (02)          Patch 2          Memory No. (Ex. 2)

144           (40)          Patch 128        Memory No. (Ex. 64)

-------------------------------------------------------------------

145            0r           r: midi receive channel of the Bank

146            ee           Checksum

147            F7           EOX

BULK DUMP REQUEST

Byte #      Data(Hex)      Comments

1            F0           Status

2            43           Yamaha ID

3            2n           n - Receive Channel

4            7E           Format No.

--------------------------------------------------------

5            4C           "L"

6            4D           "M"

7            20           " "

8            20           " "

9            38           "8"

10            33           "3"

11            33           "3"

12            32           "2"

---------------------------------------------------------

13            xx           xx = 4D

14            yy           yy = 1F to 5A  Dump of memory parameters

-------------------                       of one program (31 to 90)

15            F7

xx = 54

yy = 01 to 04  Dump of program memory

Bank A to D

xx = 55

yy = 01 to 04  Request for bank change