Async.Asm

Operation Notes

by Jerry D. Stuckle

Userid: 72205,234

Async.Asm is an example assembler program to drive the Asynchronous Adapter

in the IBM-PC.  It accepts the same parameters as the ROM BIOS Interrupt 14

code, with the exception that only one port (COM1:) is available.  The code

can be easily modified to handle two communications ports, but for 1 only

wrote the code for one port to keep things simple.

This routine provides a buffered byte-stream interface to the

communications port.  The following parameters can be used.

AH=0  Open communications port.

AL has initialization parameters

7       6       5       4       3       2       1       0

----Baud Rate----      --Parity--     -Stop-    -Word Len-

000 -  110          X0 - None     0 - 1     00 - 5 bits

001 -  150          01 - Odd      1 - 2     01 - 6 bits

010 -  300          11 - Even               10 - 7 bits

011 -  600                                  11 - 8 bits

100 - 1200

101 - 2400

110 - 4800

111 - 9600

On return, conditions set as in call to comm status (AH=3)

AH=1 Send the character in AL to the port.  (AL is preserved).  On exit,

Bit 7 of AH is set if the buffer is full.  If Bit 7 of AH is not set, the

remainder of AH is set as in status request, reflecting the current status of

the line.

AH=2 Receive a character in AL from the port before returning to the

caller. On Exit, if AH is 0FF, the buffer was found to be empty and AL is

indeterminate.  Otherwise, AH has the current line status as set by the status

routine, except the only bits left on are the error bits (7,4,3,2,1).  If AH

has bit 7 on (time out) the remaining bits are unpredictable.  Thus, AH is

non-zero only if errors have occurred.

AH=3 Return the port status in AX.

AH contains the line status

Bit 7 = Time out

Bit 6 = Trans buffer empty

Bit 5 = Trans buffer not full

Bit 4 = Break detect

Bit 3 = Framing error

Bit 2 = Parity error

Bit 1 = Receive buffer overrun

Bit 0 = Data ready in receive buffer

AH=FF = Invalid parameter in request.

AL contains the modem status

Bit 7 = Received line signal detect

Bit 6 = Ring indicator

Bit 5 = Data set ready

Bit 4 = Clear to send

Bit 3 = Delta receive line signal detect

Bit 2 = Trailing edge ring indicator

Bit 1 = Delta data set ready

Bit 0 = Delta clear to send

AH=4 Close communications port. Resets the communications line and flushes

the buffers.

For all values of AH. DX = parameter indicating which communications port

to use (0 allowed).

Other than AX, all other registers returned unchanged for all calls.

Note: The following changes have been made from the BIOS interrupt 14

driver:

1: This routine is completely interrupt driven, with transmit and receive

buffers so the program does not have to wait for data.

2: Function call 0 (Initialize) will immediately return control to the

program. It is the programmer's responsibility to ensure the port is actually

ready before sending data, although data can be sent.  This allows the program

to open the port with no carrier received.

3: AH=1 will return with Bit 7 of AH set only if the transmit buffer is

full.  Otherwise, the character will be placed in the buffer and transmitted

when the port catches up with the buffer.

4: AH=4 is a new call.  It will disable the interrupts and flush the

transmit and receive buffers.  It should be called before program termination

to disable the port.

5: On all calls, the following bits in AH have changed:

Bit 6 is now Transmit buffer empty.

Bit 5 is now Transmit buffer not full.

Bit 1 is now Receive buffer overrun.

Bit 0 is now Receive buffer not empty.

These bits correspond to equivalent bits in the original code, i.e. Bit 0

was Data ready, and now indicates something in the receive buffer (data

ready).

The receive buffer overrun bit should not occur, if the receive buffer is

large enough and called frequently enough.  If it does occur, the buffer data

is still good, but data will be missing and integrity compromised. The data

will be lost starting X bytes past the current buffer, where X is the buffer

size.

The program was written in a modular form, with mainline code performing

parameter verification checks and routine selection.  All work is done in

procedures.  Some simple macros are included for commonly used functions, and

the transmit and receive buffers are defined using a structure.

Also, I like to use both upper and lower case characters for coding in

assembler.  All registers are in upper case only (AX,BX, etc.) while

everything else is capitalized (Functble, Cinit, etc.).  I feel this makes the

code more readable, but it is up to the programmer writing the code what

format he/she uses.  The only time it will make a difference is in comparing

parameters in a macro call (I.E. using the Ifidn <A>,<a>). This function is

case sensitive, and will fail in the example above.

Since we are working with mostly bytes and port address, this routine uses

.Radix 16.  It could have also been written with the default of .Radix 10, but

then many values would have had to have an 'h' (hexadecimal) notation

following.  As it stands, the only values which need the 'h' appended are

those ending in a 'b' or 'd', which would otherwise signify binary or decimal

representation, respectively.

There are two main procedures in the code.  Int14 is the interrupt 14

handler, and is the user interface to the program.  It replaces the existing

Int14 handler in BIOS and DOS, and for the most part contains the same

functions as the original handler.  (See Async.Doc for specific information).

The other main procedure is Int0C, which is the machine's interface.  It

uses the machine's hardware interrupt 0C from the communications port to drive

it.  When status changes (i.e. character received, modem dropped ready, etc.)

the Int0C code is driven to handle the resultant hardware interrupt.  This

allows the program to update the buffers and/or status indicators when they

occur, and relieves the program of the need to check except when it is ready

to receive the data.

The macros used in the program are defined at the beginning.  These macros

are used in place of coding common code several times in the program, saving

time and the possibility of "finger checks".  They also make reading the code

and debugging easier.  The macros will expand in the listing of the code, so

you can see exactly what is generated for later debugging.

Liberal use of Subttl and Page pseudo-ops have been used to make output

formatting logical and easier to read.  I have found the time saved trying to

find an area of code is much worth the few pages of extra paper used in a

large program.  Also, it starts a procedure at the top of the page instead of

in the middle, making it easier to find later.

Unless absolutely necessary, all jumps should be in a forward direction. If

a jump must be made backwards (occasionally necessary for looping), it should

be as short a distance as possible.  This will minimize the possibility of

loops which cannot be exited, and the resultant reboot which will be

necessary.

One other note: It is possible to program in a structured fashion in MASM,

if procedures are used effectively.  The use of a procedure does not mean it

is called from multiple places in the program.  Rather, it should be a logical

and complete unit or work - that is, a specific function should be desired

upon entry, and that function completed at the end.  This modularity can allow

some common routines to be used in other programs, and definitely makes the

code easier to write and debug.

Operation of the code follows:

Upon entry, the program does a JMP to the initialization code at the end.

This code is placed at the end as it is used once (when invoked from DOS) and

not needed after that.  If placed at the end, its storage can be reused at a

later time by other programs.

At label START, we first initialize the buffers pointers.  This is one

fault with using structures - you can't set an offset from the start of the

program.  Using the pseudo-op '$' (instruction counter) just gives you the

offset from the beginning of the structure. So we have to initialize the

pointers.  After initializing the pointers, we set the interrupt 0C and 14

vectors and terminate.  However, the Int 27 (used for DOS 1.x compatibility)

allows the storage to remain allocated, and the program resident in memory.

Once the program is installed, it is accessed through software through

interrupt 14.  The hardware transfers control to procedure Int14, which

verifies the parameters and calls the correct handling routine.  When control

is returned to this routine, it restores the registers saved on entry and

returns control to the caller through the Iret (Interrupt return) statement.

When control is passed to Int14, it will select a routine to call based on

the value in AH (request code).  It moves the value to BX, multiplies by 2 and

uses this as an offset into a function call table.  The use of a table like

this eliminates the need for checking every possible value of for the request,

and returns control to one common place in the code.  It also makes it easier

to add routines.  All you have to do is add your new routine and a pointer to

it in the table.  The assembler will take care of everything else.

AH = 0.  Copen is called which must be made before any other call.  It will

initialize the system and asynch interrupt regs, flush the buffers of any

residual data, and set the initial status.  Note that the system interrupt

register (port 21) must be enabled BEFORE the asynch board, or you will get a

lockout condition where you never get the interrupt processed.  (This one cost

me four days of debugging!).

AH = 1.  Csend will take the character in AL and place it in the transmit

buffer.  If the buffer is full, it returns with the timeout bit set so the

program can resend at a later time.  If the buffer is not full, it checks to

see if transmit interrupts are enabled.  If they are, it returns with the

status in AH.  If not, it enables the transmit interrupt so the interrupt 0C

routine can send the character.

AH = 2. Crcve will return a character from the buffer in AL and the status

in AH.  If a character is not available, 0FF (-1) will be returned in AH so

the program can check later.

AH = 3. Cstat returns the current status in AH and modem status in AL.

AH = 4. Cclos will disable interrupts from the asynch port and system and

flush the buffers.  It must be the last function called when the program is

through with the port.  Also note that the asynch port must be disabled BEFORE

the system, or an incoming interrupt may cause problems next time the

interrupts are enabled.

The hardware interrupt processing procedure, Int0C, gets control whenever

the communications port interrupts the system.  It runs asynchronously (pardon

the pun) with the rest of the system, and is completely hardware controlled.

The main routine resets the hardware controller at port 20 by putting an x'20'

to it, determines what kind of interrupt is pending, and calls the appropriate

routine.  This loop will continue until all outstanding interrupts are

handled.  This code also uses a branch table and is the only place we have a

backward jump in the code.  The correct routine is called, and the loop

continues until the interrupt id register contains 01 (no interrupts pending).

Procedure Modemst handles interrupt code 00 (modem status change).  This

reads the modem status port and sets it into Mstatus for use by the program.

Procedure Xmithrg handles interrupt code 02 (transmitter holding register

empty).  If a byte is available in the transmit buffer, it is placed in the

transmit register.  If not, the transmit holding reg empty interrupt is

disabled.

Procedure Rdatint handles interrupt code 04 (receive data available).  This

is probably the most important interrupt to handles, as if this is not handled

quickly enough, another character will overlay the one currently in the

register and data will be lost.  If the receive buffer is full, the character

is discarded and the overrun bit set in Lstatus.

Procedure Rcvrlst handles interrupt code 06 (receive line status).  All it

has to do is read the line status register and set it into Lstatus.

This is a quick overview of the major blocks of the program.  This is a

simple program with many changes which can be made.  For example, the routine

could have XON/XOFF functions built in, the modem and line status could be

enhanced, and other changes made.  However, this is a good base to start for

someone who wants to play with the communications port interrupts.  Any simple

routine which will pass characters from the keyboard to the Int 14 handler,

and from the Int 14 handler to the display screen will work nicely to test

this program.  This can be done in 50 lines or so of assembler code, and works

nicely as a test for the program.

===============================================================================

Changes in this upload:

1. The extra CLI at the beginning of the COPEN routine has been removed.

2. Extra code in the CSEND routine has been removed.

3. The line status code has been reworked to operate properly (and more

efficiently).

4. The only operational change was to set the high order bit (80h) if a

character was received with a parity error.  As the parity error cannot

occur if running with 8 data bits, this will have no effect on binary

transfers of data.

#################################################################################################