BBS Modem Initializer

By Steven R. Sterling

Archive inventory:   SETMODEM.COM - Program file

MODEM.DAT    - User defined init. string file

SETMODEM.PAS - Turbo Pascal 3.1 source

SETMODEM.DOC - This doc.

This program will allow you to send any Hayes command up to 8 characters

long (for example, ATS16=1O, the = counts as a character) to your Hayes/

compatible modem. It was written and timed for an 8mhz IBM AT using a

Hayes Smartmodem 1200, it should work satisfactorily on any compatible.

Although originally intended as a modem initializer for the Collie 1.20

BBS, it may be used as a convenient way to send modem commands without

starting up a terminal program. I use it to send the offhook command when

I am doing board maintenance.

COLLIE 1.20 users who have had no success in getting their board to

handle the modem properly (i.e. ANSWER THE PHONE!) will find this a useful

addition to their C.BAT file. SETMODEM should be inserted into the batch

file just before :LOOP, as you only need to run SETMODEM once.

Also before you use SETMODEM with Collie 1.20 you must modify COLLIE.COM

with a sector editor (Norton Utilities, PC-Tools etc.).

To modify COLLIE.COM....

1) MAKE A BACK-UP OF COLLIE.COM!

2) Load a sector editor and view relative sector 68.

3) Change the string 'ATZ' to 'ATH'.

If you have followed these instructions correctly you will have a

COLLIE.COM file that will not wipe out the initialization strings that

SETMODEM sends.

The only remaining task is to determine what initialization strings you

are going to need to send to the modem. Generally the ones that you had

in your MODMINIT.BBS are satisfactory. All you have to do is rename

MODMINIT.BBS to MODEM.DAT. The file structure is the same.

Please address any comments/suggestions to:

Steve Sterling

561 Nottingham Dr.

Yardley, Pa. 19067

SYSOP, THE ABSTRACTION - Collie 804/3

GEnie Mail - [S.STERLING]