Current Video and Printer Drivers

=================================

VIDEO DRIVERS: as of May, 1991

=============

This is a current listing of the video drivers included with CompuShow. Modes

in (parentheses) are simulated.  Note that some modes may not be available on

your video adapter. For example:

-  16 color 1024x768 requires a 512K VGA,

- 256 color  640x480 requires a 512K VGA,

- 256 color 1024x768 reguires a   1M VGA, and

- 256 color modes on an 8514/A adapter require the memory expansion.

Also, VGA adapters  that are built-in  to a  system board or  bundled with  a

computer (unless you're getting a specific name-brand board), are  frequently

low-end (cheap) adapters, and probably don't have "Super-VGA" capabilities.

Driver                          Notes

=====================================================================

A1=ATI VGA Wonder

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

(640x480x256), 800x600x256

A2=ATI Graphics Solution

320x200x16, 640x200x16

A3=Ahead Systems

800x600x16, 1024x768x16, 640x400x256, 640x480x256, (640x480x256),

800x600x256, 1024x768x256

A4=Amstrad PC1512

(320x200x16), 640x200x16

A5=ATI Vga Wonder Plus

800x600x16, 1024x768x16, 640x400x256, 640x480x256, (640x480x256),

800x600x256, 1024x768x256

C1=Chips & Technologies         Cardinal

800x600x16, (320x400x256), 640x400x256, 640x480x256, (640x480x256),

800x600x256

C2=Compaq VGA

640x480x256

D1=DGIS

(Available modes depend on the video adapter and DGIS interface)

E1=Everex VGAs

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

G1=Genoa 6400

800x600x16, 1024x768x16, 512x512x256, 640x350x256, 640x400x256,

640x480x256, 800x600x256

[DRIVERS.DOC] Page 1

H1=Hercules Graphics Station

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 1024x768x256,

640x480x32k, 512x480x16m

[ The first 1024x768x256 (mode "V") is interlaced,

the second (mode "W") is non-interlaced ]

H2=Hercules InColor

720x348x16, (360x174x16)

I1=Generic CGA                  (any CGA)

160x100x16, (320x200x16)

I3=IBM PGC                      IBM Professional Graphics Controller,

Vermont Microsystems

(320x200x256), (320x240x256), (320x400x256), (320x480x256),

(640x400x256), 640x480x256

I4=IBM 8514/A                   Single monitor

I5=IBM 8514/A                   8514+VGA monitor

640x480x16, 1024x768x16, 640x480x256, 1024x768x256

I6=Generic EGA

(320x350x16)

I7=IBM XGA

1024x768x16, 640x480x256, 1024x768x256, 640x480x64k

N1=NCR VGA

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256

O1=Oak Technologies

800x600x16, 1024x768x16, (320x480x256), 640x480x256, 800x600x256

P1=Paradise                     AST, Compaq, Dell

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

(640x480x256), 800x600x256

P2=Plantronics

320x200x16

S1=STB EM16+                    This driver is identical to T4, except for

one "non-standard" mode number

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

S2=STB Ergo + Sierra DAC

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 320x200x32k,

640x480x32k, 800x600x32k

T1=Tseng 3000                   Genoa, STB, Orchid

800x600x16, 1024x768x16, (320x350x256), (320x480x256), 640x350x256,

640x480x256, 800x600x256

T2=Tandy 1000SL/TL              "ETGA"

640x200x16, (320x200x16)

[DRIVERS.DOC] Page 2

T3=Trident                      Logix, ZyMOS

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

(640x480x256), 800x600x256

T4=Tseng 4000 series            Orchid ProDesigner II

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

T5=Trident 8900 series

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

T6=Tseng 4000 series VGAs with Sierra DAC

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 640x400x32k,

640x480x32k, 800x600x32k

V1=Video 7

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

V2=VBE (VESA Bios Extensions)

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256,

1024x768x256, 1280x1024x256

[Available modes depend BOTH on the capabilities of the video adapter

and the VESA Bios Extensions]

[DRIVERS.DOC] Page 3

Printer Drivers: as of May, 1991

----------------

The following printer drivers are included with the enhanced program:

(monochrome drivers are listed first, followed by color drivers)

=============================================================================

IBMGRAPH.DRV This is "vanilla", the closest thing to an actual Graphics

Printer "standard". It will work with virtually all IBM/Epson

compatible printers.

Initialization: esc,'U',1             Turns uni-directional print on.

Line End:       cr,esc,'J',24         Carriage return, line feed 24/216"

Page End:       ff,esc,'U',0          Form-feed, uni-directional off.

Single Density: esc,'K',n1,n2         Prints at  60 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

GENERIC.DRV  An IBM Graphics Printer Driver plus Quad-Density.

Quad Density:   esc,'Z',n1,n2         Prints at 240 dots/inch.

=============================================================================

GENERIC2.DRV Identical to the Generic driver, except that it does a 1/9" line

feed with "esc,'J',20", for printers (such as the Panasonic KX-

P1124) which express line feeds in 1/180" increments.

Line End:       cr,esc,'J',20         Carriage return, line feed 20/180"

=============================================================================

CITIZEN.DRV  A driver for Citizen MSP-10,15,20,25 printers.

Initialization: esc,'@',esc,'U',1     Reset, uni-directional print on.

Line End:       cr,esc,'J',24         Carriage return, line feed 24/216"

Page End:       ff,esc,'@'            Form-feed, reset.

Single Density: esc,'*',0,n1,n2       Prints at  60 dots/inch.

Double Density: esc,'*',1,n1,n2       Prints at 120 dots/inch.

One-to-one      esc,'*',5,n1,n2       Prints at  72 dots/inch.

Two-to-one      esc,'*',7,n1,n2       Prints at 144 dots/inch.

Hi-Res CRT      esc,'*',6,n1,n2       Prints at  90 dots/inch.

=============================================================================

HPLASER1.DRV Hewlett Packard Laser printer at 75 dpi resolution.

Initialization: esc,'*t75R',esc,'*r0A'  Sets 75 dpi and left margin.

Line End:                                  (none)

Page End:       esc,'*rB',ff            End graphics, form-feed.

Line start:     esc,'*bnnnnW'           Print nnnn bytes

=============================================================================

HPLASER2.DRV Hewlett Packard Laser printer at 150 or 300 dpi resolution.

Initialization: esc,'*t150R',esc,'*r0A' (^P) Sets 150 dpi and left margin.

(or)         esc,'*t300R',esc,'*r0A'  (P) Sets 300 dpi and left margin.

=============================================================================

HPTHINK.DRV The Hewlett Packard ThinkJet.

Initialization: esc,'A',8               Sets line feed to 8 dots

Line End:       cr,lf                   Simple carriage-return/line-feed

Page End:       ff,esc,'@'              Form-feed, software reset

Single Density: esc,'K',n1,n2           Prints at  96 dots/inch.

Double Density: esc,'L',n1,n2           Prints at 192 dots/inch.

[DRIVERS.DOC] Page 4

=============================================================================

IBMCOMPC.DRV The IBM Compact Printer. At single-density (70 dpi), MacPaint

must be truncated at 560 dots (8" wide).

Initialization: esc,'0'               Set 1/9" line-spacing.

Line End:       cr,lf                 Carriage return, line feed.

Page End:       ff,esc,'2'            Form-feed, set 1/6" line spacing.

Single Density: esc,'<',esc,'L',n1,n2 Home the head, prints at 70 dots/inch.

=============================================================================

NEC8023.DRV  May also work with the C.Itoh Prowriter. Fast and dark print

modes provide a choice of print sizes, with no difference in

"darkness".

Initialization: esc,']',esc,'T17',cr  Set L/S, 17/144" line spacing.

Line End:       cr,lf                 Carriage return, line feed.

Page End:       ff,esc,'T24'          Form-feed, set 1/6" line spacing.

Bit-Image:      esc,'S',a1,a2,a3,a4   Prints at 80 dots/inch.

=============================================================================

ROLAND.DRV   The Roland 1012 is identical to the Generic driver, with the

addition of a one-to-one (72 dpi) mode for fast print of

MacPaints.

One-To-One:     esc,'*',5,n1,n2       Prints at  72 dots/inch.

=============================================================================

TSHBP321.DRV A very complicated driver for the Toshiba P321 24 pin printer.

In order to correctly size the graphics print, five rows of

eight pixels are printed as four rows of 24 pins.

Initialization: esc,'>',esc,'L07      Uni-Directional, set 7/18" line spacing

Line End:       cr,lf                 Carriage return, Line feed

Page End:       ff,esc,26,'I'         Form-feed, reset

Single Density: esc,';',a1,a2,a3,a4   Prints at 180 dots/inch

Double Density: esc,29,a1,a2,a3,a4    Prints at 360 dots/inch

=============================================================================

TSHBP351.DRV Toshiba P351, set for Qume emulation. Identical to IBMGRAPH,

except that it turns Qume emulation off in Initialization, and

on at Page End.

Initialization: esc,'_1',esc,'U1'     Turn Qume off, Uni-Directional on

Page End:       ff,esc,'U0',esc,'_0'  Form-feed, Uni-Dir off, Qume on

=============================================================================

OKI92.DRV    The Okidata 92/93 printer. Supports 60 and 72 dpi graphics. A

very complicated driver, as it only prints 7 dots at a time.

Line End:       3,14                  graphics line feed

Page End:       3,2,ff                APA graphics off, form-feed

60 dpi:         30,3                  Set 10 pitch, APA graphics on

72 dpi:         28,3                  Set 12 pitch, APA graphics on

[DRIVERS.DOC] Page 5

Color Printer Drivers

=============================================================================

IBMCOLOR.DRV The IBM Color Printer.

Initialization: esc,'3',14,esc,'U',1  Set 14/144 line spacing, uni-dir on.

Line End:       cr,lf                 carriage return, line feed

Page End:       esc,'U',0,esc,'2',ff  uni-dir off, set 6 lpi, form-feed.

Yellow:         esc,'y'

Cyan:           esc,'c'

Magenta:        esc,'m'

Black:          esc,'b'

Double Density: esc,'L',n1,n2         Prints at 168 dots/inch.

=============================================================================

COLOR8A.DRV  "Generic" 8 pin color printer. Tested on a Star NX-1000 Rainbow

and Citizen MSP-50. Similar printers/emulations: EX-800, LX-800,

FX-86e, LX-86, LX-80.

Initialization: esc,'3',24,esc,'U1'    Set 24/216 line spacing, uni-dir on

Line End:       cr,lf                  carriage return, line feed

Page End:       ff,esc,'@'             form-feed, software reset

Yellow:         esc,'r',4

Cyan:           esc,'r',2

Magenta:        esc,'r',1

Black:          esc,'r',0

Quad density:   esc,'*',3,n1,n2        Prints at 240 dots/inch.

=============================================================================

COLOR8B.DRV  Identical to COLOR8A, except for line spacing. Similar

printers/emulations: JX-80, LQ-1500, ALPS ALQ200

Initialization: esc,'3',20,esc,'U1'    Set 20/180 line spacing, uni-dir on

=============================================================================

COLOR24.DRV  "Generic" 24 pin color printer. Tested on Okidata 393C. Similar

printers/emulations: LQ-2550.

Initialization: esc,'3',20,esc,'U1'    Set 20/180 line spacing, uni-dir on

Line End:       cr,lf                  carriage return, line feed

Page End:       ff,esc,'@'             form-feed, software reset

Yellow:         esc,'r',4

Cyan:           esc,'r',2

Magenta:        esc,'r',1

Black:          esc,'r',0

Quad density:   esc,'*',39,n1,n2       Prints at 240 dots/inch.

=============================================================================

OKI290.DRV   For Okidata 290 series color printers. Tested on a 292.

Initialization: esc,'#A',esc,'A',8,    Set 8/72 line spacing,

esc,'2',esc,'U',1        uni-dir on

Line End:       cr,lf                  carriage return, line feed

Page End:       ff,esc,'U',0,          form-feed, uni-dir off,

esc,'A',12,esc,'2'       reset 6 lpi

Yellow:         esc,'r',4

Cyan:           esc,'r',2

Magenta:        esc,'r',1

Black:          esc,'r',0

Double density: esc,'L',n1,n2          Prints at 120 dots/inch.

[DRIVERS.DOC] Page 6