Common Problems:

When using the B command to send a file to the TNC (for instance, sending

TNC1.SET), the responses back from the TNC are just buffered up.  If you dont

then use the T command to go into terminal mode, but do an A to start the

MailBox, the program sees one of the first "cmd:" prompts that was buffered

up, figures that it is the response to the "conok on , mon on" series of

commands that it is sending and proceeds to wait for a connect message.  While

it is waiting, the input is the string of "cmd: XXXXX was ON..:" prompts.  The

final one "cmd:" is not followed by a return so the input routine thinks it is

getting a line from the TNC and waits for the balance of the line and a C/R.

When it finally does get something (*** CONNECTED for instance) it doesnt

catch the connect message due to the "cmd:" tacked on the front.  As long as

you go into Terminal mode after sending the file, then to auto-answer there

is no problem.

If you experience ERROR F3 during startup, you will need to add the statement

FILES=20 in your CONFIG.SYS file (See your DOS manual for details).

I dont recommend running with limited memory (less than 256K), but understand

it has been done.  If you run into a heap/stack collision due to limited memory

you might try setting the maximum number of callsigns saved in CALLS.TNC way

down, and be sure not to try to load any other resident software (Sidekick and

the like.)

Setting up file paths for Mail and Files:

If you are having problems understanding how to set up the paths for

the Mail and File subdirectories, I'm sorry I cant explain it better.

If you are running on a hard disk you probably know what I mean.  If

you are running on floppies I would suggest you format a disk and

create one subdirectory on it called files (MD FILES).  Then if you

set the path name for mail to B: instead of MAIL\ and the path for

files to B:\FILES\ your mail messages will be stored in the main

directory on drive B and your downloadable files in the FILES sub-

directory also on B.  If anyone running on floppies has a more

convenient setup or can explain this better, please do so!  I only

have one floppy drive so I cant even test and see if I explained the

above correctly.  (It really is possible to get this stuff running, in

spite of trying to figure out what I'm saying!)

If you want people to be able to download files such as LOG.TNC,

FWD.TNC just place then in the downloadable file area and specify

the entire path name for them in BBSxxx.CNF

for example:

...

WA7MBL        (Owner call sign)

Jeff          (Owner name)

FILES\CALLS.TNC     (List of monitored callsigns)

FILES\FWD.TNC       (forwarding file)

FILES\LOG.TNC       (file for logging)

FILES\MON.TNC       (Save file for J lists)

FILES\        (path/drive for files)

...