FM Software 12/90 update -- Implementation notes

================================================

Changes have been made to all of the FM Software packages since

the June 1990 release.  Note especially that extensive MIDI

driver modifications have resulted in new command line options.

If you used any command line options in previous releases, be

sure to check the "Command Line Changes" section below for new

formats before running any of the revised programs.

This document covers all available packages and some sections may

not apply to the distribution you have received.

New MIDI Drivers

----------------

The MIDI device interface code has been completely rewritten for

this release.  Support has been added for the Key Electronics

MIDIator and the Creative Labs SOUND BLASTER (with MIDI adapter).

A universal MIDI driver is employed which allows each program to

operate with any of the supported interfaces.  Default device

selection can be overridden on the command line for systems using

multiple interfaces.  All I/O is now interrupt-driven by default,

rather than via polled access (the previous default).

New Programs

------------

* Play/R -- Memory resident MIDI file player.

See PLAYR.DOC for details if this program has been included

with your distribution.

* FM2MDX -- FM-Edit to MIDIEX bank converter.

* Play/B -- Batch Play version (non-interactive).

Replaces Play15.

Help - All interactive programs

-------------------------------

* String search speed has been improved.

* Search words can now be selected from the text using a mouse.

Watch program

-------------

* The Alt-0 send key has been modified to ask for the send string

each time it is pressed.  This permits quicker entry of strings

while on the Monitor screen.

* The available buffer memory has been increased to approx. 80K

bytes.  The size of MIDI files loaded is now only limited by

the amount of free memory.

Play program

------------

* Play list support has been added for continuous, automatic

play.

* Function keys and mouse selectors have been altered due to new

functions associated with play lists.  Check the help screen

for the new definitions.

* Files are now sorted alphabetically in the file list display,

and play lists can be created by selecting files from that

display.

* There is now a meter display mode available (Alt-F6) which

shows song length and position in measures and beats.  Time

signature is also shown in this mode.  The time display is now

more consistent when advancing.

* A delay between songs in a play list can be specified using the

-w option on the command line.

* MIDI file size is now only limited by the amount of available

memory.

File extensions

---------------

* Programs which read MIDI files will now check for any of the

following extensions if none is specified: .mff, .mid, .std,

.smf.

* The default extension (.mff) for wildcard directories searches

can be changed via the mff environment variable.

Ex: set mff=mid

Command Line Changes

--------------------

All programs which do MIDI input/output now use the "-d" option

to select the active device (if more than one is present):

-dmfc      selects IBM Music Feature

-dmpu      selects MPU-401 compatible

-dmidiator selects MIDIator

-dsbmidi   selects Sound Blaster MIDI

(The first 3 letters of the device name are sufficient.)

Optional IRQ and I/O port address information can be added

following the device id if necessary:

-dxxx:<irq>:<ioaddr>

For example, to define an MPU-compatible interface using IRQ

5 and I/O address 332 (hex), use:

-dMPU:5:332

If only the I/O address is required but the default IRQ is

alright, use -dMPU::332.

Other new options include "-i" which will try to determine the

IRQ in use automatically (MPU and MFC only), "-v" which lists

the type of interface actually selected before starting, and

"-s" which doubles the size of the MIDI input buffer.

Command line options (those beginning with "-") may be set

permanently by including DOS environment variable definitions in

your autoexec.bat file.  Use the variables opt1, opt2, opt3, and

so on.  Be to include the "-" character before the option letter.

For example, "set opt1=-dmpu:5:332".  This has the same effect as

including the -d option described above on each command.

Device restrictions

-------------------

MIDIator

With the current drivers, the MIDIator is not automatically

detected as are the other device types.  The -dmid option must

always be specified on any of the program command lines to

utilize this device.  This can be set by default by defining an

environment variable before use (usually in the autoexec.bat

file):

set opt1=-dmid

SOUND BLASTER

Technically, the SOUND BLASTER uses a half-duplex MIDI

implementation, meaning that it cannot simultaneously send and

receive MIDI data.  The MIDI driver "simulates" full-duplex

operation by rapidly switching between input and output modes.

This is not 100% effective, however, and some input data may

be lost during "thru" operations.  In particular, the

thru/patchbay operations in the Watch program may not prove

useful, depending on the amount of data being received.  Thru

operations are disabled entirely if not using interrupt-driven

mode (i.e., -dsbm:0 is specified on the command line indicating

no interrupt).  Because of the i/o restriction, the FM-Tune

program will also not operate correctly and so is not supported

for use with the SOUND BLASTER.

Questions

---------

Questions or problems regarding this software should be addressed to:

Kevin Weiner

FM Software

1413 Spring St.

Bethlehem, PA 18018

(215) 865-7956

CompuServe: 72416,2465

BIX or PAN: KWEINER