Product: Port Finder               mcTRONic Systems

Date: September 20, 1990        7426 Cornwall Bridge Ln.

Document: Product Information       Houston, TX  77041-1709

Version: PF.COM R2.53              (713) 462-7687

PF.SYS R1.11              Hours: 5-10 PM M-F CST

Port  Finder is a COM and LPT port utility program.   Its

primary  purpose is to activate COM3 or 4 on computers  that

have  the  hardware and also use PC or MS DOS  3.3,  4.0  or

higher.   In  addition to COM port activation,  Port  Finder

allows  all possible COM to COM and LPT to LPT  port  swaps.

Port swapping allows use of COM3 and 4 in programs that only

permit  entry of COM1 or 2.  In addition, both COM  and  LPT

port swapping is useful for programs that are  preconfigured

to  use a port other than the one you wish to use.  You  may

find that it is easier to swap ports instead of  reconfigur-

ing your program or swapping cables.

Port  Finder  comes in two forms.  The  first  is  PF.COM

which performs all the aforementioned functions.  PF.COM  is

usually  added to your AUTOEXEC.BAT file and run  each  time

you  startup your computer.  PF.COM can also be run  at  any

time from the DOS prompt or used in batch files.  The second

form is PF.SYS which is a device driver and must be run from

your  CONFIG.SYS file on startup only.  PF.SYS  allows  only

COM  to  COM port swaps and is intended for use  with  other

device  drivers that might use COM3 or 4 directly  or  indi-

rectly after a port swap.

In  addition to activating ports and port swapping,  both

PF.COM and PF.SYS will display actual COM port addresses  in

HEX. Also PF.COM will display the LPT port addresses in HEX,

your computer type, BIOS date, DOS version, display  adapter

type,  total  base and extended memory, and  total  expanded

memory  with  EMS version.  All of this information  can  be

very helpful when setting up a computer or troubleshooting a

port or other problems.

For  your information, any program that bypasses DOS  and

directly addresses the COM or LPT ports will not be affected

in  any way by Port Finder.  Many modem programs bypass  DOS

in  order  to achieve higher port speeds or the  ability  to

multitask both COM and LPT ports for simultaneous use.  This

(bypass DOS) is not a problem with such a program as PROCOMM

Plus  as  it works with both COM3 and 4.   There  are  other

programs  which do not permit entry of COM3 and 4,  who  di-

rectly  address  the hardware, and will not work  with  Port

Finder.  At least with Port Finder, your computer will  know

that  COM3 or 4 is present as there will be many  more  pro-

grams  that will work properly with COM3 or 4 after  running

Port Finder.

Port Finder has gained some fame after being  recommended

in  two different magazine articles.  The first was  in  the

Advisor  column  of PC Magazine on October  17,  1989.   The

second in the July 1990 issue of Vulcan's Computer  Monthly.

The article in PC Magazine is very significant because it is

most  likely  the number one computer  magazine  sold.   The

article  in Vulcan's Computer Monthly was the first to  rank

Port  Finder as the most useful Shareware utility out of  10

reviewed.   Many thanks to these two publications for  their

choice of Port Finder.

USAGE:

Neither PF.COM nor PF.SYS require any runtime  parameters

to  display  COM  port addresses or to activate  COM3  or  4

ports.   PF.COM  will also automatically  display  LPT  port

addresses  as well as other system information.  DOS 3.3  or

4.0  only  support access to a maximum of 4 COM  and  3  LPT

ports (though your BIOS does allocate a location for LPT4).

The PF.COM runtime parameters in an AUTOEXEC.BAT file are:

Syntax: [d:\path\]pf [parm1] [parm2]

Do  not include the brackets "[" or "]" which  indicate  op-

tional information.  d:\path\ = optional drive and path.

[parm1] and/or [parm2] = optional parameters

H   = Display help screen        C23 = Swap COM ports 2 & 3

RC  = Reset COM port addressing  C24 = Swap COM ports 2 & 4

RL  = Reset LPT port addressing  C34 = Swap COM ports 3 & 4

C12 = Swap COM ports 1 & 2       L12 = Swap LPT ports 1 & 2

C13 = Swap COM ports 1 & 3       L13 = Swap LPT ports 1 & 3

C14 = Swap COM ports 1 & 4       L23 = Swap LPT ports 2 & 3

The PF.SYS runtime parameters in a CONFIG.SYS file are:

Syntax: device=[d:\path\]pf.sys [parm1] [parm2]

Do  not include the brackets "[" or "]" which  indicate  op-

tional information.  d:\path\ = optional drive and path.

[parm1] and/or [parm2] = optional parameters

C12 = Swap COM ports 1 & 2       C23 = Swap COM ports 2 & 3

C13 = Swap COM ports 1 & 3       C24 = Swap COM ports 2 & 4

C14 = Swap COM ports 1 & 4       C34 = Swap COM ports 3 & 4

NOTES:

All  parameters can be in upper or lower case.   The  COM

and  LPT port addresses displayed always indicate the  final

location  after  all port activations, port  swaps  or  port

resets have been executed.  Enter all port numbers with  low

first,  then high.  If you become confused as to which  port

address  belongs  where, just use the RC or RL  commands  to

restore the ports to their correct locations.

Port Finder performs a checksum when it loads.  If it  is

not  correct, the program will terminate with an error  mes-

sage.   If  this happens to you stop using the  version  you

have. Request a new copy of the program if you purchased  it

from  mcTRONic Systems or download a new copy from the  RBBS

you got it from.  I would like to hear about any such  prob-

lems.

In  addition  to the other features mentioned,  when  you

load either PF.COM or PF.SYS, Port Finder will determine  if

you  have a joystick port and add that information  to  your

equipment list.  When IBM first came out with the PC and its

BIOS,  the  BIOS would do this for you.  When IBM  came  out

with  the AT BIOS they left out this feature.  Whether  your

joystick  is  added to your equipment list or not  does  not

seem  to  make any difference in your ability  to  use  that

port.  The only problem is that there are many programs that

list  your equipment as found by your BIOS and if  the  joy-

stick presence is not included, these programs will say  you

do  not  have a joystick, even when you do.   I  don't  know

about  you,  but if a program does indicate  a  joystick  is

present or not, I would like it to say YES if I do have one.

TECHNICAL DATA:

Your  computer  BIOS assigns logical COM  and  LPT  ports

numbers,  from low too high, to all responding I/O ports  in

order  as  they are found, and non-responding I/O  port  ad-

dresses are skipped.  Thus if you had but one COM port,  and

it was the address for COM2, your computer BIOS would assign

it  to COM1 because no real COM1 address was present.   Port

Finder, which is used as an extension of your computer BIOS,

also  activates  COM3 and 4 ports in the  same  manner.   In

fact, if you only had a COM3 or 4 port in your system,  Port

Finder would assign these ports to COM1 and 2.  In addition,

Port  Finder  will activate up to 4 COM port  addresses,  if

your  system  had four ports and two of them  were  not  the

normal COM1 and 2 addresses.

COM PORT ADDRESSES:

MACHINE     COMx       I/O      INT      MEM   ACT PORT

ALL         COM1     3F8  HEX   IRQ4    40:00     NO

ALL         COM2     2F8  HEX   IRQ3    40:02     NO

PC/XT/AT    COM3     3E8  HEX   IRQ4    40:04     YES

PC/XT/AT    COM4     2E8  HEX   IRQ3    40:06     YES

PC/XT/AT    COM3     3E0  HEX   IRQ4    40:04     YES

PC/XT/AT    COM4     2E0  HEX   IRQ3    40:06     YES

PC/XT/AT    COM3     338  HEX   IRQ4    40:04     YES

PC/XT/AT    COM4     238  HEX   IRQ3    40:06     YES

PS/2        COM3     3220 HEX   IRQ3    40:04     YES

PS/2        COM4     3228 HEX   IRQ3    40:06     YES

PS/2        COM5     4220 HEX   IRQ3    NONE      NO

PS/2        COM6     4228 HEX   IRQ3    NONE      NO

PS/2        COM7     5220 HEX   IRQ3    NONE      NO

PS/2        COM8     5228 HEX   IRQ3    NONE      NO

SERIAL PORT COUNT IS AT MEM LOCATION 40:11 BITS 2, 3 & 4

LPT PORT ADDRESSES:

LPTx   MEM     I/O    COMMENT

LPT1  40:08  3BC HEX  MAY BE INCLUDED ON MDA OR OTHER CARD

LPT2  40:0A  378 HEX  MAY BE ADDRESSED AS LPT1 ON PC/XT/AT

LPT3  40:0C  278 HEX  MAY BE ADDRESSED AS LPT2 ON PC/XT/AT

BATCH FILE ERRORLEVEL:

The  Port  Finder program PF.COM can be used in  a  batch

file  to  determine the COM ports that  are  present.   Port

Finder exits with an errorlevel number which corresponds  to

the ports active in your system.  All possible  combinations

are covered.

ERRORLEVEL 0 - NO COM PORTS  ERRORLEVEL 8  - COM4

ERRORLEVEL 1 - COM1          ERRORLEVEL 9  - COM1,4

ERRORLEVEL 2 - COM2          ERRORLEVEL 10 - COM2,4

ERRORLEVEL 3 - COM1,2        ERRORLEVEL 11 - COM1,2,4

ERRORLEVEL 4 - COM3          ERRORLEVEL 12 - COM3,4

ERRORLEVEL 5 - COM1,3        ERRORLEVEL 13 - COM1,3,4

ERRORLEVEL 6 - COM2,3        ERRORLEVEL 14 - COM2,3,4

ERRORLEVEL 7 - COM1,2,3      ERRORLEVEL 15 - COM1,2,3,4

COMMTEST.BAT IS AN EXAMPLE COM PORT ERRORLEVEL BATCH FILE

INSTALLATION:

Installation of Port Finder on your Hard Disk is easy us-

ing supplied INSTALL program.   If you are  unfamiliar  with

ASCII files and how to edit or create them, you should  pur-

chase a book about DOS commands and file formats.  The  file

PF.COM  is normally added to your AUTOEXEC.BAT file  towards

the  beginning, but at least before any file  that  requires

COM3  or  4.   The  file PF.SYS  should  be  added  to  your

CONFIG.SYS file before any other device driver that may need

COM3 or 4.  You do not need to load both PF.SYS and  PF.COM.

If you do not have another device driver that could use COM3

or  4 then use PF.COM in your AUTOEXEC.BAT file.  If you  do

have a device driver that can use COM3 or 4, load PF.SYS  in

your CONFIG.SYS file.

As  an  example  of how to install Port  Finder,  let  us

assume the following.  You do not need to use PF.SYS in your

CONFIG.SYS  file.  You are not sure how to modify  your  AU-

TOEXEC.BAT  file that does exist on your C: hard drive  root

directory.   You  have the Port Finder disk  and  its  files

located in the A: drive.  You would use the following  steps

to install Port Finder in your AUTOEXEC.BAT file.

A:

copy pf.com c:\

c:

cd\

copy con+autoexec.bat autoexec.new

pf

(press F6)^Z

rename autoexec.bat autoexec.old

rename autoexec.new autoexec.bat

You  are  done!   Restart your  computer  and  sit  back.

Normally Port Finder might be the second or third command in

your  AUTOEXEC.BAT file, after such commands as  @echo  off,

however  the above procedure will work if you  know  nothing

about editing your AUTOEXEC.BAT file.

ORDERING INFORMATION:

Port Finder cost just $10 plus shipping.  Shipping  costs

are $2.50 inside the U.S. and $7.50 outside the U.S. in U.S.

funds only.  If you order Port Finder from outside the  U.S.

you  must  send a money order or add another  $5  for  money

transfer.   Port  Finder  comes on 3.5 or  5.25  inch  disk,

please specify size with order.

CONSIDERATIONS ON COM PORT SWAPPING:

Port Finder allows all possible COM to COM port swapping,

however  not all possible COM port swaps will work  on  your

system.  Without regard to the following rules, any COM port

swap combination may work in your situation.  I have includ-

ed  all possible swap combinations just in case your  system

does not work like the norm.

First  of all, even though Port Finder will place  up  to

four COM ports to your disposal, the IBM PC/XT/AT  structure

was  only  designed to have two COM ports work at  the  same

time.   The  main restriction is caused by only  having  two

hardware  interrupts  available  to  all  four  COM   ports.

Through  the use of special software and/or hardware  it  is

possible  to  run up to four or more  COM  ports.   Normally

however this will not be the case.

If  you talk to other experts on COM ports you may get  a

different  opinion  of what is proper in port  swapping  and

what is not.  My view on the subject is as follows:

1.  Swap only ports that share the same interrupt.  A  mouse

driver set for  COM1 (IRQ4) will have problems if the  mouse

is on COM4 (IRQ3) which was swapped with COM1 (PF C14).   If

the mouse was on COM3 (IRQ4) and swapped with COM1 (PF  C13)

it should work fine.  Therefore swap COM1 with COM3 (PF C13)

and  COM2  with COM4 (PF C24).  This appears to  go  against

what  Lori Grunin stated in the PC Magazine article.   Swap-

ping COM1 with COM2 (PF C12) or COM3 with COM4 (PF C34)  may

still work on your system.  It depends both on the  software

used and the actual interrupts your COM ports use.

2.  If you are going to use two COM ports at one  time  they

should  be on opposite interrupts.  If you are going to  use

two ports at once the    possibilities are COM1 (IRQ4)  with

COM2 (IRQ3) (same as COM2 with COM1), COM1 (IRQ4) with  COM4

(IRQ3),  COM2 (IRQ3) with COM3 (IRQ4) and COM3  (IRQ4)  with

COM4 (IRQ3).  Divide your COM port inputs into two groups of

two.  Pare serial devices that you will not use at the  same

time  on  the same interrupt (IRQ4 with IRQ4 and  IRQ3  with

IRQ3).

MICROSOFT WINDOWS CONSIDERATIONS:

If  you  load either PF.SYS in your  CONFIG.SYS  file  or

PF.COM in your AUTOEXEC.BAT before you run Windows your  COM

ports  will be activated and Windows will use  them  without

further  entries  in the Windows INI files.  It is  best  to

have loaded PF.SYS or PF.COM before you INSTALL Windows  for

the  first time so it will fill in the .INI files  for  you.

You  can run PF.COM from within Windows and the file  PF.PIF

will allow it in a Window (The PF211.PIF file is for Windows

2.11).

However  any Port Swaps that are made will be ignored  by

Windows  if  made after Windows is loaded.  Any  port  swaps

made before Windows is loaded will be honored by Windows.  I

am  working  on a fix for Port Finder swaps while  still  in

Windows.

ADVERTISEMENT:

If  you  need  a good mail list  program  then  send  for

my  Shareware program MAILALL.   It is the best  and   least

expensive mail list program you can buy for a small  company

or personal use.  Also each copy of MAILALL includes a  copy

of PORT FINDER!

MAILALL will maintain the names of up to 20 address files

(per  address name file as you can have more than one)  each

with   4000 addresses on a hard disk or 1440  addresses  per

floppy  disk.   There are 12 data fields and you   can  sort

the   address file by any two that you wish.   MAILALL  will

print  Labels (up  to four across), Reports, and  Envelopes.

You  can  edit  or print  all functions   based  on  address

search  matches from one to  all  12 fields.   MAILALL   can

import  or  export WORDSTAR  mailmerge  data files  and  can

export  MULTIMATE ADVANTAGE II mailmerge files.  MAILALL  is

network  compatible  and supports any  Microsoft  compatible

mouse.   The user can select any printer port that he  would

like  and the program colors are also user  selectable.   It

cost   just  $27.50 shipped FIRST CLASS MAIL to  you.   Send

for  it today!

-------------------------------------------------------------

To  register your copy of Port Finder, fill out  this  form

and  send  to  mcTRONic Systems, 7426  Cornwall  Bridge  Ln.

Houston, TX 77041-1709.

Date: __/__/__    Serial Number: _________   Version: _____

Name: _____________________________________________________

Company Name: _____________________________________________

Address: __________________________________________________

City: ____________________ State: ___ Zip Code: ___________

Purchased From: ___________________________________________

City: ____________________ State: ___ Zip Code: ___________

-------------------------------------------------------------