File Descriptions:

1. GSCSI4.SYS   -- DOS Driver for 51-2/3150/3180 host adapter. This driver

handles non_bootable configuration, and support

removable media feature ( DTC TAK10/TAK20, Syquest). The

driver follows DOS standard BPB structure. The driver

support logical drive size over than 32 Mbyte, if

it run under DOS 4.00 version or above. It also fully

support the utility comes with the DOS, such as FORMAT

SYS, XCOPY, CHKDSK. It dosn't support DOS FDISK

2. GSCSI4.EXE   -- DOS Driver for 51-2/3150/3180 host adapter. This driver

handles bootable configuration, and support functions

descpibe in GSD40.SYS

3. GSDIAG.EXE  -- Provide with SCSI device Low Level Format, and Diagnostic

function.

4. GSFDISK.EXE -- SCSI disk DOS partition utility. This utility support

multiple DOS partition create function. It also support

display partition information, delete exist DOS partition,

and active partitioncan function. The partition size can

over than 32 Mbyte.

Installatin Note:

1.  Use GSDIAG.EXE to do the SCSI disk low level format.

2.  Follow the DOS  "installable device driver"  procedure and

put the following stament in CONFIG.SYS file.

DEVICE=GSCSI4.SYS

3   If install Removeable Drive as first two drive (i.e. no ST-506 attach

in system) put the following statement in AUTOEXEC.BAT file to support

removeable drive feature.

GSCSI4.EXE

Otherwise, if systen has ST-506 drive attach, or if system has no

ST-506 attach and no any DTC TAK10/HF20 attach as first two drive

( base on drive SCSI ID sequence ), you don't need execute GSCSI4.EXE

file.

4.  Use GSFDISK to create drive partitions.

5.  Use DOS FORMAT utility to hight level the logical drives.