���FINDINIT���������������������������������v1.0�Ŀ

�      FIND the debug INITialization routine      �

�    for Seagate Controllers and Host Adapters    �

����������� By R. Dimick and B. Rudock ������������

Seagate Technology, Inc. - Scotts Valley, CA

FINDINIT.EXE and FINDINIT.DOC are both Copyright Seagate 1991

Seagate Tech Support Bulletin Board (408)438-8771 [300-9600, N-8-1]

Courtesy of the Seagate Telephone Technical Support Group

Note: This software is provided AS IS, AND NO WARRANTY OF

MERCHANTABILITY OR FITNESS FOR A SPECIFIC PURPOSE IS EXPRESS OR

IMPLIED.

This software is intended to be protected against viruses by FSHIELD,

a product of McAfee Associates (BBS 408/988-4004).  Even so, all newly

acquired files should be scanned for viruses.

Seagate specifically DISCLAIMS ANY WARRANTY, EXPRESS OR IMPLIED, that

FINDINIT is free from computer viruses.

--------------------------------------------------------------------

IMPORTANT! This program, FINDINIT.EXE, helps to initiate a Low-Level

format. Low-level formatting WILL destroy all user-data on a drive.

Always keep floppy and/or tape backups of you data.

-------------------------------------------------------------------

FINDINIT

Many of the calls received to Seagate's Technical Support Department

focus on the initialization (low-level format) routines and the BIOS

memory addresses used on our Seagate Controllers and Host Adapters.

The two most common questions asked are, "What is the BIOS memory

address of MY card?", and "How do I start the low level format on this

controller?".

The reasons we need to know the BIOS memory address are:

Case #1: To troubleshoot possible memory conflicts with other

peripheral cards that have their own BIOS ROM chips, such as

VGA video, enhanced floppy, network, and other hard drive

controllers or host adapters. On Seagate Controllers and Host

Adapters, the BIOS memory address is controlled by the

physical placement or removal of jumpers on the card itself.

(Refer to your Installation Guide for charts and diagrams

detailing this information.)

or

Case #2. Need to perform an initialization (low-level) format which

requires the DOS DEBUG utility, and the command Go function

(as in "go" to a particular memory address).  This is to

begin the Seagate format routine that is stored the ROM BIOS

chip on the Seagate card (called firmware instead of software,

by the way).

FINDINIT can help with both of these cases. When executed, the program

will scan the high memory address ranges traditionally dedicated to

adapter ROM BIOS's, and indicates which Seagate Controller or Host

Adapter it finds and the BIOS memory address (Case #1). Next, it will

ask if you REALLY want to format the drive. If you answer "Y", a

second warning will appear reminding you that ALL USER DATA WILL BE

LOST! It will ask again if you REALLY want to format this drive. This

time, if yes, then you must type all three letters y-e-s. Any

misspelling of 'yes' will immediately abort the program.  When 'yes'

is given, FINDINIT will issue the appropriate DEBUG-type of command to

begin the format routine of the particular Controller or

Host Adapter (Case #2). The format routines on all Seagate Controllers

and Host Adapters are step-by-step, friendly, and menu driven.

In the event that you have two Seagate BIOS's and you want to work

with the second one (which will have to have a higher memory address),

be sure to say NO to the format question for the first drive. The

program will then continue to scan and prompt for any other Seagate

BIOS.

One of the most common questions that is asked once the routine is

started, is the answer to the question of INTERLEAVE. All Seagate hard

disc drives can support to a 1 to 1 (fastest) interleave

------- BUT! ------     not all Controllers or Host Adapters are able

support a high speed 1 to 1 interleave. Interleave is always a

conditional factor of the controller or host adapter card AND in

conjunction the computer system, not the drive.

When determining what INTERLEAVE to select, refer to the following

chart for a suggested value.

Seagate   Inter-  Record   Slot    Interleave in:

Product   face    Method   size    XT's   or AT's       Type

====================================================================

ST-01     SCSI    RLL     8-bit    3 or 4    2 or 3     Hard

ST-02     SCSI    RLL     8-bit    3 or 4    2 or 3     Hard/Floppy

ST-05X    XT      RLL     8-bit    4         n/a        Hard

ST-07A    AT      RLL    16-bit    NO BIOS, USES CMOS   Hard

ST-08A    AT      RLL    16-bit    NO BIOS, USES CMOS   Hard/Floppy

ST-10     ST412   MFM     8-bit    5         6          Hard

ST-11M    ST412   MFM     8-bit    3 or 4    5 or 6     Hard

ST-11R    ST412   RLL     8-bit    3 or 4    5 or 6     Hard

ST-21M    ST412   MFM    16-bit    n/a       1          Hard

ST-21R    ST412   RLL    16-bit    n/a       1          Hard

ST-22M    ST412   MFM    16-bit    n/a       1          Hard/Floppy

ST-22R    ST412   RLL    16-bit    n/a       1          Hard/Floppy

Note: In AT systems, set the drive type to "0 or None or Not

Installed" when using  a Seagate Controller or Host Adapter that

has a BIOS ROM.

Note: When using two controllers in the same system, they must have

different BIOS memory addresses. In an AT system with a 16-bit

controller and an 8-bit controller, the 16-bit must have the lower

address and be the boot drive.

For reasons of comparison, without FINDINIT, the

process would be as follows:

1. Open up the system and inspect the controller card to determine

what BIOS address pins are jumpered. Then determine the memory address

from the pin chart in the install guide. The traditional default

memory address is C800 with the associated BIOS Address jumpers open on

the ABCD pins.

2. Put everything back together, make sure all the cards are seated

firmly in the motherboard slots, make sure that the cables are

attached correctly with pin-1 to pin-1.

3. Power on the system and boot to a floppy A:>

4. Find the DOS DEBUG utility on one of your DOS diskettes and run

DEBUG.

5. At the DEBUG "-" hyphen prompt (that's its user-friendly

interface!) type the GO command to the BIOS memory address.

Depending on the jumper settings of the controller or host adapter it

would look like G=C800:5 and press <ENTER>. If the address is correct

and the command syntax was typed out correctly the routine for

initialization on the controller or host adapter will start.

------------------------------------------------------------

This 5 step process is how we've been doing things for many years and

since it is obviously a little "techy" and cumbersome, we decided to

write the FINDINIT program.

-= EOF FINDINIT.DOC =-