File Descriptions:

1. GSCSI4.SYS  -- DOS Driver for 51-2/3150/3180 host adapter. This driver

handles non_bootable configuration, and supports

removable media feature (DTC HF24, Syquest). The

driver follows DOS standard BPB (BIOS Parameter Block)

structure. The driver support logical drive size greater

than 32 Mbyte, if driver is run under DOS 4.00 version or

above. It also fully support the utility comes with the

DOS, such as FORMAT SYS, XCOPY, CHKDSK. It dosn't support

DOS FDISK.

2. GSCSI4.EXE  -- DOS Driver for 51-2/3150/3180 host adapter. This driver

handles bootable configuration, and supports functions

described in GSCSI4.SYS.

3. GSDIAG.EXE  -- Provide with SCSI device Low Level Format, and Diagnostic

function.

4. GSFDISK.EXE -- SCSI disk DOS partition utility. This utility support

multiple DOS partition create function. It also support

display partition information, delete exist DOS partition,

and active partitioncan function. The partition size can

greater than 32 Mbyte.

Installatin Note:

1.  Use GSDIAG.EXE to do the SCSI disk low level format.

2.  Follow the DOS  "installable device driver"  procedure and

put the following statement in CONFIG.SYS file.

DEVICE=GSCSI4.SYS

3   If install Removeable Drive as first two drive (i.e. no ST-506 attach

in system) put the following statement in AUTOEXEC.BAT file to support

removeable drive feature.

GSCSI4.EXE

Otherwise, if systen has ST-506 drive attached, or if system has no

ST-506 attached and no any DTC HF24 attached as first two drive

( base on drive SCSI ID sequence ), you don't need to execute the

GSCSI4.EXE file.

4.  Use GSFDISK to create drive partitions.

5.  Use DOS FORMAT utility to high level format the logical drives.