USER DOCUMENTATION

WESTERN DIGITAL USER DIAGNOSTICS

This document is intended to accompany the utility USERDIAG.EXE

placed on this bulletin board for your convenience.

The utility is a safe, non-destructive way for your customer

(or your self) to verify the configuration of your AT

compatible computer, and to specifically test the hard drive/

controller subsystem.

Upon loading the program, the low-order ram pointer bytes are

presented to the screen. The machine's power-on self tests

test installed peripherals, and if valid ones exsist, the

tables are updated.

NOTE : THIS UTILITY DOES NOT PERFORM THE SETUP OR FORMAT

FUNCTIONS. THESE FUNCTIONS MUST BE PERFORMED BY

THE MACHINE'S MANUFACTURER-SUPPLIED SETUP AND

FORMAT UTILITIES. IN THE EVENT A FORMATTER IS

REQUIRED, WE SUGGEST THE OBTAINING OF THE WD

UTILITY WDFMT.

A sample of the start-up screen follows:

System contains the following :

640K bytes of RAM

2 Floppy disk drive(s)

1 Parallel printer port(s)

2 Asynchronous serial port(s)

1 Hard disk drive(s) of the following configuration(s) ...

Drive    Cylinders    Heads    Precomp cyl    Landing zone    Sectors/track

0          820        10         65535           820              17

Total formatted capacity of drive  0  is  71,372,800 bytes

If the above information is not correct, run your system's setup utility

The following diagnostic will use the configuration information above

Do you want to do a hard drive / controller diagnostic ( y / n) ??

Note that the total count for all devices, as well as the

physical capacity of the hard drive is indicated. The useable

capacity is somewhat less, due to the partitioning method

that DOS uses.

You can proceed to the controller/drive diagnostic by typing

'y' now. No return is necessary.

The disk diagnostic is useful for a problem installation,

or to verify absolute disk addresses in the event of a disk

error later.

The user can directly enter any cylinder, sector, or head

he desires to test. The drive number can also be selected

if TWO drives are present. The amount of sectors to test

(sector count) is valid only for a (R)ead operation. If

no value is entered for any parameter, the program

automatically chooses:

drive        =   0

cylinder     =   0

head         =   0

sector no.   =   1

sector count =   256

command      =   re(Z)ero

The currently supported commands are :

re(Z)ero ....... recalibrates drive to cylinder 0

(S)eek   ....... positions drive to any cylinder

(R)ead   ....... transfers data from drive starting

at location chosen, and continues

for duration of sector count

(W)rite / read . transfers a random pattern of

data to the drive, then reads

it back, and checks for errors.

THIS IS DONE ONLY AT THE LAST

SECTOR OF THE DRIVE. YOU CANNOT

CHANGE THE LOCATION.

(A)ll .......... each of the preceeding commands

done in order. Any error will

terminate the sequence, and you

have the option of trying a

single command.

As you may notice, the sector number, sector count, and

head number have no bearing on re(Z)ero and (S)eek

commands.

IMPORTANT NOTE : Illegal disk addresses have been screened

for, and the following is how the program

handles them.

ILLEGAL SECTOR NUMBER : results when sector 0 or a sector number

greater than the actual sector per track

count is selected. Sector 0 is fixed

to 1, and the sector per track value

from the system BIOS is the maximum.

ILLEGAL HEAD NUMBER   : results when a head number greater

than ONE LESS THAN THE BIOS VALUE

is selected. Head (<0) is fixed to 0

and head numbers are truncated at

one less than the BIOS value.

ILLEGAL CYLINDER NO.  : results when a cylinder number less

than zero or greater than ONE LESS

than the BIOS table. Handled exactly

like the heads, above. Can also

result when a read operation will

span many cylinders. The actual

cylinder entered is tested against

the selected value PLUS the sector

count divided by the number of sectors

in a cylinder PLUS 1. If the result

would be greater, than the max cylinder

count of the drive, the difference is

truncated.

ILLEGAL SECTOR COUNT .. results when less than zero or

greater than 255 is entered. The

program makes 0 equal 256, and

<0 equal 1.

A sample session follows:

Drive 0 selected automatically

Select a cylinder between		 0  and     819   ? 819

Select a head (surface) between	 0  and       9   ? 9

Select a starting sector between	 1  and      17   ? 1

Select a number of sectors between  1  and     256   ? 17

The following commands are available :

re(Z)ero,  (S)eek,  (R)ead,  (W)rite/read verify,  (A)ll

ENTER YOUR OPTION	   :	  ? a

Now issuing the rezero (recal) command ......... command successful !

Now issuing the seek command ................... command successful !

Now issuing the read command ................... command successful !

For your protection, this part of our show will only do the LAST SECTOR

That will be ... cylinder	 819  , head   9  , sector   17

Now issuing the write command .................. command successful !

Now issuing the read command to compare data ... command successful !

Now comparing data ............................. data compared O.K. !

Want another test	( y / n )	   ?

IMPORTANT NOTE : In the event of an error, the status and

error registers are presented to the screen.

An English interpretation of the HIGHEST

PRIORITY ERROR will follow. If there is a

real problem, such as an XT controller

installed in the system, or a timeout error

that "hangs" the controller, the status

and error register values may be the same.

The sector count, head number, and cylinder

number are also displayed.

Possible error codes listed below:

POSSIBLE ERROR CODES :

ERROR                   PROBABLE CAUSE

DRIVE NOT READY         Improper drive select option on drive.

TRACK 0 ERROR           Drive not up to speed, or no seek

complete signal received. Cable fault.

Power the machine off and re-verify

drive select option and cable connections.

WRITE FAULT FROM DRIVE  Improper drive type selected in SETUP.

Above maximum cylinder number or head

number was selected. Verify the drive

type selected in SETUP.

CONTROLLER STAYED BUSY  No index signal was present from the

INDEX STAYED ACTIVE     drive. If this signal is missing, the

controller will wait forever if a

(R)ead command is selected. Power the

machine off and re-verify drive select

option and cable connections.

DRQ FAILURE             The controller either still has data

that was unread or is waiting for

data to write. If this persists, the

controller is probably defective.

BAD BLOCK ON DRIVE      These are either unreadable or marked as

DATA CORRECTED ON       bad at format time. BE SURE THESE AREAS

DRIVE		       ARE NOT USED. Use a format utility (WDFMT)

UNCORRECTABLE DATA      and enter these areas as bad. If the

ERROR		       disk contains data, try to relocate the

ID FIELD (address)      cluster that contains this area using a

NOT FOUND	       DOS disk utility. Verify that the drive

type selected in SETUP matches the actual

configuration of the hard drive, and

that a low-level format was performed on

the ENTIRE drive.

ILLEGAL COMMAND	       The controller was unable to recognize

the command you issued. This could be

caused by incorrect jumper configuration

on controller (check USER'S GUIDES

section of this BBS). Also could be

defective controller.

IF PROBLEMS PERSIST ... 24 hour Technical Support number 1-800-777-4787

==============