T E S T   D R I V E  (tm)

A Comprehensive Disk Drive Diagnostic Tool

OPERATOR'S MANUAL

Version 1.2

Microsystems Development

4100 Moorpark Ave.  #104

San Jose, CA  95117  USA

(408) 296-4000

Copyright (C) Microsystems Development 1987,1988.

All Rights Reserved

TEST DRIVE (tm)                                     Version 1.2

TABLE OF CONTENTS

I.   INTRODUCTION............................................1

II.  WARRANTY................................................3

III. COPYRIGHT AND TRADEMARK INFORMATION.....................3

IV.  ORDERING INFORMATION....................................4

V.   BASIC DISK INFORMATION..................................5

VI.  OPERATION...............................................7

F1   Program Information................................7

F2   General Test.......................................8

A) Sample display for a good drive.................9

B) Sample display for a bad drive.................10

F3   Alignment Test....................................11

A) Sample display for a good drive................13

B) Sample display for a bad drive.................14

F4   Spindle Speed.....................................15

A) Sample display for a good drive................16

B) Sample display for a bad drive.................16

F5   Write / Read Test.................................17

A) Sample display for a good drive................18

B) Sample display for a bad drive.................19

F6   Hysteresis Test...................................20

A) Sample display for a good drive................21

B) Sample display for a bad drive.................22

F7   Head Azimuth Test.................................23

A) Sample display for a good drive................24

B) Sample display for a bad drive.................25

F8   Hub Centering.....................................26

A) Sample display for a good drive................27

B) Sample display for a bad drive.................28

F9   Continuous Alignment..............................29

A) Sample display for a good drive................30

B) Sample display for a bad drive.................31

F10  Cleaning Utility..................................32

GLOSSARY OF TERMS...........................................34

CUSTOMER FEEDBACK FORM......................................37

Copyright (C) Microsystems Development 1987, 1988.            i

TEST DRIVE (tm)                                     Version 1.2

I.   INTRODUCTION

Floppy disk drives are your computer's lifeline.  They

provide the easiest and in many cases the only method of

transferring data from one computer to another.  It is extremely

important that your computer's disk drives be operating at peak

performance at all times.

TEST DRIVE is a comprehensive floppy disk drive diagnostic

tool.  It accurately measures and displays important operating

parameters, so that the exact operating condition of a drive can

be determined.  In addition, this manual will help in

understanding the basics of disk operation, will help to analyze

the test results, and recommend corrective procedures.

Have you ever gotten the dreaded "Abort, Retry, Ignore?"

message when using a disk you thought was good?  TEST DRIVE will

let you know if your drive is functioning properly.

Have you ever had problems in transferring files on disk

between different computers?  Perhaps one computer can reliably

read and write to a diskette, but that same diskette will not

work in another computer?  TEST DRIVE will pinpoint the problem.

Have you ever experienced unexplainable data loss on

diskettes?  Perhaps a backup disk that your computer generated

months ago will no longer work in your computer?  Periodic

testing with TEST DRIVE will help avoid this problem.

Do you need to be absolutely sure that the diskettes your

disk drive writes to can be read by another computer?  TEST DRIVE

will confirm proper operation of your drive.

Have you ever had repairs made only to have the same or new

problems occur?  TEST DRIVE will keep the technician honest by

reporting and quantifying the drive's performance, before and

after repair.

Want to dig in and make adjustments yourself?  TEST DRIVE

provides continuous tests which can be run on your computer while

adjustments are made.

Ever wonder if you should be cleaning your drives, and if

so, how often?  TEST DRIVE will indicate when cleaning may be

necessary.  In addition, TEST DRIVE provides a helpful utility to

be used with a standard cleaning disk.

Does cleaning really help?  TEST DRIVE can measure your

drive's performance before and after cleaning to determine if the

cleaning has been helpful.

How important is the diskette media?  Bulk purchased generic

disks are generally tested to a lesser extent that the more

expensive name brands.  In general, the generics are tested for

Copyright (C) Microsystems Development 1987, 1988.            1

TEST DRIVE (tm)                                     Version 1.2

media defects only where a perfectly aligned drive would read and

write.  If your drive's alignment is off, the probability of a

data loss using an inferior disk is greatly increased.

In addition, as drive densities continue to increase, data

is packed closer and closer together on the magnetic media.  This

makes precise operation of your drive even more crucial.

The TEST DRIVE disk drive diagnostic package consists of the

TEST DRIVE program disk and an Dysan Digital Diagnostic Disk

(DDD).  The program diskette contains the following files:

testdriv.com   -    Executable program

testdriv.doc   -    This documentation

To use the full capabilities of TEST DRIVE you will need the

Dysan precision alignment disk manufactured by Xidex Corporation,

available for sale from Microsystems Development (see ordering

information).  The Dysan Digital Diagnostic Disk (DDD) is a test

disk with data written at precise distances and angles along the

tracks.  TEST DRIVE will analyze the disk drive's ability to read

the specialized data patterns contained on the DDD.  Some of TEST

DRIVE's tests require the use of the DDD, some do not.

Together, these two disks allow you, the user, to perform

comprehensive diagnostic tests, and perform certain adjustments

on floppy disk drives.  The use of expensive and special test

equipment, oscilloscopes, etc., is unnecessary.  The following

tests are performed by TEST DRIVE:

*  General Test

*  Alignment Test

Spindle Speed

Write/Read Test

*  Hysteresis Test

*  Head Azimuth Test

*  Hub Centering

*  Continuous Alignment          * These tests require the

Cleaning Utility                Dysan diagnostic disk

This version of TEST DRIVE will test 5 1/4" (360KB  or less)

disk drives.  Updates for high density 5 1/4" and 3 1/2" drives

are under development.  Please inquire for availability.

System Requirements:

IBM PC/XT/AT or Compatible

64K RAM

DOS 2.0 or greater

An 80 column monitor (Color or Monochrome)

One working 5 1/4" disk drive that can load the program.

Once loaded, TEST DRIVE can later be run from a hard disk.

In fact, this is recommended for periodic testing to insure your

drives are in proper operating condition.

Copyright (C) Microsystems Development 1987, 1988.            2

TEST DRIVE (tm)                                     Version 1.2

II.  WARRANTY

Microsystems Development makes no warranty of any kind,

express or implied, including without limitation, any warranties

of merchantability and/or fitness for any specific application or

use.  Microsystems Development shall not be liable for ANY loss

or damage arising from a failure of this program to operate in

the manner described, or in a manner desired by the user.

Microsystems Development shall not be liable for any damage to

data or property which may be caused directly or indirectly by

use of the program.

IN NO EVENT SHALL MICROSYSTEMS DEVELOPMENT OR IT'S SUPPLIERS

BE LIABLE FOR ANY CONSEQUENTIAL, INCIDENTAL OR EXEMPLARY DAMAGES

INCLUDING, BUT NOT LIMITED TO LOSS OF PROFITS OR REVENUES, LOSS

OF USE OF ANY PRODUCT, SERVICE OR REPLACEMENT COSTS, OR CLAIMS BY

ANY THIRD PARTY.

III. COPYRIGHT AND TRADEMARK INFORMATION

This version of TEST DRIVE may be copied and distributed

freely, with the following restrictions:

1)  The program and documentation may not be modified,

2)  The entire documentation file must be included with the

program,

3)  No fee for the program may be charged outside of a

reasonable and customary charge for diskette media,

copying and/or shipping, and

4)  It may not be packaged or sold in conjuction with any

other product.

The software code and screen displays used in TEST DRIVE are

the sole property of Microsystems Development, and may not be

copied, in any form, in whole or in part, or included in any

other program or document without the express written permission

of Microsystems Development.

If you'd like to make use of the full capabilities of TEST

DRIVE, you are encouraged to purchase the Dysan Diagnostic Disk

directly from us (see ordering information).  This will help

support our effort, make you a registered user, and ensure that

you are kept up to date on new versions and enhancements.

Digital Diagnostic Diskette (DDD) is a trademark of Dysan.

Dysan is a registered trademark of Xidex Corporation.

IBM, IBM PC, IBM XT and IBM AT are registered trademarks of

International Business Machines Corporation.

MS-DOS is a trademark of Microsoft Corporation.

TEST DRIVE is a trademark of Microsystems Development.

Copyright (C) Microsystems Development 1987, 1988.            3

TEST DRIVE (tm)                                     Version 1.2

IV.  ORDERING INFORMATION

TEST DRIVE ORDER FORM

____ Dysan Digital Diagnostic Disk,

includes TEST DRIVE registration.......US$50.00 ea _______

____ Dysan disk, Latest version of TEST DRIVE,

Printed manual, and Registration........US$75.00 ea _______

Subtotal..............._______

Calif. residents add 6%, 6.5% or 7% sales tax........_______

Shipping charges.  $2.50 within the continental

USA, $5.00 outside the continental USA..............._______

TOTAL...............$_________

______ Enclosed is a check for the above amount.

______ Please charge the above amount to my:  Visa( )  MC( )

Account number:__________________________________Exp._____

Cardholder signature:_____________________________________

Purchase orders are accepted from qualified corporations.

Please call first for approval.

Name:    _____________________________________________

Company: _____________________________________________

Address: _____________________________________________

_____________________________________________

Phone:   (________) __________________________________

Remit to:  Microsystems Development

4100 Moorpark Ave.  #104

San Jose, CA  95117  USA

Phone:  (408) 296-4000  10 AM to 4 PM  Pacific Time.

Please inquire for availability of high density

5 1/4" and 3 1/2" versions of TEST DRIVE.

Copyright (C) Microsystems Development 1987, 1988.            4

TEST DRIVE (tm)                                     Version 1.2

V.   BASIC DISK INFORMATION

It is not necessary to have a degree in computer engineering

to use or benefit from TEST DRIVE.  However, a basic

understanding of disk operation is helpful in identifying

problems.  This section is meant to introduce you to basic disk

operation and terminology.

Diskette density specifies the amount of data the disk is

capable of holding.  Densities are measured in kilobytes (KB) or

megabytes (MB).  A kilobyte is 1024 bytes and a megabyte is 1000

kilobytes.

Information is written on a floppy disk magnetically, in

much the same way as audio information is stored on cassettes or

reel to reel tapes.  The disk is rotating so data is written in

circles around the disk.  These circles are called tracks.  Each

track is divided into a number of sectors.

Each sector contains addressing marks, a fixed amount of

data, and a Cyclic Redundancy Check (CRC) for the data.  The CRC

is an integrity check on the data.  To retrieve data, the disk

drive controller must position the head over the desired track

and wait for the correct sector address to come around.  Once the

sector is found, the disk controller reads the information from

the sector and stores it in a buffer.  Then the CRC is computed

and compared to the CRC read from the disk.  If this does not

compare an error is reported.  A write operation occurs in much

the same manner except that the CRC is computed and stored with

the data and no verification is performed.

One indicator of disk density is the TPI specification.

This stands for Tracks Per Inch.  In a 48TPI disk the tracks are

spaced 1/48 of an inch apart.  Since a 360K diskette has 40

tracks, the width of the media containing data is somewhat less

than an inch.  A 96 TPI diskette usually contains 80 tracks or

double the 40 tracks of a 360K.   This would tend to make us

think that the density would be 720K, and this is true for the 3

1/2 inch technology.  But the high density 5 1/4 inch standard

has 15 sectors per track instead of the 9 on a 360 KB disk.  This

makes 1.2 MB (see table below).  It seems that as the technology

becomes available to store more information on a disk, someone

will.  However, the basics for data storage and retrieval is

relative constant no matter what the density is.

Copyright (C) Microsystems Development 1987, 1988.            5

TEST DRIVE (tm)                                     Version 1.2

A 360KB diskette is formatted with 40 tracks, 9 sectors per

track, on both sides.  512 bytes are written in each sector.

Total    40 tracks   9 sectors   2 sides    512 bytes

bytes  =    ------ X   ------- X          X     ------

side       track                   sector

OR

Total bytes = 368,640

This is usually abbreviated to 360 KB, which is 360 x 1024

bytes.

The computer's operating system normally uses a few sectors

for overhead such as label and directory information.  This is

why there are slightly less than 368,640 usable bytes on a

formatted disk.

The address marks and CRC are stored between the data and

are not included in the formatted capacity of a disk.

The following table illustrates the storage specifications

for the most commonly used disk densities:

Storage   Physical  Sectors     Bytes       Tracks     Number

Size      Size      per Track   per Sector  per Side   of Sides

----------------------------------------------------------------

160 KB    5 1/4"       8          512         40         1

180 KB    5 1/4"       9          512         40         1

320 KB    5 1/4"       8          512         40         2

360 KB    5 1/4"       9          512         40         2

1.2 MB    5 1/4"      15          512         80         2

720 KB    3 1/2"       9          512         80         2

1.44 MB    3 1/2"      18          512         80         2

This version of TEST DRIVE is designed to test 360 KB

drives.  It will also test the first 3 types, however.  Since the

first 2 types (160 and 180 KB) have only 1 head in the disk

drive, TEST DRIVE will report errors for Head 1, but the Head 0

results are correct.

Copyright (C) Microsystems Development 1987, 1988.            6

TEST DRIVE (tm)                                     Version 1.2

VI.  OPERATION

To operate, first load the operating system, then type

'testdriv', followed by the enter key.  After the opening screen

appears, press any key to proceed to the main menu.  You will see

a list of available selections.  The number of drive selections

you see is dependent on the number of drives available in your

system.  To make a selection, press the key indicated to the left

of the selection, or highlight your choice by using the up and

down arrows, then press enter.

Some tests require the use of the Dysan DDD.  The speed test

will work with any standard formatted diskette OR the DDD.  The

Write/Read test will only work a normal diskette, not the DDD.

TEST DRIVE always checks for the type of diskette inserted.

Data is never written to the DDD.

F1   Program Information

This screen gives a very brief "on-line" description of TEST

DRIVE.  This document is provided for more detailed information.

After the information screen, the user is given the option

of displaying an 'Instant Order Form'.  If a printer is attached,

simply use the PrtSc (print screen) function of your computer to

print a copy of the order form.

Copyright (C) Microsystems Development 1987, 1988.            7

TEST DRIVE (tm)                                     Version 1.2

F2  General Test

In this test, six areas of drive performance are displayed

and measured, with Pass / Fail results given for each.

The measured results are also displayed, along with the

criteria used for Pass / Fail determination.  For more detailed

information on each of the tests, see the appropriate section

describing that particular test in detail.

The TEST DRIVE general test is a good overall test which

should usually be run first to determine if there are any

problem areas.  You encouraged to read the rest of this document

for a detailed description of each test.

Display Information:

Alignment and Sensitivity are combined in the first part of

the test.  Three tracks for each head are measured.  The drive

will pass the test if it can read data written at least +/- 8

milli-inches from the track centerline.  Some manufacturers

specify +/- 9 milli-inches as passing criteria.

The next part of the test measures the drive's ability to

precisely seek to a specified track from either direction.  This

is called hysteresis.  Only one track is used for this test.  If

the hysteresis is less than or equal to 1.5, it passes.

Next, the spindle speed is measured.  For a 360KB disk

drive, the speed must be between 295 and 305 RPM.  Some

manufacturers specify a smaller range.

Hub centering is the next test.  The alternate offset track

which has data written at +/- 8 milli-inches offset around the

track is used for this part of the test.  No errors allowed for

the drive to pass.

The last test in the general test is Head Azimuth.  If the

drive can read data placed on the disk at an angle of +/- 39

minutes of a degree from centerline, it is considered good.

The Pass / Fail criteria used in the General Test is in

accordance with generally accepted standards.  However, different

manufacturers, equipment, testing labs, etc., may use different

criteria.  The criteria used in TEST DRIVE are not meant to imply

absolute minimums or maximums, but were simply chosen to help

interpret the data.  In all cases, the measured data is also

displayed for the user's own interpretation.

In addition, the measured data displayed for the General

Test, as well as the other tests may vary slightly each time they

are run.  This is because the tests push the drive past its

normal operating points, and operation outside the normal ranges

may not be consistent.

Copyright (C) Microsystems Development 1987, 1988.            8

TEST DRIVE (tm)                                     Version 1.2

A) Sample display for a good drive

TEST DRIVE

General Test

Criteria   Test     Result

-------- --------- --------

Alignment /    Track  0 Head 0     +/- 8   +10 -10    Pass

Sensitivity    Track  0 Head 1     +/- 8   +11 -10    Pass

Track 19 Head 0     +/- 8   +10 -10    Pass

Track 19 Head 1     +/- 8   +10 - 9    Pass

Track 39 Head 0     +/- 8   +12 -10    Pass

Track 39 Head 1     +/- 8   + 8 - 9    Pass

Hysteresis     Track 19 Head 0      1.5       0.5     Pass

Spindle Speed  . . . . . . . .   295 - 305    301     Pass

Hub Centering  . . . .  Head 0   0 Errors  0 Errors   Pass

Head Azimuth   . . . .  Head 0    +/- 39'  +42 -42    Pass

Head 1    +/- 39'  +42 -42    Pass

Analysis:

The above drive has passed all the tests, and the drive

appears to be in good condition.

Alignment     - the drive was able to read data 8 milli-inches

or greater from the track centerline.

Hysteresis    - is less than 1.5.

Spindle Speed - is in the acceptable range at 301 RPM.

Hub Centering - no errors were detected.

Head Azimuth  - both heads were able to read data angled at

39 minutes of a degree or greater.

Copyright (C) Microsystems Development 1987, 1988.            9

TEST DRIVE (tm)                                     Version 1.2

B) Sample display for a bad drive

TEST DRIVE

General Test

Criteria   Test     Result

-------- --------- --------

Alignment /    Track  0 Head 0     +/- 8   + 6 -12    Fail

Sensitivity    Track  0 Head 1     +/- 8   +11 -10    Pass

Track 19 Head 0     +/- 8   + 7 -11    Fail

Track 19 Head 1     +/- 8   +11 - 9    Pass

Track 39 Head 0     +/- 8   + 6 -11    Fail

Track 39 Head 1     +/- 8   + 8 - 9    Pass

Hysteresis     Track 19 Head 0      1.5       2.0     Fail

Spindle Speed  . . . . . . . .   295 - 305    309     Fail

Hub Centering  . . . .  Head 0   0 Errors  2 Errors   Fail

Head Azimuth   . . . .  Head 0    +/- 39'  +42 -42    Pass

Head 1    +/- 39'  +42 -21    Fail

Analysis:

In the above example, Head 0 is poorly aligned.  It is

shifted slightly away from the spindle as indicated by the high

negative values and low positive values.

The above drive has also failed the hysteresis test

indicating a possible problem with the positioning mechanism.

The TEST DRIVE General Test indicates that the spindle

speed is too high.  Hub centering has also failed indicating a

possible clamping problem.  It was not able to read some of the

data written at 8 milli-inches from track center.

Head 1 is also rotated slightly in one direction.  It cannot

read data written in a position greater than 21 minutes of a

degree from track centerline.

For all the above problem areas, each individual test should

be run in full to determine the extent of each problem.

Copyright (C) Microsystems Development 1987, 1988.            10

TEST DRIVE (tm)                                     Version 1.2

F3  Alignment Test

Alignment is a measure of how centered the read/write head

is over the track center.  This is sometimes called Radial

Alignment.  For a 48TPI disk, each track is 1/48 inch wide which

is 20.833 milli-inches.  Data on the progressive offset tracks of

the DDD are progressively displaced from 6 to 13 milli-inches

toward and away from the spindle.  Therefore, data  will span 26

milli-inches which is slightly wider than a 48 TPI track.  The

tracks adjacent to the progressive offset track of the DDD are

not used.

Sensitivity is a measure of how much data on a given track

is readable, or the width to which the head is sensitive to.  If

a head is able to read data from +10 to -10 milli-inches on a 20

milli-inch track, you have a drive performing very well.  If the

same drive read +13 to -7, we still have perfect sensitivity but

the alignment is now shifted.

The successful reading of data greater than +/- 10 milli-

inches will cover the entire track.  If a disk drive is able to

read data that is progressively offset to +8 and only to -7, the

drive is slightly mis-aligned and suffers from poor alignment

sensitivity.  A drive that will read -11 to +8 milli-inches,

shows good sensitivity but is poorly aligned.  In the latter

case, the drive will likely function reliably until the head

becomes dirty and the sensitivity decreases.

The alignment test will push the drive's operation beyond

normal limits and report back the results,  You will certainly

see errors in this test simply because the drive is not supposed

to read data placed beyond the normal track width.

Display Information:

The alignment test uses all of the progressive offset tracks

available on the Dysan Digital Diagnostic test.  These are tracks

0, 5, 16, 19, 30, and 39.  It attempts to read all the data on

the tracks, from 6 to 13 milli-inches off center in either

direction.  If the read was successful, it will display four

solid characters, represented in this document as '~~~~'.  If the

read was not successful, it will display 'erxx', where xx is the

hex error code returned by the BIOS read routine.

Copyright (C) Microsystems Development 1987, 1988.            11

TEST DRIVE (tm)                                     Version 1.2

The following is a list of errors than can be reported:

ERROR ERROR

CODE  TYPE                REASON

----  ------------------  ---------------------------------

er80  time_out            Drive did not respond

er40  bad_seek            Drive failed to seek to track

er20  bad_nec             Drive controller chip failed

*    er10  bad_crc             Cyclic redundancy check failed

er09  dma_boundary        Attempt to direct memory

across 640K boundary

er08  bad_dma             DMA overrun on operation

er06  media_change        media not identified

on multiple media drive

*    er04  record_not_found    sector not found

*    er02  bad_addr_mark       address mark not found

* Commonly induced errors in alignment test.

Adjustments for poor Alignment Sensitivity:

Poor alignment sensitivity indicates that the heads are not

picking up as much data as they should.  This could be due in

part to a build up of dirt on the heads.  The correction for this

to simply clean the heads.  If the problem is a dirty head,

cleaning will eventually correct the problem.  Cleaning, however,

generally will not have a drastic effect.  If the problem

persists, it may be necessary to replace the drive since

replacing the head assembly usually is not done economically.

Adjustments for poor Alignment:

Most drives come equipped with an adjustment screw which

will translate a turning motion into a linear head adjustment.

Since drives are becoming so inexpensive however, manufacturers

are saving costs by eliminating expendable mechanisms such as

alignment hardware.  It is more cost effective for the

manufacturer to set the alignment once, than it is to provide

adjustment screws.

If your drive has such an adjustment, we suggest you use the

Continuous Alignment Test if you wish to make adjustments.

Please see the explanation of that test for more information.

If the alignment is just slightly off, we recommend that you

leave it alone, and monitor its condition with TEST DRIVE.

Copyright (C) Microsystems Development 1987, 1988.            12

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display for a good drive

TEST DRIVE Alignment Test

<------------------------  Track  ------------------------->

39   30   19   16   5    0       0    5    16   19   30   39

- 13 er02 er02 er02 er02 er02 er02    er02 er02 er02 er04 er10 er04

- 12 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

- 11 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

- 10 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ er02 er02

-  9 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ er02

-  8 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

-  7 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

-  6 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

Head  0                          Head  1

+  6 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  7 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  8 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  9 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+ 10 er02 er02 er02 ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ er02

+ 11 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

+ 12 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

+ 13 er02 er04 er04 er04 er04 er04    er04 er04 er02 er02 er04 er02

Analysis:

This drive is in good shape, the '~~~~' pattern is

symmetrical between the + and - readings.  This indicates very

good alignment.  This test also indicates the sensitivity of the

head.  For the most part, the heads are sensitive to data from -

10 to +10 milli-inches or a total of 20 milli-inches.  On track

39 of head 1, the head is sensitive only from -8 to +9 milli-

inches or 17 milli-inches.  Since these tracks on the diskette

are 20 milli-inches wide, this drive is performing well.

Copyright (C) Microsystems Development 1987, 1988.            13

TEST DRIVE (tm)                                     Version 1.2

C)  Sample display for a bad drive

TEST DRIVE Alignment Test

<------------------------  Track  ------------------------->

39   30   19   16   5    0       0    5    16   19   30   39

- 13 er02 er02 er02 er02 er02 er02    er02 er10 er04 er10 er10 er04

- 12 er02 er02 er02 er02 er02 er02    er02 er04 er04 er02 er02 er02

- 11 er02 er02 er02 er02 er02 er02    er02 er04 er02 er02 er02 er02

- 10 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

-  9 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

-  8 er02 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ er02 er02 er02 er02 er02

-  7 er02 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ er02 er02 er02 er02

-  6 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ er02 er02 er02

Head  0                          Head  1

+  6 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  7 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  8 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+  9 ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+ 10 er02 er02 er02 ~~~~ ~~~~ ~~~~    ~~~~ ~~~~ ~~~~ ~~~~ ~~~~ ~~~~

+ 11 er02 er02 er02 er02 er02 er02    er02 er02 er02 ~~~~ ~~~~ ~~~~

+ 12 er02 er02 er02 er02 er02 er02    er02 er02 er02 er02 er02 er02

+ 13 er02 er04 er04 er04 er04 er04    er04 er04 er02 er02 er04 er02

Analysis:

This drive shows problems in both alignment and possibly in

sensitivity as well.  The alignment pattern shows a shift of the

successful read operations toward the positive side.  This

indicates that the physical positioning of the head is shifted

off the center of the track.  This problem could be remedied by

adjusting the head positioning to make the pattern symmetrical.

For most of the disk the total sensitivity reads about +10

to -7 or 17 milli-inches of read sensitivity.  However, head 1

cannot even read data at -6 milli-inches on tracks 19, 30 and 39.

In this case, the sensitivity cannot be determined until the

alignment is corrected.

Copyright (C) Microsystems Development 1987, 1988.            14

TEST DRIVE (tm)                                     Version 1.2

F4   Spindle Speed

This test measures the speed at which the diskette rotates

in the drive.  It can use either the Dysan DDD or a standard

formatted diskette.  If an error occurs when the test starts, the

user is given the opportunity to override by pressing 'O'.  If

errors still occur, speed will be displayed as '---'.

Speed is measured by the time it takes for the diskette to

rotate from a given sector back around to the same sector.  This

time is then converted to revolutions per minute, or RPM.  The

acceptable range is 295 to 305 RPM.  A drive can operate outside

this range and cause no apparent problems, but it should be

adjusted to be within range.  Otherwise, it may be difficult to

interchange data written by it with another drive.

Other types of drives operate at different speeds as

follows:

high density 5 1/4"    355 to 365 RPM

3 1/2"    295 to 305 RPM

Display Information:

The test will run for several trials, continually updating

the speed and the pointer on the display.  It then stops in order

to save wear on the disk.  If you wish to run it continuously,

press "C".  Any other key will return you to the main menu.  If

you select "C" for continuous testing, the test will not stop

until you press another key.

Adjustment:

On most drives,there is a small screw adjustment for spindle

speed.  This adjustment can be made while this test is running,

and TEST DRIVE will continually update the display accordingly.

If the speed is adjusted too far out of range so that the program

can no longer read any data, TEST DRIVE will report '---'.

If the speed varies erratically, say 4 RPM or more, the

problem could be a loose or bad drive belt.  In many cases,

replacement of the belt will help or cure the problem.

Some drives have direct drive from the motor so do not have

belts.  Some do not even have a speed adjustment.  In these

cases, the drive should either be repaired professionally or

replaced.

Copyright (C) Microsystems Development 1987, 1988.            15

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display for a good drive

TEST DRIVE Spindle Speed Measurement

|---- Acceptable ---|

|       Range       |

285       290       295       300       305       310       315

|---------|---------|---------|---------|---------|---------|

^

Speed =   300 RPM

Using Dysan DDD Model 508-400

Analysis:

Here, the speed is directly in the middle of the acceptable

range, at 300 revolutions per minute.

B)  Sample display for a bad drive

TEST DRIVE Spindle Speed Measurement

|---- Acceptable ---|

|       Range       |

285       290       295       300       305       310       315

|---------|---------|---------|---------|---------|---------|

^

Speed =   291 RPM

Using Dysan DDD Model 508-400

Analysis:

The above drive is running slightly slow, at 291 revolutions

per minute.

Copyright (C) Microsystems Development 1987, 1988.            16

TEST DRIVE (tm)                                     Version 1.2

F5   Write / Read Test

This test measures the drive's ability to correctly write,

then read back, each sector of the disk.  It can also be used to

verify the operation of a particular diskette, that is, its

ability to store data on all parts of it.

The computer's operating system normally checks both the

write and the read processes for errors.  TEST DRIVE checks for

those errors and reports them.  In addition, each byte that was

written is compared with what was read back to determine if it's

exactly the same.  Any discrepancy is reported as a compare

error.

Display Information:

This test requires the use of a formatted scratch diskette.

It writes 512 random bytes of data to each sector on the disk,

reads them back and compares them.  Errors can occur while

Writing, Reading or Comparing the data, and are reported sector

by sector.  There are several types of write and read errors

returned by the system's BIOS, such as bad seek or bad CRC.

These errors are not reported individually in this test.  Only

'W' or 'R' are displayed, depending on whether the error was the

result of a write or a read operation.

Since TEST DRIVE writes random data to all sectors including

those used by the operating system for file and directory

information, any diskette used for this test will have to be re-

formatted before it can be used for normal purposes.

Insure that the media used is of high quality and does not

introduce errors into the test.

No errors should be tolerated for this test.

Adjustment:

There is no adjustment.  In some cases, the drive heads may

simply need to be cleaned.  Use a good quality cleaner and try

the test again.

Copyright (C) Microsystems Development 1987, 1988.            17

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display for a good drive

TEST DRIVE Write / Read Test

Sector-+   Track -->

|   0 - - -  9  - - - 19 - - -  29 - - -  39

1 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~-

2 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

~ = Sector OK     3 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

4 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

5 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~| Head

W = Write error   6 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|   0

7 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

R = Read error    8 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

9 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

C = Compare error 1 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~-

2 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

3 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

Total errors = 0  4 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

5 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~| Head

6 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|   1

7 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

8 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

9 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~-

Analysis:

The above display indicates that the program was able to

successfully write, then read back the same data, to every sector

on the diskette.

Copyright (C) Microsystems Development 1987, 1988.            18

TEST DRIVE (tm)                                     Version 1.2

B) Sample display for a bad drive

TEST DRIVE Write / Read Test

Sector-+   Track -->

|   0 - - -  9  - - - 19 - - -  29 - - -  39

1 - ~~C~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~-

2 - ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

~ = Sector OK     3 - W~~~~~~~~~~~~~~~W~~~~~~~~~~~~~~~~~~~~~~~|

4 - WR~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

5 - WR~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~| Head

W = Write error   6 - WW~~~~~~~~~~~~~~~W~~~~~~~~~~~~~~~~~~~~~~|   0

7 - WR~~~~~~~~~~~~~~~R~~~~~~~~~~~~~~~~~~~~~~|

R = Read error    8 - R~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

9 - ~~~~~~~~~~~~~~~~~~~~~~~~R~~~~~~~~~~~~~~~|

C = Compare error 1 - ~~~~~C~~~~~~~~~~~~~~~~~~WR~~~~~~~~~~~~~~-

2 - ~~~WRW~~~~~~~~~~~~~~~~~~WWR~~~~~~~~~~~~~|

3 - ~~~WWWW~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

Total errors = 54 4 - ~~~WWWW~~~~~~~~~~~WWW~~~~~~~~~~~~~~~~~~~|

5 - ~~~WWRW~~~~~~~~~~~WWW~~~~~~~~~~~~~~~~~~~| Head

6 - ~~~~RWW~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|   1

7 - ~~~~WRR~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

8 - ~~~~~RR~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|

9 - ~~~~~RRWC~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~-

Analysis:

The above display shows many errors of different types.  The

problem could simply be a bad diskette.  If the problem occurs

with more than 1 formatted diskette of good quality, there is a

problem with the read/write mechanism of the drive.

Copyright (C) Microsystems Development 1987, 1988.            19

TEST DRIVE (tm)                                     Version 1.2

F6  Hysteresis Test

Hysteresis is a measure of the drive mechanism's ability to

seek a specified track from either direction and position the

head in precisely the same position.

The head is moved to an inner track (toward the hub) and

then sent to the middle of the disk.  Measurements of alignment

are recorded and displayed.  The head is then sent to an outer

track and subsequently sent to the center again.  Measurements

are taken and compared to the previous center measurements to

determine the hysteresis.

The hysteresis error is determined by the difference in the

center of the alignment pattern.  For example:  The first

alignment readings are +10 and -9 milli-inches.  This is to say

that the head is in such a position that data offset +10 and -9

milli-inches can be read.  The head is then sent to the outer

track and back to the center to again take readings of +10 and -9

milli-inches.  This is simply 0 milli-inches of hysteresis.  The

head had positioned itself at precisely the same position when

seeking a track from either direction.  If the second set of

readings were +9 and -10 the hysteresis would be 1.0 milli-inch.

In general, the center of each reading is compared to determine

the hysteresis.  The center of +10 and -9 is +0.5 and the center

of +9 and -10 is -0.5.  The difference is 1 milli-inch.

Although these numbers are confusing the graphical

representation will make more sense.

In this test, the alignment sensitivity is ignored and only

the center of the data pattern is used in determining hysteresis.

The actual alignment of the drive is unimportant in this test but

the repeatability of the positioning is crucial.  Disk drives

with excessive hysteresis are unreliable.

Display Information:

TEST DRIVE uses two of the middle progressive offset tracks

on the DDD for this test:  tracks 16 and 19.  The horizontal bars

on the display span the distance from track centerline that the

drive was able to read successfully.  Each set of bars is for a

particular head and track, with the head having been positioned

there from opposite directions.  Ideally, each set of bars should

be equal in length.

Adjustment:

There is no adjustment and the drive should be discarded or

sent to the manufacturer for reconditioning.

Copyright (C) Microsystems Development 1987, 1988.            20

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display of a good drive

TEST DRIVE Hysteresis Test

<----------- m i l l i - i n c h e s ---------->

H Tr Dir Error  -13-12-11-10 -9 -8 -7 -6 +6 +7 +8 +9+10+11+12+13

- -- --- -----  |--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|

0 16 -->                 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  0.0            ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1 16 -->                    ~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  0.5            ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

0 19 -->              ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  0.5         ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1 19 -->                ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  0.0           ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Analysis:

The above drive has no significant hysteresis.  Some error

is normal since data written too far off track center may be

read unreliably.  If the error is consistently greater, the drive

has a problem.

The drive also exhibits good sensitivity since it can read to

a minimum of +10 and -9 milli-inches.

Copyright (C) Microsystems Development 1987, 1988.            21

TEST DRIVE (tm)                                     Version 1.2

B)  Sample display of a bad drive

TEST DRIVE Hysteresis Test

<----------- m i l l i - i n c h e s ---------->

H Tr Dir Error  -13-12-11-10 -9 -8 -7 -6 +6 +7 +8 +9+10+11+12+13

- -- --- -----  |--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|

0 16 -->                 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  2.0      ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1 16 -->                    ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  2.0         ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

0 19 -->                 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  1.5      ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1 19 -->                       ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

<--  2.5      ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Analysis:

This drive has more hysteresis than normal and indicates a

possible problem with its positioning mechanism.

For example, when head 1 is moved toward the spindle to

track 19 it can read data at 8 milli-inches away from the spindle

and 12 milli-inches toward the spindle.  When head 1 is moved

away from the spindle and is stopped at track 19, it can read

data at 12 milli-inches away from the spindle and 9 inches toward

the spindle.  This indicates that the head was not in the same

physical location.

This drive would probably pass the alignment test however,

since in all the above cases it can read data to +/-8 milli-inches.

Copyright (C) Microsystems Development 1987, 1988.            22

TEST DRIVE (tm)                                     Version 1.2

F7  Head Azimuth Test

Head rotation or azimuth is an indication of the angular

position of the head with respect to a line tangent to the track

centerline. If the head is "twisted", or "rotated", with respect

to the track centerline the data written to the diskette will be

"slanted" and will eventually cause problems when interchanging

diskettes with other drives.

For these measurements, TEST DRIVE uses the azimuth rotation

track (track 34) of the DDD.  This track has data written on

track centerline, but at increasingly greater angles, both toward

and away from the spindle.  The angle range from 21 to 42 minutes

of a degree.  If the drive's head is angled slightly out of

position, it will be able to read data on the tracks at one

angle, but will have problems with data written at the opposite

angles.

Display information:

For each successful read operation, the display shows four

slashes '/' or four reverse slashes '\'.  It will show four

capital x's if it was unable to read data written at a particular

angle.

Adjustment:

There is no adjustment for head azimuth in most drives, and

the head assembly or the entire drive should be replaced.

Copyright (C) Microsystems Development 1987, 1988.            23

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display of a good drive

TEST DRIVE Head Azimuth Test

/

\                                                         /  /

\  \                                                 /  /  /

\  \  \                                         /  /  /  /

\  \  \  \                                 /  /  /  /

\  \  \  \                         /  /  /  /          /

X          \  \  \  \                 /  /  /  /          /  /

X  \          \  \  \  \         /  /  /  /          /  /  /

X  \  \          \  \  \  Head /  /  /          /  /  /  /

X  \  \  \          \  \  1  /  /          /  /  /  /

\  \  \  \          \   /          /  /  /  /

\  \  \  \                 /  /  /  /

\ = OK             \  \  \  \         /  /  /  /

\  \  \  Head /  /  /

X = Error                  \  \  0  /  /

\   /

-42 -39 -36 -33 -30 -27 -24 -21    +21 +24 +27 +30 +33 +36 +39 +42

<---------------------  Angular Rotation  ---------------------->

Angular position of head in minutes of a degree.

Analysis:

The above drive shows no problems with head azimuth.  Both

heads were able to acceptably read the data on the test tracks.

Copyright (C) Microsystems Development 1987, 1988.            24

TEST DRIVE (tm)                                     Version 1.2

B)  Sample display of a bad drive

TEST DRIVE Head Azimuth Test

\                                                                 /

\  \                                                         /  /

\  \  \                                                 /  /  /

\  \  \  \                                         /  /  /  /

\  \  \  \                                 /  /  /  /

\          \  \  \  \                         /  /  /  /          X

\  \          \  \  \  \                 /  /  /  /          X  X

\  \  \          \  \  \  \         /  /  /  /          X  X  X

\  \  \  \          \  \  \  Head /  /  /          X  X  X  X

\  \  \  \          \  \  1  /  /          X  X  X  X

\  \  \  \          \   /          X  X  X  X

\  \  \  \                 /  X  X  X

\ = OK             \  \  \  \         /  /  X  X

\  \  \  Head /  /  X

X = Error                  \  \  0  /  /

\   /

-42 -39 -36 -33 -30 -27 -24 -21    +21 +24 +27 +30 +33 +36 +39 +42

<---------------------  Angular Rotation  ---------------------->

Angular position of head in minutes of a degree.

Analysis:

The above display indicates that Head 1 does not have an

azimuth problem, but that Head 0 may be rotated slightly in one

direction,  Head 0 was able to read all the data angled in one

direction, but could not read data angled at 27' or greater in

the opposite direction.

Copyright (C) Microsystems Development 1987, 1988.            25

TEST DRIVE (tm)                                     Version 1.2

F8  Hub Centering

Hub Centering is a measure of how well the disk clamping

mechanism holds the disk media centered and rotates it in a

perfect circle.

The DDD has three test tracks for three levels of clamping

sensitivity, 7, 8 and 9 milli-inches.  These tracks are called

the alternate offset tracks.  Data is alternately offset +/- 7,

+/- 8 or +/- 9 milli-inches all around the disk.  All disks

should pass the 7 milli-inch test and some with perfect clamping

and good sensitivity will pass the 9 milli-inch test.  A drive

should be considered to be in good shape if the 8 milli-inch test

is passed.

Display information:

The display for hub centering shows 3 sinusoidal waveforms

each indicating results from different tracks.  A solid block

(indicated by a '~' in this document) is displayed for a

successful read, and an 'X' is displayed for an unsuccessful

read.

The results for head 0 are displayed first, and the user is

given the option to test head 1 also.

Adjustment:

There generally is no adjustment for disk clamping or hub

centering.  Poor centering us usually an indication of a worn

out drive.

Note:  The DDD does not have a reinforcing hub.  This is to

avoid any centering error that could be introduced by

the hub reinforcer ring.

Copyright (C) Microsystems Development 1987, 1988.            26

TEST DRIVE (tm)                                     Version 1.2

A)  Sample of a good drive

TEST DRIVE Hub Centering Head 0

~   ~

~           ~

~

~       ~   ~

~           ~       X

~                                   ~

9 -->  ~   ~       ~   ~

~           ~       ~   ~

~                               ~   ~

8 -->  ~   ~                               ~

~   ~       ~           ~

~   ~       ~   ~

7 -->  ~                                   ~

~       ~           ~

~   ~       ~

~

~ = OK  X = Error                              ~           ~

~   ~

<-----------------  One Disk Revolution  ------------------->

Analysis:

This drive is functioning properly.  The 9 milli-inch test

shows an occasional error but this is acceptable.

Copyright (C) Microsystems Development 1987, 1988.            27

TEST DRIVE (tm)                                     Version 1.2

B)  Sample of a bad drive

TEST DRIVE Hub Centering Head 0

~   X

X           ~

X

~       ~   X

X           ~       ~

X                                   ~

9 -->  X   ~       ~   ~

X           ~       ~   X

~                               X   ~

8 -->  X   ~                               ~

~   X       X           ~

~   X       X   ~

7 -->  ~                                   ~

~       X           ~

~   ~       ~

~

~ = OK  X = Error                              ~           ~

~   ~

<-----------------  One Disk Revolution  ------------------->

Analysis:

The drive is not rotating the disk in a perfect circular

fashion.  Errors may be tolerated at 9 milli-inches but never at

7 milli-inches.  This indicates that as the disk is rotating,

with the head in a fixed position, the data track is not

consistently under the head.

Copyright (C) Microsystems Development 1987, 1988.            28

TEST DRIVE (tm)                                     Version 1.2

F9  Continuous Alignment

The continuous alignment test is provided as a way to `zero

in' on a particular head and track and take continuous

measurements.

The test defaults to head 0, track 19, but by using the

left, right, up and down arrows, the user can select any of the

progressive offset tracks on the DDD.

When the continuous alignment test begins to run, it will

first test for the presence of the Dysan DDD.  If it is present,

the test will proceed.  If it is not, the operator is given the

opportunity to override and continue anyway.  If the drive is so

poorly adjusted that TEST DRIVE cannot sense the Dysan disk, the

override feature allows the operator to at least start the test,

presumably so that a technician can attempt adjustment.  If the

user selects the override feature, and a non-Dysan DDD is

inserted, TEST DRIVE will report "Unable to read any data on

disk" for each measurement trial.

Display information:

The horizontal bars on the display span the distance from

track centerline that the drive was able to read.  When new

measurements are taken, the previous bars are scrolled down, and

the current measurements are displayed on the top line.  If the

head or track is switched, all previous measurements are cleared

before new data is displayed.

Adjustments:

If you attempt to make alignment adjustments, you can do so

while the continuous alignment test is running.  The test will

continually report any change.  Keep in mind that a disk drive is

a precision instrument.  If the data pattern shows a 2 milli-inch

shift, this is indicating the physical positioning needs to be

shifted 1/1000 of an inch to gain symmetry.  This is a very small

distance.  Once you start to move the head assembly, the

alignment may worsen.

In any case, when adjusting the heads be careful not to

cause damage to the DDD.

Copyright (C) Microsystems Development 1987, 1988.            29

TEST DRIVE (tm)                                     Version 1.2

A)  Sample display of a good drive

TEST DRIVE Continuous Alignment Test

<----------- m i l l i - i n c h e s ---------->

-13-12-11-10 -9 -8 -7 -6 +6 +7 +8 +9+10+11+12+13

|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|

Current >         ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Previous           ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Use the up & down arrows >  Head 0  Track 19 < Use the left & right

to switch heads                             arrows to switch tracks

Analysis:

The above drive appears to be perfectly aligned, as

evidenced by the fact that it can read data at equal distances on

either side of track center.  If other tracks and the other head

also display good results, alignment can be considered excellent.

Sensitivity is 20 (10 + 10) which is also very good.

Keep in mind, that some variations in the above display are

normal, particularly at the sides.  Data written greater than 8

milli-inches from track center may be read unreliably.  If this

test display were to show some slight variation in alignment or

sensitivity, it does not necessarily mean that the drive is in

need of adjustment.

Some variation is normal since data written too far off

track center may be read unreliably.

Copyright (C) Microsystems Development 1987, 1988.            30

TEST DRIVE (tm)                                     Version 1.2

B)  Sample display of a bad drive

TEST DRIVE Continuous Alignment Test

<----------- m i l l i - i n c h e s ---------->

-13-12-11-10 -9 -8 -7 -6 +6 +7 +8 +9+10+11+12+13

|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|--|

Current >   ~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Previous  ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Use the up & down arrows >  Head 0  Track 19 < Use the left & right

to switch heads                             arrows to switch tracks

Analysis:

The above drive shows poor alignment since the horizontal

bars are all skewed to one side.  It is definitely in need of

adjustment.

The head is shifted slightly away from the spindle as

indicated by the higher negative values.  Sensitivity is 19 (6 +

13 or 7 + 12), which is good.  Therefore, this drive can probably

be adjusted to within acceptable tolerances.

Copyright (C) Microsystems Development 1987, 1988.            31

TEST DRIVE (tm)                                     Version 1.2

F10  Cleaning Utility

The disk read/write heads may in time build up a deposit of

oxide, smoke or dust particles that will adversely effect the

sensitivity of the heads.  Depending on the amount of buildup,

cleaning may improve the performance of the disk drive.

The TEST DRIVE cleaning utility is designed to be used with

a standard cleaning disk and cleaning solution.  It will rotate

the disk for a set length of time, and move the heads across the

surface of the cleaning disk.

Most manufacturers of cleaning disks recommend that you

invoke a DIR command and continue the drive motion for about 30

seconds.  This will cause the head to seek to track zero looking

for the directory information.  In doing this, the only portion

of the cleaning disk that is used is the portion physically

located where track zero is supposed to be (the outer edge).  The

TEST DRIVE cleaning utility will move the head across the disk

from track 0 to track 39.  Each cycle from track 0 to 39 takes

approximately 10 seconds.  By using the cleaning utility in

conjunction with a standard cleaning disk you will get more

efficient usage of the cleaning disk and more efficient cleaning

of the heads.

Most drive cleaners recommend that you use the cleaning disk

only 10 to 20 times before discarding.  Since the head itself is

physically less that 1/8 inch wide, and it can be moved almost 1

inch across the disk, it stands to reason that if the head

positioning is varied and you follow manufacturers advice, you

can expect to obtain 80 or more cleanings from a single cleaning

disk.

Drive cleaning tips:

Does cleaning really help?  We recommend that you monitor

head sensitivity with TEST DRIVE before and after you clean the

heads to illustrate what effect the cleaning has on the

performance of the heads.  Sensitivity is a measure of how well

the heads read.  The General Test and the Alignment Test indicate

sensitivity in the +x to -x readings.  The larger the span

between the + and - numbers the greater the sensitivity.  Take

note of the sensitivity before cleaning and after.  If the

sensitivity did not increase the heads didn't need cleaning.

Are cleaning disks harmful to the drives?  The cleaning

media in the revolving  disk cleaners must be stiff to allow for

the hub to clamp and spin it.  This stiff characteristic

generally means that the cleaner is somewhat abrasive also.

Excessive usage of abrasive cleaning disks may cause excessive

wear on the heads, but occassional cleaning is not harmful.

Copyright (C) Microsystems Development 1987, 1988.            32

TEST DRIVE (tm)                                     Version 1.2

How often should floppy disk drives be cleaned?  One

cleaning disk manufacturer recommends daily cleaning, most all

recommend cleaning at least once a week.  However, we recommend

that you clean your drive only when needed based on usage and

environment.  If the environment is smoky or dusty the heads will

build up a deposit quickly.  In an extremely clean location the

heads will require less cleaning.  The amount of usage the drive

receives will only slightly effect the cleaning requirements.

Since the time the head is in contact with the media is generally

so short, this does not have as much effect as the environment.

In a generally clean environment with moderate usage a disk drive

may require cleaning every 3 to 4 months, but there are no hard

and fast rules.  The best indicator of when to clean your drives

can be obtained by using TEST DRIVE to periodically check their

performance.

What is the best way to clean disk drives?  Any standard

cleaning disk is acceptable.  It appears as though there are only

a few manufacturers that provide several name brand distributors.

We recommend that a lint free cloth saturated in a good cleaning

solvent be used if access to the head assembly if possible.  If

the access is limited be careful not to disturb the alignment by

moving the head assembly.  The commercially available cleaning

solution provided with cleaning disks is generally a form of

isopropyl alcohol and freon.  This is fine to use but isopropyl

alcohol by itself is sufficient to do the job.  The purer it is,

the better.  Ninety-one percent isopropyl alcohol is available in

many drug stores, and 99% is rarer but still available.

Copyright (C) Microsystems Development 1987, 1988.            33

TEST DRIVE (tm)                                     Version 1.2

GLOSSARY OF TERMS

1.2MB          1200 x 1024 or 1,228,800 bytes.  The amount of

data stored on a high density diskette,  High

density drives are normally found in AT type

computers.

360KB          360 * 1024 or 368,640 bytes.  The amount of data

stored on standard 5 1/4 inch diskette.

720KB          720 * 1024 or 737,280 bytes.  The amount of data

stored on a 3 1/2 inch diskette.

Alignment      The measure of how centered the Read/Write head is

over the track centerline.  Sometimes referred to

as Radial Alignment.

Alignment      The measure of how much of the track the head will

Sensitivity    respond to.  In a 48 TPI drive, if the head reads

-11 to +10 milli-inches the head is sensitive to

21 milli-inches of data and is very good.  If the

head reads -8 to +7 milli-inches of data the

sensitivity is 15 milli-inches and is poor.

Alternate      Alternate offset refers to the data pattern on the

Offset         centering test tracks.  There are 3 tracks used

for this test.  Data is written at +/- 7 milli-

inches on the first, +/- 8 on the second, and

+/- 9 on the third.

Azimuth        The angular displacement of the Read/Write Head

relative to a line tangent to the track center

line.

BIOS           Basic Input/Output System.  The built functions of

a computer to facilitate many functions including

disk reads and writes.

Centering      The ability of the disk clamping mechanisms to

clamp on the diskette precisely in the center of

the disk and spin the disk in a perfect circle.

See Alternate Offset.  Sometimes referred to as

eccentricity.

Clamping       See Centering.

CRC            Cyclic Redundancy Check.  A type of error checking

used by the computer's BIOS to help insure

integrity of data on a diskette.

Cylinder       A set of concentric equal size tracks, stacked on

top of one another.  Usually used when discussing

hard disks, but sometimes with diskettes also.

Copyright (C) Microsystems Development 1987, 1988.            34

TEST DRIVE (tm)                                     Version 1.2

Diskette       The magnetic media inserted into a floppy disk

drive, used to store information.

DDD            Digital Diagnostic Disk, manufactured by Dysan

Corporation.  Contains data patterns written at

precise angles and distances from track center.

Eccentricity   See Centering.

Head           The portion of the drive that reads and writes the

stored magnetic information from the diskette.

Hub            The center of the diskette sometimes reinforced

with ring.

Hysteresis     A measure of the drive mechanism's ability to seek

to a specified track from either direction and

position the head at precisely the same distance

from the center.

Milli-inch     One 1/1000th of an inch.

Minute         One 1/60th of a degree.  Used to measure the

rotational position or azimuth of a head relative

to track centerline.

Progressive    Progressive offset refers to the data pattern on

Offset         the alignment test tracks.  The pattern starts at

-/+6 milli-inches and progresses to -/+13 milli-

inches.  The data progressively gets farther away

from track center.

Radial         See Alignment

Alignment

RPM            Revolutions per minute, the number of times the

disk spins around in 1 minute.

Sector         A slice of a track containing data, like a piece

of a pie.  Each track contains a number of

sectors.  For 360KB drives, there are 9 sectors

per track.  High density 1.2MB drives use 15

sectors per track.

Seek           The process of moving the drive's head to a

particular track.

Sensitivity    See Alignment Sensitivity

Spindle        The mechanism in the disk drive used to spin the

diskette.

Copyright (C) Microsystems Development 1987, 1988.            35

TEST DRIVE (tm)                                     Version 1.2

Spindle Speed  The angular rotational speed of the diskette

media, or how fast the disk spins around.  This is

measured in revolutions per minute (rpm).

TPI            Tracks per inch.  The density of tracks on a disk.

For example:  A 48 TPI disk will have 48 tracks

per inch, each track spaced 1/48 of an inch apart.

A disk with 40 tracks will utilize 40/48ths of an

inch on the disk.

Track          A circle around a disk made up of a number of

sectors.  Sometimes called a cylinder.

Copyright (C) Microsystems Development 1987, 1988.            36

TEST DRIVE (tm)                                     Version 1.2

CUSTOMER FEEDBACK FORM

Your comments, suggestions, criticisms, etc. are most welcome.

They help us bring you a quality product at a reasonable price.

Just fill out this form and send it to us.

Brand and model of computer:_____________________________________

Comment( ) Suggestion( ) Criticism( ) Problem( ) Other___________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Name:    ________________________________________________

Company: ________________________________________________

Address: ________________________________________________

City, State, Zip: _______________________________________

Phone:  (__________) ____________________________________

Do you need a reply?_________ Are you a registered user?_________

We guarantee a reply to all registered users who request one.

Microsystems Development may use the information you supply

without incurring any obligation whatsoever.

Mail this form to:  Microsystems Development

4100 Moorpark Ave.  #104

San Jose, CA  95117  USA

Copyright (C) Microsystems Development 1987, 1988.            37

#####################################################